| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #41 |
| Source:* | Bei Xu, Huawei Technologies Co.,Ltd., echo.xubei@huawei.com Yongjing Zhang, Huawei Technologies Co.,Ltd., zhangyongjing@huawei.com |
| Date:* | 2019-06-28 |
| Reason for Change/s:* | The CR is to add new feature of 3GPP V2X interworking. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0082 - 3GPP V2X Interworking MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0055 |
| Clauses * | 8.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | NA |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

### -----------------------Start of change 1-------------------------------------------

## 8. 2 Solution B: Supporting PC5 Continuity

### 8.2.1 Solution Description

In order to address the Key Issues 2, we can rely on the IN-CSE to manage the task to support the continuity for PC5 communication.

### 8.2.2 Solution Applicability

This solution applies to Key Issue 2.

### Solution Details

#### 8.2.3.1 Impacted Resources and Attributes

##### 8.2.3.1.1 <taskMgmt> new resource

The <taskMgmt> resource is used to initiate and manage the atomic and consistent processing of a continuity task which could be devided into different subtasks according the policy of <taskMgmt> resource hosting CSE.

Table 8.2.3.1.1-1: New attributes of <taskMgmt> resource

| Attributes of <taskMgmt> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| targetedNode | 1 | RW | Indicates the node ID which the task apply for. |
| taskType | 1 | RW | Indicates the task type: - DeterminingSteps - VariableSteps |
| subTaskSequence | 0..1 | RW |  |
| targetedNodestatus | 1 | RW | Indicates the information of the targeted Node. It includes the |
| taskContentByteSize | 1 | RW | The size in bytes of taskContent. |
| taskContent | 1 | RW | Indicastes the task content. e.g. The software package for downloading and upgraging. |

##### 8.2.3.1.2 <task> new resource

The <task> resource is used to initiate and manage the atomic and consistent processing of a single oneM2M subtask of the oneM2M continuity task.

Table 8.2.3.1.2-2: New attributes of <task> resource

| Attributes of <taskMgmt> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| taskID | 1 | WO | This attribute is configured with the identifier of the task. This attribute is configured by the <taskMgmt> Hosting CSE with the resource identifier of the <taskMgmt> resource. |
| targetedNode | 1 | RW | Indicates the node ID which the task apply for. |
| taskContentByteSize | 1 | RW | The size in bytes of taskContent. |
| task RequestPrimitive | 1 | RW | This attribute contains the request primitive to be executed on the parent of this <task> resource. |
| taskResponsePrimitive | 1 | RO | This attribute contains the oneM2M response primitive associated with this transaction. This attribute is updated by the <task> Hosting CSE after it executes the requestPrimitive on the parent resource. |

#### 8.2.3.2 procedure

Step 0: The ADN-AE/ASN-CSE (Vehicle) and the ASN/MN-CSEs (RSUs) perform the registration procedure.

Step 1: The IN-AE send CREATE <taskMgmt> for the targeted ADN-AE/ASN-CSE.

Step 2: The IN-CSE check the taskType and targetedNodestatus of <taskMgmt>, if the taskType is Determining steps and the targetedNodestatus includes the route information, then IN-CSE devides the taskContent into different tasks:

The IN-CSE calculates the duration how long the the targeted ADN-AE/ASN-CSE (Vehicle) stays in the coverage scope of the each ASN/MN-CSE (RSU) according to PC5 coverage information and speed of the  targeted ADN-AE/ASN-CSE (Vehicle).

The IN-CSE calculates each subtask according to the duration, the taskContentByteSize and the capability of each ASN/MN-CSE (RSU).

The IN-CSE determins each ASN/MN-CSE (RSU) according to the location and route information of the targeted ADN-AE/ASN-CSE (Vehicle).

For example:

| RSU | bandwidth | Access Duration | Volume |
| --- | --- | --- | --- |
| RSU1 | 1M/s | 15s | 15M |
| RSU2 | 1M/s | 5min | 300M |

Step 3: The IN-CSE send CREATE <task> resource including the first subtask to the first ASN/MN-CSE (RSU) according to the location and PC5 coverage of the first ASN/MN-CSE (RSU), and location of the targeted ADN-AE/ASN-CSE (Vehicle). The IN-CSE may use its location services or other location information.

Step 4: The first ASN/MN-CSE (RSU) creates the <task> resource and broadcasts the taskRequestPrimitive to the broadcastAddress of <Node> resource when it detects the targeted ADN-AE/ASN-CSE (Vehicle) enters its coverage scope .

Step 5: The targeted ADN-AE/ASN-CSE (Vehicle) performs the operation and broadcasts the response by PC5.

Step 6: The first ASN/MN-CSE (RSU) updates the taskResponsePrimitive according to the response from the targeted ADN-AE/ASN-CSE (Vehicle) and sends response to the IN-CSE.

Step 7: The IN-CSE send CREATE <task> resource including the second subtask to the second ASN/MN-CSE (RSU) according to the location, route information of the targeted ADN-AE/ASN-CSE (Vehicle) and the response of the first subtask.

Step 8: The second ASN/MN-CSE (RSU) creates the <task> resource and broadcasts the taskRequestPrimitive to the broadcastAddress of <Node> resource when it detects the targeted ADN-AE/ASN-CSE (Vehicle) enters its coverage scope .

Step 9: The targeted ADN-AE/ASN-CSE (Vehicle) performs the operation and broadcasts the response by PC5.

Step 10: The second ASN/MN-CSE (RSU) updates the taskResponsePrimitive according to the response from the targeted ADN-AE/ASN-CSE (Vehicle) and sends response to the IN-CSE.

Step 11: The IN-CSE sends response to the IN-AE.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?