| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | SDS #41 |
| Source:* | Convida Wireless |
| Date:* | 2019-07-01 |
| Contact:* | Catalina Mladin, Convida, Mladin.Catalina@convidawireless.com Lu Liu, Convida, Liu.Lu@convidawireless.com Dale Seed, Convida, Seed.Dale@convidawireless.com Xu Li, Convida, Li.Xu@convidawireless.com |
| Reason for Change/s:* | Study solutions for dynamic instantiation of services |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0080> MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0052 V0.6.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This solution uses the M2M Service concepts introduced in solution C, including the definition and composition of an M2M service, csType and csfType, associated IDs, etc. Specifically, solution C provides the definition: “M2M Service: one or more Common Services, Application Services or combination thereof.”

This contribution proposes resources which can be used for the management of any M2M Service as defined above. In order to be able to abstract ANY M2M Service (as combination of Common Services (CS) and Application Services (instantiated by AEs), this solution proposes new resources to represent the components of an M2M Service and the M2M Service itself. In addition, a new resource for providing dynamic instantiation-related policies is introduced.

Given that the oneM2M framework for implementing Device Management provides several procedures which may be re-used within the framework for M2M Service Management, in this solution several new resources are proposed as new mgmtObj specializations: [serviceInstance], [serviceComponent] and [servicePolicy].

Note that while [serviceInstance] represents an instance of a M2M service defined above, [serviceComponent] abstracts representations of either CSs or AEs, for the purpose of service management. An alternate solution may be envisioned with two separate/dedicated objects for this purpose, i.e. [commonServiceInstance] and [applicationServiceInstance].

The relationships between the mgmtObj specializations proposed in this contribution and existing ones, as well as types of specializations proposed for future work are captured in the figure below

The relationships between the concepts of M2M Service/CSE/CSF and the service components AE and CS, as described in solution C, are depicted in the figure below. This figure also shows the correspondence between these entities and the newly introduced resources. Specifically, it shows the value of the instanceType attribute for the [serviceInstance] mgmtObj may be for managing different types of entities such as:

- M2M Service composed of multiple CSF/CSs and AEs

- CSE composed of multiple CSF/CSs

- CSF composed of multiple CSs

- M2M Service composed of one AE

- M2M Service composed of one CS

It is up to implementation if a set of CSs is managed as CSF, CSE or M2M service.

Note that a CSE or CSF instance can be viewed as a special case of M2M service instance.

The figure below describes the relationship between the management actions proposed in this contribution and the M2M Service or service component states associated. As shown in the figure, some new actions are proposed, and currently defined states are used but need to be harmonized with existing functionality.

- -----------------------Start of change 1 (new text) -------------------------------------

  - Solution X: Dynamic Service Management

    - Solution Applicability

This solution applies to Key Issue Y: “Dynamic Service Instantiation”

    - Solution Description

#### 9.4.2.1 Impacted Resources and Attributes

##### 9.4.2.1.1 Overview

This solution uses the M2M Service concepts introduced in solution C, including the definition and composition of an M2M service, csType and csfType, associated IDs, etc.

To implement this solution, several resources need to be modified, and some new resource types need to be defined These are further detailed in the following clauses.

##### 9.4.2.1.2 New <mgmtObj> specialization type: [serviceInstance]

A new [serviceInstance] specialization of the <mgmtObj> resource type is proposed for management of different types of service instances, such as M2M services (e.g. combinations of AEs and CSs), CSFs or CSEs.  It is up to implementation if a set of CSs is managed as CSF, CSE or M2M service.

Editor’s Note. It is currently FFS where the [serviceInstance] object should be instantiated.

The [serviceInstance] resource shall contain the attributes specified in table below.

Table: Attributes of [serviceInstance] resource

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| universal and common attributes | See clause 9.6.1.3 in [2] | See clause 9.6.1.3 in [2] | See clause 9.6.1.3 in [2] |
| mgmtDefinition | 1 | WO | See clause 9.6.15. Has fixed value "serviceInstance" to indicate the resource is for service entity management. |
| instanceType | 1 | RW | Choice of “CSE”, “Service”, “CSF”, etc. It is up to implementation if a set of CSs is managed as CSF, CSE or M2M Service. |
| instanceID | 0..1 | RW | The ID of the instance, may be a CSE-ID, Serv-ID, or CSF-ID. This attribute is a specialization of [objectAttribute] attribute. Editor’s Note. The format of a Serv-ID associated with a M2M service is FFS and to be addressed in Solution C. |
| componentIDs | 1(L) | RW | CS-IDs or AE-IDs of the service components. This attribute is a specialization of [objectAttribute] attribute. |
| instantiate | 1 | RW | The action that instantiates (installs) the service instance in a single operation. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| remove | 1 | RW | The action that removes (uninstalls) an instance. The action is triggered by assigning value "TRUE" to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| status | 1 | RW | Indicates the status of the instance, including “instantiated”, “enabled”, “disabled”, “removed”. This attribute is a specialization of [objectAttribute] attribute. Editor’s Note. The serviceInstance status (e.g. “installed”, “active”) need to be harmonized with Device Management procedures and TS-0004 clauses D.2, D.3 |
| enable | 0..1 | RW | The action that enables an instance that has been instantiated. The action is triggered by assigning the tuple ("TRUE”, instanceID) to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| disable | 0..1 | RW | The action that disables an instance that has been instantiated. The action is triggered by the tuple ("TRUE”, instanceID) to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| resetInstance | 0..1 | RW | The action that resets an instance that has been instantiated. The action is triggered by the tuple ("TRUE", instanceID) to this attribute. This attribute is a specialization of [objectAttribute] attribute. If this attribute exists, the attribute rebootLink is either not present or ignored |
| managementAtomicityIndicator | 1 | RW | Indicates if the operations for multiple components are to be treated atomically or not. The indicator applies to instantiate, remove, enable and disable operations. If the indicator specifies that the operation atomicity is FALSE, the Hosting CSE will manage the state of the service based on component states and local policies. If the indicator specifies that the operation atomicity is TRUE, the Hosting CSE will update the state of the service only when all the components achieve the new state. If a component operation is not successful it is rolled back, using install/remove and activate/deactivate operations as rollback pairs. Other atomicity behaviour (e.g. number of tries, timeouts) are implementation or local policy dependent |
| rebootLink | 0..1 | RW | Link to the [reboot] object used to reboot the instance. This attribute is a specialization of mgmtLink attribute. If the resetInstance attribute exists, the attribute rebootLink is either not present or ignored |
| servicePolicyLink | 0..1(L) | RW | Link(s) to the [servicePolicy] object used for the instance. If individual components have associated policies, the individual policies take precedence. If individual components do not have associated policies, the policy associated with the service instance is applied. If there are no service policies provided, local policies apply. This attribute is a specialization of mgmtLink attribute |
| instanceProvisioningLink | 0..1(L) | RW | Link(s) to management objects providing provisioning information for the component, e.g. [MAFClientRegCfg], [MEFClientRegCfg]. This attribute is a specialization of mgmtLink attribute Editor’s Note. The management objects providing provisioning information are FFS. For example, new security-related provisioning objects may provide security identifiers, credentials, lifetimes, default ACPs, etc. |
| componentLink | 1(L) | RW | Link(s) to [serviceComponent] management objects for managing each component. This attribute is a specialization of mgmtLink attribute |

##### 9.4.2.1.3 New <mgmtObj> specialization type: [serviceComponent]

A new [serviceComponent] specialization of the <mgmtObj> resource type is proposed for management of service components. Service components are either CSs or AEs.

The [serviceComponent] specialization shall contain the attributes specified in table below.

Table Attributes of [serviceComponent] resource

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| universal and common attributes | See clause 9.6.1.3 in [2] | See clause 9.6.1.3 in [2] | See clause 9.6.1.3 in [2] |
| mgmtDefinition | 1 | WO | See clause 9.6.15 in [2]. Has fixed value "serviceComponent" to indicate the resource is for the management of a single service component, i.e. either CS or AE. |
| componentype | 1 | RW | Choice of “CS” or “AE”. This attribute is a specialization of [objectAttribute] attribute. |
| csType | 0..1 | RW | If componentype is CS, this attribute provides the csType of the component. If componentype is AE, this attribute is ignored. This attribute is a specialization of [objectAttribute] attribute. |
| componentID | 0..1 | RW | CS-ID or AE-ID of the component. This attribute is a specialization of [objectAttribute] attribute. |
| reset | 0..1 | RW | The action that resets the component. The action is triggered by "TRUE to this attribute. This attribute is a specialization of [objectAttribute] attribute. |
| componentSoftwareLink | 0..1 | RW | Link to the [software] object used to install, uninstall, activate and deactivate the component, and monitor its status. This attribute is a specialization of mgmtLink attribute. |
| componentProvisioningLink | 0..1(L) | RW | Link to management objects providing provisioning information for the component, e.g. [MAFClientRegCfg], [MEFClientRegCfg]. This attribute is a specialization of mgmtLink attribute. |
| servicePolicyLink | 0..1(L) | RW | Link to the [servicePolicy] object used for the instance. If this policy is provided it take precedence over policies associated with the service instance. If this policy is not provided, the policy associated with the service instance is applied. If there are no service policies provided, local policies apply. This attribute is a specialization of mgmtLink attribute. |

##### New <mgmtObj> specialization type: [servicePolicy]

The [servicePolicy] management object specialization provides criteria for the dynamic management of a M2M services or service components (AEs or CSs). A [servicePolicy] resource may apply to one or more services or service components.

Table: Attributes of [servicePolicy]

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| policyID | 1 | RW | This is the unique ID of the service policy. It is created when the resource is created. |
| serviceID | 1(L) | RW | Identifier of the service(s) which this policy could be applied to. (e.g. AE-ID, CS-ID) |
| dynamicManagementEnable | 1 | WO | Indicates whether the dynamic management operations of the service are enabled or not. |
| eligibleNodeType | 0..1 | RW | Indicates the type of node that is eligible to host the service, e.g., M2M server. |
| serviceProvisionScope | 0..1 | RW | Indicates the scope/service domain where the service may be instantiated and managed. |
| managementProtocol | 0..1(L) | RW | Indicates alternative device management protocols preferred to be supported at the node where the service is instantiated, e.g. OMA DM protocol. |
| underlyingNetworkProtocol | 0..1(L) | RW | Indicates the preference for the underlying network protocols to be supported by the node where the service is instantiated. |

#### 9.4.2.2 Impacted Information Flows

##### 9.4.2.2.1 Overview

This Information Flows in the following sub-clauses describe how to implement dynamic service management procedures such as:

- Service Component management uses the install/uninstall/activate/deactivate attributes of the [software] mgmtObj associated with the [serviceComponent] mgmtObj via the softwareLink attribute. The reset operation is performed using the reset attribute of the [serviceComponent] mgmtObj.

If componentype is “AE”, an AE registration will be performed after a successful AE component installation.

- Service Instance management uses the instantiate/remove/enable/disable attributes of the [serviceInstance] corresponding to the M2M Service. Each action is translated into Service Component actions for each of the components of the M2M Service.

Note that CSE and CSF management are accomplished using the [serviceInstance] corresponding to the CSE or CSF, as CSEs and CSFs are special cases of M2M Services.

The Hosting CSE manages the state of the Service Instance based on the atomicity indicator and the states of the components.

If instanceType is “CSE”, a CSE registration will be performed after a successful CSE instance installation.

- ----------------------- End of change 1 (new text) -------------------------------------