| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#40.4 |
| Source:* | NTT |
| Date:* | 2019-06-27 |
| Contact:* | Kei Harada(kei.harada.by@hco.ntt.co.jp), NTT, Hiroyuki Maeomichi(maeomichi.hiroyuki@lab.ntt.co.jp), NTT, |
| Reason for Change/s:* | To add the description about JSON Path for improving its usability. |
| CR against: Release* | R4 |
| CR against: WI* | Active < Work Item number(optional)> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No STE Small Technical Enhancements / < Work Item number(optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 V4.1.0 |
| Clauses * | 8.1.2 and Annex K |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Instaed of introducing new type of filters (LDAP filters, attribute above, attribute below) use of current content based filtering was suggested at session of  SDS#39.8.

Use case for the change is to discover with newly introduced attributes or attributes which are not defined yet, in concrete, dataGenerationTime attribute of <timeSeriesInstance> or custom attributes of <flexContainer>.

Current filter criteria does not  support these type of discoveries.

The main change is meaning of $. Existing specification $ would point to content attribute of <contentInstance> or <timeSeries>, and this is a bit blur. This CR is going to change it as $ points to an entire oneM2M resource.

Due to this change, small backward compatibility issue arises.If existing user need to insert just “.con” after $ to point content attribute.

There is a similar specification at  https://goessner.net/articles/JsonPath/. The relation is not clear and it may introduce confusion, so I also added note for it.

This proposal is based on ARC-2018-0098R01-Modifying_condition_tags_of_Filter_Criteria.

## ------------------------------Start of change 1-------------------------------------

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0011: "Common Terminology".

[2]	oneM2M TS-0003: " Security Solutions".

[3]	oneM2M TS-0004: "Service Layer Core Protocol Specification".

[4]	W3C RDF 1.1 Concepts and Abstract Syntax.

[5]	W3C SPARQL 1.1 Query Language.

[6]	oneM2M TS-0012: "oneM2M Base Ontology".

[7]	oneM2M TS-0021: "oneM2M and AllJoyn Interworking".

[8]	oneM2M TS-0023: "Home Appliances Information Model and Mapping".

[9]	oneM2M TS-0016: “Secure Environment Abstraction”

[10]	oneM2M TS-0022: "Field Device Configuration".

[11]                     IETF RFC 5771: “IANA Guidelines for IPv4 Multicast Address Assignments”.

[12]                     IETF RFC 2357: “IPv6 Multicast Address Assignments”.

[13]	oneM2M TS-0032: ‘MAF and MEF Interface Specification”.

[14]	oneM2M TS-0034: "Semantics Support".

[15]	oneM2M TS-0026: "3GPP Interworking".

Void

[17]	IETF RFC 4566: "SDP: Session Description Protocol".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M TS-0002: "Requirements".

[i.2]	Broadband Forum TR-069: "CPE WAN Management Protocol Issue": 1 Amendment 5, November 2013.

[i.3]	OMA-DM: "OMA Device Management Protocol", Version 1.3, Open Mobile Alliance.

[i.4]	LWM2M: "OMA LightweightM2M", Version 1.0, Open Mobile Alliance.

[i.5]	OMA-TS-MLP-V3-4-20130226-C: "Mobile Location Protocol", Version 3.4.

[i.6]	OMA-TS-REST-NetAPI_TerminalLocation-V1_0-20130924-A: "RESTful Network API for Terminal Location", Version 1.0.

[i.7]	IETF RFC 1035: "Domain names - Implementation and specification".

[i.8]	IETF RFC 3588: "Diameter Base Protocol".

[i.9]	IETF RFC 3596: "DNS Extensions to Support IP Version 6".

[i.10]	IETF RFC 3986: "Uniform Resource Identifier (URI): General Syntax".

[i.11]	IETF RFC 4006: "Diameter Credit-Control Application".

[i.12]	IETF RFC 6895: "Domain Name System (DNS) IANA Considerations".

[i.13]	GSMA-IR.67: "DNS/ENU Guidelines for Service Providers & GRX/IPX Providers".

[i.14]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications (Release 13)".

[i.15]	ETSI TS 132 240: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Charging management; Charging architecture and principles (3GPP TS 32.240)".

[i.16]	ETSI TS 132 299: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Telecommunication management; Charging management; Diameter charging applications (3GPP TS 32.299)".

[i.17]	3GPP2.S0068: "Network Enhancements for Machine to Machine (M2M)".

[i.18]	JNI 6.0 API Specification: "Java Native Interface 6.0 Specification". .

Void

Void

Void

Void

[i.19]	3GPP TS 23.003: "Numbering, addressing and identification".

[i.20]	Recommendation ITU-T X.660 | ISO/IEC 9834-1: "Information technology - Procedures for the operation of object identifier registration authorities: General procedures and top arcs of the international object identifier tree".

[i.21]	oneM2M TR-0008: "Analysis of Security Solutions for oneM2M System".

[i.22]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[i.23]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf

[i.24]	            oneM2M TR-0007: "Study of Abstraction and Semantics Enablement".

Void

Void

[i.31]	OMA-TS-REST-NetAPI-CommunicationPatterns-V1-0: '"RESTful Network API for Communication Patterns'", Version 1.0, Open Mobile Alliance.

[i.32]	3GPP TS 23.246: Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description; (Release 14).

[i.33]	3GPP TS 23.468: Group Communication System Enablers for LTE (GCSE_LTE); (Release 14).

[i.34]                    IETF RFC 3171: “IANA Guidelines for IPv4 Multicast Address Assignments”, 2001

[i.35]                    IETF RFC 4291: “IP Version 6 Addressing Architecture”, 2006

[i.36]                    IETF RFC 6838: “Media Type Specifications and Registration Procedures”, 2013

[i.37]	IETF RFC 3987: "Internationalized Resource Identifiers (IRIs)".

NOTE:	Available at https://www.ietf.org/rfc/rfc3987.txt.

[i.38]				JSONPath

NOTE:	Available at  https://goessner.net/articles/JsonPath/

## ------------------------------End of change 1-------------------------------------

## ------------------------------Start of change 2-------------------------------------

### 8.1.2	Request

Table 8.1.2-2: Filter Criteria conditions

| Condition tag | Multiplicity | Description |
| --- | --- | --- |
| Matching Conditions |  |  |
| createdBefore | 0..1 | The creationTime attribute of the matched resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the matched resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically before the specified value. |
| stateTagSmaller | 0..1 | The stateTag attribute of the matched resource is smaller than the specified value. |
| stateTagBigger | 0..1 | The stateTag attribute of the matched resource is bigger than the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the matched resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the matched resource is chronologically after the specified value. |
| labels | 0..1 | The labels attribute of the matched resource matches the specified value. |
| labelsQuery | 0..1 | The value is an expression for the filtering of labels attribute of resource when it is of key-value pair format. The expression is about the relationship between label-key and label-value which may include equal to or not equal to, within or not within a specified set etc. For example, label-key equals to label value, or label-key within {label-value1, label-value2}. Details are defined in [3] |
| childLabels | 0..1 | A child of the matched resource has labels attributes matching the specified value. The evaluation is the same as for the labels attribute above. Details are defined in [3]. |
| parentLabels | 0..1 | The parent of the matched resource has labels attributes matching the specified value. The evaluation is the same as for the labels attribute above. Details are defined in [3]. |
| resourceType | 0..n | The resourceType attribute of the matched resource is the same as the specified value. It also allows differentiating between normal and announced resources. |
| childResourceType | 0..n | A child of the matched resource has the resourceType attribute the same as the specified value. |
| parentResourceType | 0..1 | The parent of the matched resource has the resourceType attribute the same as the specified value. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> matched resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> matched resource is smaller than the specified value. |
| contentType | 0..n | The contentInfo attribute of the <contentInstance> matched resource matches the specified value. |
| attribute | 0..n | This is an attribute of resource types (clause 9.6). Therefore, a real tag name is variable and depends on its usage and the value of the attribute can have wild card *. E.g. creator of container resource type can be used as a filter criteria tag as "creator=Sam", "creator=Sam*", "creator=*Sam". |
| childAttribute | 0..n | A child of the matched resource meets the condition provided. The evaluation of this condition is similar to the attribute matching condition above. |
| parentAttribute | 0..n | The parent of the matched resource meets the condition provided. The evaluation of this condition is similar to the attribute matching condition above. |
| semanticsFilter | 0..n | Both semantic resource discovery and semantic query use semanticsFilter to specify a query statement that shall be specified in the SPARQL query language [5]. When a CSE receives a RETRIEVE request including a semanticsFilter, and the Semantic Query Indicator parameter is also present in the request, the request shall be processed as a semantic query; otherwise, the request shall be processed as a semantic resource discovery. In the case of semantic resource discovery targeting a specific resource, if the semantic description contained in the <semanticDescriptor> of a child resource matches the semanticFilter, the URI of this child resource will be included in the semantic resource discovery result. In the case of semantic query, given a received semantic query request and its query scope, the SPARQL query statement shall be executed over aggregated semantic information collected from the semantic resource(s) in the query scope and the produced output will be the result of this semantic query. Examples for matching semantic filters in SPARQL to semantic descriptions can be found in [i.24]. |
| filterOperation | 0..1 | Indicates the logical operation (AND/OR) to be used for different condition tags. The default value is logical AND. |
| customFilterSyntax | 0..1 | Indicates the Identifier for syntax to be applied for discovery. |
| customFilterQuery | 0..1 | The query string shall be specified when customFilterSyntax parameter is present. |
| Filter Handling Conditions |  |  |
| filterUsage | 0..1 | Indicates how the filter criteria is used. If provided, possible values are 'discovery' and 'IPEOnDemandDiscovery'. If this parameter is not provided, the Retrieve operation is a generic retrieve operation and the content of the child resources fitting the filter criteria is returned. If filterUsage is 'discovery', the Retrieve operation is for resource discovery (clause 10.2.6), i.e. only the addresses of the child resources are returned. If filterUsage is 'IPEOnDemandDiscovery', the other filter conditions are sent to the IPE as well as the discovery Originator ID. When the IPE successfully generates new resources matching with the conditions, then the resource address(es) shall be returned. This value shall only be valid for the Retrieve request targeting an <AE> resource that represents the IPE. |
| limit | 0..1 | The maximum number of resources to be included in the filtering result. This may be modified by the Hosting CSE. When it is modified, then the new value shall be smaller than the suggested value by the Originator. |
| level | 0..1 | The maximum level of resource tree that the Hosting CSE shall perform the operation starting from the target resource (i.e. To parameter). This shall only be applied for Retrieve operation. The level of the target resource itself is zero and the level of the direct children of the target is one. |
| offset | 0..1 | The number of direct child and descendant resources that a Hosting CSE shall skip over and not include within a Retrieve response when processing a Retrieve request to a targeted resource. |
| applyRelativePath | 0..1 | - This attribute contains a resource tree relative path (e.g. ../tempContainer/LATEST). This condition applies after all the matching conditions have been used (i.e. a matching result has been obtained). The attribute determines the set of resource(s) in the final filtering result. The filtering result is computed by appending the relative path to the path(s) in the matching result. All resources whose Resource-IDs match that combined path(s) shall be returned in the filtering result. If the relative path does not represent a valid resource, the outcome is the same as if no match was found, i.e. there is no corresponding entry in the filtering result. |

## ------------------------------End of change 2--------------------------------------

## ------------------------------Start of change 3------------------------------------

## Annex K (Normative): Syntaxes for discovery

- K.1		Introduction

This annex specifies the syntax for customtFilterQuery filterCriteria (see clause 8.1.2).

The syntax of string for customFilterQuery parameter shall be chosen by customFilterSyntax parameter.

- K.2		'jsonpath' query syntax

This syntax of query is applicable to the resource by interpreting as encoded in JSON.

The target of evaluation shall be located by JSON path like addressing, which is constructed following rules:

- The entire oneM2M resource shall be referred by '$(dollar sign)' character. For example, when access to content attribute of <contentIinstance> or <timeSeriesinstance>, the path is “$. con”.

- The notation '[n]' shall refer n-th member of JSON Array.

- The operator '.(dot)' followed by name shall refer member of JSON Objects.

- The name shall be surrounded with '"(quote)' characters when the name contains special characters, such as '$', '.', ' (space)', '[', ']', '{', and '}'.

- The ' (space)' character shall be inserted between reserved keyword and other component of query string.

- The operator '! (exclamation mark)' shall expand string, what is stringified expression of JSON, to the corresponding JSON structure. In case that content attributed has stringified JSON, $.con! points to the corresoponding JSON structure.

- Note: There is a site for explaining JSONpath [i.38]. There are some common expressions with this section, but basically this specification is independent on one at the page.

- The following keywords shall be used to construct query string when customFilterSyntax parameter was 'JSON_PATH_SYNTAX'.

Table K.2-1: Reserved keywords for JSON-Path query syntax

| Keyword | Condition | Applicability |
| --- | --- | --- |
| EQ (Equals) | When the target value equals with query | String or number |
| NE (Not Equals) | When the target value does not equal with query. | String or number |
| GT (Greater Than) | When the target value was greater than number given as query. | Number or m2m:timestamp |
| LT (Less Than) | When the target value was less than number given as query. | Number or m2m:timestamp |
| GE (Greater or Equals) | When the targeted value was greater or equals with number given as query. | Number or m2m:timestamp |
| LE (Less or Equals) | When the targeted value was less or equals with number given as query. | Number or m2m:timestamp |
| MATCH | When the targeted value contains given string. | String only |
| AND | Concatenation of query which evaluated as AND combination logic. | Query-string |
| OR | Concatenation of query-string which evaluated as OR combination logic. | Query-string |
| () | Separation of the each query-string. | Query-string |

When discovery for the resources which are created before March 21 2019, creationTime attribute is used like this:

$.ct LT 20190321T123011

Moreover, when discovery for the HAIM [8] based devices which user is Sam or Anne, containertDefinition attribute and creator attribute are used like this:

($.cnd MATCH “org.onem2m.home.device.”) AND ($.cr EQ Sam OR $.cr EQ Anne)

## ------------------------------End of change 3-------------------------------------