| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #40 |
| Source:* | Catalina Mladin, Convida Wireless, Mladin.Catalina@convidawireless.com Leila Lebrun, Orange, leila.lebrun@orange.com Dale Seed, Convida Wireless, Mladin.Catalina@convidawireless.com Bob Flynn, Chordant, Bob.Flynn@chordant.io Lu Liu, Convida Wireless, Lu.Liu@convidawireless.com |
| Date:* | 2019-05-15 |
| Reason for Change/s:* | Discovery-based operations |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <WI-0046> MNT maintenance / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 v3.11.1 |
| Clauses * | 6.3.4.2.7, 6.3.4.2.31, 7.2.1.2., 7.2.1.1, 7.2.2.2, 7.3.3.2, 7.3.3.14, 7.2.2.17, 7.5.2. |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This is a companion CR to ARC-2018-0348, implementing protocol -level details for Discovery-based operations, as well as some corrections and clarifications for the discovery operation.

R05 includes the Form Group request parameter

R06 addresses discussions and tentative agreements from SDS #40.4. the text is marked as Mladin07

- Keeping FilterUsage ‘discovery’ and ‘discoveryBasedOperation’ separate.

- Clarifying access control: see sections highlighted in magenta (in both ARC and PRO contributions) for how this is addressed.

- Clarifying handling conditions i.e. offset and level apply only to “Discovery”, not Discovery-based operations.

R07 addresses comments received at SDS 41 and over email. Changes done at the meeting are tracked under Leila’s name. Changes done afterwards are highlighted in green for ease of reading and are tracked under Mladin-update. Some previous changes were undone if that addressed the comment.

### ------------------ start of change 1 ----------------------------------------------------------

##### 6.3.4.2.7	m2m:resultContent

Used for Result Content parameter in the request primitive.

Table 6.3.4.2.7-1: Interpretation of resultContent

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | nothing |  |
| 1 | attributes |  |
| 2 | hierarchical address |  |
| 3 | hierarchical address and attributes |  |
| 4 | attributes and child resources |  |
| 5 | attributes and child resource references |  |
| 6 | child resource references |  |
| 7 | original resource |  |
| 8 | child resources |  |
| 9 | modified attributes |  |
| 10 | semantic content |  |
| 11 | discovery result resource references |  |
| NOTE: See clause 6.4.1 Request primitive parameter data types. |  |  |

### ------------------ end of change 1 ----------------------------------------------------------

### ------------------ start of change 2 ----------------------------------------------------------

##### 6.3.4.2.31	m2m:filterUsage

Used in m2m:filterCriteria.

Table 6.3.4.2.31-2: Interpretation of filterUsage

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Discovery |  |
| 2 | Conditional Operation | This is the default value when the filterUsage condition is not present in a request. |
| 3 | IPE On-demand Discovery |  |
| 4 | Discovery-based Operation |  |

### ------------------ end of change 2 ----------------------------------------------------------

### ------------------ start of change 3 ----------------------------------------------------------

#### 6.6.3.6	Receiver error response class

Table 6.6.3.6-1 specifies the RSCs for Receiver error responses.

51xx codes are oneM2M specific, which are used in generic procedures.

52xx codes are oneM2M specific, which are used in resource specific procedures.

Table 6.6.3.6-3: RSCs for Receiver error response class

| Numeric Code | Description |
| --- | --- |
| 5000 | INTERNAL_SERVER_ERROR |
| 5001 | NOT_IMPLEMENTED |
| 5103 | TARGET_NOT_REACHABLE |
| 5105 | RECEIVER_HAS_NO_PRIVILEGE |
| 5106 | ALREADY_EXISTS |
| 5203 | TARGET_NOT_SUBSCRIBABLE |
| 5204 | SUBSCRIPTION_VERIFICATION_INITIATION_FAILED |
| 5205 | SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE |
| 5206 | NON_BLOCKING_SYNCH_REQUEST_NOT_SUPPORTED |
| 5207 | NOT_ACCEPTABLE |
| 5208 | DISCOVERY_DENIED_BY_IPE |
| 5209 | GROUP_MEMBERS_NOT_RESPONDED |
| 5210 | ESPRIM_DECRYPTION_ERROR |
| 5211 | ESPRIM_ENCRYPTION_ERROR |
| 5212 | SPARQL_UPDATE_ERROR |
| 5214 | TARGET_HAS_NO_SESSION_CAPABILITY |
| 5215 | SESSION_IS_ONLINE |
| 5216 | JOIN_MULTICAST_GROUP_FAILED |
| 5217 | LEAVE_MULTICAST_GROUP_FAILED |
| 5218 | TRIGGERING_DISABLED_FOR_RECIPIENT |
| 5219 | UNABLE_TO_REPLACE_REQUEST |
| 5220 | UNABLE_TO_RECALL_REQUEST |
| 5221 | CROSS_RESOURCE_OPERATION_FAILURE |
| 5222 | TRANSACTION_PROCESSING_IS_INCOMPLETE |

### ------------------ end of change 3 ----------------------------------------------------------

### ------------------ start of change 3i ----------------------------------------------------------

### 6.4.1	Request primitive parameter data types

The data types of request primitive parameters are specified in this clause.

Detailed request primitive parameter descriptions and usage can be found in clause 8.1.2 of the oneM2M TS-0001 [Error: Reference source not found]. Further details on the representation of request primitives are specified in clauses Error: Reference source not found and Error: Reference source not found of the present document.

Table 6.4.1-4 shows the structure of the request primitive. This is defined as the m2m:requestPrimitive element in the XSD file CDT-requestPrimitive-v3_11_0.xsd.

Table 6.4.1-4: Data Types for Request primitive parameters

| Primitive Parameter | Data Type | Multiplicity | Default Handling (note 2) | Note |
| --- | --- | --- | --- | --- |
| Operation | m2m:operation | 1 | Not applicable | See clause Error: Reference source not found |
| To | xs:anyURI | 1 | Not applicable |  |
| From | m2m:ID | 0..1 | Not applicable | See clause Error: Reference source not found Also see note 2 below |
| Request Identifier | m2m:requestID | 1 | Not applicable | See clause Error: Reference source not found |
| Resource Type | m2m:resourceType | 0..1 | No default | See clause Error: Reference source not found |
| Content | m2m:primitiveContent | 0..1 | No default | See clause Error: Reference source not found |
| Role IDs | list of m2m:roleID | 0..1 | Not applicable |  |
| Originating Timestamp | m2m:timestamp | 0..1 | No default |  |
| Request Expiration Timestamp | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clause D.12) | "Result Expiration Timestamp" shall be later than "Request Message Expiration Timestamp" |
| Result Expiration Timestamp | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clause D.12) |  |
| Operation Execution Time | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clause D.12) |  |
| Response Type | m2m:responseTypeInfo | 0..1 | Use "blockingRequest" | See clause Error: Reference source not found |
| Result Persistence | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clause D.12) |  |
| Result Content | m2m:resultContent | 0..1 | The default value depends on a given operation. See Table 8.1.2-1 of oneM2M TS-0001 [Error: Reference source not found] | See clause |
| Event Category | m2m:eventCat | 0..1 | No default | See clause Error: Reference source not found |
| Delivery Aggregation | xs:boolean | 0..1 | Can be given by CMDH policy (clause D.12), otherwise false |  |
| Group Request Identifier | xs:string | 0..1 | No default |  |
| Filter Criteria | m2m:filterCriteria | 0..1 | No default | See clause Error: Reference source not found |
| Desired Identifier Result Type | m2m:desIdResType | 0..1 | Use "structured" | See clause Error: Reference source not found |
| Tokens | list of m2m:dynAuthJWT | 0..1 | Not applicable | See clause Error: Reference source not found |
| Token IDs | list of m2m:tokenID | 0..1 | Not applicable |  |
| Local Token IDs | list of xs:NCName | 0..1 | No default |  |
| Token Request Indicator | xs:boolean | 0..1 | No default |  |
| Group Request Target Members | list of xs:anyURI | 0..1 | No default |  |
| Authorization Signature Indicator | xs:boolean | 0..1 | No default |  |
| Authorization Signatures | m2m:signatureList | 0..1 | No default | See clause Error: Reference source not found |
| Authorization Relationship Indicator | xs:boolean | 0..1 | No default |  |
| Semantic Query Indicator | xs:boolean | 0..1 | No default | Semantic Query Indicator |
| Release Version Indicator | m2m:releaseVersion | 1 | No default | This parameter is set to the release version that the primitive complies with |
| Vendor Information | xs:string | 0..1 | No default | This parameter is used to convey vendor specific information. No procedures are defined |
| NOTE 1: Default handling is the request handling procedure on a Transit/Hosting CSE when the request parameter is not included in a request primitive. This is not applicable for mandatory parameters which are marked as 'M' in Error: Reference source not found. NOTE 2: The From parameter shall be present for all requests except for <AE> CREATE where it is optional. |  |  |  |  |

### ------------------ end of change 3i ----------------------------------------------------------

### ------------------ start of change 4 ----------------------------------------------------------

### 7.2.1	Primitive format

#### 7.2.1.1	Request primitive format

Error: Reference source not found summarizes the primitive parameters of the Request primitive, indicating their presence depending on the C, R, U, D or N operations. "M" indicates mandatory, "O" indicates optional, "NP" indicates not present.

Refer to clause 8.1.2 of the oneM2M TS-0001 [Error: Reference source not found] for additional information on the request primitive parameters.

Table 7.2.1.1-5: Request Primitive Parameters

| Primitive Parameter | CREATE | RETRIEVE | UPDATE | DELETE | NOTIFY |
| --- | --- | --- | --- | --- | --- |
| Operation | M | M | M | M | M |
| To | M | M | M | M | M |
| From | O See note | M | M | M | M |
| Request Identifier | M | M | M | M | M |
| Resource Type | M | NP | NP | NP | NP |
| Content | M | O | M | NP | M |
| Role IDs | O | O | O | O | O |
| Originating Timestamp | O | O | O | O | O |
| Request Expiration Timestamp | O | O | O | O | O |
| Result Expiration Time | O | O | O | O | O |
| Operation Execution Time | O | O | O | O | O |
| Response Type | O | O | O | O | O |
| Result Persistence | O | O | O | O | NP |
| Result Content | O | O | O | O | NP |
| Event Category | O | O | O | O | O |
| Delivery Aggregation | O | O | O | O | O |
| Group Request Identifier | O | O | O | O | O |
| Filter Criteria | NP | O | O | O | NP |
| Desired Identifier Result Type | O | O | O | O | NP |
| Tokens | O | O | O | O | O |
| Token IDs | O | O | O | O | O |
| Local Token IDs | O | O | O | O | O |
| Token Request Indicator | O | O | O | O | O |
| Group Request Target Members | O | O | O | O | NP |
| Authorization Signature Indicator | O | O | O | O | NP |
| Authorization Signature | O | O | O | O | NP |
| Authorization Relationship Indicator | O | O | O | O | NP |
| Semantic Query Indicator | NP | O | NP | NP | NP |
| Release Version Indicator | M | M | M | M | M |
| Vendor Information | O | O | O | O | O |
| NOTE: The From parameter is Mandatory for all requests except for AE CREATE. For AE CREATE, it is Optional. |  |  |  |  |  |

The Content parameter in a Request shall contain one of the following:

- A partial Resource. This applies to Create and Update request primitives. In the case of Create request the Content parameter shall contain a single root element whose name is the name of the Resource and whose content consists of one or more attributes, child Resources or childResource references. In the case of an Update request primitive, the Content parameter shall contain the attribute and new values. Attributes to be deleted from the resource shall be indicated without a value. In both cases the resource type is as defined in clause Error: Reference source not found, however since a partial resource is being transferred it is not required to be valid according to the XSD for that resource in terms of the presence of resource attributes. Any attribute that is present, however, shall comply to the data type defined in the XSD of that resource.

- A Notification Data Object. This applies to Notification request primitives. The data type of the data object is named <m2m:notification> and is described in clause Error: Reference source not found.

- An Aggregated Notification. This applies to Notification request primitives. The data type of the data object is named <m2m:aggregatedNotification> and contains multiple <m2m:notification> objects. This is described in clause Error: Reference source not found.

- An AttributeList element, as described in clause 7. This is used in partial retrieve request primitives to indicate a list of attribute names whose values shall be retrieved in the response.

- A ResponsePrimitive object as described in clause Error: Reference source not found. This applies to Notification request primitives which are sent when accessing resources in asynchronous non-blocking mode.

#### 7.2.1.2	Response primitive format

Table 7.2.1.2-6 summarizes the primitive parameters for Response primitive, indicating their presence depending on the C, R, U, D or N operations of the associated Request primitive and whether this operation was successful or caused an error. "M" indicates mandatory, "O" indicates optional, "NP" indicates not present.

Refer to clause 8.1.3 of oneM2M TS-0001 [Error: Reference source not found] for additional information on the request primitive parameters.

NOTE:	Response Code and Status Code parameters are merged into the Response Status Code parameter.

Table 7.2.1.2-6: Response Primitive Parameters

| Primitive parameter | Ack | CREATE Success | RETRIEVE Success | UPDATE Success | DELETE Success | NOTIFY Success | Error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Response Status Code | M | M | M | M | M | M | M |
| Request Identifier | M | M | M | M | M | M | M |
| Content | O | O | M | O | O | O | O |
| To | O | O | O | O | O | O | O |
| From | O | O | O | O | O | O | O |
| Originating Timestamp | O | O | O | O | O | O | O |
| Result Expiration Timestamp | O | O | O | O | O | O | O |
| Event Category | O | O | O | O | O | O | O |
| Content Status | NP | NP | O | NP | NP | NP | NP |
| Content Offset | NP | NP | O | NP | NP | NP | NP |
| Assigned Token Identifiers | NP | O | O | O | O | O | O |
| Token Request Information | NP | NP | NP | NP | NP | NP | O |
| Authorization Signature Request Information | NP | NP | NP | NP | NP | NP | O |
| Release Version Indicator | M | M | M | M | M | M | M |
| Vendor Information | O | O | O | O | O | O | O |

The Content parameter in a Response shall contain one of the following:

- A complete or partial Resource. This applies to a response primitive sent in reply to create and retrieve request message. A partial resource also applies to a response primitive sent in reply to update request message. The Content parameter shall contain a single root element whose name is the name of the Resource and whose content consists of one or more attributes, child resources or childResource references. In this case the resource type is as defined in clause Error: Reference source not found. However if a partial resource is being transferred, it is not required to be valid according to the XSD for that resource, in terms of the presence of resource attributes. Any attribute that is present, however, shall comply to the data type defined in the XSD of that resource.

- The URI of a resource. This is included directly as the content of the Content parameter (like in case 6).

- A partial resource and its hierarchical URI. These are included in a root element called m2m:resource defined in clause 7. The URI is included as an attribute of m2m:resource.

- A list of URIs. This can be used for transferring the childResource URIs in a Discovery response. These are included in an element called m2m:URIList defined in clause 7.

- A list of childResourceRef. This can be used for transferring the child resource references in a Discovery response. These are included in an element called m2m:resourceRefList defined in clause 7.

- An Aggregated Response. This is sent as a result of a Group operation or as a result of a Discovery-based Operation with Result Content not present or different than discoveryResultResourceReferences. The response uses the element m2m:aggregatedResponse defined in clause 7.

- A request primitive. A pending request is sent in a polling response. This uses the element m2m:requestPrimitive defined in clause 6.4.1.

- Human-readable error message. This is included in an element called m2m:debugInfo defined in clause 7.

### ------------------ end of change 4 ----------------------------------------------------------

### ------------------ start of change 5 ----------------------------------------------------------

#### 7.2.2.2	Generic procedure for handling a Request at a receiver

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an error response" (refer to clause Error: Reference source not found for details) and then "Send Response primitive" (refer to clause Error: Reference source not found for details). The corresponding Response Status Code shall be included in the Response primitive.

Recv-1.0: "Check the validity of received request primitive"

Recv-2.0: Communication method?

Recv-6.0: Resource handling procedures

Recv-3.0: "Create <request> resource locally"

Recv-4.0: "Create a success Response"

Recv-5.0: "Send Response primitive"

nonBlockingRequestAsynch

nonBlockingRequestSynch

Recv-6.0: Resource handling procedures

Recv-7.0: "Update <request> resource"

Finish

Start

Recv-3.0: "Create <request> resource locally"

Recv-4.0: "Create a success Response"

Recv-5.0: "Send Response primitive"

Recv-6.0: Resource handling procedures

Recv-7.0: "Update <request> resource"

Recv-8.0: "Send Notification"

Recv-9.0: "Wait for Response primitive"

blockingRequest

Recv-10.0: "Send Response Primitive"

Figure 7.2.2.2-1: Generic procedure of Receiver

Recv-1.0 "Check the validity of received request primitive": See clause Error: Reference source not found for details.

Recv-2.0 "Communication method?": The Receiver CSE checks whether a received request is blockingRequest, nonBlockingRequestSynch or nonBlockingRequestAsynch by using the Response Type parameter (see detail in clause 8.1.2 in oneM2M TS-0001 [Error: Reference source not found]). If the request is blockingRequest or the Response Type parameter is not included, it goes to step Recv-6.0 "Resource handling procedure". If the request is nonBlockingRequestSynch, it goes to step Recv-3.0 "Create <request> resource locally". If the request is nonBlockingRequestAsynch, it goes to step Recv-3.0 "Create <request> resource locally". If the request is flexBlocking, the Receiver CSE shall make the decision to respond using blocking or non-blocking based on its own local context (memory, processing capability, etc.) unless specified further in the resource-specific procedure.

Recv-3.0 "Create <request> resource locally": Refer to clause Error: Reference source not found for details.

Recv-4.0 "Create a successResponse": Refer to clause Error: Reference source not found for details.

Recv-5.0 "Send Response Primitive": Refer to clause Error: Reference source not found for details.

Recv-6.0 "Resource handling procedure": Refer to Figure 7.2.2.2-2 for details.

Recv-7.0 "Update <request> resource": Refer to clause Error: Reference source not found for details. This step is only valid when the request is non-blocking.

Recv-8.0 "Send Notification": Refer to clause Error: Reference source not found for details.

Recv-9.0 "Wait for a Response primitive": Refer to clause Error: Reference source not found for details.

Recv-10.0 "Send Response Primitive": Refer to clause Error: Reference source not found for details.

Recv-6.10: "Queue request primitive and execute CMDH message forwarding procedure"

Recv-6.1: Hosting CSE of the targeted resource?

Start

Recv-6.3: "Check authorization of the Originator"

Recv-6.4: "Check validity of resource representation for the given resource type"

Recv-6.2: "Check existence of the addressed resource"

Recv-6.5: "Create/Update/Retrieve/Delete/Notify operation is performed"

Recv-6.6: "Announce/De-announce the resource"

Finish

Yes

No

Recv-6.7: "Create a success response"

Recv-6.9: CMDH processing supported?

Recv-6.11: "Forwarding"

No

Yes

Recv-6.0.1: Requested operation is an AE registration?

Recv-6.0.2: "Check Service Subscription Profile"

Yes

No

Recv-6.6.1: "Communication Method?"

Else

blockingRequest

Figure 7.2.2.2-2: Resource handling procedure

Figure 7.2.2.2-2 describes the generic procedure to resource handling procedures.

Recv-6.0.1 "Requested operation is an AE registration?": If the requested operation is an AE registration, then it goes to Recv-6.0.2 "Check Service Subscription Profile". Otherwise, it goes to Recv-6.1.

Recv-6.0.2 "Check Service Subscription Profile": Refer to clause Error: Reference source not found for details.

Recv-6.1 "Hosting CSE of the targeted resource?": The step checks if the receiver is a transit CSE or the Hosting CSE of the received Request by examining the To parameter of the Request primitive. If the receiver hosts the resource that the address in the To parameter represents, the receiver is the Hosting CSE (goes to Recv-6.2"Check existence of the addressed resource", Yes branch). Otherwise, the receiver is the Transit CSE (goes to Recv-6.9 "CMDH processing supported?", No branch). Refer to clause Error: Reference source not found for details.

Recv-6.1.1 "filterUsage?": The Hosting CSE checks if the request includes steps for discovery or identification of target resources or not. If filterUsage is configured as “Discovery”, "Discovery-based Operation" or "IPE On-demand Discovery" more than one targeted resources may be identified, and the “Discovery-related procedure” in clause 7.3.3.14 applies. Otherwise, it goes to Recv-6.2

Recv-6.2 "Check existence of the targeted resource(s)": Refer to clause 7.3.3.2 for details.

Recv-6.3 "Check authorization of the Originator": Refer to clause Error: Reference source not found for details.

Recv-6.4 "Check validity of resource representation": Refer to clause  and clause Error: Reference source not foundError: Reference source not found for details. Notify is not applicable for this step.

Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed": The step represents five common operations which are "Create the resource (clause Error: Reference source not found)", "Retrieve the resource (clause Error: Reference source not found)", "Update the resource (clause Error: Reference source not found)", "Delete the resource (clause Error: Reference source not found)" and "Notify processing (clause Error: Reference source not found)".

Recv-6.6 "Announce/De-announce the resource": The step represents two common operations which are "Announce the resource" and "De-announce the resource". Refer to clause  and clause Error: Reference source not foundError: Reference source not found for details. Notify is not applicable for this step.

Recv-6.6.1 "Communication method?": The Receiver CSE checks whether a received request is blockingRequest or not by using Response Type parameter (see detail in clause 8.1.2 in oneM2M TS-0001 [Error: Reference source not found]). If the request was blockingRequest or Response Type parameter was not included, it goes to step Recv-6.7 "Create a success response". Otherwise, it goes back to the generic procedure of the receiver (Figure 7.2.2.2-1).

Recv-6.7 "Create a success response": Refer to clause Error: Reference source not found for details.

If filterUsage is configured as “Discovery” or "IPE On-demand Discovery", after the “Discovery-related procedure” in clause 7.3.3.14 the Receiver shall include addresses for all the resources in the target resource set, with the resource identifier format based on the Desired Identifier Result Type parameter setting (see clause 6.3.4.2.8). If there are no resources in the resource target set, the Hosting CSE shall respond with successful response with an empty Content parameter as follows:

- If rcn is not present the primitive Content is an m2m:URIList  containing no entries

- If  rcn=6 the primitive Content is an m2m:resourceRefList  containing no entries

If filterUsage is configured as “Discovery-based Operation", after the “Discovery-related procedure” in clause 7.3.3.14 the Receiver shall respond with successful response as follows:

- If rcn is not present or other than 11 all the individual primitives created either as successful or as error responses are aggregated to be sent as a single response primitive. If the target resource set is empty the primitive content is an m2m:aggregatedResponse containing no entries in the successful response.

- If rcn=11 a single response primitive is composed, with content that includes the resource references of all the target resources. If the target resource set is empty the primitive content is an m2m:URIList  containing no entries in the successful response.

- Recv-6.9 "CMDH processing supported?": This step checks whether the Receiver supports the CMDH processing. If the receiver supports CMDH processing, it goes to Recv-6.10 "Queue request primitive and execute CMDH message forwarding procedure" otherwise, it goes to Recv-6.11 "Forwarding".

Recv-6.10 "Queue request primitive and execute CMDH message forwarding procedure": the Receiver CSE shall queue the received request primitive and execute the "CMDH message forwarding procedure". Refer to clause H.2.4 for details.

Recv-6.11 "Forwarding": carry out message forwarding as defined in clause Error: Reference source not found.

### ------------------end of change 5 ----------------------------------------------------------

### ------------------start of change 6 ----------------------------------------------------------

#### 7.3.3.2	Check existence of the targeted resource

If the Request Expiration Timestamp is given in the request and has expired, the Hosting CSE shall reject the request with a "REQUEST_TIMEOUT" Response Status Code parameter value. Otherwise, the Hosting CSE should handle the request before the time specified in Request Expiration Timestamp.

The Hosting CSE shall check if the resource addressed by the To parameter exists in the repository. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

If filterUsage is configured as "Conditional Operation" the Hosting CSE shall also check the existence of target resource based on conditions specified in the Filter Criteria parameter of the Create/Retrieve/Update/Delete request. If there are no matching target resources, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

If the Hosting CSE does not support the content format (i.e. type of serialization) requested by the originator, the request shall be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

If the Hosting CSE does not support the content format sent by the originator, the request shall be rejected with a Response Status Code indicating "UNSUPPORTED_MEDIA_TYPE" error.

### ------------------end of change 6 ----------------------------------------------------------

### ------------------start of change 7 ----------------------------------------------------------

#### 7.3.3.14	Discovery related procedure

If the Operation Execution Time is given in the request, the Hosting CSE should perform the following procedures at the time and shall not perform the procedures before the time.

If the parameter filterUsage is configured as "Discovery", " Discovery-based Operation " or "IPE On-demand Discovery"  the request is a Discovery-related request and t

he Hosting CSE shall perform the following steps for discovery or identification of target resources in the resource tree.

The parameter filterUsage indicates the type of procedure to be performed, as follows:

- Discovery: initiated by a Retrieve request with filterUsage configured as "Discovery”.

- Discovery-based Operations: initiated by Create, Update or Delete requests with filterUsage configured as "Discovery-based Operation".

- IPE On-demand Discovery: initiated by a Retreieve request with filterUsage configured as “IPE On-demand Discovery”.

These request may include other Filter Criteria parameters as well.

Discovery related procedures shall be comprised of the following actions.

Originator:

The Originator shall follow the steps from Orig-1.0 to Orig-6.0 specified in clause Error: Reference source not found Generic Resource Request Procedure for Originator.

In addition to Orig-1.0, the following steps shall be performed.

The To parameter in the Request indicates the root of where the discovery begins.

The Request shall include a Filter Criteria request parameter that includes a filterUsage element configured with either “Discovery”, "Discovery-based Operation" or "IPE On-demand Discovery".

The Retrieve Request may include other elements of Filter Criteria.

Receiver:

The Receiver shall follow the steps from Recv-1.0 to Recv-7.0 specified in clause .

The Hosting CSE shall perform the following as part of step Recv 6.2 specified in clause 7.3.3.2  in order to identify the operation target resource set based on conditions specified in Filter Criteria.

The Receiver shall find the resources that match all the configured Filter Criteria and to which the Originator has "Discover" privilege, among all the children/descendent resources of the addressed resource. As part of this search, the Receiver will not consider :

- any child/descendent resources of resources to which the Originator does not have "Discover" privilege,

- <AE> resources with registrationStatus attribute set to INACTIVE, and

- any child/descendent resources of INACTIVE <AE> resources

The scope of the search is subject to filter handling conditions specified by the Originator in the Filter Criteria as follows (see also clause 8.1.2 of oneM2M TS-0001 [Error: Reference source not found]):.

- Based on the filter handling confitions level and offset valid only for Retrieve operations: The Hosting CSE shall skip over and not include in scope the number of child and descendant resources indicated by the offset parameter.  The scope stops being enlarged when the Hosting CSE reaches the maximum level in the resource tree starting from the target resource (i.e. To parameter), as indicated by the level parameter.

- Based on the filter handling condition limit: The scope stops being enlarged when the maximum number of resources to be included, as indicated by the limit parameter, is reached. This maximum may be modified by the Hosting CSE based on local policies. When it is modified, the new value shall be smaller than the suggested value by the Originator.

For filterUsage ‘discovery’ or 'discoveryBasedOperation' if the filter handling condition applyRelativePath is provided, the Hosting CSE appends the specified relative path to each resource in the matching result. If the compounded path does not correspond to a valid resource the matched resource is not considered in scope. See 7.3.3.17.17 for details about applyRelativePath.

If the request is a retrieve request with filterUsage set to "IPE On-demand Discovery” and addressing an <AE> resource representing the IPE by its labels attribute, the Hosting CSE shall first find resources using the Filter Criteria as detailed above. If the Hosting CSE finds no match, the Hosting CSE shall send a Notify request to the IPE to trigger the external discovery procedure (see clause Error: Reference source not found for more details). If the Hosting CSE receives a successful Notify response, the Hosting CSE shall check the Originator's "Discover" privilege for the resources listed in the Notify response. If the Hosting CSE receives an unsuccessful Notify response from the IPE, then the Hosting CSE shall use the same Response Status Code in the response to the Originator.

If filterUsage is configured as “Discovery” or "IPE On-demand Discovery", the Receiver shall proceed to step Recv-6.7

If filterUsage is configured as "Discovery-based Operation", the Receiver shall repeat steps Recv-6.3- Recv-6.7 for each of the resources in the target resource set as “addressed” or “targeted” resources.

### ------------------end of change 7  ----------------------------------------------------------

### ------------------start of change 8 ----------------------------------------------------------

#### 7.3.3.17	Using Filter Criteria for identification of target resources

##### 7.3.3.17.0	Introduction

When the Filter Criteria primitive parameter is present in a request primitive, it shall be applied for identification of the applicable target resources of the respective operation. This may apply to Create, Retrieve, Update, Delete, Discovery and Semantic Resource Discovery operations as specified in clauses 7.3.3.5, Error: Reference source not found, 7.3.3.7, Error: Reference source not found, 0 and Error: Reference source not found respectively.

The Filter Criteria primitive parameter defines Matching Conditions on resource attributes (table 7.3.3.17.0-1) and Filter Handling Conditions (table 7.3.3.17.0-2).

Matching Conditions are evaluated against resources and, when true, determine the matched resources. The Filter Handling Conditions provide additional input applied to the matched resource set to determine the filtering result (e.g. maximum number of resources to be included in the filtering result). The filtering result may be composed of one or more resources and shall be used as the target of the operation. Table 7.3.3.17.0-7 summarizes the various filter criteria and conditions. Each row in the table represents a different filter condition type.

If multiple matching conditions of the same type (i.e. same condition tag) are present in the Filter Criteria parameter, these shall be combined by applying logical OR operation. This applies to the condition tags labels, resourceType, contentType or attribute with multiplicity n > 1.

If multiple matching conditions of different type (i.e. different condition tags) are present in the Filter Criteria parameter, then the combined condition shall be derived by applying the logical operation specified by the filterOperation condition. By default logical AND operation shall be used if the filterOperation condition is not present.

EXAMPLE:

- labels=floor1, stateTagSmaller=3 will match if both conditions are true [default AND when filterOperation is not specified]

- labels=floor1, stateTagSmaller=3, filterOperation=1 will match if both conditions are true

- labels=floor1, stateTagSmaller=3, filterOperation=2 will match if either condition is true

- labels=floor1, labels=floor2, filterOperation=1 will match if either condition is true [ filterOperation has no effect when all condition tags are the same]

- labels=floor1, stateTagSmaller=3, labels=floor2, filterOperation=2 will match if any of these conditions are true resource has [labels with value "floor1" OR "floor2"] OR stateTagSmaller than 3

Table 7.3.3.17.0-7: Summary of Filter Matching conditions

| Condition Tag | Multiplicity | Targeted Resource Attribute | Matching Condition |
| --- | --- | --- | --- |
| createdBefore | 0..1 | creationTime | creationTime < createdBefore, see clause 7.3.3.17.1 |
| createdAfter | 0..1 |  | createdAfter ≤ creationTime, see clause 7.3.3.17.1 |
| unmodifiedSince | 0..1 | lastModifiedTime | lastModifiedTime < unmodifiedSince, see clause Error: Reference source not found |
| modifiedSince | 0..1 |  | unmodifiedSince ≤ lastModifiedTime, see clause Error: Reference source not found |
| stateTagSmaller | 0..1 | stateTag | stateTag < stateTagSmaller, see clause Error: Reference source not found |
| stateTagBigger | 0..1 |  | stateTagBigger ≤ stateTag, see clause Error: Reference source not found |
| expireBefore | 0..1 | expirationTime | expirationTime < expireBefore, see clause Error: Reference source not found |
| expireAfter | 0..1 |  | expireAfter ≤ expirationTime, see clause Error: Reference source not found |
| labels | 0..1 | labels | see clause Error: Reference source not found |
| childLabels | 0..1 |  | see clause Error: Reference source not found |
| parentLabels | 0..1 |  | see clause Error: Reference source not found |
| resourceType | 0..1 | resourceType | see clause Error: Reference source not found |
| childResourceType | 0..1 |  | see clause Error: Reference source not found |
| parentResourceType | 0..1 |  | see clause Error: Reference source not found |
| sizeBelow | 0..1 | contentSize | contentSize < sizeBelow, see clause Error: Reference source not found |
| sizeAbove | 0..1 |  | sizeAbove ≤ contentSize, see clause Error: Reference source not found |
| typeOfContent | 0..n | contentInfo | matched with typeOfContent component in contentInfo, see clause Error: Reference source not found |
| attribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause Error: Reference source not found |
| childAttribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause Error: Reference source not found |
| parentAttribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause 7.3.3.17.9 |
| semanticsFilter | 0..n | (not applicable) | Filtering conditions expressed in SPARQL [Error: Reference source not found]. These are applicable to the descriptor attribute of <semanticDescriptor> children associated with discoverable resources. When multiple semanticsFilter elements are provided, the matching condition is fulfilled if any of the individual conditions is matched |
| contentFilterSyntax | 0..1 | (not applicable) | Indicates the Identifier for syntax to be applied for content-based discovery |
| contentFilterQuery | 0..1 | content | The query string shall be specified when contentFilterSyntax parameter is present. See clause Error: Reference source not found for applicable syntax for content-based discovery |

Table 7.3.3.17.0-2: Summary of Filter Handling Conditions

| Condition Tag |  | Multiplicity |  | Usage |
| --- | --- | --- | --- | --- |
| filterUsage | 0..1 |  | Indicator specifying the use case of Filter Criteria parameters |  |
| limit |  | 0..1 |  | Constraint on maximum number of targeted resources, see clause Error: Reference source not found |
| filterOperation |  | 0..1 |  | Indicates the logical operation (AND/OR) to be used for different conditions. The default value is logical AND. |
| level |  | 0..1 |  | Constraint on maximum number of levels in the resource tree that the operation shall span and applies only to retrieve operations, see clause Error: Reference source not found. |
| offset |  | 0..1 |  | The number of direct child and descendant resources the Hosting CSE shall skip over and not process in a discovery operation and applies only to retrieve requests, see clause Error: Reference source not found. |
| applyRelativePath |  | 0..1 |  | A resource tree relative path (e.g. /tempContainer/la) which applies after all the matching conditions have been used (i.e. a matching result has been obtained). This shall only be applied for Discovery and Discovery-based operations. See clause 7.2.2.17.17. |

### ------------------end of change 8 ----------------------------------------------------------

### ------------------start of change 9 ----------------------------------------------------------

### 7.5.2	Elements contained in the Content primitive parameter

Clauses Error: Reference source not found and  enumerate the forms that the Content primitive parameter takes in various Request and Response cases. Note that the Content primitive parameter is denoted as primitiveContent in both CDT-requestPrimitive-v3_11_0.xsd and CDT-responsePrimitive-v3_11_0.xsd.

This clause details the Objects (elements) used in some of these cases. in the tables below.

The following elements are defined for use in the Content parameter of a request:

Table 7.5.2-1: Elements used for request content

| Element Name | Applicable Operations | Data Type | Defined in |
| --- | --- | --- | --- |
| m2m:<resourceType> {other namespace identifier}:<resourceType> | C U | See element declaration | CDT-<resourceType>-v3_11_0.xsd |
| m2m:notification | N | m2m:notification | CDT-notification-v3_11_0.xsd |
| m2m:aggregatedNotification | N | m2m:aggregatedNotification | CDT-notification-v3_11_0.xsd |
| m2m:securityInfo | N | m2m:securityInfo | CDT-notification-v3_11_0.xsd |
| m2m:attributeList | R | m2m:attributeList | CDT-requestPrimitive-v3_11_0.xsd |
| m2m:responsePrimitive | N | Anonymous data type defined in the responsePrimitive declaration | CDT-responsePrimitive-v3_11_0.xsd |

The following elements are defined for use in the Content parameter of a response sent in reply to a request message with Operation and Result Content (rcn) parameters as given in the column "Applicable Operations" (the settings of the Result Content parameters are defined in clause ; NP means the rcn parameter is not present).

Table 7.5.2-2: Elements used for response content

| Element Name | Applicable Operations/rcn | Data Type | Element is Defined in |
| --- | --- | --- | --- |
| m2m:<resourceType> {other namespace identifier}:<resourceType> See note 6 | C/1,9,NP R/1,4,5,6,7,8,NP U/1,9,NP D/1,4,5,6,8 See note 1 | See element declaration | CDT-<resourceType>-v3_11_0.xsd |
| m2m:resource | C/3 | m2m:resourceWrapper | CDT-responsePrimitive-v3_11_0.xsd |
| m2m:URIList | C R U D/NP See note 2 | list of xs:anyURI | CDT-responsePrimitive-v3_11_0.xsd |
| m2m:resourceRefList | R/6 See note 2 | m2m:listOfChildResourceRef | CDT-responsePrimitive-v3_11_0.xsd |
| m2m:aggregatedResponse | C R U D See note 3 a) C U D/11 See note 3 b) | m2m:aggregatedResponse | CDT-responsePrimitive-v3_11_0.xsd |
| m2m:URI | C/2 See note 4 | xs:anyURI | CDT-responsePrimitive-v3_11_0.xsd |
| m2m:requestPrimitive | See note 7 | Anonymous data type defined in the requestPrimitive declaration | CDT-requestPrimitive-v3_11_0.xsd |
| m2m:debugInfo | See note 5 | xs:string | CDT-responsePrimitive-v3_11_0.xsd |
| m2m:securityInfo | N/NP | m2m:securityInfo | CDT-notification-v3_11_0.xsd |
| m2m:queryResult | R/10 See note 8 | xs:string | CDT-responsePrimitive-v3_11_0.xsd |
| NOTE 1: The case rcn = 7 applies to Retrieve operation only (R/7). It retrieves the original resource in case the To parameter points to an announced resource. The case R/NP applies to Retrieve operation (Non-Discovery) only. NOTE 2: This applies to discovery or discovery-based operations only. The format of the address (structured, unstructured) depends on the Desired Identifier Result Type parameter setting (see clause Error: Reference source not found). NOTE 3: This applies for two distinct cases: - CRUD operations on a <fanOutPoint> child resource of a <group> parent resource, independent of rcn value. The Content parameter of each response primitive included in aggregatedResponse is set as given in one of the other rows of this table. - Discovery-based CUD operations (filterUsage = 4) with rcn not presen or other than = 11. NOTE 4: This also applies to the response ("acknowledgement") to non-blocking requests in asynchronous and synchronous modes for any CRUD operation. NOTE 5: This is a plain text messages which can optionally be included as debugging information in error responses. The language and content of the message is determined by the Service Provider. NOTE 6: "{other namespace identifier}" refers to a namespace other than m2m. NOTE 7: This applies to a polling response that contains a request for polling mechanism (see clause 7.4.22.2.2). NOTE 8: This applies to semantic query operation only. The Originator may use the Accept option to indicate which media types are acceptable for the semantic query result, e.g. application/sparql-results+xml, or application/sparql-results+json. |  |  |  |

The XML schema definition of the Content primitive parameter (i.e. datatype m2m:primitiveContent) allows to include XML wildcard elements. An XML representation of the Content primitive parameter shall include a root element which is associated with an XSD Global Element. The root element shall be prefixed with a namespace prefix identifier (e.g. m2m:) specified in the associated XSD which defines the respective Global Element. The Content primitive parameter allows to include namespaces other than m2m.

### ------------------end of change 9 ----------------------------------------------------------

### ------------------start of change 10 ----------------------------------------------------------

### 8.2.2	Primitive parameters

In protocol bindings primitive parameter names shall be translated into short names of Table 8.2.2-8.

Table 8.2.2-8: Primitive parameter short names

| Parameter Name | XSD long name | Occurs in | Short Name |
| --- | --- | --- | --- |
| Operation | operation | Request | op |
| To | to | Request, Response | to |
| From | from | Request, Response | fr |
| Request Identifier | requestIdentifier | Request, Response | rqi |
| Resource Type | resourceType | Request | ty |
| Content | primitiveContent | Request, Response | pc |
| Role IDs | roleIDs | Request | rids |
| Originating Timestamp | originatingTimestamp | Request, Response | ot |
| Request Expiration Timestamp | requestExpirationTimestamp | Request | rqet |
| Result Expiration Timestamp | resultExpirationTimestamp | Request, Response | rset |
| Operation Execution Time | operationExecutionTime | Request | oet |
| Response Type | responseType | Request | rt |
| Result Persistence | resultPersistence | Request | rp |
| Result Content | resultContent | Request | rcn |
| Event Category | eventCategory | Request, Response | ec |
| Delivery Aggregation | deliveryAggregation | Request | da |
| Group Request Identifier | groupRequestIdentifier | Request | gid |
| Filter Criteria | filterCriteria | Request | fc |
| Desired Identifier Result Type | desiredIdentifierResultType | Request | drt |
| Response Status Code | responseStatusCode | Response | rsc |
| Tokens | tokens | Request | tkns |
| Token IDs | tokenIDs | Request | tids |
| Token Request Indicator | tokenRequestIndicator | Request | tqi |
| Local Token IDs | localTokenIDs | Request | ltids |
| Group Request Target Members | groupRequestTargetMembers | Request | grtm |
| Assigned Token Identifiers | assignedTokenIdentifiers | Response | ati |
| Token Request Information | tokenRequestInformation | Response | tqf |
| Content Status | contentStatus | Response | cnst |
| Content Offset | contentOffset | Response | cnot |
| Authorization Signature Indicator | authorSignIndicator | Request | asi |
| Authorization Signature Request Information | authorSignReqInfo | Response | asri |
| Authorization Signature | authorSigns | Request | aus |
| Authorization Relationship Indicator | authorRelIndicator | Request | auri |
| Semantic Query Indicator | semanticQueryIndicator | Request | sqi |
| Release Version Indicator | releaseVersionIndicator | Request, Response | rvi |
| Vendor Information | vendorInformation | Request, Response | vsi |

XML serialized representations of primitives employ root elements to differentiate between request and response primitive types (see clause Error: Reference source not found). These elements are also embedded in some oneM2M-defined complex datatypes. These root element names shall be translated into short names as in Table 8.2.2-9. Their short name serialization shall include the namespace prefix.

Table 8.2.2-9: Primitive root element short names

| Root Element Name | Occurs in | Short Name |
| --- | --- | --- |
| m2m:requestPrimitive | Request, transaction | rqp |
| m2m:responsePrimitive | Response, aggregatedResponse, transaction | rsp |

### ------------------end of change 10 ----------------------------------------------------------