| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#41.2 |
| Source:* | Kenichi Yamamoto, KDDI, kc-yamamoto@kddi.com |
| Date:* | 2019-07-26 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0080 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0052 Study on Edge and Fog Computing in oneM2M systems V0.8.0 |
| Clauses * | 9.7 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to update descriptions in the solution of tightly coupled Edge/Fog Computing with 3GPP T8 API #2 in order to support Session QoS API of TS-0026, which was agreed at SDS41.

### -----------------------Start of change 1-------------------------------------------

## Solution G: Tightly coupled Edge/Fog Computing with 3GPP T8 API #2

### Solution Applicability

This solution addresses Key Issue 3.

### Solution Description

### Introduction

This solution addresses Edge/Fog computing in a deployment where the T8 interface is exposed to ASN/MN-CSE (SCS), therefore there is a “tight coupling” between the Edge/Fog node and the Underlying Network. Unlike Solution E, which adopt a new resource <nwMonitoringReq> for Network Status Reports API and Monitoring Event API (Number of UEs in an area), this solution introduces the procedure with existing resources.

Table 9.7.2.1-1 shows the summary of interworking between ASN/MN-CSE (SCS) and T8 APIs. Note that the table does not contains the procedures for 3GPP connectivity establishment and oneM2M registration, which is depicted in Solution F.

Table 9.7.2.1-1:  Summary of Interworking between ASN/MN-CSE (SCS) and T8 APIs

| TS-0026 [Error: Reference source not found] |  | Interworking with ASN/MN-CSE (SCS) |  |  |
| --- | --- | --- | --- | --- |
| Clause | T8 APIs | Clause | Applicability | Remarks |
| 7.1 | Cellular IoT non-IP data delivery(NIDD) | 9.7.2.2.1 | T.B.D | How to interwork with ASN/MN-CSE (SCS) and NIDD API is FFS. |
| 7.4.1 | UE Reachability monitoring of Monitoring events | 9.7.2.2.2 | ✓ |  |
| 7.4.2 | UE Availability after DDN Failure of Monitoring events | 9.7.2.2.3 | ✓ |  |
| 7.4.3 | UE Communication Failure of Monitoring events | 9.7.2.2.4 | ✓ |  |
| 7.4.4 | UE Loss of Connectivity of Monitoring events | 9.7.2.2.5 | ✓ |  |
| 7.4.5 | Roaming Status of Monitoring events | 9.7.2.2.6 | ✓ |  |
| 7.4.6 | Detecting Change of IMSI-IMEI(SV) Association of Monitoring events | 9.7.2.2.7 | ✓ |  |
| 7.4.7 | Location Reporting of Monitoring events | 9.7.2.2.8 | ✓ |  |
| 7.4.8 | Number of UEs in an Area of Monitoring events | 9.7.2.2.9 | ✓ |  |
| 7.5 | 3GPP Based Device triggering | 9.7.2.2.10 | ✓ | The descriptions of triggerReferenceNumber for <remoteCSE> and <AE>, and the description of the triggerReference for <triggerRequest> need to be modified to support ASN/MN-CSE (SCS). |
| 7.6 | Configuration of Traffic Patterns | 9.7.2.2.11 | ✓ |  |
| 7.7 | Group message delivery using MBMS | 9.7.2.2.12 | T.B.D | No MBMS use case with Edge/Fog Computing is described in current oneM2M TR. Further consideration for this solution may be needed. |
| 7.8 | Informing about Potential Network Issues | 9.7.2.2.13 | ✓ |  |
| 7.9 | Setting up an AS session with required QoS procedure | 9.7.2.2.14 | ✓ |  |
| 7.10 | Background Data Transfer | 9.7.2.2.15 | ✓ | The description of transferSelectionGuidance for <backgroundDataTransfer> resource needs to be modified to support ASN/MN-CSE (SCS). |
| 7.11 | Network Parameter Configuration | 9.7.2.2.16 | ✓ |  |

### Solution procedures

The clause introduces the interworking procedures between ASN/MN-CSE (SCS) and T8 APIs described in TS-0026 [Error: Reference source not found].

### Cellular IoT non-IP data delivery(NIDD)

T.B.D.

NOTE: How to interwork between ASN/MN-CSE (SCS) and NIDD API is FFS.

### UE Reachability monitoring of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.1 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### UE Availability after DDN Failure of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.2 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### UE Communication Failure of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.3 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### UE Loss of Connectivity of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.4 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Roaming Status of Monitoring events

The procedure is depicted in clause 7.4.5 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Detecting Change of IMSI-IMEI(SV) Association of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.6 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Location Reporting of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.7 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Number of UEs in an Area of Monitoring events

The procedure with IN-CSE is depicted in clause 7.4.8 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### 3GPP Based Device triggering

The procedure with IN-CSE is depicted in clause 7.5 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Configuration of Traffic Patterns

The procedure with IN-CSE is depicted in clause 7.6 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Group message delivery using MBMS

T.B.D.

NOTE: No MBMS use case with Edge/Fog Computing is described in current oneM2M TR. Further consideration for this solution may be needed.

### Informing about Potential Network Issues

The procedure with IN-CSE is depicted in clause 7.8 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Setting up an AS session with required QoS procedure

- The procedure with IN-CSE is depicted in clause 7.9 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

        - Background Data Transfer

The procedure with IN-CSE is depicted in clause 7.10 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Network Parameter Configuration

The procedure with IN-CSE is depicted in clause 7.12 of TS-0026 [Error: Reference source not found]. In this solution, the ASN/MN-CSE is operated as SCS and the procedure related to IN-CSE is replaced by the ASN/MN-CSE (SCS).

### Impacted Resources

### Introduction

To implement this solution, the following attributes are proposed:

- The descriptions of the triggerReferenceNumber for <remoteCSE> and <AE> resources and the description of the triggerReference for <triggerRequest> resource are updated to support ASN/MN-CSE (SCS). Those attributes are used for the device triggering Recall/Replace procedures as described in clause 7.5 of TS-0026 [Error: Reference source not found].

- The description of the transferSelectionGuidance for <backgroundDataTransfer> resource is updated to support ASN/MN-CSE (SCS). This attribute is used for the procedures of requesting and selecting a Background Data Transfer Policy as described in clause 7.10 of TS-0026 [Error: Reference source not found].

### Modified <remoteCSE> resource

The description of the triggerReferenceNumber is updated to support ASN/MN-CSE (SCS) as shown in the table below.

Table 9.7.2.3.2-2: Modified attribute of <remoteCSE> resource

| Attributes of <remoteCSE> | Multiplicity | RW/ RO/ WO | Description | <remoteCSEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| triggerReferenceNumber | 0..1 | RW | This is to identify device trigger procedure request. This attribute is used only for device trigger and assigned by the CSE. | NA |

### Modified <AE> resource

The description of the triggerReferenceNumber is updated to support ASN/MN-CSE (SCS) as shown in the table below.

Table 9.7.2.3.3-3: Modified attribute of <AE> resource

| Attributes of <AE> | Multiplicity | RW/ RO/ WO | Description | <AEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| triggerReferenceNumber | 0..1 | RW | This is to identify device trigger procedure request. This attribute is used only for device trigger and assigned by the CSE. | NA |

### Modified <triggerRequest> resource

The descriptions of the triggerReference is updated to support ASN/MN-CSE (SCS) as shown in the table below.

Table 9.7.2.3.4-4: Attributes of <triggerRequest> resource

| Attributes of <triggerRequest> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| triggerReference | 0..1 | RO | This attribute is a reference number which is allocated by the CSE of a transaction and is used in all subsequent messages related to that transaction to support device triggering. |

### Modified <backgroundDataTransfer> resource

The description of the transferSelectionGuidance is updated to support ASN/MN-CSE (SCS) as shown in the table below.

Table 9.7.2.3.5-5: Attributes of <backgroundDataTransfer> resource

| Attributes of < backgroundDataTransfer > | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| transferSelectionGuidance | 0..1(L) | WO | List that includes guidance to CSE in selecting from multiple transfer policies provided by underlying network. Possible values include:” lowest cost”, “highest throughput given maximum cost of X”, etc. If not included, the CSE may independently choose from among multiple transfer policies. |

### -----------------------End of Change 1 ---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?