| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 41.2 |
| Source:* | Preeti Rani, C-DOT, preetir@cdot.in, Siddharth Trikha, C-DOT, strikha@cdot.in Poornima Shandilya, C-DOT, poornima@cdot.in Anupama Chopra, C-DOT, anupama@cdot.in |
| Date:* | 2019-08-06 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0003-Security_Solutions-V3_10_2 |
| Clauses * | 7.6 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | TS-0001, TS-0004, TS-0008, TS-0009 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

A related Change Request SDS-2019-0327R02-MALICIOUS_AE_AND_BLOCKED_ORIGINATOR was introduced in release 3 in SDS39. A short presentation with overall concept was presented and discussed (SDS-2019-0199R02 :MALICIOUS AE AND BLOCKED/BARRED ORIGINATOR) in SDS 39.9. This CR is now prepared to introduce the corresponding concept in Security based on the inputs received in SDS 39 and SDS 39.9.

The CR proposes to add prevention of Barred originator in the TS-0003, corresponding to new concepts added in TS-0001: Malicious AE and Blocked/Barred Originator.

Supporting Points:

- An AE or CSE can act maliciously in an environment. Then the receiver CSE should have capability to deny the service according to its policies.

- To recognise and block such entities, is a function of Security of the system, before it reaches the deeper layers of the architecture.

- Any channel skipping secure communication shall have to either make its system robust so that it is not vulnerable to such attacks or implement such recognising and blocking of such entities by itself.

The key idea is to be able to identify a malicious Originator that is attempting to perform some malicious activity through an oneM2M Request. If such an Originator is identified, then it should be blocked/barred from performing any further action in the oneM2M network. The implementation of blocking barring an AE, identified as Malicious AE, requires the implementation of a blocked/barred originator list.

### The proposal would be an extension of existing requirement: TS-0002

| SER-001 | The oneM2M System shall incorporate protection against threats to its availability such as Denial of Service attacks. | Partially Implemented in Rel-1 |
| --- | --- | --- |

### -----------------------Start of change 1-------------------------------------------

## 7.6 Malicious Originator Prevention

### 7.6.1	Barred originator verification

Originators can behave maliciously and may send request containing malformed patterns, or sending frequent requests exceeding a threshold defined by the hosting CSE’s local policy i.e. DDOS Attack, etc. the Receiver CSE needs prevention mechanism to block such originators.

When the Receiver CSE receives a request, it shall perform the following procedure.

Figure 7.6.1-1: Malicious AE/CSE prevention procedure

- Security association establishment may be performed. Clause 6.1.2.2.1 describes the scenarios when security association establishment between an AE and CSE is mandatory, and describes the scenarios when security association establishment between an AE and CSE is recommended. The security association between a CSE and CSE is mandatory. The subsequent procedures shall be performed if a security association has been established.

- The AE or CSE sends a request to Hosting CSE via its Registrar CSE as specified in oneM2M TS-0001[] (Hosting CSE is not represented on this figure and can either be the Registrar CSE or another CSE). The malicious AE or CSE sends a request to a CSE as specified in oneM2M TS-0001[]

- The Receiver CSE checks if the value in the From parameter is the one of the barred Originators.

- If it is present in the CSE’s barred Originator list, then the CSE shall send a response with Response Status Code '4103' ("ORIGINATOR_HAS_NO_PRIVILEGE").

- If the originator is not present in the barred originator list, then Receiver CSE shall perform the procedures specified in clause 8.2 of oneM2M TS-0001 [Error: Reference source not found]. Depending on the number of Transit CSEs, the Receiver CSE shall either process the request or forward it to the Hosting CSE or to another Transit CSE.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?