| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr Il-Yeup Ahn, KETI, iyahn@keti.re.kr |
| Date:* | 2019-09-23 |
| Reason for Change/s:* | Getting N latest/oldest contentInstance resources |
| CR against: Release* | 4 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v4.2.0 |
| Clauses * | 10.2.4.11 Retrieve <latest> 10.2.4.12 Delete <latest> 10.2.4.13 Retrieve <oldest> 10.2.4.14 Delete <oldest> |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Currntly in the oneM2M specs, latest or oldest contentInstance resource can be retrieved and deleted using <latest> and <oldest> virtual resources. This contribution proposes to allow multiple instances also can be retrieved or deleted by pending “/n” after the name of the virtual resource.

E.g. The retrieval of “root/cnt/la/3” will return 3 latest <contentInstance> resources of “cnt” container.

In R01:

- Adds “cnt/la/3” retrieval response primitive example in the introduction. Basically the array of multiple <contentInstance> resources having the parent resource object key on top. This is like the Result Content parameter with “child resources” option related response.

{ "m2m:cnt":[ { "m2m:cin":{ "ty":4, "et":"20200911T112511", "ct":"20190911T112511", "lt":"20190911T112511", "ri":"5d785b07a571b01e90d618e7", "rn":"cin01", "pi":"5d6b057b8c899034017709ba", "cs":8, "con":"test" } }, { "m2m:cin":{ "ty":4, "et":"20200912T112511", "ct":"20190912T112511", "lt":"20190912T112511", "ri":"5d785b07a571b01e90d618e8", "rn":"cin02", "pi":"5d6b057b8c899034017709ba", "cs":8, "con":"test" } } ] }

In R02:

- updates the cover page: clauses, impacted TR/TS (this CR impacts TS-0004)

### ********** Start of change 1 **********

#### 10.2.4.11	Retrieve <latest>

If locationID of <container> is configured, the procedure specified in clause 10.2.9.7 shall apply.

If locationID of <container> is not configured, this procedure shall apply to the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If there is no <contentInstance> resource in the parent, then the Receiver shall response with an error.

This procedure shall apply to the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If there is no <contentInstance> resource in the parent, then the Receiver shall response with an error.

When the To parameter of the retrieve request has a number suffix (e.g. cnt1/la/3), then the Hosting CSE shall return the number of latest <contentInstance> resources. If there are smaller number of instances, then those instances shall be returned.

10.2.4.12	Delete <latest>

This procedure shall apply to the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If there is no <contentInstance> resource in the parent, then the Receiver shall response with an error.

When the To parameter of the delete request has a number suffix (e.g. cnt1/la/3), then the Hosting CSE shall remove the number of latest <contentInstance> resources. If there are smaller number of instances, then those instances shall be removed.

#### 10.2.4.13	Retrieve <oldest>

This procedure shall apply to the oldest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If there is no <contentInstance> resource in the parent, then the Receiver shall response with an error.

When the To parameter of the retrieve request has a number suffix (e.g. cnt1/ol/3), then the Hosting CSE shall return the number of oldest <contentInstance> resources. If there are smaller number of instances, then those instances shall be returned.

10.2.4.14	Delete <oldest>

This procedure shall apply to the oldest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If there is no <contentInstance> resource in the parent, then the Receiver shall response with an error.

When the To parameter of the delete request has a number suffix (e.g. cnt1/ol/3), then the Hosting CSE shall remove the number of oldest <contentInstance> resources. If there are smaller number of instances, then those instances shall be removed.

### ********** End of change 1 **********

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?