| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | Kenichi Yamamoto, KDDI, kc-yamamoto@kddi.com |
| Date:* | 2019-09-12 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0080 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 3GPP Interworking V4.2.0 |
| Clauses * | 7.x |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | TS-0001 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes how an AE (Originator) exchanges with a 3GPP Underlying Network parameters to be used for optimizing the data traffic over the Underlying Network by using a new resource <nwMonitoringReq>. The <nwMonitoringReq> resource provides the characteristics of the Underlying Network status in a particular geographic area such as congestion status and number of devices.

The CR is based on solution E:Tightly coupled Edge/Fog Computing with 3GPP T8 API of TR-0052. In the solution E, a oneM2M MN retrieves the 3GPP underlying network information in a particular area from a SCEF directly and adjusts data processing/transfer for the Field Domain Nodes. As a result, oneM2M System can avoid the need for an IN-CSE to process data for the Field Domain Nodes and the IN-CSE can be freed from the burden for processing of 3GPP T8 API information.

In this proposed procedure, a oneM2M IN or ASN can also retrieve the 3GPP underlying network information in a particular area from a SCEF for supporting various deployment scenarios.

R01 updates based on SDS discussion.

### -----------------------Start of change 1-------------------------------------------

## 7.x	Network Monitoring Request

This clause provides details on how an AE (Originator) exchanges with a 3GPP Underlying Network parameters to be used for optimizing the data traffic over the Underlying Network for a set of Field Domain Nodes hosted on UEs. If the AE (Originator) sets the type of network request with the associated attributes such as a geographic area, congestion threshold and External Group ID, the Hosting CSE determines the corresponding T8 API(s) based on the type of network request, maps the attributes to the T8 API(s), and communicates with the SCEF. When the SCEF returns a response to the Hosting CSE, the Hosting CSE maps the response to the specified oneM2M resource and sends a response to the AE (Originator). Based on the information, the AE (Originator) may adjust data processing/transfer for the Field Domain Nodes (ASN/MN/ADN).

### 7.x.2 Resource Structure

Refer to the clause 9.6.x Resource Type <nwMonitoringReq> of oneM2M TS-0001[1].

### 7.x.3	Procedures

Figure 7.x.3.1 depicts a procedure to retrieve an Underlying Network information in a particular geographic area initiated by a request from an AE. The following T8 APIs are applicable for this procedure.

- Network Status Reports API

- Monitoring Event API (Monitoring Type: Number of UEs in an Area)

Figure 7.x.2-1: Procedure to retrieve an Underlying Network information in a particular geographic area

Pre-conditions:

There is a relationship in place between the Service Provider and MNO allowing the AE (Originator) to request 3GPP T8 API information from the 3GPP Underlying Network. The method for establishing this relationship is outside the scope of the present document.

If the deployment uses External Group Identifier (externalGroupId) as described in 3GPP TS29.122 [4], when ASN/MN-CSEs or ADN-AEs register with the Hosting CSE (SCS), then they use externalGroupId information to configure the externalGroupID of the corresponding <remoteCSE> or <AE> resources (see clause 6.3 when externalGroupID is configured).

The Hosting CSE is configured with system defaults as described in clause 7.8 and/or clause 7.4.8.

Step 1: CREATE <nwMonitoringReq> Request & Response, Subscription creation

An Originator (AE) requests the creation of a <nwMonitoringReq> resource at the Hosting CSE and, if the operation is successful, the Originator receives a response message. And the Originator subsequently subscribes to updates of the <nwMonitoringReq> resource.

Step 2: UPDATE <nwMonitoringReq> for enable network monitoring

In order to initiate a monitoring request, the Originator sends a request to update the monitorEnable attribute of the <nwMonitoringReq> resource.

- monitorEnable shall be set to the type of network monitoring request (e.g. congestion status in an area, the number of devices in an area, both congestion status and the number of devices in an area, disable).

- geographicArea shall be set to the geographic area where the Originator wants to retrieve an Underling Network information.

- externalGroupID shall be set to the group of interest in the request, in which case the Monitoring Event Request is for the number of group-member UEs present in the area of interest. The Hosting CSE gets the externalGroupID information according to the attribute externalGroupID of the resource <remoteCSE> and <AE> of the UEs which location are in the area of interest. If there are multiple externalGroupIDs, the Hosting CSE uses local policies to determine the value sent in this request. For example, the Hosting CSE may determine  to send separate requests for each externalGroupID or it may determine to send this request without an externalGroupID and filter the received information.

- congestionLevel shall be set to one of following values:

  - The list of congestion level(s) with exact value and specify what congestion threshold(s) the Originator wants to receive a report for.

  - The list of enumerated types with values HIGH, MEDIUM and LOW that specify the type of congestion status the Originator would like to receive a report for.

Step 3a: Process Network Status Reports Request

If the monitorEnable attribute is set to “enable congestion status in an area” or “enable both number of devices and congestion status in an area”, the Hosting CSE maps the attributes of the <nwMonitoringReq> resource to the following attributes of Network Status Reports API as described in clause 7.8.

- The Hosting CSE sets the fixed parameters with the corresponding attributes of the API (e.g. URI, monitorExpireTime, supportedFeatures).

- geographicArea of the <nwMonitoringReq> resource shall be set to locationArea.

- If congestionLevel indicates an abstracted value for congestion status (e.g. HIGH, MEDIUM or LOW), thresholdTypes shall be set to the abstracted value of the congestionLevel. If congestionLevel indicates an exact value for congestion status (e.g. between 0 and 31), thresholdValues shall be set to the exact value of the congestionStatus.

Then the Hosting CSE communicates with SCEF by using the procedures for Network Status Reports API described clause 7.8.

Step 3b: Process Monitoring Event (Number of UEs in an area) Request

If the monitorEnable attribute of the <nwMonitoringReq> resource is set to “enable number of devices in an area” or “enable both number of devices and congestion status in an area”, the Hosting CSE maps the attributes of the <nwMonitoringReq> resource to the following attributes of Monitoring Event API (Number of UEs in an area) described clause 7.4.8.

- The Hosting CSE shall set the fixed parameters with the corresponding attributes of the API (e.g. URI, supportedFeatures).

- geographicArea of the <nwMonitoringReq> resource shall be set to locationArea or locationArea5G.

- externalGroupId shall be set to the externalGroupID if in step 2 the Hosting CSE monitoring request targets identifying the number of UEs from a specific group in the area and the Hosting CSE determined an externalGroupID to be monitored.

Then the Hosting CSE communicates with SCEF by using the procedures for Monitoring Event API (Number of UEs in an area)  described clause 7.4.8.

Step 4: Notification of <nwMonitoringReq>

After completion of Step 3a or 3b, the Hosting CSE shall map the response of 3GPP T8 API to the following attributes of the <nwMonitoringReq> resource as detailed below. The update <nwMonitoringReq> resource generates a corresponding notification response to the Originator.

- If the response indicates nsiValue or nsiType, the Hosting CSE shall set the congestionStatus parameter of the <nwMonitoringReq> resource.

- If the response indicates ueCount, the Hosting CSE shall set the numberOfDevices parameter of the <nwMonitoringReq> resource. If an externalGroupId has been provided in the request, the count indicates the number of UEs from the given group which are found at the location.

- If the response indicates externalIds, the Hosting CSE shall configure to indicate M2M-Ext-ID attribute of the <nwMonitoringReq> resource.

Step 5: The Originator adjusts data processing/transfer for Field Domain Nodes (ASN/MN/ADN)

The Originator may use the information provided in step 4 in order to adjusts data processing/transfer for Field Domain Nodes (ASN/MN/ADN).

Step 6 (Optional): DELETE <nwMonitoringReq> Request

The Originator sends a request to delete the <nwMonitoringReq> resource.

Step 7 (Optional): Process deletion of Network Status Reports

If the monitorEnable attribute is set to “enable congestion status in an area” or “enable both number of devices and congestion status in an area”, and the congestionStatus is set to a value for congestion status or congestion threshold, the Hosting CSE sends a DELETE request of the Network Status Reports API to the SCEF as described in clause 7.8.

Step 8  (Optional): The Hosting CSE deletes the <nwMonitoringReq> resource

If the monitorEnable attribute is set to “enable number of devices in an area” or “enable both number of devices and congestion status in an area”, and the congestionStatus is not indicated, the Hosting CSE deletes <nwMonitoringReq> resource.

If in step 7 the Hosting CSE receives a 204 No Content response code from the SCEF, it deletes <nwMonitoringReq> resource. Otherwise, the Hosting CSE does not delete the <nwMonitoringReq> resource.

Step 9 (Optional): The Hosting CSE returns response to the Originator.

The Hosting CSE sends a DELETE response back to the Originator.

See clause 8.3 for a list of possible error scenarios and error handling options for the Hosting CSE.

### -----------------------End of Change 1 ---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?