| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | Dale Seed, Convida, Seed.Dale@convidawireless.com |
| Date:* | 2019-09-23 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0046 (Vehicular Domain Enablement) MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v4.2.0 |
| Clauses * | 9.6.X, 10.2.X |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to move one of the time synchronization solutions defined in clause 10.8 of TR-0026 to TS-0001 as normative texts.

This solution addresses Key Issue 7 which highlights that services requiring time-sensitive operations rely on the various Service Layer entities being time-synchronized.

CSEs can be configured by one or more entities to send SL time synchronization beacon notifications. The beacons can be customized to be sent at periodic frequencies (e.g. every 5 minutes) or in response to loss of synchronization exceeding a specified time offset threshold.   The beacons can be useful for keeping an entity synchronized with the current time of a CSE especially when the entity is communicating with the CSE on an intermittent basis where clock drift and synchronization issues can arise.

Time synchronization beacon notifications contain information such as the current time of the Hosting CSE.  In the future, oneM2M could also consider adding additional information to beacon notifications such as scheduling information to program sleep and/or low power schedules.

The solution relies upon the following elements:

- <timeSynchBeacon> resources are created by entities requesting time synchronization services from the Hosting CSE. The Hosting CSE provides time synchronization services in the form of beacon notifications that are sent to the entities.

- The Hosting CSE sends notifications with the time-synchronization information (beacon message) based on the configured parameters.

- The beacon requester can extract the current time of the Hosting CSE, compare it against its own current time to compute an offset.  The offset can then be used to synchronize the current time of the beacon requester to the current time of the Hosting CSE.

Note that nodes hosting oneM2M entities should keep their time synchronized to Coordinated Universal Time (UTC) using mechanisms such as NTP or GPS whenever possible.  However if this is not possible, then this proposed oneM2M time synchronization method can be used.

R02 : Made timeSyncBeacon attributes as optional announce.

### -----------------------Start of change 1-------------------------------------------

### 9.6.X	Resource Type timeSyncBeacon

A <timeSyncBeacon> resource is created by an entity requesting time synchronization services from the Hosting CSE. As a result, the Hosting CSE provides time synchronization services in the form of beacon notifications. The beacon notifications include time-synchronization information (e.g. current time of the Hosting CSE).

When receiving a beacon notification, the receiver can extract the Hosting CSE’s current time, compare it against its own current time and compute an offset.  This offset can then be used to synchronize the current time of the receiver to the current time of the Hosting CSE.

The <timeSyncBeacon> resource shall contain the child resources specified in table 9.6.X-1.

Table 9.6.X-1: Child resources of <timeSyncBeacon> resource

| Child Resources of <timeSyncBeacon> | Child Resource Type | Multiplicity | Description | <timeSyncBeaconAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | None |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |

The <timeSyncBeacon> resource shall contain the attributes specified in table 9.6.X-2.

Table 9.6.X-2: Attributes of <timeSyncBeacon> resource

| Attributes of <timeSyncBeacon> | Multiplicity | RW/ RO/ WO | Description | <timeSyncBeaconAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| beaconRequester | 0..1 | WO | This attribute is configured with the AE-ID or CSE-ID of the entity requesting beacon notifications from the Hosting CSE (see note). This attribute shall be configured if the beaconCriteria attribute is set to “Loss of Synchronization”. This attribute is not applicable if the beaconCriteria attribute is set to “Periodic”. | OA |
| beaconCriteria | 1 | RW | Criteria for the beacon generation. - Periodic (Default) – Beacon is sent periodically based on the period defined by timeSynchBeaconInterval - Loss of Synchronization – Beacon is sent as a result of a detection in a loss of synchronization exceeding the threshold defined by beaconThreshold | OA |
| beaconInterval | 0..1 | RW | The requested frequency of beacons expressed as the minimum time between two beacon notifications sent by the Hosting CSE to the beaconRequester. This attribute is applicable if beaconCriteria is set to “Periodic”. If this attribute is not configured and beaconCriteria is set to “Periodic”, then the Hosting CSE shall use a default value based on local policies. | OA |
| beaconThreshold | 0..1 | RW | Synchronization time offset threshold. This attribute is applicable if beaconCriteria is set to “Loss of Synchronization”. When the delta between the current time of the Hosting CSE and the current time of the beaconRequester exceeds this defined threshold, then the Hosting CSE shall send beacon notification(s) to the beaconNotificationURIs. If this attribute is not configured and beaconCriteria is set to “Loss of Synchronization” then the Hosting CSE shall use a default value based on local policies. The current time of a beaconRequester can be monitored by the Hosting CSE using Originating Timestamp parameters present in the request and response messages that a beaconRequester sends to the Hosting CSE. If these parameters are not present and the Hosting CSE is unable to monitor the current time of the beaconRequester, then the Hosting CSE shall not send beacon notifications. | OA |
| beaconNotificationURIs | 1 (L) | RW | This attribute shall be configured as a list consisting of one or more beacon notification targets. This attribute uses the same format as the notificationURI attribute defined for the <subscription> resource (see Table 9.6.8-2). | OA |
| NOTE: The AE-ID and CSE-ID identifiers should be shortened to use CSE-Relative or SP-Relative formats when appropriate based on their context of use (see clause 7.2). |  |  |  |  |

### -----------------------End of Change 1 ---------------------------------------------

### -----------------------Start of Change 2 ---------------------------------------------

### -----------------------End of Change 2 ---------------------------------------------

### 10.2.XX	Time Management

#### 10.2.XX.1	Introduction

This clause describes procedures for the management of time between entities in the oneM2M system.

#### 10.2.XX.2	Time Synchronization Beaconing

When an Originator creates a <timeSyncBeacon> resource, the Hosting CSE shall generate time synchronization beacon notifications based on the information specified within the <timeSyncBeacon> resource.

If the beaconCriteria attribute of the <timeSyncBeacon> resource has a value of “Periodic”, which is the default value,  then the Hosting CSE shall send periodic notifications to the specified beaconNotificationURIs with a periodic interval defined by the beaconInterval attribute. If the beaconInterval attribute is not configured, then the Hosting CSE shall use a default periodic interval value based on local policies.

If the beaconCriteria attribute of the <timeSyncBeacon> resource has a value of “Loss of Synchronization”, then the Hosting CSE shall monitor the current time of the entity specified by the beaconRequester attribute and compare it to its own current time.   The current time of the entity shall be monitored by the Hosting CSE using Originating Timestamp parameters present in the request and response messages sent by the entity to the Hosting CSE.  If these parameters are not present and the Hosting CSE is unable to monitor the current time of the entity, then the Hosting CSE shall not send beacon notifications.  If the Hosting CSE computes an offset between its current time and the current time of the entity that exceeds the value specified by the beaconThreshold attribute, then the Hosting CSE shall send notifications to the specified beaconNotificationURIs. If the beaconThreshold attribute is not configured, then the Hosting CSE shall use a default threshold value based on local policies

The beacon notifications sent by the Hosting CSE shall contain the current time of the Hosting CSE.  If the beacon notification is sent due to a “Loss of Synchronization”, it shall also include the computed time offset.

Upon receiving the beacon notification, the receiving entity may extract the current time of the Hosting CSE and offset (if present) and use this information to synchronize its current time to the current time of the Hosting CSE.

#### 10.2.XX.3	Create <timeSyncBeacon>

This procedure shall be used for creating a <timeSyncBeacon> resource.

Table 10.2.XX.3-1: <timeSyncBeacon> CREATE

| <timeSyncBeacon> CREATE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: The resource content shall provide the information as defined in clause 9.6.XX. |
| Processing at Originator before sending Request | The Originator shall request to Create a <timeSyncBeacon> resource by using the CREATE operation. The request shall address a <CSEBase>, <remoteCSE> or <AE> resource of a Hosting CSE. The Originator shall be an AE or CSE. The Originator shall specify one or more beaconNotificationURIs. The Originator may also configure other optional attributes defined in clause 9.6.XX. |
| Processing at Receiver | According to clause 10.1.2 In addition, the Receiver shall process the request as described in clause 10.2.XX.2 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: Address of the created <timeSyncBeacon> resource, according to clause 10.1.2 |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | - According to clause 10.1.2 |

#### 10.2.XX.4	Retrieve <timeSyncBeacon>

This procedure shall be used for retrieving <timeSyncBeacon> resource.

Table 10.2.XX.4-1: <timeSyncBeacon> RETRIEVE

| <timeSyncBeacon> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: void. |
| Processing at Originator before sending Request | According to clause 10.1.3. |
| Processing at Receiver | - According to clause 10.1.3 |
| Information in Response message | - According to clause 10.1.3 |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | - According to clause 10.1.3 |

#### 10.2.XX.5	Update <timeSyncBeacon>

This procedure shall be used for updating an existing <timeSyncBeacon> resource.

Table 10.2.XX.5-1: <timeSyncBeacon> UPDATE

| <timeSyncBeacon> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the target <timeSyncBeacon> resource Content: The attributes which are to be updated |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | - According to clause 10.1.4 |
| Information in Response message | According to clause 10.1.4 |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | - According to clause 10.1.4 |

#### 10.2.XX.6	Delete <timeSyncBeacon>

This procedure shall be used for deleting an existing <timeSyncBeacon> resource.

Table 10.2.XX.6-1: <timeSyncBeacon> DELETE

| <timeSyncBeacon> DELETE |  |
| --- | --- |
| Associated Reference Point | Mcc, Mca and Mcc' |
| Information in Request message | From: Identifier of the AE or the CSE that initiates the Request To: The address of the target <timeSyncBeacon> resource |
| Processing at Originator before sending Request | The Originator shall request to delete an existing <timesyncBeacon> resource by using the DELETE operation. The request shall address the specific <timeSyncBeacon> resource of a Hosting CSE. |
| Processing at Receiver | - According to clause 10.1.5 |
| Information in Response message | - According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | - According to clause 10.1.5 |

### -----------------------Start of Change 3 ---------------------------------------------

## Resource Types

### 9.6.1	Overview

#### 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, pollingChannel, semanticDescriptor, timeSeries, transaction, transactionMgmt, triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, locationPolicy, action, timeSyncBeacon | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor, transaction | Container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, subscription, semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.35 |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, flexContainer, timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action, timeSyncBeacon | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription, transaction | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription, transaction | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription, transaction | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance> and <timeSeriesInstance> child resource within a <container> and a <timeSeries> resource | None specified | container, timeSeries | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription, transaction | CSEBase, AE | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription, transaction | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, transaction, semanticDescriptor | node, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription, transaction | CSEBase | 9.6.19 |
| node | Represents specific Node information | mgmtObj, subscription, semanticDescriptor, schedule, transaction, action | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription, transaction | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules, transaction | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance> and <timeSeriesInstance> child resource within a <container> and a <timeSeries> resource | None specified | container, timeSeries | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription, transaction | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, contentInstanceAnnc flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, timeSeries, timeSeriesAnnc, timeSeriesInstanceAnnc, mgmtObjAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc, semanticMashupInstance, semanticMashupInstanceAnnc, action, timeSyncBeacon | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription, transaction | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription, transaction | subscription, CSEBase, - node | 9.6.9 |
| serviceSubscribedNode | Node information | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription, transaction | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription, transaction | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribed-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc, timeSyncBeacon | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription, transaction | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources. | subscription, transaction | AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource | transaction | group | 9.6.14a |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities. | timeSeriesInstance, subscription, semanticDescriptor, latest, oldest, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, container, containerAnnc, flexContainer, flexContainerAnnc | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | transaction | timeSeries, timeSeriesAnnc | 9.6.37 |
| authorizationDecision | Represents an access control decision point | subscription, transaction | CSEBase | 9.6.41 |
| authorizationPolicy | Represents an access control policy retrieval point | subscription, transaction | CSEBase | 9.6.42 |
| authorizationInformation | Represents an access control information point | role token subscription, transaction | CSEBase | 9.6.43 |
| localMulticastGroup | Stores local multicast group information of member hosting CSE. | transaction | CSEBase | 9.6.44 |
| AEContactList | Contains information about a CSE that has resources that referencing an AE-ID | AEContactListPerCSE, subscription, transaction | CSEBase | 9.6.45 |
| AEContactListPerCSE | Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes | None specified | AEContactList | 9.6.46 |
| transactionMgmt |  | subscription | CSEBase, AE, remoteCSE | 9.6.47 |
| transaction |  | action | All non-virtual resource types with the exception of the following: request, delivery, pollingChannel, transactionMgmt, transaction, timeSyncBeacon | 9.6.48 |
| triggerRequest | Used by an AE to initiate, replace or recall a device trigger request | subscription | AE | 9.6.49 |
| ontologyRepository | Represents the collection of the managed ontologies and the semantic validation service | ontology, semanticValidation, subscription | CSEBase | 9.6.50 |
| ontology | Store the representation of an ontology | subscription | ontologyRepository | 9.6.51 |
| semanticValidation | A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology. | None specified | ontologyRepository | 9.6.52 |
| semanticMashupJobProfile | Represents the profile and description of a semantic mashup service | semanticMashupInstance, semanticDescriptor, subscription | CSEBase, remoteCSE | 9.6.53 |
| semanitcMashupInstance | Represents a semantic mashup instance | semanticMashupResult, semanticDescriptor, mashup, subscription | semanticMashupJobProfile, AE, remoteCSE, CSEBase | 9.6.54 |
| Mashup | A virtual resource use to trigger the calculation and generation of new mashup result | Not specified | semanticMashupInstance | 9.6.55 |
| semanticMashupResult | Represent semantic mashup results | semanticDescriptor, subscription | semanticMashupInstance | 9.6.56 |
| multimediaSession | Stores a representation of a multimedia session information requested by a registering AE | subscription | AE | 9.6.57 |
| crossResourceSubscription | represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | CSEBase, remoteCSE, AE | 9.6.58 |
| backgroundDataTransfer | Stores information for a background data transfer request | None specified | AE, remoteCSE, CSEBase | 9.6.60 |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | dependency | CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction | 9.6.61 |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | None specified | action | 9.6.62 |
| e2eQosSession | Specifies the end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities. | subscription | CSEBase, remoteCSE, AE | 9.6.63 |
| timeSyncBeacon | Specifies criteria that a Hosting CSE uses to generate time synchronization beacon notifications | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.XX |
| NOTE: See clause 9.6.12 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### -----------------------End of Change 3 ---------------------------------------------

### -----------------------Start of Change 4 ---------------------------------------------

### 10.1.6	NOTIFY (N)

The NOTIFY operation shall be used for notifying information. All the specific notification procedures defined in this present document are listed in clause 10.1.6 (Notification procedures).

The Originator requests to notify an entity by using NOTIFY method. See clause 8.1.2 for the information to be included in a Request message.

The Receiver responds to the Originator with the operation results as specified in clause 8.1.3.

Figure 10.1.6-1: Procedure for NOTIFYing Information

Step 001: A notification to be sent to the Receiver is triggered in the Originator.

Step 002: The Originator shall send  mandatory parameters and may send optional parameters in Request message for NOTIFY operation as specified in clause 8.1.2.

Step 003: Local Processing.

Step 004: The Receiver  shall respond with mandatory parameters and may send optional parameters in Response message for NOTIFY operation as specified in clause 8.1.3.

General Exceptions:

- See oneM2M TS-0003 [Error: Reference source not found].

In the present specification, notification procedures are defined in the following procedures:

- <subscription> resource handling (clause 10.2.10)

  - to notify Receiver(s) of modifications of a resource for an associated <subscription> resource

  - to notify aggregated notifications from <subscription> member resources of <group> resource

  - to request Receiver(s) to perform resource subscription verification

  - to notify deletion of the <subscription> resource

  - to seek authorization from the subscription creator during a notification target deletion

- Asynchronous non-blocking request handling (clause 8.2.2.3)

  - to send the result of the request

- <pollingChannelURI> resource handling (clause 10.2.5.19)

  - to send the response corresponding to a request delivered via service layer long polling

- IPE on-demand discovery handling (clause 10.2.6)

  - to notify Receiver(s)(i.e., IPE) for on-demand discovery request.

- End-to-end security handling (clause 11.4)

  - to send the request/response that cannot be readable by Transit CSEs

- Dynamic authorization consultation handling (clause 11.5)

  - to seek authorization to access a resource from Dynamic Authorization Server

- Change in AE Registration Point (clause 10.2.16)

- to notify Receivers that an AE has changed registration point

- Change in a resource with reference to an Application Entity Resource ID (clause 10.1.2 (CREATE), 10.1.4 (UPDATE), 10.1.5 (DELETE), 9.6.1.3.1 (expiration timer expiry)

- to notify IN-CSE that the Originator has a new/updated reference to an Application Entity Resource identifier

- <crossResourceSubscription> resource handling (clause 10.2.10)

  - to notify Receiver(s) of cross-resource notification generated by a <crossResourceSubscription> Hosting CSE

  - to notify deletion of the <crossResourceSubscription> resource

  - to request Receiver(s) to perform cross-resource subscription verification

  - to seek authorization from the cross-resource subscription creator during a notification target deletion

- <timeSyncBeacn> handling (clause 10.2.XX)

  - to notify Receivers of time synchronization information of the Hosting CSE

### -----------------------End of Change 4 ---------------------------------------------

### -----------------------Start of Change 5 ---------------------------------------------

### 8.1.2	Request

Requests over the Mca and Mcc reference points, from an Originator to a Receiver, shall contain mandatory and may contain optional parameters. Certain parameters may be mandatory or optional depending upon the Requested operation. In this clause, the mandatory parameters are detailed first, followed by those that are operation dependent, and then by those that are optional:

- To: Address of the target resource or target attribute for the operation. The To parameter shall conform to clause 9.3.1.

NOTE 1:	To parameter can be known either by pre-provisioning (clause 11.2) or by discovery (clause 10.2.6 for discovery). Discovery of <CSEBase> resource is not supported in this release of the document. It is assumed knowledge of <CSEBase> resource is by pre-provisioning only.

NOTE 2:	The term target resource refers to the resource which is addressed for the specific operation. For example, the To parameter of a Create operation for a resource <example> would be "/m2m.provider.com/exampleBase". The To parameter for the Retrieve operation of the same resource <example> is "/m2m.provider.com/exampleBase/example".

NOTE 3:	For Retrieve operation (clause 10.1.3), the To parameter can be the URI of an attribute to be retrieved.

- From: Identifier representing the Originator.

The From parameter is used by the Receiver to check the Originator identity for access privilege verification.

- Operation: operation to be executed: Create (C), Retrieve (R), Update (U), Delete (D), Notify (N).

The Operation parameter shall indicate the operation to be executed at the Receiver:

- Create (C): To is the address of the target resource where the new resource (parent resource).

- Retrieve (R): an existing To addressable resource is read and provided back to the Originator.

- Update (U): the content of an existing To addressable resource is replaced with the new content as in Content parameter. If some attributes in the Content parameter do not exist at the target resource, such attributes are created with the assigned values. If some attributes in the Content parameter are set to NULL, such attributes are deleted from the addressed resource.

- Delete (D): an existing To addressable resource and all its sub-resources are deleted from the Resource storage.

- Notify (N): information to be sent to the Receiver, processing on the Receiver is not indicated by the Originator.

- Request Identifier: request Identifier (see clause 7.1.7).

Example usage of request identifier includes enabling the correlation between a Request and one of the many received Responses.

Operation dependent Parameters:

- Content: resource content to be transferred.

The Content parameter shall be present in Request for the following operations:

- Create (C): Content is the content of the new resource with the resource type ResourceType.

- Update (U): Content is the content to be replaced in an existing resource. For attributes to be updated at the resource, Content includes the names of such attributes with their new values. For attributes to be created at the resource, Content includes names of such attributes with their associated values. For attributes to be deleted at the resource, Content includes the names of such attributes with their value set to NULL.

- Notify (N): Content is the notification information.

The Content parameter may be present in Request for the following operations:

- Retrieve (R): Content is the list of attribute names from the resource that needs to be retrieved. The values associated with the attribute names shall be returned.

- Resource Type: type of resource.

The ResourceType parameter shall be present in Request for the following operations:

- Create (C): Resource Type is the type of the resource to be created.

Optional Parameters:

- Role IDs: optional, required when role based access control is applied. A list of Role-IDs that are allowed by the service subscription shall be provided otherwise the request is considered not valid.

The Role IDs parameter shall be used by the Receiver to check the Access Control privileges of the Originator.

- Originating Timestamp: optional originating timestamp of when the message was built.

Example usage of the originating timestamp includes: to measure and enable operation (e.g. message logging, correlation, message prioritization/scheduling, accept performance requests, charging, etc.), to enable time synchronization and compensation between oneM2M entities, and to measure performance (distribution and processing latency, closed loop latency, SLAs, analytics, etc.)

- Request Expiration Timestamp: optional request message expiration timestamp. The Receiver CSE should handle the request before the time expires. If a Receiver CSE receives a request with Request Expiration Timestamp with the value indicating a time in the past, then the request shall be rejected.

- …

### -----------------------End of Change 5 ---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?