| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | Yongjing Zhang, Huawei (zhangyongjing@huawei.com) |
| Date:* | 2019-09-23 |
| Reason for Change/s:* | To bring the procedures of ontology mapping into TS-0034 based on the study outcome in TR-0033. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0053> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0034 V4.0.0 |
| Clauses * | 6.x, 6.y, 6.z, 7.x |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR introduces into TS-0034 the resource types, brief CSF & procedure description for the feature of ontology mapping and the usage of ontology mapping result for semantic query and semantic resource discovery based on the study outcome in TR-0033.

All changes made against the original content from TR-0033 are editorial with regard to normalized language and reference updates.

R01: withdrawn

R02: merge the two approaches into one flow in 7.x.2.2. Different implementation choices are described as non-normative text in step 3b.

### -----------------------Start of change 1 (new clause) ----------------------------

- 7.x	Ontology mapping

- 7.x.1	Introduction

There are already many standardized or proprietary ontologies defined for various vertical domains or cross-domain scenarios. Each ontology specifies the common vocabulary and relationships between concepts within its own namespace, but may sometimes overlap conceptually with other ontologies due to the independent design. This is often true if two ontologies are designed for the same knowledge domain or under a common high level domain. Different terminologies may mean the same or similar concept (e.g. lamp vs. light), or one is the actually the sub-class of another (e.g. device vs. thing).

To enable the semantic interoperability between different ontologies, ontology mapping is a prerequisite. It’s an important ontology management method to identify the commonality, similarity as well as inclusion relationships between ontologies, so that the data described in one ontology can be consumed meaningfully by the application who understand only another ontology. Ontology mapping can also help to build a global knowledge base and enhance the system intelligence by linking together a collection of ontologies via the anchors of equal/similar/inclusive concepts.

Ontology mapping can be implemented by either manual approaches or automatic approaches.  For example, in Annex B.1 of oneM2M TS-0012 [5], the ontology mapping between Base Ontology and SAREF is specified by manually configured mapping rules (in the format of mapping tables) according to the experts’ common understanding on both ontologies.

However, discovering proper mapping relationships manually is often too labour-intensive, error-prone, and impractical for large ontologies, especially for non-standardized and unstable ones. Therefore, oneM2M provides automatic means of ontology mapping to discover, create and save the mapped relationships between semantically related ontologies by using industry-proven mapping algorithms, e.g. the edit distance, language-based similarity, structural-based similarity, or external- resources-based similarity etc.

The solution is based on the <ontologyMapping> resource to configure the input parameters for executing the ontology mapping task and to store the mapping result. Meanwhile, the <ontologyMappingAlgorithmRepository> resource and its child resources <ontologyMappingAlgorithm> are used to host a collection of algorithms for automatic ontology mapping that can be selected for individual ontology mapping tasks. The detailed procedures related to ontology mapping are specified in clause 6.x, 6.y and 6.z.

Based on the generated ontology mapping result contained in the <ontologyMapping> resource, semantically equivalent operations such as semantic resource discovery and semantic query can be realized in multi-ontology scenarios as specified in clause 7.x.2.

- 7.x.2 Semantic query and semantic resource discovery based on the ontology mapping result

#### 7.x.2.1 Introduction

Semantic query and semantic resource discovery operations can be enhanced by leveraging the ontology mapping result stored in the <ontologyMapping> resource, so that an application which understands only one ontology (e.g. Ontology-A) can get the resulting content or resource described in another (e.g. Ontology-B).

This feature requires the issuer to provide an additional request parameter - Ontology Mapping Resources as specified in [1] in a normal semantic query and semantic resource discovery request. This request parameter contains a list of resource identifiers of existing <ontologyMapping> resources that are used as the base of converting the query statement or the semantic description of the target resources into their equivalents.

The following clause describes the detailed procedures using the example ontology mapping result in clause 6.x.2.

Note: There is no difference between the semantic query operation and the semantic resource discovery operation in terms of applying the feature of ontology mapping in addition to the respective original procedures. So the following procedures combine the two operations in to one message flow.

#### 7.x.2.2 Procedures

Figure 7.x.2.2-1: The semantic query (or semantic resource discovery) procedure with ontology mapping

The detailed message flow is depicted in Figure 7.x.2.2-1 and explained as follows:

- The hosting CSE (e.g. an oneM2M platform) receives a semantic query (or semantic resource discovery) request from an Originator (e.g. an oneM2M application). The request shall carry a semanticsFilter request parameter that contains the original query statement described in the first ontology (e.g. Ontology-A), as well as an Ontology Mapping Resources request parameter that contains the resource identifiers of one or multiple <ontologyMapping> resources. 
Note that the originator may or may not be the one who created the <ontologyMapping> resources. The use of the <ontologyMapping> resources is subject to the associated access control policy against the originator.

For example, the original query statement may be:

SELECT ?device

WHERE {

?device rdf:type Ontology-A:LightSensor.

}

- The hosting CSE shall locate the <ontologyMapping> resources according to the resource identifiers in the Ontology Mapping Resources request parameter, and retrieves the mapping results between the first ontology (Ontology-A) and the second ontology (Ontology-B) from the mappingResult attribute of the <ontologyMapping> resources.

For example, the mappingResult may contain the following triple (mapping relationship):

Ontology-A:LightSensor  owl:equivalentClass Ontology-B:Light_Sensor

- The hosting CSE shall perform the semantic query (or semantic resource discovery) upon the semantic description of the target resources which reference the first ontology (Ontology-A) or the second ontology (Ontology-B), according to the ontology mapping result and the original query statement. The result shall contain both the original outcome of a normal semantic query (or semantic resource discovery) operation as well as additional outcome of the semantic query (or semantic resource discovery) by using the ontology mapping result.

- This step comprises the following sub-steps:

  - The hosting CSE shall perform the semantic query (or semantic resource discovery) procedure upon the semantic description of the target resources that reference the first ontology (Ontology-A) using the original query statement, and collect the outcome.

For example, a <container-x> resource may have a <semanticDescriptor> child resource that references Ontology-A (by the ontologyRef attribute) and contains the following triple that matches the original query statement:

dev-x rdf:type Ontology-A:LightSensor.

  - The hosting CSE shall perform the semantic query (or semantic resource discovery) procedure upon the semantic description of the target resources that reference the second ontology (Ontology-B) using the ontology mapping result (provided in the <ontologyMapping> resources), and get the additional outcome.

- This step may be performed in two different approaches that are implementation specific (non-normative):

- Approach-1: converting the semantic query statement using the ontology mapping result.

- The hosting CSE can determine the equivalent query statement described in the second ontology (Ontology-B) by converting from the original query statement described in the first ontology (Ontology-A), according to the ontology mapping result between the first ontology (Ontology-A) and the second ontology (Ontology-B). This can be done by replacing all the ontology class and properties of the first ontology (Ontology-A) in the query statement with the corresponding equivalents of the second ontology (Ontology-B).

According to the examples above, the equivalent query statement becomes:

SELECT ?device

WHERE {

?device rdf:type Ontology-B:Light_Sensor.

}

- The hosting CSE then performs the semantic query (or semantic resource discovery) procedure upon the semantic description of the target resources that reference the second ontology (Ontology-B) using the equivalent query statement, and collect the second query (or semantic resource discovery) result.

For example, a <container-y> resource may have a <semanticDescriptor> child resource that references Ontology-B (by the ontologyRef attribute) and contains the following triple that matches the converted equivalent query statement:

dev-y rdf:type Ontology-B:Light_Sensor.

- Approach-2: converting the semantic description of the target resources according to the <ontologyMapping> resources.

- The hosting CSE can determine the equivalent semantic description in the first ontology (Ontology-A) for the target resources which reference the second ontology (Ontology-B), by converting from the original semantic description in the second ontology (Ontology-B) according to the ontology mapping result. This can be done by replacing all the ontology class and properties of the second ontology (Ontology-B) in the related <semanticDescriptor> resources of the target resources into the equivalents of the first ontology (Ontology-A).

For example, a <container-y> resource may have a <semanticDescriptor> child resource that references Ontology-B (by the ontologyRef attribute) and contains the following triple:

dev-y rdf:type Ontology-B:Light_Sensor.

This triple is then converted into the equivalent triple in Ontology-A that matches the original query statement:

dev-y rdf:type Ontology-A:LightSensor.

- The hosting CSE then performs the semantic query (or semantic resource discovery) procedure upon the converted equivalent semantic description using the original query statement, and collect the second query (or semantic resource discovery) result.

  - The hosting CSE

- The hosting CSE shall combine the query/ discoveryquery (or semantic resource discovery) results from both step 3a and step 3b, and return the combined results to the originator.

For example, in the case of semantic query, the results are the IRIs of dev-x and dev-y. In the case of semantic resource discovery, the results are the resource identifiers of <container-x> and <container-y>.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2 (new clause) -----------------------------------

## 6.x	<ontologyMapping> Operations

### 6.x.1 Introduction

The ontology mapping task shall be performed by the Create or Update operation against an <ontologyMapping> resource on a Hosting CSE. A Retrieve operation against the same <ontologyMapping> resource shall be used to get the result of ontology mapping. A Delete operation against a <ontologyMapping> resource shall follow the basic procedure as specified in clause [1].

### 6.x.2 Create <ontologyMapping> (Ontology Mapping)

This procedure shall be used for performing the ontology mapping task by creating a <ontologyMapping> resource as described in Table 6.x.2-1. Detailed message flows are described in Figure 6.x.2-1.

Table 6.x.2-1: <ontologyMapping> CREATE

| <ontologyMapping> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in [Error: Reference source not found1] table 8.1.2-2 apply with the specific details for: Content: The resource content shall provide the information about an <ontologyMapping> resource (e.g. attribute values) as described in [1]. |
| Processing at Originator before sending Request | According to clause 10.1.2 in [Error: Reference source not found1]. |
| Processing at Receiver | The receiver shall follow the basic procedure according to clause 10.1.2 of [Error: Reference source not found1], with the following specific details: - Determine the source <ontology> and target <ontology> resources to be mapped according to the sourceOntology and targetOntology attributes provided in the request; - Determine the ontology mapping method according to the mapping method description including the mappingPolicy and mappingAlgorithmLinks attributes provided in the request; - Retrieve the source <ontology> and target <ontology> resources from a remote CSE if needed; - Retrieve the <ontologyMappingAlgorithm> resources from a remote CSE if needed; - Create the ontology mapping relationships between the source <ontology> and the target <ontology>; - Store the mapping result in the <ontologyMapping> resource in the successful case. |
| Information in Response message | All parameters defined in table 8.1.3-1 in [Error: Reference source not found1] shall apply with the specific details for: Content: Address of the created <ontologyMapping> resource, according to clause 10.1.2 in [Error: Reference source not found1]. |
| Processing at Originator after receiving Response | According to clause 10.1.2 in [Error: Reference source not found1]. |
| Exceptions | According to clause 10.1.2 in [Error: Reference source not found1]. |

Figure 6.x.2-1: The ontology mapping procedure by Create/Update a <ontologyMapping> resource

The normal message flow Create <ontologyMapping> procedure is described as follows:

- The hosting CSE (e.g. an oneM2M platform) receives an ontology mapping request from an Originator (e.g. an oneM2M application) in the form of a Create operation against an <ontologyMapping> resource. The request shall contain the resourceIDs of the source and target <ontology> resources indicated by the sourceOntology and targetOntology attributes in the <ontologyMapping> resource. It shall also contain the information of mapping method description including the mapping policy and the mapping algorithms indicated by the attributes of mappingPolicy and mappingAlgorithmLinks respectively.

- The hosting CSE shall determine the source and target ontologies according to the resourceIDs of the source and target <ontology> resources provided in the request. It shall also determine the ontology mapping method according to the information of mapping method description provided in the request. Specifically, the hosting CSE shall first determine the mapping algorithm policy (single, multiple or traversal) according to the mappingPolicy attribute provided in the request. Then it shall determine the mapping algorithm(s) to be used according to the determined mapping algorithm policy and the available mapping algorithm(s) provided by the mappingAlgorithmLinks attribute, and shall perform the ontology mapping process according to Table 6.x.2-2.

Table 6.x.2-2: ontology mapping process according to different mapping policies and the provided mapping algorithms

| mappingPolicy | mappingAlgorithmLinks | ontology mapping process by the hosting CSE |
| --- | --- | --- |
| single | contains the resourceID(s) of one or more existing <ontologyMappingAlgorithm> resources, or the resourceID of an existing <ontologyMappingAlgorithmRepository> resource. | Decide to use a single ontology mapping algorithm for the ontology mapping between the source and target ontologies. If more than one <ontologyMappingAlgorithm> resource is provided by the mappingAlgorithmLinks, or contained in the referenced <ontologyMappingAlgorithmRepository> resource, the hosting CSE may decide to use one of the provided algorithms according to its local policy. |
| multiple | contains the resourceIDs of two or more existing <ontologyMappingAlgorithm> resources of different types, or the resourceID of an existing <ontologyMappingAlgorithmRepository> resource which contains two or more <ontologyMappingAlgorithm> resources of different types. | Decide to use two or more different types of mapping algorithms (based on the algorithmType attribute of the <ontologyMappingAlgorithm> resource) for the ontology mapping between the source and target ontologies. The hosting CSE may decide to use a subset (at least two types) of the provided algorithms according to its local policy. If the number of the types of the provided algorithms is less than two, the hosting CSE shall reject the request with an error. |
| traversal | contains the resourceID(s) of one or more existing <ontologyMappingAlgorithm> resources, or the resourceID of an existing <ontologyMappingAlgorithmRepository> resource. | Decide to use all the provided ontology mapping algorithms in a traversal way for the ontology mapping between the source and target ontologies. |
| Note1: Any combination of mappingPolicy and mappingAlgorithmLinks not covered by this table shall be considered as an exceptional case, and the hosting CSE shall reject the request with an error. Note2: For a pre-configured algorithm already stored in the system, the <ontologyMappingAlgorithm> resource may not contain the executable of the algorithm. In this case, the hosting CSE may invoke the algorithm from the system locally according to the resourceName or resourceID attribute. Note3: If more than one algorithms are used, the final ontology mapping result shall be a union of all the results from each algorithm. |  |  |

- The hosting CSE may need to retrieve the source and/or target <ontology> resources from a remote CSE the sourceOntology and targetOntology attributes if they are not hosted locally.

- The hosting CSE may need to retrieve the used <mappingAlgorithm> resources from a remote CSE according to the mappingAlgorithmLinks attribute if they are not hosted locally.

- The hosting CSE shall create the ontology mapping relationships between the source and target ontologies according to the determined mapping method, and shall store the resulted mapping relationships in the <ontologyMapping> resource.

Figure 8.12.2.2-2 shows an example of the ontology mapping between the source ontology (Ontology-A) and the target ontology (Ontology-B). Assuming the mappingPolicy=multiple and the mappingAlgorithmLinks points to two <ontologyMappingAlgorithm> resources which are a “linguistic-feature extraction algorithm” and an “external resource acquisition algorithm” respectively. The hosting CSE first performs the “linguistic-feature extraction algorithm” against Ontology-A and Ontology-B, and generates three mapping relationships (as formatted by Triple#1, Triple#2 and Triple#3 below). The hosting CSE then performs the “external resource acquisition algorithm” with the support from external resources (e.g. the WordNet dictionary), and generates a fourth mapping relationship (as Triple#4 below).

- RDF Triple #1: Ontology-A:Thing  owl:equivalentClass Ontology-B:Thing

- RDF Triple #2: Ontology-A:Devices  owl:equivalentClass Ontology-B:Device

- RDF Triple #3: Ontology-A:LightSensor  owl:equivalentClass Ontology-B:Light_Sensor

- RDF Triple #4: Ontology-A:Switch_off  owl:equivalentProperty Ontology-B:Turn_off

The mapping relationships (RDF Triple #1/2/3/4) are stored in the mappingResult attribute of the <ontologyMapping> resource to be created.

- The hosting CSE shall return the successful response to the Originator with the resourceID of the created <ontologyMapping> resource.

Figure 8.12.2.2-2: Example of the mapping result between ontology A and ontology B

### 6.x.3 Retrieve <ontologyMapping> (Get the ontology mapping result)

The ontology mapping result can be retrieved by the Retrieve operation against an <ontologyMapping> resource as described in Table 6.x.3-1. The mapping result is contained in the mappingResult attribute. No resource specific process is required.

Table 6.x.3-1: <ontologyMapping> RETRIEVE

| <ontologyMapping> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in [Error: Reference source not found1] table 8.1.2-2. |
| Processing at Originator before sending Request | According to clause 10.1.3 in [Error: Reference source not found1]. |
| Processing at Receiver | The receiver shall follow the basic procedure according to clause 10.1.2 of [Error: Reference source not found1]. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: attributes of the <ontologyMapping> resource as specified in [1]. The resulted mapping relationships are contained in the mappingResult attribute of the <ontologyMapping> resource. |
| Processing at Originator after receiving Response | According to clause 10.1.3 in [Error: Reference source not found1]. |
| Exceptions | According to clause 10.1.3 in [Error: Reference source not found1]. |

### 6.x.4 Update <ontologyMapping> (Ontology Mapping)

The ontology mapping task may also be performed by the Update operation against an <ontologyMapping> resource. This operation shall be used to generate new mapping results based upon an existing configuration of the <ontologyMapping> with only necessary modifications. The procedure is similar to the Create operation described in clause 6.x.2.

Table 6.x.4-1: <ontologyMapping> UPDATE

| <ontologyMapping> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in [1] table 8.1.2-2 apply with the specific details for: Content: The resource content shall provide the information about an <ontologyMapping> resource (e.g. attribute values) as described in [1]. |
| Processing at Originator before sending Request | According to clause 10.1.4 in [1]. |
| Processing at Receiver | The receiver shall follow the basic procedure according to clause 10.1.4 of [1], with the following specific details: - Determine the source <ontology> and target <ontology> resources to be mapped according to the sourceOntology and targetOntology attributes provided in the request; - Determine the ontology mapping method according to the mapping method description including the mappingPolicy and mappingAlgorithmLinks attributes provided in the request; - Retrieve the source <ontology> and target <ontology> resources from a remote CSE if needed; - Retrieve the <mappingAlgorithm> resources from a remote CSE if needed; - Create the ontology mapping relationships between the source <ontology> and the target <ontology>; - Store the mapping result in the <ontologyMapping> resource in the successful case. |
| Information in Response message | According to clause 10.1.4 in [Error: Reference source not found]. |
| Processing at Originator after receiving Response | According to clause 10.1.4 in [1]. |
| Exceptions | According to clause 10.1.4 in [1]. |

### 6.x.5 Delete <ontologyMapping>

This procedure shall be used for deleting a <ontologyMapping> resource.

Table 6.x.5-1: <ontologyMapping> DELETE

| <ontologyMapping> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-2 in [Error: Reference source not found] apply |
| Processing at Originator before sending Request | According to clause 10.1.5 in [Error: Reference source not found] |
| Processing at Receiver | According to clause 10.1.5 in [Error: Reference source not found] |
| Information in Response message | According to clause 10.1.5 in [Error: Reference source not found] |
| Processing at Originator after receiving Response | According to clause 10.1.5 in [Error: Reference source not found] |
| Exceptions | According to clause 10.1.5 in [Error: Reference source not found] |

## 6.y	<ontologyMappingAlgorithm> procedure

Ontology mapping algorithms are represented as <ontologyMappingAlgorithm> resources under an <ontologyMappingAlgorithmRepository> resource. They can be added, updated, retrieved and deleted by the CRUD operation against a <ontologyMappingAlgorithm> resource following the basic procedures as specified in clause 10.1 in [1]. There is no resource-specific process to be defined.

## 6.z	<ontologyMappingAlgorithmRepository> procedure

The CRUD operation against a <ontologyMappingAlgorithmRepository> resource following the basic procedures as specified in clause 10.1 in [1]. There is no resource-specific process to be defined.

### -----------------------End of change 2-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?