| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr SeongYun KIM, KETI, seongyun.kim@keti.re.kr |
| Date:* | 2019-09-16 |
| Reason for Change/s:* | New feature to support geo-query |
| CR against: Release* | 4 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 v4.0.0 (does not exist at the moment) |
| Clauses * | 6.3.3 Simple data types 6.3.4 Enumeration data types 6.3.5 Complex data types 6.3.6 Universal and common attributes |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0008 and TS-0009 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes the new attribute, filter conditions and handling procedures for the geo-query feature to fullfill the new requirements from RDM#40 agreed contribution “RDM-2019-0061R01-Requirements_on_geometry_objects_and_geospatial_functions”.

| Requirement ID | Description | Release |
| --- | --- | --- |
| OSR-189 | The oneM2M System shall support geometry objects (e.g. Point, Polygon) to represent the geo-location of a M2M Device, M2M Gateway and a Thing. |  |
| OSR-190 | The oneM2M System shall support syntax and semantics of geometry objects referring existing standards (e.g. OGC) to assure location information interoperability for M2M Applications. |  |
| OSR-191 | The oneM2M System shall support discovery of identifiers of Things with geospatial function execution against geometry objects representing the Things. |  |

For OSR-189 and 190, this CR proposes to add the location attribute as the common attribute to AE, CSE and Content Sharing Resource types (i.e. container, flexContainer, timeSeries). The new attribute represents the geo-location in GeoJSON format. The data type for the new attribute is proposed as the complex data type.

For OSR-191, the new filter conditions for requests are suggested to deliver geometry type, geo-coordinates and geospatial function for the geo-query. Correponding query handling is proposed in the discovery procedure.

In R01:

- Adds the short names (from Change 14)

In R02:

- adds missing shortnames for Filter Criteria conditions (in Change 14)

### ********** Start of change 1 **********

### 6.3.5	Complex data types

#### 6.3.5.X	m2m:geoCoordinates

Used for the location attribute of the <CSEBase>, <remoteCSE>, <AE>, <container>, <flexContainer> and <timeSeries> resources. The data type refers GeoJSON [x].

Table 6.3.5.X-1: Type Definition of m2m:geoCoordinates

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| type | m2m:geometryType | 1 |  |
| coordinates | m2m:listOfCoordinates | 1 | An example for Point type: [100.0, 0.0] An example for LineString type: [[100.0, 0.0], [101.0, 1.0]] |

### ********** End of change 1 **********

### ********** Start of change 2 **********

### 6.3.3	oneM2M simple data types

Table 6.3.3-2 describes oneM2M-specific simple data type definitions. XML Schema data type definitions for these data types can be found in the XSD file called CDT-commonTypes-v3_13_0.xsd.

The types in Table 6.3.3-2 are either:

- Atomic data types derived from XML Schema data types by restrictions (other than enumeration) or union.

- List data types constructed from other XML Schema or oneM2M-defined atomic data types.

The oneM2M-defined enumeration data types are defined in clause Error: Reference source not found.

Table 6.3.3-2: oneM2M Simple Data Types

| XSD type name | Type Name | Examples | Description |
| --- | --- | --- | --- |
| m2m:listOfCoordinates | Geo-coordinates | Point: [100.0, 0.0] LineString: [[100.0, 0.0], [101.0, 1.0]] Polygon: [[0.0, 0.0], [0.0, 100.0], [100.0, 100.0], [100.0, 0.0], [0.0, 0.0]] | Longitude and latitude (optionally altitude as well), coordinates in an array. The last coordinates of the Polygon type shall have the last item as the same as the first item in the array. |

### ********** End of change 2 **********

### ********** Start of change 3 **********

#### 6.3.4.2	Enumeration type definitions

##### 6.3.4.2.X	m2m:geometryType

Table 6.3.4.2.X-3: Interpretation of geometryType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Point |  |
| 2 | LineString |  |
| 3 | Polygon |  |
| 4 | MultiPoints |  |
| 5 | MultiLineStrings |  |
| 6 | MultiPoligons |  |

##### 6.3.4.2.Y	m2m:geoSpatialFunctionType

Table 6.3.4.2.Y-4: Interpretation of geoSpatialFunctionType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Within |  |
| 2 | Contains |  |
| 3 | Intersects |  |

### ********** End of change 3 **********

### ********** Start of change 4 **********

### 6.3.6	Universal and Common attributes

oneM2M TS-0001 [Error: Reference source not found] defines a number of Universal Attributes (which appear in all resources) and Common Attributes (which appear in more than one resource and have the same meaning whenever they do appear). Their types and values are described in Table 6.3.6-5.

If a Resource is represented as an XML document then the resource attributes (if present) appear in the order listed in this table. They appear before any resource-specific attributes.

Table 6.3.6-5: Universal and Common Attributes

| Attribute Name | Data Type | Value restrictions and Notes |
| --- | --- | --- |
| resourceType | m2m:resourceType | This attribute is only determined at creation time by the Hosting CSE |
| resourceID | m2m:ID | This attribute is determined at creation time by the Hosting CSE and used for the non-hierarchical addressing method |
| parentID | m2m:nhURI | This attribute is determined by the Hosting CSE and specified in all resource types. For <CSEBase> however, the value of this attribute shall be an empty string |
| creationTime | m2m:timestamp | This attribute is determined by the Hosting CSE when the resource is created locally |
| lastModifiedTime | m2m:timestamp | This attribute is determined by the Hosting CSE when the addressed resource is modified by means of the UPDATE operation |
| labels | m2m:labels | Absence of this attribute means there are no labels |
| accessControlPolicyIDs | m2m:acpType | accessControlPolicyIDs |
| expirationTime | m2m:timestamp | expirationTime |
| link | xs:anyURI | Absence of this attribute means that this is not an announced resource |
| announceTo | list of xs:anyURI | Absence of this attribute means that this is not an announced resource |
| announcedAttribute | list of xs:NCName | Absence of this attribute means that this is not an announced resource |
| stateTag | xs:nonNegativeInteger | This attribute is determined by the Hosting CSE. When a resource is created this counter is set to 0 and it will be incremented on every modification of the resource |
| resourceName | m2m:resourceName |  |
| dynamicAuthorizationConsultationIDs | list of xs:anyURI | dynamicAuthorizationConsultationIDs |
| creator | m2m:ID | The AE-ID or CSE-ID of the entity which created the resource containing this attribute |
| location | m2m:geoCoordinates |  |

Table 6.3.6-6 describes some complex types that group together the universal and common attributes, to be used by Resource Type definitions. Note that stateTag and creator only appear in a limited number of resource types, and therefore are not included in these definitions, instead they are declared in the corresponding XSD files of the resources that support them.

Table 6.3.6-6: Complex Data Types declaring groups of resource common attributes

| XSD type name | Child Elements | Child Element Datatype | Multiplicity | Description |
| --- | --- | --- | --- | --- |
| m2m:resource | @resourceName | m2m:resourceName | 1 |  |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
| m2m:regularResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes included in the non-announceable resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
| m2m:announceableResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes included in the majority of announceable resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | xs:anyURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
|  | announceTo | list of xs:anyURI | 0..1 |  |
|  | announcedAttribute | list of xs:NCName | 0..1 |  |
| m2m:announcedResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes in the announced variant of the preceding resources. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | link | xs:anyURI | 1 |  |
|  | dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
| m2m:subordinateResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes included in the non-announceable resource types without accessControlPolicyIDs. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
| m2m:announceableSubordinateResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes used by resource types that are subordinate children of other resources. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | xs:anyURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | announceTo | list of xs:anyURI | 0..1 |  |
|  | announcedAttribute | list of xs: NCName | 0..1 |  |
| m2m:announcedSubordinateResource | @resourceName | m2m:resourceName | 1 | Declares the common / universal attributes used in the announced variants of the subordinate resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | link | xs:anyURI | 1 |  |

NOTE:	In Table 6.3.6-6, names of XML schema attributes are prefixed with a "@" character to differentiate these from Resource attribute names. The "@" character is not part of the actual attribute name.

### ********** End of change 4 **********

### ********** Start of change 5 **********

### 7.4.3	Resource Type <CSEBase>

#### 7.4.3.1	Introduction

A <CSEBase> resource shall represent a CSE. This <CSEBase> resource shall be the root for all the resources that are residing on the CSE. The detailed description can be found in clause 9.6.3 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.3.1-7: Data type definition of <CSEBase> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| CSEBase | CDT-CSEBase-v3_13_0.xsd |  |

Table 7.4.3.1-8: Universal/Common Attributes of <CSEBase> resource

Attribute Name

@resourceName

resourceType

resourceID

parentID

creationTime

lastModifiedTime

labels

location

### ********** End of change 5 **********

### ********** Start of change 6 **********

### 7.4.4	Resource Type <remoteCSE>

#### 7.4.4.1	Introduction

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> of the Registrar CSE.

In addition each registered CSE shall have a <remoteCSE> resource representing its Registrar CSE. This is located directly under the registered CSE's <CSEBase>.

The detailed description can be found in clause 9.6.4 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.4.1-9: Data type definition of <remoteCSE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| remoteCSE | CDT-remoteCSE-v3_13_0.xsd |  |

Table 7.4.4.1-10: Universal/Common Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

### ********** End of change 6 **********

### ********** Start of change 7 **********

### 7.4.5	Resource Type <AE>

#### 7.4.5.1	Introduction

The <AE> resource represents information about an Application Entity known to a given Common Services Entity.

The detailed description can be found in clause 9.6.5 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.5.1-11: Data type definition of <AE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| AE | CDT-AE-v3_13_0.xsd | XSD schema for AE resource |

Table 7.4.5.1-12: Universal/Common Attributes of <AE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

### ********** End of change 7 **********

### ********** Start of change 8 **********

### 7.4.6	Resource Type <container>

#### 7.4.6.1	Introduction

This resource represents a container for data instances. It is used to share information among other entities and potentially to track the data. A <container> resource has no associated content, only attributes and child resources.

The detailed description can be found in clause 9.6.6 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.6.1-13: Data type definition of <container> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| container | CDT-container-v3_13_0.xsd |  |

Table 7.4.6.1-14: Universal/Common Attributes of <container> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| stateTag | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |
| location | O | O |

### ********** End of change 8 **********

### ********** Start of change 9 **********

### 7.4.37	Resource Type <flexContainer>

#### 7.4.37.1	Introduction

This resource represents a customizable container for data instances. It is a template for the definition of flexible specializations of data containers.

The detailed description can be found in clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found].

There are multiple specializations of <flexContainer> specified by oneM2M. Each of these specializations has its own schema file. There is no separate schema file just for <flexContainer>, however the XML schema types for the specializations all conform to the pattern described in this clause. The XSD of <flexContainer> specializations may use a targetNamespace other than the one identified by the m2m: prefix. Specializations of <flexContainer> which employ the namespace prefix m2m: are defined in Error: Reference source not found. Specialization of <flexContainer> which employ the namespace prefix hd: for Home Domain use cases are specified in oneM2M TS-0023 [Error: Reference source not found].

The following resource types are allowed to include <flexContainer> specializations as children: <CSEBase>, <AE>, <remoteCSE> and <container>.

Table 7.4.37.1-15: Universal/Common Attributes of <flexContainer> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| stateTag | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| creator | O | NP |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

### ********** End of change 9 **********

### ********** Start of change 10 **********

### 7.4.38	Resource Type <timeSeries>

#### 7.4.38.1	Introduction

The resource represents a container for Time Series Data instances. It is used to share information with other entities and potentially to track, detect and report the missing data in Time Series. A <timeSeries> resource has no associated content, only attributes and child resources.

The detailed description can be found in clause 9.6.36 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.38.1-16: Data type definition of <timeSeries> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| timeSeries | CDT-timeSeries-v3_13_0.xsd |  |

Table 7.4.38.1-17: Universal/Common Attributes of <timeSeries> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |
| location | O | O |

### ********** End of change 10 **********

### ********** Start of change 11 **********

#### 7.3.3.17	Using Filter Criteria for identification of target resources

##### 7.3.3.17.0	Introduction

When the Filter Criteria primitive parameter is present in a request primitive, it shall be applied for identification of the applicable target resources of the respective operation. This may apply to Retrieve, Delete, Discovery and Semantic Resource Discovery operations as specified in clauses Error: Reference source not found, Error: Reference source not found, Error: Reference source not found and Error: Reference source not found respectively.

The Filter Criteria primitive parameter defines matching conditions on resource attributes and filter handling conditions. Matching conditions are evaluated against resources and, when true, determine the matched resources. The filter handling conditions provide additional input applied to the matched resource set to determine the filtering result (e.g. maximum number of resources to be included in the filtering result). The filtering result may be composed of one or more resources and shall be used as the target of the operation. Table 7.3.3.17.0-18 summarizes the various filter criteria and conditions. Each row in the table represents a different filter condition type.

If multiple matching conditions of the same type (i.e. same condition tag) are present in the Filter Criteria parameter, these shall be combined by applying logical OR operation. This applies to the condition tags labels, resourceType, contentType or attribute with multiplicity n > 1.

If multiple matching conditions of different type (i.e. different condition tags) are present in the Filter Criteria parameter, then the combined condition shall be derived by applying the logical operation specified by the filterOperation condition. By default logical AND operation shall be used if the filterOperation condition is not present.

EXAMPLE:

- labels=floor1, stateTagSmaller=3 will match if both conditions are true [default AND when filterOperation is not specified]

- labels=floor1, stateTagSmaller=3, filterOperation=1 will match if both conditions are true

- labels=floor1, stateTagSmaller=3, filterOperation=2 will match if either condition is true

- labels=floor1, labels=floor2, filterOperation=1 will match if either condition is true [filterOperation has no effect when all condition tags are the same]

- labels=floor1, stateTagSmaller=3, labels=floor2, filterOperation=2 will match if any of these conditions are true resource has [labels with value "floor1" OR "floor2"] OR stateTagSmaller than 3

Table 7.3.3.17.0-18: Summary on Filter conditions

| Condition Tag | Multiplicity | Targeted Resource Attribute | Matching Condition |
| --- | --- | --- | --- |
| createdBefore | 0..1 | creationTime | creationTime < createdBefore, see clause 7.3.3.17.1 |
| createdAfter | 0..1 |  | createdAfter ≤ creationTime, see clause 7.3.3.17.1 |
| unmodifiedSince | 0..1 | lastModifiedTime | lastModifiedTime < unmodifiedSince, see clause Error: Reference source not found |
| modifiedSince | 0..1 |  | unmodifiedSince ≤ lastModifiedTime, see clause Error: Reference source not found |
| stateTagSmaller | 0..1 | stateTag | stateTag < stateTagSmaller, see clause Error: Reference source not found |
| stateTagBigger | 0..1 |  | stateTagBigger ≤ stateTag, see clause Error: Reference source not found |
| expireBefore | 0..1 | expirationTime | expirationTime < expireBefore, see clause Error: Reference source not found |
| expireAfter | 0..1 |  | expireAfter ≤ expirationTime, see clause Error: Reference source not found |
| labels | 0..1 | labels | see clause |
| childLabels | 0..1 |  | see clause |
| parentLabels | 0..1 |  | see clause |
| resourceType | 0..1 | resourceType | see clause Error: Reference source not found |
| childResourceType | 0..1 |  | see clause Error: Reference source not found |
| parentResourceType | 0..1 |  | see clause Error: Reference source not found |
| sizeBelow | 0..1 | contentSize | contentSize < sizeBelow, see clause Error: Reference source not found |
| sizeAbove | 0..1 |  | sizeAbove ≤ contentSize, see clause Error: Reference source not found |
| typeOfContent | 0..n | contentInfo | matched with typeOfContent component in contentInfo, see clause Error: Reference source not found |
| attribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause Error: Reference source not found |
| childAttribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause Error: Reference source not found |
| parentAttribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause 7.3.3.17.9 |
| limit | 0..1 | (not applicable) | Constraint on maximum number of targeted resources, see clause Error: Reference source not found |
| filterUsage | 0..1 | (not applicable) | Indicator specifying the use case of Filter Criteria parameters |
| semanticsFilter | 0..n | (not applicable) | Filtering conditions expressed in SPARQL [Error: Reference source not found]. These are applicable to the descriptor attribute of <semanticDescriptor> children associated with discoverable resources. When multiple semanticsFilter elements are provided, the matching condition is fulfilled if any of the individual conditions is matched |
| filterOperation | 0..1 | (not applicable) | Indicates the logical operation (AND/OR) to be used for different conditions. The default value is logical AND |
| contentFilterSyntax | 0..1 | (not applicable) | Indicates the Identifier for syntax to be applied for content-based discovery |
| contentFilterQuery | 0..1 | content | The query string shall be specified when contentFilterSyntax parameter is present. See clause Error: Reference source not found for applicable syntax for content-based discovery |
| level | 0..1 | (not applicable) | Constraint on maximum number of levels in the resource tree that retrieve operation shall span, see clause Error: Reference source not found |
| offset | 0..1 | (not applicable) | The number of direct child and descendant resources the Hosting CSE shall skip over and not include within a retrieve response, see clause Error: Reference source not found |
| applyRelativePath | 0..1 | (not applicable) | A resource tree relative path (e.g. ../tempContainer/LATEST) which applies after all the matching conditions have been used (i.e. a matching result has been obtained). See clause 7.2.2.17.17 |
| geometryType | 0..1 | location | see clause 7.3.3.17.X |
| geometry | 0..1 | location |  |
| geoSpatialFunction | 0..1 | location |  |

### ********** End of change 11 **********

### ********** Start of change 12 **********

##### 7.3.3.17.X	Conditions on geo-query

When a geo-query is requested, geometryType, geometry and geoSpatialFunction conditions shall be present together. The geometryType defines the geometry type (e.g. Polygon) of the coordinates in the geometry element. The geoSpatialFunction indicates the function type (e.g. Within) to be performed by the Hosting CSE.

The filter conditions above shall be used to perform geo-query against the location common attribute of a resource (e.g. container). When a geo-location information of the location attribute matches with the geo-query conditions, the Hosting CSE shall return the list of the resources.

### ********** End of change 12 **********

### ********** Start of change 13 **********

## 2.2	Normative references

[x]	IETF RFC 7946: "The GeoJSON Format".

NOTE:	Available at https://tools.ietf.org/html/rfc7946

### ********** End of change 13 **********

### ********** Start of change 14 **********

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

Table 8.2.3-19: Resource attribute short names (1/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| location | CSEBase, remoteCSE, AE, container, flexContainer, timeSeries | loc |

### ********** End of change 14 **********

### ********** Start of change 15 **********

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-20: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| type | location | gty |
| coordinates | location | crd |
| geometryType | filterCriteria | gmty |
| geometry | filterCriteria | geom |
| geoSpatialFunction | filterCriteria | gsf |

### ********** End of change 15 **********

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?