| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | Rakesh Singla C-DOT, rakeshs@cdot.in Aurindam Bhattacharya, aurindam@cdot.in |
| Date:* | 2019-09-16 |
| Reason for Change/s:* | See Introduction |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001-V2_25_0 |
| Clauses * | Table 9.6.20-2: Attributes of <serviceSubscribedNode> resource |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | - |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This change request is for adding a new device identifier for devices which do not have OUI issued from IEEE. This change request shall add PEN based device identifier in onem2m specifications.

-----------------------Start of change 1-------------------------------------------

Presently, onem2m specifies 7 types of device identfiers as mentioned in Table 9.6.20-2: Attributes of <serviceSubscribedNode>, resource of oneM2M TS-0001-v3.16.0. One of the format of the URN is urn:dev:ops:<OUI> "-" <ProductClass> "-" <SerialNumber>.

An organizationally unique identifier (OUI) is a 24-bit number that uniquely identifies a vendor, manufacturer, or other organization.OUIs are purchased from the Institute of Electrical and Electronics Engineers (IEEE) Registration Authority by the assignee (IEEE term for the vendor, manufacturer, or other organization). They are used to uniquely identify a particular piece of equipment through derived identifiers such as MAC addresses,Subnetwork Access Protocol protocol identifiers, World Wide Names for Fibre Channel devices  (Organizationally unique identifier, 2019)

There is one more parameter called PEN(Private Enterprise Number) which also uniquely defines an organization. Private Enterprise Numbers (PENs) are created and maintained by the Internet Assigned Number Authority (IANA) in a public registry, including a publicly revealed email address and "contact name". Any private enterprise (organization) may request, at no cost, a Private Enterprise Number (PEN) to be assigned  (Private Enterprise Number, 2019)

The proposal is to use PEN alongwith OUI in ‘o’ field of ‘ops’ as per section 4 and section 5 of draft-ietf-core-dev-urn-03 (J. Arkko (Ericson), 2019) . The reasons for proposing PEN alongwith OUI in urn:dev:ops device identifier are following:

- For an organization to get OUI from IEEE, some fee needs to be paid whereas PEN is issued by IANA free of cost, so all the companies can easily get PEN number. Hence making device identifier using PEN number shall be useful.

- For constrained devices that do not have MAC address or any other addressing like EUI-48,EUI-64 etc and they do not have IMEI or UUID as well,  but have serial numbers, for such devices it will not be possible to use ‘ops’ as device identifier but if PEN is integrated in ‘ops’ then such devices can also be uniquely identified.

So following change can be incorporated in oneM2M specifications :

Table 9.6.20-2: Attributes of <serviceSubscribedNode> resource

| Attributes of <serviceSubscribedNode> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| deviceIdentifier | 0..1 (L) | WO | A list of device identifiers that uniquely identify a device. The format of a device identifier is one of the following: - Case 1: Identify a device using the format <OUI> "-" <ProductClass> "-" <SerialNumber> as defined in section 3.4.4 of BBF TR-069 [Error: Reference source not found]. The format of the URN is urn:dev:ops:<OUI>"-" <ProductClass> "-" <SerialNumber>. - Case 2: Identify a device using the format <OUI> "-"<SerialNumber> as defined in section 3.4.4 of BBF TR-069 [Error: Reference source not found]. The format of the URN is urn:dev:os:<OUI>"-"<SerialNumber>. - Case 3: Identify a device using an International Mobile Equipment Identifiers of 3GPP TS 23 003 [Error: Reference source not found]. This URN specifies a valid, 15 digit IMEI. The format of the URN is urn:imei:###############. - Case 4: Identify a device using an Electronic Serial Number. The ESN specifies a valid, 8 digit ESN. The format of the URN is urn:esn:########. - Case 5: Identify a device using a Mobile Equipment Identifier. This URN specifies a valid, 14 digit MEID. The format of the URN is urn:meid:##############. - Case 6: Identify a device using an Object IDentifier (OID). This URN specifies a valid OID - see Annex H for one possible naming convention. The format of the URN is urn:oid:<OID>. - Case 7: Identify a device using a Universally Unique IDentifier (UUID). The UUID specifies a valid, hex digit character string as defined in RFC4122 [i.26]. The format of the URN is urn:uuid:########-####-####-############. - Case 8 : Identify a device using the format <PEN> "-" <ProductClass> "-" <SerialNumber> as defined in section 4 of draft-ietf-core-dev-urn-03. The format of the URN is urn:dev:ops:<PEN> "-" <ProductClass> "-" <SerialNumber>. - Case 9 : Identify a device using the format <PEN> "-" <SerialNumber> as defined in section 4 of draft-ietf-core-dev-urn-03. The format of the URN is urn:dev:os:<PEN> "-" <SerialNumber>. - cdot |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?

# References

J. Arkko (Ericson), C. J. (2019, 09 05). draft-ietf-core-dev-urn-03. Retrieved from IETF: https://tools.ietf.org/html/draft-ietf-core-dev-urn-03

Organizationally unique identifier. (2019, 09 05). Retrieved from Wikipedia: https://en.wikipedia.org/wiki/Organizationally_unique_identifier

Private Enterprise Number. (2019, 09 05). Retrieved from Wikipedia: https://en.wikipedia.org/wiki/Private_Enterprise_Number