| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | SDS #42 |
| Source:* | Convida Wireless |
| Date:* | 2019-09-17 |
| Contact:* | Dale Seed, Convida, Seed.Dale@convidawireless.com Lu Liu, Convida, Liu.Lu@convidawireless.com Catalina Mladin, Convida, Mladin.Catalina@convidawireless.com |
| Reason for Change/s:* | Study solutions for dynamic service management |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0080> MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0052 V0.8.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The contribution provides improvements for Solution H: dynamic service management.

- Management objects [serviceInstance] and [serviceComponent] have been consolidated to [serviceInstance], and the corresponding attributes are clarified.

- New attributes are introduced for the [servicePolicy] management object to enable dynamic and automatic management of service instances and to provide support for advanced features.

- A high-level flow diagram is presented to show the fundamental procedures of service management based on the management objects [serviceInstance] and [servicePolicy].

R01: Added more description about these two resources and updated the call flow.

- -----------------------Start of change 1 -------------------------------------

  - Solution H: Dynamic Service Management

    - Solution Applicability

This solution applies to Key Issue 6: “Dynamic Service Instantiation”

    - Solution Description

      - Impacted Resources and Attributes

        - Overview

This solution uses the M2M Service concepts introduced in solution C, including the definition and composition of an M2M service, csType and csfType, associated IDs, etc.

To implement this solution, several resources need to be modified, and some new resource types need to be defined These are further detailed in the following clauses.

        - New <mgmtObj> specialization type: [serviceInstance]

A new [serviceInstance] specialization of the <mgmtObj> resource type is proposed for management of different types of service instances, such as M2M services (e.g. combinations of AEs and CSs), CSFs, CSEs, CSs, or AEs.  It is up to implementation if a set of CSs is managed as CSF, CSE or M2M service. [serviceInstance] management object can be used by an originator AE or CSE to install/uninstall/activate/deactivate an M2M Services on an Edge/Fog Node.

Editor’s Note. It is currently FFS where the [serviceInstance] object should be instantiated.

The [serviceInstance] resource contains the attributes specified in table below.

Table 9.8.2.1.2-1: Attributes of [serviceInstance] resource

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| universal and common attributes | See clause 9.6.1.3 in TS-0001 [Error: Reference source not found] | See clause 9.6.1.3 in TS-0001 [Error: Reference source not found] | See clause 9.6.1.3 in TS-0001 [Error: Reference source not found] |
| mgmtDefinition | 1 | WO | See clause 9.6.15 in TS-0001 [Error: Reference source not found]. This attribute has a fixed value "serviceInstance" to indicate the resource is for service entity management. |
| instanceType | 1 | RW | Choice of “CSE”, “Service”, “CSF”, “CS”, “AE”, etc. It is up to implementation if a set of CSs is managed as CSF, CSE or M2M Service. |
| instanceID | 0..1 | RW | The ID of the instance, may be a CSE-ID, Serv-ID, CSF-ID, CS-ID, or AE-ID. This attribute is a specialization of [objectAttribute] attribute. Editor’s Note. The format of a Serv-ID associated with a M2M service is FFS and to be addressed in Solution C. |
| install | 1 | RW | The action that installs/instantiates the service instance in a single operation. The action is triggered by assigning value "TRUE" to this attribute. If this service instance is linked to a [software] object via softwareLink, the Hosting CSE links this attribute to the install attribute of [software]. |
| uninstall | 1 | RW | The action that uninstalls an instance. The action is triggered by assigning value "TRUE" to this attribute. If this service instance is linked to a [software] object via softwareLink, the Hosting CSE links this attribute to the uninstall attribute of [software]. |
| activate | 0..1 | RW | The action that activates/enables an instance that has been installed. The action is triggered by assigning value "TRUE" to this attribute. If this service instance is linked to a [software] object via softwareLink, the Hosting CSE links this attribute to the activate attribute of [software]. |
| deactivate | 0..1 | RW | The action that deactivates/disables an instance that has been installed. The action is triggered by assigning value "TRUE" to this attribute. If this service instance is linked to a [software] object via softwareLink, the Hosting CSE links this attribute to the “deactivate” action of [software]. |
| instanceStatus | 1 | RW | Indicates the status of the instance. If this service instance is linked to a [software] object via softwareLink, the Hosting CSE keeps the value of this attribute consistent with the value of the installStatus and activeStatus attributes of [software]. Editor’s Note. The serviceInstance status (e.g. “installed”, “activated”) need to be harmonized with Device Management procedures and TS-0004 clauses D.2, D.3. |
| resetInstance | 0..1 | RW | The action that resets an instance that has been instantiated. The action is triggered by assigning value "TRUE" to this attribute. |
| rebootInstance | 0..1 | RW | The action that reboots an instance that has been instantiated. The action is triggered by assigning value "TRUE" to this attribute. |
| managementAtomicityIndicator | 1 | RW | Indicates if the operations for multiple component instances are to be treated atomically or not. The indicator applies to install, uninstall, activate and deactivate operations. If the indicator specifies that the operation atomicity is TRUE, the Hosting CSE will update the state of the service only when all the components achieve the new state. If a component operation is not successful it is rolled back, using install/uninstall and activate/deactivate operations as rollback pairs. Other atomicity behaviour (e.g. number of tries, timeouts) are implementation or local policy dependent. If the indicator specifies that the operation atomicity is FALSE, the Hosting CSE will manage the state of the service based on component states and local policies. Editor’s Note. The non-atomic management of service instance is FFS. |
| rebootLink | 0..1 | RW | Link to the [reboot] object used to reboot the instance. This attribute is a specialization of mgmtLink attribute. If the resetInstance attribute exists, the attribute rebootLink is either not present or ignored. |
| servicePolicyLink | 0..1 | RW | The resourceID of the [servicePolicy] object used for the instance. If there are no service policies provided, local policies apply. |
| instanceProvisioningLink | 0..1(L) | RW | Link(s) to management objects providing provisioning information for the instance, e.g. [MAFClientRegCfg], [MEFClientRegCfg]. Editor’s Note. The management objects providing provisioning information are FFS. For example, new security-related provisioning objects may provide security identifiers, credentials, lifetimes, default ACPs, etc. |
| componentLink | 0..1(L) | RW | This attribute contains reference to a list of [serviceInstance](s) which are the components of this service instance. For example, in Figure 9.3.2-2, Service S may have components: CSE 123, CSE 456, AE 1, AE N. CSE 123 may have components: CS A1, CS Ak. |

- resource

|  | Error: Reference source not found | Error: Reference source not found | Error: Reference source not found |
| --- | --- | --- | --- |
|  |  |  | Error: Reference source not found |

        - New <mgmtObj> specialization type: [servicePolicy]

The [servicePolicy] management object specialization provides policies and rules for the dynamic management of a service instance. [servicePolicy] management object can be used by Hosting CSE on behalf of AE based on certain criteria or external command to install/uninstall/activate/deactivate an M2M Service on an Edge/Fog Node. .

Table 9.8.2.1.4-3: Attributes of [servicePolicy]

| Attributes | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| instanceLink | 0..1 | RW | Link to the [serviceInstance] management object that is associated with this policy. |
| servicePolicyEnable | 1 | RW | Indicates whether this policy is enabled or not. For example, the dynamic management operations defined by this policy can be temporarily disabled to limit the impact on regular operations. |
| eligibleNodeType | 0..1 | RW | Indicates the type of node that is eligible to host the service, e.g., MN, IN, ASN, ADN. |
| serviceScope | 0..1 | RW | Indicates the scope/service domain where the service may be instantiated and managed, such as a M2M Service Domain or a list of nodes defined by their M2M-Node-IDs. |
| managementProtocol | 0..1(L) | RW | Indicates alternative device management protocols preferred to be supported at the node where the service is instantiated, e.g. OMA DM protocol. |
| underlyingNetworkProtocol | 0..1(L) | RW | Indicates the preference for the underlying network protocols to be supported by the node where the service is instantiated. |
| serviceTarget | 0..1(L) | RW | Specifies a list of targets that the service management is to be performed upon (e.g. one or more M2M-Node-IDs, CSE-IDs, CSF-IDs, CS-IDs, AE-IDs). |
| serviceManagementOperation | 0..1 | RW | Defines the management operations that the Hosting CSE is to perform, such as install, uninstall, enable, disable. |
| serviceManagementTrigger | 0..1 | RW | Defines the conditions/events that the Hosting CSE can monitor and detect if/when to perform a specified management operation. |
| serviceManagementSchedule | 0..1 | RW | Defines a schedule for when the Hosting CSE is to perform a specified management operation. |
| serviceManagementCommand | 0..1 | RW | Used by an originator to manually trigger the Hosting CSE to perform the specified management operation. |
| serviceManagementStatus | 0..1 | RW | Used by the Hosting CSE to reflect the status of management operations that it performs. |
| servicePolicyLink | 0..1(L) | RW | This attribute contains reference to a list of other <servicePolicy> resources in case a hierarchy of <servicePolicy> is needed. For example, to manage a set of services which are related and/or dependent on one another. |

      - Impacted Information Flows

        - Management Procedures

The following figure shows the high-level flow for service management on an Edge/Fog Node. Assume the management is performed by a service management function in an IN-CSE.

NOTE – Enhancing the existing Application and Service Layer Management (ASM) CSF to function as the “service management function” is FFS

NOTE - The procedures for managing a service on multiple nodes are FFS

Step 1: An Originator may send a discovery request to a Service Management Function of a Hosting CSE to discover an existing [servicePolicy] mgmtObj.

Step 2a: If an Originator successfully discovers existing [servicePolicy] it can send a request to UPDATE an existing [servicePolicy].

Step 2b: If discovery fails, then Originator can CREATE a new [servicePolicy] to instantiate a M2M service on an Edge/Fog Node (e.g. MN, ASN or ADN).

Step 3: According to the policies defined in the [servicePolicy], the Hosting CSE’s Service Management Function creates one or more [serviceInstance] mgmtObj at the target node where the service will be hosted. The link to the [serviceInstance] are added to the serviceLink attribute of the [servicePolicy] mgmtObj.

Step 4: When service management is triggered (e.g. as specified by the serviceManagementTrigger or serviceManagementCommand attribute), the following management operations are performed by the Hosting CSE’s Service Management Function.

- The Hosting CSE configures the install/uninstall/activate/deactivate/reset attributes of the [serviceInstance] corresponding to the Service.

- If the [serviceInstance] is linked to a [software] mgmtObj, the install/uninstall/activate/deactivate attributes of the [software] mgmtObj will be used by the Hosting CSE when performing the corresponding operation.

- The Hosting CSE manages the state of the Service Instance based on the atomicity indicator and the states of the component instances.

Note: If instanceType is “AE” or “CSE”, an AE or CSE registration will be performed when the AE/CSE instance is activated.

- ----------------------- End of change 1 (new text) -------------------------------------