| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#42 |
| Source:* | Yongjing Zhang, Huawei (zhangyongjing@huawei.com) |
| Date:* | 2019-09-27 |
| Reason for Change/s:* | To provide CoAP status code and header mapping for ontology mapping. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0053> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0008 V3.4.0 (shall be applied to R4 baseline which is not available yet) |
| Clauses * | 6.2.4, 6.2.2.4 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR provide CoAP status code and header mapping for ontology mapping.

### -----------------------Start of change 1-------------------------------------------

### 6.2.4	Response Codes Mapping

Table 6.2.4-1 defines a mapping between oneM2M Response Status Code parameter specified in [Error: Reference source not found] and CoAP Response Code.

In case of where multiple oneM2M Response Status Code parameters are mapped to a single CoAP Response Code, the Response Status Code parameter shall be specified in oneM2M-RSC Option.

Table 6.2.4-1: Mapping between oneM2M Response Status Code and CoAP Response Code

| oneM2M Response Status Code | Description | CoAP Response Code | Description |
| --- | --- | --- | --- |
| 1000 | ACCEPTED | None | Not used |
| 1001 | ACCEPTED for nonBlockingRequestSynch | 2.01 | Created (indicates that a <request> resource has been created |
| 1002 | ACCEPTED for nonBlockingRequestAsynch | 2.01 or 2.04 | 2.01 (Created) is used if a <request> resource was created, otherwise 2.04 (Changed) is used |
| 2000 (for RETRIEVE operation) | OK | 2.05 | Content |
| 2000 (for NOTIFY operation) | OK | 2.04 | Changed. CoAP payload shall be empty. |
| 2001 | CREATED | 2.01 | Created |
| 2002 | DELETED | 2.02 | Deleted |
| 2004 | UPDATED | 2.04 | Changed |
| 4000 | BAD_REQUEST | 4.00 | Bad Request |
| 4001 | RELEASE_VERSION_NOT_SUPPORTED | 4.00 | Bad Request |
| 4004 | NOT_FOUND | 4.04 | Not Found |
| 4005 | OPERATION_NOT_ALLOWED | 4.05 | Method Not Allowed |
| 4008 | REQUEST_TIMEOUT | 4.04 | Not Found |
| 4015 | UNSUPPORTED_MEDIA_TYPE | 4.15 | Unsupported Content-Format |
| 4101 | SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 4102 | CONTENTS_UNACCEPTABLE | 4.00 | Bad Request |
| 4103 | ORIGINATOR_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 4104 | GROUP_REQUEST_IDENTIFIER_EXISTS | 4.00 | Bad Request |
| 4105 | CONFLICT | 4.03 | Forbidden |
| 4106 | ORIGINATOR_HAS_NOT_REGISTERED | 4.03 | Forbidden |
| 4107 | SECURITY_ASSOCIATION_REQUIRED | 4.03 | Forbidden |
| 4108 | INVALID_CHILD_RESOURCE_TYPE | 4.03 | Forbidden |
| 4109 | NO_MEMBERS | 4.03 | Forbidden |
| 4110 | GROUP_MEMBER_TYPE_INCONSISTENT | 4.00 | Bad Request |
| 4111 | ESPRIM_UNSUPPORTED_OPTION | 4.03 | Forbidden |
| 4112 | ESPRIM_UNKNOWN_KEY_ID | 4.03 | Forbidden |
| 4113 | ESPRIM_UNKNOWN_ORIG_RAND_ID | 4.03 | Forbidden |
| 4114 | ESPRIM_UNKNOWN_RECV_RAND_ID | 4.03 | Forbidden |
| 4115 | ESPRIM_BAD_MAC | 4.03 | Forbidden |
| 4116 | ESPRIM_IMPERSONATION_ERROR | 4.03 | Forbidden |
| 4117 | ORIGINATOR_HAS_ALREADY_REGISTERED | 4.03 | Forbidden |
| 4118 | ONTOLOGY_NOT_AVAILABLE | 4.04 | Not Found |
| 4119 | LINKED_SEMANTICS_NOT_AVAILABLE | 4.04 | Not Found |
| 4120 | INVALID_SEMANTICS | 4.02 | Bad Option |
| 4121 | MASHUP_MEMBER_NOT_FOUND | 4.04 | Not Found |
| 4122 | INVALID_TRIGGER_PURPOSE | 4.02 | Bad Option |
| 4123 | ILLEGAL_TRANSACTION_STATE_TRANSITION_ATTEMPTED | 4.00 | Bad Request |
| 4124 | BLOCKING_SUBSCRIPTION_ALREADY_EXISTS | 4.00 | Bad Request |
| 4125 | SPECIALIZATION_SCHEMA_NOT_FOUND | 4.00 | Bad Request |
| 5000 | INTERNAL_SERVER_ERROR | 5.00 | Internal Server Error |
| 5001 | NOT_IMPLEMENTED | 5.01 | Not Implemented |
| 5103 | TARGET_NOT_REACHABLE | 4.04 | Not Found |
| 5105 | RECEIVER_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 5106 | ALREADY_EXISTS | 4.00 | Bad Request |
| 5203 | TARGET_NOT_ SUBSCRIBABLE | 4.03 | Forbidden |
| 5204 | SUBSCRIPTION_VERIFICATION_INITIATION_FAILED | 5.00 | Internal Server Error |
| 5205 | SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE | 4.03 | Forbidden |
| 5206 | NON_BLOCKING_SYNCH_REQUEST_NOT_SUPPORTED | 5.01 | Not Implemented |
| 5207 | NOT_ACCEPTABLE | 4.06 | Not Acceptable |
| 5208 | DISCOVERY_DENIED_BY_IPE | 4.03 | Forbidden |
| 5209 | GROUP_MEMBERS_NOT_RESPONDED | 5.00 | Internal Server Error |
| 5210 | ESPRIM_DECRYPTION_ERROR | 5.00 | Internal Server Error |
| 5211 | ESPRIM_ENCRYPTION_ERROR | 5.00 | Internal Server Error |
| 5212 | SPARQL_UPDATE_ERROR | 5.00 | Internal Server Error |
| 5213 | MASH_UP_OPERATION_FAILED | 5.00 | Internal Server Error |
| 5214 | TARGET_HAS_NO_SESSION_CAPABILITY | 4.03 | Forbidden |
| 5215 | SESSION_IS_ONLINE | 4.03 | Forbidden |
| 5216 | JOIN_MULTICAST_GROUP_FAILED | 5.00 | Internal Server Error |
| 5217 | LEAVE_MULTICAST_GROUP_FAILED | 5.00 | Internal Server Error |
| 5218 | TRIGGERING_DISABLED_FOR_RECIPIENT | 5.03 | Service Unavailable |
| 5219 | UNABLE_TO_REPLACE_ REQUEST | 5.03 | Service Unavailable |
| 5220 | UNABLE_TO_RECALL_REQUEST | 5.03 | Service Unavailable |
| 5221 | CROSS_RESOURCE_OPERATION_FAILURE | 5.00 | Internal Server Error |
| 6003 | EXTERNAL_OBJECT_NOT_REACHABLE | 4.04 | Not Found |
| 6005 | EXTERNAL_OBJECT_NOT_FOUND | 4.04 | Not Found |
| 6010 | MAX_NUMBER_OF_MEMBER_EXCEEDED | 4.00 | Bad Request |
| 6020 | MGMT_SESSION_CANNOT_BE_ESTABLISHED | 5.00 | Internal Server Error |
| 6021 | MGMT_SESSION_ESTABLISHMENT_TIMEOUT | 5.00 | Internal Server Error |
| 6022 | INVALID_CMDTYPE | 4.00 | Bad Request |
| 6023 | INVALID_ARGUMENTS | 4.00 | Bad Request |
| 6024 | INSUFFICIENT_ARGUMENTS | 4.00 | Bad Request |
| 6025 | MGMT_CONVERSION_ERROR | 5.00 | Internal Server Error |
| 6026 | MGMT_CANCELLATION_FAILED | 5.00 | Internal Server Error |
| 6028 | ALREADY_COMPLETE | 4.00 | Bad Request |
| 6029 | MGMT_COMMAND_NOT_CANCELLABLE | 4.00 | Bad Request |
| 6030 | EXTERNAL_OBJECT_NOT_REACHABLE_BEFORE_RQET_TIMEOUT | 5.04 | Gateway Timeout |
| 6031 | EXTERNAL_OBJECT_NOT_REACHABLE_BEFORE_OET_TIMEOUT | 5.04 | Gateway Timeout |
|  | ONTOLOGY_MAPPING_ALGORITHM_FAILED | 5.00 | Internal Server Error |
|  | ONTOLOGY_CONVERSION_FAILED | 5.00 | Internal Server Error |
|  | ONTOLOGY_MAPPING_ALGORITHM_NOT_AVAILABLE | 4.04 | Not Found |
|  | ONTOLOGY_MAPPING_NOT_AVAILABLE | 4.04 | Not Found |
|  | ONTOLOGY_MAPPING_POLICY_NOT_MATCH | 4.03 | Forbidden |

The Receiver shall use this table to determine the CoAP response code that is to be used in the response, based on the value of the oneM2M Response Status Code parameter.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### 6.2.2.4	Definition of New Options

##### 6.2.2.4.0	Introduction

This clause describes new CoAP Options used for binding several oneM2M request/response parameters. Table 6.2.2.4.0-1 contains definitions of the new CoAP Options and sub-clauses specify oneM2M parameter mapping with the newly defined CoAP Options in the table 6.2.2.4.0-1.

Table 6.2.2.4.0-1: Definition of New Options

| No | C | U | N | R | Name | Format | Length | Default |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  |  |  | oneM2M-FR | string | 0-255 | (None) |
| 257 |  |  |  |  | oneM2M-RQI | string | 0-255 | (None) |
| 259 |  |  |  |  | oneM2M-OT | string | 15 | (None) |
| 260 |  |  |  |  | oneM2M-RQET | string | 15 | (None) |
| 261 |  |  |  |  | oneM2M-RSET | string | 15 | (None) |
| 262 |  |  |  |  | oneM2M-OET | string | 15 | (None) |
| 263 |  |  |  |  | oneM2M-RTURI | string | 0-255 | (None) |
| 264 |  |  |  |  | oneM2M-EC | uint | 1 | (None) |
| 265 |  |  |  |  | oneM2M-RSC | uint | 2 | (None) |
| 266 |  |  |  |  | oneM2M-GID | string | 0-255 | (None) |
| 267 |  |  |  |  | oneM2M-TY | uint | 2 | (None) |
| 268 |  |  |  |  | oneM2M-CTO | uint | 2 | (None) |
| 269 |  |  |  |  | oneM2M-CTS | uint | 2 | (None) |
| 270 |  |  |  |  | oneM2M-ATI | string | 0-255 | (None) |
| 271 |  |  |  |  | oneM2M-RVI | string | 1 | (None) |
| 272 |  |  |  |  | oneM2M-VSI | string | 0-255 | (None) |
| 273 |  |  |  |  | oneM2M-GTM | string | 0-512 | (None) |
| 274 |  |  |  |  | oneM2M-AUS | string | 0-255 | (None) |
| 275 |  |  |  |  | oneM2M-ASRI | string | 0-255 | (None) |
|  |  |  |  |  | oneM2M-OMR | string | 0-255 | (None) |
| NOTE 1: C, U, N, R means Critical, Unsafe, NoCacheKey and Repeatable respectively [Error: Reference source not found]. Table 6.2.2.4.0-1 follows the template used in clause 5.10 Option Definitions of CoAP specification [Error: Reference source not found]. NOTE 2: CoAP Option numbers specified in table 6.2.2.4.0-1 are subject to change after review by IANA registration. |  |  |  |  |  |  |  |  |

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

##### 6.2.2.4.x	Ontology Mapping Resources

The Ontology Mapping Resources parameter shall be mapped to the oneM2M-OMR Option. The format of the oneM2M-OMR option shall be represented as a sequence of oneM2M resource identifiers separated by a '+'.

EXAMPLE:	The option looks as follows:

oneM2M-OMR: /IN-CSE-0001/omr1+/IN-CSE-0001/omr2+…

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

##### 6.2.2.4.15	Assigned Token Identifiers

The Assigned Token Identifiers parameter shall be mapped to the oneM2M-ATI Option. The format of the oneM2M-ATI option shall be represented as a sequence of lti-value:tkid-value pairs separated by a colon ':' and multiple pairs separated by '+' character.

EXAMPLE:	The header looks as follows:

oneM2M-ATI: lti-value1:tkid-value1+lti-value2:tkid-value2+…

if the XML representation of the Assigned Token Identifiers parameter is given as (using short element names):

<ati>

<ltia>

<lti>lti-value1</lti>

<tkid>tkid-value1</tkid>

</ltia>

<ltia>

<lti>lti-value2</lti>

<tkid>tkid-value2</tkid>

</ltia>

…

</ati>

The data type m2m:dynAuthlocalTokenIdAssignments of the Assigned Token Identifiers parameter is defined in clause 6.3.5.43 of oneM2M TS-0004 [Error: Reference source not found].

### -----------------------End of change 4-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?