| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 43 |
| Source:* | Cyrille Bareau, Orange, cyrille.bareau@orange.com Leila Le Brun, Orange, leila.lebrun@orange.com Marianne Mohali, Orange, marianne.mohali@orange.com Bob Flynn, Chordant, Bob.Flynn@chordant.io Andreas Kraft, Deutsche Telekom, Andreas.Kraft@t-systems.com |
| Date:* | 2019-11-21 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0084 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v.4.3.0 |
| Clauses * | 9.6.1.1, 9.6.27, 9.6.28, 9.6.35, 10.2.4.1, 10.2.4.15, 10.2.4.16, 10.2.4.17, 10.2.4.18, 10.2.4.19 New clauses 9.6.xx, 10.2.4.ya, 10.2.4.yb,10.2.4.yc,10.2.4.yd |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 as impact of the flexContainerInstance introduction. |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The resource flexContainer is used to model devices defined in TS-0023. The flexContainer can also be used to model the data generated by the devices, in the case of an AE that provides an Interworking Proxy Entity to this device (or a native oneM2M device). However, it is not possible to maintain the history of the data in the flexContainer when updates occur (or this can be done using a child timeSeries resource, but in this case we lose the facility of exploiting custom attributes, i.e. fixed, specialized attributes for a given specialization of a flexContainer).

The need for this was discussed in RDM and agreed with the action to bring relevant contributions to TS-0001 and TS-0004.

This is particularly important in the case of devices with low connectivity that upload a bunch of measures at regular intervals: currently the flexContainer can only retain one of the measures and loses all the others

This CR hence proposes to introduce a new resource, flexContainerInstance, child of flexContainer, that stores a copy of the flexContainer when it is updated. A <flexContainerInstance> cannot be created nor updated via the Mca/Mcc interface. Only the <flexContainer> Hosting CSE can create a <flexContainerInstance>. However a <flexContainerInstance> can be retrieved and deleted. Virtual <latest> and <oldest> resources also are created for the <flexContainer> resource; they return the latest / oldest <flexContainerInstance> child resource of a <flexContainer> resource.

### ********************* Start of Change  1 ******************************************

#### 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- flexContainerInstance;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, pollingChannel, semanticDescriptor, timeSeries, transaction, transactionMgmt, triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, locationPolicy, action, multimediaSession, dynamicAuthorizationConsultation, nwMonitoringReq, timeSyncBeacon | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor, transaction | Container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, flexContainerInstance, latest, oldest,subscription, semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.35 |
| flexContainerInstance | Represents a timestamped Data instance in the <flexContainer> resource | transaction, semanticDescriptor | flexContainer, flexContainerAnnc | 9.6.xx |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, semanticRuleRepository, flexContainer, timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action, m2mServiceSubscriptionProfile, dynamicAuthorizationConsultation, localMulticastGroup, serviceSubscribedAppRule, authorizationPolicy, authorizationDecision, authorizationInformation, ontologyRepository, nwMonitoringReq, timeSyncBeacon | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription, transaction | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription, transaction | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription, transaction | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource | None specified | container, timeSeries, or flexContainer | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription, transaction | CSEBase, AE | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription, transaction | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, transaction, semanticDescriptor | node, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription, transaction, serviceSubscribedUserProfile | CSEBase | 9.6.19 |
| node | Represents specific Node information | mgmtObj, subscription, semanticDescriptor, schedule, transaction, action | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription, transaction | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules, transaction | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource | None specified | container, timeSeries, or flexContainer | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription, transaction | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, contentInstanceAnnc flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, timeSeries, timeSeriesAnnc, timeSeriesInstanceAnnc, mgmtObjAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc, semanticMashupInstance, semanticMashupInstanceAnnc, action, dynamicAuthorizationConsultation, dynamicAuthorizationConsultationAnnc, nwMonitoringReq, timeSyncBeacon | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription, transaction | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription, transaction | subscription, CSEBase, - node | 9.6.9 |
| serviceSubscribedNode | Node information | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription, transaction | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription, transaction | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribed-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc, nwMonitoringReq, semanticRuleRepository, reasoningRules, reasoningJobInstance, timeSyncBeacon | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription, transaction | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources. | subscription, transaction | AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource |  | group | 9.6.14a |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities. | timeSeriesInstance, subscription, semanticDescriptor, latest, oldest, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, container, containerAnnc, flexContainer, flexContainerAnnc | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | transaction | timeSeries, timeSeriesAnnc | 9.6.37 |
| role | Represents a role that is assigned to an AE or CSE. | subscription, transaction | authorizationInformation | 9.6.38 |
| token | Used for storing a token that is issued to an AE or CSE. | subscription, transaction | authorizationInformation | 9.6.39 |
| authorizationDecision | Represents an access control decision point | subscription, transaction | CSEBase | 9.6.41 |
| authorizationPolicy | Represents an access control policy retrieval point | subscription, transaction | CSEBase | 9.6.42 |
| authorizationInformation | Represents an access control information point | role token subscription, transaction | CSEBase | 9.6.43 |
| localMulticastGroup | Stores local multicast group information of member hosting CSE. | transaction | CSEBase | 9.6.44 |
| AEContactList | Contains information about a CSE that has resources that referencing an AE-ID | AEContactListPerCSE, subscription, transaction | CSEBase | 9.6.45 |
| AEContactListPerCSE | Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes | None specified | AEContactList | 9.6.46 |
| transactionMgmt |  | subscription | CSEBase, AE, remoteCSE | 9.6.47 |
| transaction |  | action | All non-virtual resource types with the exception of the following: request, delivery, pollingChannel, transactionMgmt, transaction, timeSyncBeacon | 9.6.48 |
| triggerRequest | Used by an AE to initiate, replace or recall a device trigger request | subscription | AE | 9.6.49 |
| ontologyRepository | Represents the collection of the managed ontologies and the semantic validation service | ontology, semanticValidation, subscription | CSEBase | 9.6.50 |
| ontology | Store the representation of an ontology | subscription | ontologyRepository | 9.6.51 |
| semanticValidation | A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology. | None specified | ontologyRepository | 9.6.52 |
| semanticMashupJobProfile | Represents the profile and description of a semantic mashup service | semanticMashupInstance, semanticDescriptor, subscription | CSEBase, remoteCSE | 9.6.53 |
| semanitcMashupInstance | Represents a semantic mashup instance | semanticMashupResult, semanticDescriptor, mashup, subscription | semanticMashupJobProfile, AE, remoteCSE, CSEBase | 9.6.54 |
| mashup | A virtual resource use to trigger the calculation and generation of new mashup result | Not specified | semanticMashupInstance | 9.6.55 |
| semanticMashupResult | Represent semantic mashup results | semanticDescriptor, subscription | semanticMashupInstance | 9.6.56 |
| multimediaSession | Stores a representation of a multimedia session information requested by a registering AE | subscription | AE | 9.6.57 |
| crossResourceSubscription | represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | CSEBase, remoteCSE, AE | 9.6.58 |
| backgroundDataTransfer | Stores information for a background data transfer request | None specified | AE, remoteCSE, CSEBase | 9.6.60 |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | dependency | CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction | 9.6.61 |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | None specified | action | 9.6.62 |
| e2eQosSession | Specifies the end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities. | subscription | CSEBase, remoteCSE, AE | 9.6.63 |
| nwMonitoringReq | Specifiesed the request that an AE retrieves the Underlying Network information. The resource provides the characteristics of the Underlying Network status in a particular geographic area such as congestion status and number of devices. | subscription | CSEBase, remoteCSE, AE | 9.6.64 |
| semanticRuleRepository | Specifies one or multiple <reasoningRules> child resources to represent different sets of reasoning rules in the oneM2M system | reasoningRules, reasoningJobInstance | CSEBase | 9.6.65 |
| reasoningRules | Store a set of related reasoning rules | subscription | semanticRuleRepository | 9.6.66 |
| reasoningJobInstance | Represents a specific reasoning job instance for enabling one-time or continuous reasoning operations. | subscription | semanticRuleRepository | 9.6.67 |
| serviceSubscribedUserProfile | Stores user profile information for a given M2M Service User | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.68 |
| timeSyncBeacon | Specifies criteria that a Hosting CSE uses to generate time synchronization beacon notifications | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.69 |
| NOTE: See clause 9.6.12 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### ********************* End of Change  1 ******************************************

### **********************  Start of change 2   **********************

### 9.6.27	Resource Type latest

The <latest> resource is a virtual resource because it does not have a representation. It is the child resource of a <container>,  <timeSeries>or <flexContainer> resource. When a request addresses the <latest> resource, the Hosting CSE shall apply the request to the latest <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> resource among all existing <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> resources of the <container> or <timeSeries> or <flexContainer> resource.

The <latest> resource inherits access control policies that apply to the parent resource.

### 9.6.28	Resource Type oldest

The <oldest> resource is a virtual resource because it does not have a representation. It is the child resource of a <container>, <timeSeries> or <flexContainer> resource. When a request addresses the <oldest> resource, the Hosting CSE shall apply the request to the oldest <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> resource among all existing <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> resources of the <container>, <timeSeries> or <flexContainer> resource.

The <oldest> resource inherits access control policies that apply to the parent resource.

### ********************* End of Change  2 ******************************************

### **********************  Start of change 3   **********************

### 9.6.35	Resource Type flexContainer

The <flexContainer> resource type is a customizable container for data instances. It is a template for the definition of flexible specializations of data containers. Like a <container> resource, specializations of this <flexContainer> resource type are used to share information with other entities and potentially to track the data. While the <container> resources includes data to be made accessible to oneM2M entities inside <contentInstance> children, a specialization of the <flexContainer> resource includes associated content directly inside the <flexContainer> by means of one or more [customAttribute] attribute(s). The attribute name and attribute data type of [customAttribute] attributes are defined explicitly for each specialization of <flexContainer>, i.e. the specific set of attribute name and type are defined in a corresponding XSD-file.

Example usage of <flexContainer>: As a specialization of <flexContainer> that includes two [customAttribute] attributes, named "temperature"(xs:float type) and "humidity"(xs:positiveInteger type) can be specified in some TS. The actual data types of [customAttribute] will be described both in the specification document or XSD file which are referred by the value of containerDefinition attribute.

If a <flexContainer> resource is created with at least one of the attributes maxNrOfInstances, maxByteSize or maxInstanceAge, then the hosting CSE shall automatically create a copy of the <flexContainer> resource in the form of a <flexContainerInstance> resource, child of the current <flexContainer> resource; and each time a <flexContainer> custom attribute is modified, a new <flexContainerInstance> child resource shall be added, which contains a copy of the <flexContainer> after update. In this case, virtual resources <latest> and <oldest> shall be available.

The <flexContainer> resource shall contain the child resource specified in table 9.6.35-1.

Table 9.6.35-1: Child resources of <flexContainer> resource

| Child Resources of <flexContainer> | Child Resource Type | Multiplicity | Description | <flexContainerAnnc> Child Resource Type |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <flexContainer> | 0..n | <flexContainer> resource can include any of its specializations as child resource | <flexContainer> <flexContainerAnnc> |
| [variable] | <flexContainerInstance> | 0..n | Timestamped copies of the <flexContainer> resource. | <flexContainerInstance> <flexContainerInstanceAnnc> |
| la | <latest> | 0..1 | See clause 9.6.27 | None |
| ol | <oldest> | 0..1 | See clause 9.6.28 | None |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries>, <timeSeriesAnnc> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <action> | 0..n | See clause 9.6.61 | None |

The <flexContainer> resource shall contain the attributes specified in table 9.6.35-2.

Table 9.6.35-2: Attributes of <flexContainer> resource

| Attributes of <flexContainer> | Multiplicity | RW/ RO/ WO | Description | <flexContainerAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 0..1 (note) | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 0..1 (note) | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 0..1 (note) | RO | See clause 9.6.1.3. | NA |
| stateTag | 1 | RO | See clause 9.6.1.3. This stateTag attribute value shall be incremented when a custom attribute of the flexContainer is modified. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| location | 0..1 | RW | See clause 9.6.1.3. | OA |
| maxNrOfInstances | 0..1 | RW | Maximum number of direct child <flexContainerInstance> resources in the <flexContainer> resource. | OA |
| maxByteSize | 0..1 | RW | Maximum size in bytes of custom attributes that is allocated for the <flexContainer> resource for all direct child <flexContainerInstance> resources. | OA |
| maxInstanceAge | 0..1 | RW | Maximum age of a direct child <flexContainerInstance> resource in the <flexContainer> resource. The value is expressed in seconds. | OA |
| currentNrOfInstances | 0..1 | RO | Current number of direct child <flexContainerInstance> resource in the <flexContainer> resource. It is limited by the maxNrOfInstances. The currentNrOfInstances attribute of the <flexContainer> resource shall be updated on successful creation or deletion of direct child <flexContainerInstance> resource of <flexContainer> resource. | OA |
| currentByteSize | 0..1 | RO | Current size in bytes of custom attributes stored in all direct child <flexContainerInstance> resources of the <flexContainer> resource. It is limited by the maxByteSize. The currentByteSize attribute of the <flexContainer> resource shall be updated on successful creation or deletion of direct child <flexContainerInstance> resource of <flexContainer> resource. | OA |
| containerDefinition | 1 | WO | This contains an identifier reference (URI) to the <flexContainer> schema definition which shall be used by the CSE to validate the syntax of the <flexContainer> resource. This URI may refer to one of the oneM2M <flexContainer> definitions specified in the following documents: - Generic Interworking [Error: Reference source not found]] - AllJoyn Interworking [7]; - Home Domain Information Model [8] A list of oneM2M <flexContainer> definitions is also provided in clause 9.6.1.2.2 [3]. Other URI for other <flexContainer> definitions may be specified. | MA |
| ontologyRef | 0..1 | RW | A reference (URI) of the ontology used to represent the information that is stored in the present <flexContainer> resource. | OA |
| contentSize | 1 | RO | Sum of the size in bytes of all of the custom attributes. | NA |
| nodeLink | 0..1 | RW | The resource identifier of a <node> resource that stores the node specific information of the NoDN on which the interworked service represented by this <flexContainer> resource resides. | OA |
| [customAttribute] | 0..n | RW | Specialization-specific attribute(s). Name and data type defined in each specialization of <flexContainer> resource. | OA |
| NOTE: When an instance of <flexContainer> is a child of a <flexContainer> resource, these attributes can be optional. Their presence is determined by the respective definition referred to by the containerDefinition attribute. |  |  |  |  |

### ********************* End of Change 3 ******************************************

### ********************* Start of Change 4 ******************************************

### 9.6.xx	Resource Type flexContainerInstance

The <flexContainerInstance> resource represents a data instance in the <flexContainer> resource. The <flexContainerInstance> resource shall be created by the Hosting CSE when the parent <flexContainer> is created and modified and when at least one of policy attributes (maxNrOfInstances, maxByteSize or maxInstanceAge) is present, and shall not be modified once created. An AE shall be able to delete a <flexContainerInstance> resource explicitly or it may be deleted by the Hosting CSE. The <flexContainerInstance> resource inherits the same access control policies of the parent <flexContainer> resource, and does not have its own accessControlPolicyIDs attribute.

> resource

The <flexContainerInstance> resource shall contain the attributes specified in table 9.6.xx-2.

Table 9.6.xx-2: Attributes of <flexContainerInstance> resource

| Attributes of <flexContainerInstance> |  | Multiplicity |  | RW/ RO/ WO |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| resourceType |  | 1 |  | RO |  | See clause 9.6.1.3. |
| resourceID |  | 1 |  | RO |  | See clause 9.6.1.3. |
| resourceName |  | 1 |  | RO |  | See clause 9.6.1.3. |
| parentID |  | 1 |  | RO |  | See clause 9.6.1.3. |
| labels |  | 0..1 (L) |  | RO |  | See clause 9.6.1.3. |
| creationTime |  | 1 |  | RO |  | See clause 9.6.1.3. |
| lastModifiedTime |  | 1 |  | RO |  | See clause 9.6.1.3. |
| contentSize | 1 |  | RO |  | Sum of the size in bytes of all of the custom attributes. |  |
| [customAttribute] | 0..n |  | RO |  | Specialization-specific attribute(s). Name and data type defined in each specialization of parent <flexContainer> resource. These custom attributes are copies of all of the parent <flexContainer> custom attributes after CREATE / UPDATE, i.e. when the <flexContainerInstance> is created. |  |

### ********************* End of Change 4 ******************************************

### ********************* Start of Change 5 ******************************************

#### 10.2.4.1	Introduction

Data management is provided with different sets of Content Sharing Resources and those sets provide different functionalities that can be utilized depending on application services. The following lists each resource type set.

- container, contentInstance, latest, oldest

- flexContainer, flexContainerInstance, latest, oldest

- timeSeries, timeSeriesInstance, latest, oldest

### ********************* End of Change 5 ******************************************

### ********************* Start of Change 6 ******************************************

#### 10.2.4.15	Data management using <flexContainer>

A <flexContainer> resource can store application data in custom attributes and provide limited meta information (e.g. contentSize) compared to a <container> resource. With the custom attributes, application data can be separately stored in different attributes that can be leveraged to discover the resource with the attribute value or subscribe to specific attribute(s). A <flexContainer> resource can have one or more child(ren) <flexContainers> resource(s) in multiple levels.

A <flexContainer> resource can have one or more child(ren) <flexContainerInstance> resource(s) in order to keep track of the modifications history of its custom attributes.

The <latest> and <oldest> virtual resources are used to easily retrieve and delete the latest and oldest <flexContainerInstance> resource in a <flexContainer> resource, respectively, without any knowledge of all the <flexContainerInstance> resources in the <flexContainer>.

### ********************* End of Change 6 ******************************************

### ********************* Start of Change 7 ******************************************

#### 10.2.4.16	Create <flexContainer>

This procedure shall be used for creating a <flexContainer> resource.

Table 10.2.4.16-1: <flexContainer> CREATE

| <flexContainer> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | According to clause 10.1.2 |
| Processing at Originator before sending Request | According to clause 10.1.2 |
| Processing at Receiver | According to clause 10.1.2 If at least one of the attributes maxNrOfInstances, maxByteSize or maxInstanceAge is defined: - a child resource <flexContainerInstance> shall be created with custom attributes that take the current values of the <flexContainer>. - the attribute currentNrOfInstances shall be created with value 1. - the attribute currentByteSize shall be created with the value of contentSize. [See NOTE] |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: Address of the created <flexContainer> resource, according to clause 10.1.2 |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | According to clause 10.1.2 with the following addition: - The parent resource type of this newly created <flexContainer> resource shall follow the definition in clause 9.6.1.2.2 (Specializations of <flexContainer>). |
| NOTE: When a <flexContainerInstance> resource is created, its resourceName attribute shall be: [flexContainer’s resourceName]_[flexContainer’s stateTag] (e.g. “LightBulb_1” for a <flexContainer> with resourceName “LightBulb”). |  |

### ********************* End of Change 7 ******************************************

### ********************* Start of Change 8 ******************************************

#### 10.2.4.17	Retrieve <flexContainer>

This procedure shall be used for retrieving the attributes of a <flexContainer> resource.

Table 10.2.4.17-1: <flexContainer> RETRIEVE

| <flexContainer> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-2 apply with the specific details for: Content: void |
| Processing at Originator before sending Request | According to clause 10.1.3 |
| Processing at Receiver | According to clause 10.1.3 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: attributes of the <flexContainer> resource as defined in clause 9.6.6 |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | According to clause 10.1.3 |

##### 10.2.4.17.1	Retrieve <latest>

This procedure shall apply to the latest <flexContainerInstance> resource among all existing <flexContainerInstance> resources in the parent <flexContainer> resource. If there is no <flexContainerInstance> resource in the parent, then the Receiver shall respond with an error.

This procedure is the same as the procedures in clause 10.2.4.yb <flexContainerInstance> RETRIEVE.

##### 10.2.4.17.2	Retrieve <oldest>

This procedure shall apply to the oldest <flexContainerInstance> resource among all existing <flexContainerInstance> resources in the parent <flexContainer> resource. If there is no <flexContainerInstance> resource in the parent, then the Receiver shall respond with an error.

This procedure is the same as the procedures in clause 10.2.4.yb <flexContainerInstance> RETRIEVE.

### ********************* End of Change 8 ******************************************

### ********************* Start of Change 9 ******************************************

#### 10.2.4.18	Update <flexContainer>

This procedure shall be used for updating the attributes and the actual data of a <flexContainer> resource.

Table 10.2.4.18-1: <flexContainer> UPDATE

| <flexContainer> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-2 apply with the specific details for: Content: attributes of the <flexContainer> resource as defined in clause 9.6.6 which need be updated |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | According to clause 10.1.4 Four distinct cases can apply: 1/ At least one custom attribute is present in <flexContainer> update request, and at least one of the policy attributes maxNrOfInstances, maxByteSize or maxInstanceAge is already defined and different from 0: - a - child resource <flexContainerInstance> shall be created with custom attributes that take the new values of the <flexContainer> ones (after update); and - the currentNrOfInstances and currentByteSize) attributes shall be incremented. If the newly created <flexContainerInstance> resource violates any of the defined policy attributes, then the oldest <flexContainerInstance> resource/s shall be removed to enable the creation of this new <flexContainerInstance> resource, and the currentNrOfInstances and currentByteSize attributes shall be decremented. 2/ At least one of the policy attributes is defined in the request, with non-zero positive integer value, whereas none was previously defined, then - a child resource <flexContainerInstance> shall be created with custom attributes that take the new values of the <flexContainer> ones (after update); - the attribute currentNrOfInstances shall be created with value 1; and - the attribute currentByteSize shall be created with the value of contentSize. 3/ At least one of the policy attributes is modified in the request (i.e. given a different, non NULL value), then the set of <flexContainerInstances> children resources and the currentNrOfInstances and currentByteSize attributes shall be adjusted to reflect the new policy 4/ At least one of the policy attributes is deleted by the request (i.e. given a NULL value), then - the set of <flexContainerInstances> children resources and the currentNrOfInstances and currentByteSize attributes shall be adjusted to reflect the remaining policy attributes; and - If no policy attribute remains defined, all <flexContainerInstances> shall be deleted and the currentNrOfInstances and currentByteSize attributes shall be deleted. [See NOTE] |
| Information in Response message | According to clause 10.1.4 |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | According to clause 10.1.4 |
| NOTE: When a <flexContainerInstance> resource is created, its resourceName attribute shall be [flexContainer’s resourceName]_[flexContainer’s stateTag] (e.g. “LightBulb_3” for a <flexContainer> with resourceName “LightBulb” and stateTag=3). The value of the stateTag is taken after it has been incremented. |  |

### ********************* End of Change 9 ******************************************

### ********************* Start of Change 10 ******************************************

#### 10.2.4.19	Delete <flexContainer>

This procedure shall be used for deleting a <flexContainer> resource.

Table 10.2.4.19-1: <flexContainer> DELETE

| <flexContainer> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-2 apply |
| Processing at Originator before sending Request | According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | According to clause 10.1.5 |

##### 10.2.4.19.1	Delete <latest>

This procedure shall apply to the latest <flexContainerInstance> resource among all existing <flexContainerInstance> resources in the parent <flexContainer> resource. If there is no <flexContainerInstance> resource in the parent, then the Receiver shall respond with an error.

After deletion, the <latest> flexContainerInstance will point to the latest <flexContainerInstance> among all remaining <flexContainerInstance> resources in the parent <flexContainer> resource.

This procedure is the same as the procedures in clause 10.2.4.yd <flexContainerInstance> DELETE.

##### 10.2.4.19.2	Delete <oldest>

This procedure shall apply to the oldest <flexContainerInstance> resource among all existing <flexContainerInstance> resources in the parent <flexContainer> resource. If there is no <flexContainerInstance> resource in the parent, then the Receiver shall respond with an error.

After deletion, the <oldest> flexContainerInstance will point to the oldest <flexContainerInstance> among all remaining <flexContainerInstance> resources in the parent <flexContainer> resource.

This procedure is the same as the procedures in clause 10.2.4.yd <flexContainerInstance> DELETE.

### ********************* End of Change 10 ******************************************

### ********************* Start of Change 11 ******************************************

#### 10.2.4.ya	Create <flexContainerInstance>

The Create operation shall not apply to <flexContainerInstance> resource. Creation of <flexContainerInstance> resource is only performed by the Hosting CSE when the parent <flexContainer> is modified and at least one of the policy attributes (maxNrOfInstances, maxByteSize or maxInstanceAge) is present.

#### 10.2.4.yb	Retrieve <flexContainerInstance>

This procedure shall be used for retrieving the attributes of a <flexContainerInstance> resource.

Table 10.2.4.yb-1: <flexContainerInstance> RETRIEVE

| <flexContainerInstance> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | According to clause 10.1.3. |
| Processing at Originator before sending Request | According to clause 10.1.3. |
| Processing at Receiver | According to clause 10.1.3. |
| Information in Response message | According to clause 10.1.3. |
| Processing at Originator after receiving Response | According to clause 10.1.3. |
| Exceptions | According to clause 10.1.3. |

#### 10.2.4.yc	Update <flexContainerInstance>

The Update operation shall not apply to <flexContainerInstance> resource.

#### 10.2.4.yd	Delete <flexContainerInstance>

This procedure shall be used for deleting a <flexContainerInstance> resource residing under a <flexContainer> resource.

Table 10.2.4.yd-1: <flexContainerInstance> DELETE

| <flexContainerInstance> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.5. |
| Processing at Receiver | According to clause 10.1.5. |
| Information in Response message | According to clause 10.1.5. |
| Processing at Originator after receiving Response | According to clause 10.1.5. |
| Exceptions | According to clause 10.1.5. |

### ********************* End of Change 11 ******************************************