| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TP 43 |
| Source:* | Leila Le Brun, Orange, leila.lebrun@orange.com Marianne Mohali, Orange, marianne.mohali@orange.com |
| Date:* | 2019-11-21 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0003 v.3.10.2 |
| Clauses * | Clauses 8.5.3.3, 10.1.1.3, 10.2.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR introduces TS-0003 Security Solutions v3.10.2  enhacement by adding RSA as optional chypering.

### ----------------------Start of change 1-------------------------------------------

#### 8.5.3.3	Signature-Only ESData Security Class Protocol Details

To maintain consistency, signature types are provided which are available in both XML-Signature [Error: Reference source not found] and JSON Web Signature (JWS) [Error: Reference source not found].

- HMAC using SHA-256, SHA-384 or SHA-512.

- RSA signature using PKCS1-v1.5  or PSS and MGF1 with SHA-256, SHA-384 or SHA-512.

- ECDSA signature using P-256, P-384 or P-512 with SHA-256, SHA-284 or SHA-512 respectively.

- ECDSA signature using FRP256v1 and brainpoolP256r1 curves [Error: Reference source not found] with SHA-256 for both curves.

Table 8.5.3.3-1 identifies the algorithms that are supported in XML-SIG for Signature-only ESData Security Class.

Table 8.5.3.3-1: Algorithms that are supported in XML-Signature for
Signature-only ESData Security Class

| Signature Type | Algorithm |  | <SignatureMethod Algorithm=".."> |
| --- | --- | --- | --- |
| HMAC | SHA-256 |  | http://www.w3.org/2001/04/xmldsigmore#hmacsha256 |
|  | SHA-384 |  | http://www.w3.org/2001/04/xmldsigmore#hmacsha384 |
|  | SHA-512 |  | http://www.w3.org/2001/04/xmldsigmore#hmacsha512 |
| RSA | RSA PKCS1-v1.5 or PSS and MGF1 with: | SHA-256 | http://www.w3.org/2001/04/xmldsigmore#rsasha256 |
|  |  | SHA-384 | http://www.w3.org/2001/04/xmldsigmore#rsasha384 |
|  |  | SHA-512 | http://www.w3.org/2001/04/xmldsigmore#rsasha512 |
| ECDSA | P-256 and SHA-256 |  | http://www.w3.org/2001/04/xmldsigmore#ecdsasha256 |
|  | P-384and SHA-384 |  | http://www.w3.org/2001/04/xmldsigmore#ecdsasha384 |
|  | P-512 and SHA-512 |  | http://www.w3.org/2001/04/xmldsigmore#ecdsasha512 |
|  | FRP256v1 and SHA-256 |  | See [Error: Reference source not found] |
|  | brainpoolP256r1 and SHA-256 |  | See [Error: Reference source not found] |

The XML-Signature object may be transported "plain" - with no encoding, or may be encoded in base64.

Table 8.5.3.3-2 identifies the algorithms that are supported in JWS for Signature-only ESData Security Class.

Table 8.5.3.3-2: Algorithms that are supported in JSON Web Signature (JWS) for
Signature-only ESData Security Class

| Signature Type | Algorithm |  | "alg":".." |
| --- | --- | --- | --- |
| HMAC | SHA-256 |  | HS256 |
|  | SHA-384 |  | HS384 |
|  | SHA-512 |  | HS512 |
| RSA | RSA PKCS1-v1.5 or PSS and MGF1 with: | SHA-256 | RS256 |
|  |  | SHA-384 | RS384 |
|  |  | SHA-512 | RS512 |
| ECDSA | P-256 and SHA-256 |  | ES256 |
|  | P-384and SHA-384 |  | ES384 |
|  | P-512 and SHA-512 |  | ES512 |

The output generated by JWS conforms to either the JWS JSON Serialization or a URI-safe JWS Compact Serialization. The JWS JSON Serialization may be transported "plain" – with no encoding, or may be encoded in base64. oneM2M TS-0004 [Error: Reference source not found] defines the datatype m2m:e2eCompactJWS for the JWS Compact Serialization.

### -----------------------End of change 1-------------------------------------------

-----------------------Start of change 2-------------------------------------------

#### 10.1.1.3	Details Common to Certificates with Certificate Chains

Certificates with Certificate Chains shall conform to the following description:

- These certificates shall conform to clause 10.1.1.1 "Common Certificate Details".

- Certificates shall be signed with ECDSA using secp256r1 or optionally RSA using at least 2048 key length, and the signature shall use SHA-256.

- Certificate chains should limit the number of intermediate CA certificates to avoid having a negative impact in constrained environments.

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 10.2.3	TLS and DTLS Ciphersuites for Certificate-Based Security Frameworks

The following Security Frameworks:

- Certificate-Based Security Association Establishment Framework;

- Certificate-Based Security Bootstrap Framework;

shall use the standard TLS handshake (IETF RFC 8446 [Error: Reference source not found]) with the ECDHE_ECDSA Key Exchange (IETF RFC 8422 [Error: Reference source not found]).

TLS implementations supporting these security frameworks shall implement at least the following ciphersuite:

- TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256, IETF RFC 5289 [Error: Reference source not found].

DTLS implementations supporting these security frameworks shall implement at least the following TLS ciphersuite:

- TLS_ECDHE_ECDSA_WITH_AES_128_CCM_8, IETF RFC 7251 [Error: Reference source not found].

- Optionally TLS/DTLS can support RSA keys (2048bits or more):

- TLS_ECDHE_RSA_WITH_AES_128_GCM_SHA256, IETF RFC 5289 [32].

Implementations supporting these security frameworks shall support authenticating other entities using all available public key certificate flavours (see clause 8.1.2.1 "Public Key Certificate Flavours"):

- Raw public key certificate: using the mechanism specified in IETF RFC 7250 [Error: Reference source not found], Implementation shall support receiving and processing raw public keys compliant with section 9.1.3.2 "Raw Public Key Certificates" in IETF RFC 7252 [Error: Reference source not found].

- All other certificates: X.509 certificates including device hardware identifier. Implementation shall support receiving and processing raw public keys compliant with section 9.1.3.3 "X.509 Certificates" in IETF RFC 7252 [Error: Reference source not found].

### -----------------------End of change 3---------------------------------------------