| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #43 |
| Source:* | Cyrille Bareau, Orange, cyrille.bareau@orange.com Leila Le Brun, Orange, leila.lebrun@orange.com Marianne Mohali, Orange, marianne.mohali@orange.com Przemyslaw Ratuszek, Orange, przemyslaw.ratuszek@orange.com Bob Flynn, Chordant, Bob.Flynn@chordant.io |
| Date:* | 2019-11-25 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0084 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023 v. 4.1.0 |
| Clauses * | New clauses: 5.6.xa, 5.6.xb, 5.6.xc, 5.6.xd, 5.6.xe, 5.8 Modified clauses: 6.2.2, 6.3.2, 6.3.3, A.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

The SDT-based Harmonized Information Model is used to model the functional behavior of devices, but it does not allow operating them in other ways: control, supervision, configuration, life cycle, etc. In other words, TS-0023 does not specify how to manage these devices, in the classical sense of Device Management: this must be done through the CSF designed in TS-0001 6.2.4, using resources such as <mgmtObj> and <mgmtCmd>.

We propose here a new approach that consists in transposing the Device Management functions into SDT modules. We present some new modules that are very close to <mgmtObj> resources. The idea is not to replace the <mgmtObj> approach, but to give the users a simple, unified framework for handling both functional and management behaviors of devices.

This approach is agnostic of existing DM protocols. The IPE in charge of NoDN devices can rely on a DM technology if it deals with devices that have a specific DM stack (e.g. OMA DM / LwM2M, BBF TR-069 / USP…), or it can use a proprietary protocol, or it can use standardized IoT protocols (e.g. ZigBee, Z-Wave…), etc.

As Device Management is a transversal set of functions that may concern all devices, we do not explicitly add these new modules as possible children of all the devices that are specified in TS-0023, but we define a new, optional sdtNodeLink attribute in SDT devices flexContainers that is a link relation (similar to the nodeLink relation pointing to the CSEBase <node> child) that points to a [sdtNode] <flexContainer> specialization, child of the hosting CSEBase. This sdtNode would contain these new DM modules as children.

### **********************  Start of change 1   **********************

## 5.8 Device Management

### 5.8.1 Introduction

The modules that are specified in this section allow performing classical Device Management (DM) functions: rebooting a device, upgrading it, reading / setting its configuration, monitoring its logs, checking its memory or battery status, etc.

In the case of a NoDN, it is the IPE in charge of exposing the device to oneM2M that creates / implements these modules. It may rely on external Device Management techniques like e.g. LwM2M (from OMA) or USP (from BBF), or any other technique, proprietary or standardized, that allows performing at least some DM functions, for instance a reboot.

### 5.8.2 sdtNode

This flexContainer specialization is the root for SDT-based Device Management modules.

It is defined as a direct child of the hosting <CSEBase> resource, and it is targeted by the sdtNodeLink attribute of SDT <flexContainer> devices (see 6.2.2 Rules 1-6 and 1-7).

Table 5.7.1-1: Modules of sdtNode model

| Module Instance Name | Module Class Name | Optional | Description |
| --- | --- | --- | --- |
| dmClient | dmClient | false | See clause 5.8.3 |
| dmFirmware_i | dmFirmware | true | See clause 5.8.5 |
| dmSoftware_i | dmSoftware | true | See clause 5.8.6. |
| dmEventLog_i | dmEventLog | true | See clause 5.8.7 |

This flexContainer can contain a 0..n list of module classes [dmFirmware], specializations of <flexContainer> resources, where each module name is dmFirmware_1, dmFirmware_2, etc.

It can similarly contain 0..n lists of module classes [dmSoftware] and [dmEventLog], where each module name is dmFirmware_1, dmFirmware_2, dmEventLog_1, dmEventLog_2etc.

### 5.8.3 dmClient

This ModuleClass provides capabilities to control and monitor the Device Management of the device. It notably provides actions to handle the device’s Data Model.

Table 5.8.3-1 Actions of dmClient ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | reboot | rebootType: hd:enumRebootType | false | Execute a reboot or a factory reset. |
| none | execute | command: xs:string, arguments: xs:string | true | Execute a command. The first argument is the command name, the second one a list of optional parameters, depending on the command type. |
| xs:string | createDataModel | parameters: xs:string | true | Create new parameters. Argument: a JSon serialization of the parameters. Result: the JSon array of the created parameter names. |
| xs:string | readDataModel | parameters: xs:string | true | Read the current values of parameters. Argument: the JSon array of the parameter names. Result: a JSon serialization of the parameters. |
| xs:string | updateDataModel | parameters: xs:string | true | Update the current values of parameters. Argument: a JSon serialization of the parameters. Result: the JSon array of the modified parameter names. |
| xs:string | deleteDataModel | parameters: xs:string | true | Delete some parameters. Argument: the JSon array of the parameter names. Result: the JSon array of the deleted parameter names. |

NOTE: see section 5.8.4 for the Data Model format.

Table 5.8.3-2 DataPoints of dmClient ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| dataModel | hd:enumDataModelType | R | false |  | The type of the data model (OMA DM, OMA LwM2M, BBF TR-181, etc.). |
| deviceId | xs:string | R | true |  | Identifier of the device for the selected data model. |
| state | hd:enumDmClientState | R | false |  | The current state of the client (ready, error, etc.) |
| memAvailable | xs:integer | R | true | KB | The size of available storage memory. |
| memTotal | xs:integer | R | true | KB | The size of total storage memory. |
| ramAvailable | xs:long | R | true | KB | The size of available RAM memory |
| ramTotal | xs:long | R | true | KB | Total size of the RAM memory. |
| batteryLevel | xs:integer | R | true | % | The current battery level (range 0-100). |
| batteryStatus | m2m:batteryStatus | R | true |  | The status of the battery. |
| cpuUsage | xs:integer | R | true | % | Current CPU usage in percent. |

### 5.8.4 Device Management Data Model

The 4 actions create/read/update/deleteDataModel for handling parameters rely on the data model type defined by the dataModel data point of the dmClient module.

An example could be ’Device.WiFi.SSID.1’ in TR-181 or ’/9/1/1’ in LwM2M.

Note: the datapoints of the module (except dataModel and state) correspond to attributes in management objects defined in TS-0001 (firmware, battery, memory, reboot). They are related to fixed parameters in OMA & BBF data models. The corresponding concepts in OMA DM / LwM2M data models (resp. BBF TR-181) are specified in oneM2M TS-0005 (resp. TS-0006).

For instance the datapoint memAvailable corresponds to ‘Device.DeviceInfo.MemoryStatus.Free’ in TR-181 (see TS-0006 § 7.3) and to ‘/3/0/10’ in LwM2M (TS-0005 § 6.3.4).

The 4 CRUD actions manipulate strings that serialize JSon objects (arrays or structures). The parameter value types are the valid JSon primitive types (string, number, boolean, null).

Examples:

createDataModel({"Foo": 123})

- ["Foo"]

readDataModel(["/3/0/1", "/3/0/2"])

- {"/3/0/1": "MyCompany_MyModel_Version", "/3/0/2": "SN376575A86"}

readDataModel(["Device.WiFi.SSID.SSID", "Device.DeviceInfo.MemoryStatus.Free"])

- {"Device.DeviceInfo.MemoryStatus.Free": 123456,

"Device.WiFi.SSID.SSID": "SSIDName"}

updateDataModel(["Device.DeviceInfo.FriendlyName": "my device",

"Device.WiFi.SSID.SSID": "NewSSIDName"])

- {["Device.DeviceInfo.FriendlyName", "Device.WiFi.SSID.SSID"]}

updateDataModel(["/3/0/15": "Europe/Paris"])

- {["/3/0/15"]}

### 5.8.5 dmFirmware

This ModuleClass provides DM capabilities to control and monitor firmware images of the device.

Device firmware is comprised of firmware modules and artefacts (e.g. configuration files) that are maintained on a device. A device can maintain more than one firmware image and the capability to manage individual firmware images. The hosting CSE will ensure that only one firmware image is indicated as ‘active’. The firmware lifecycle includes actions to download, update or remove a firmware image.

Table 5.8.5-1 Actions of dmFirmware ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | upgrade | none | true | Download if needed and install the firmware image. |

Notes:

- The firmware image can be pre-downloaded when the [dmFirmware] resource is created.

- When the [dmFirmware] resource is deleted, the firmware image shall be removed from the device.

- When the ‘upgrade’ action is performed, the ‘fwVersion’ attribute of the [deviceInfo] <mgmtObj> resource associated to the device shall be updated by the IPE/AE.

Table 5.8.5-2 DataPoints of dmFirmware ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| state | hd:enumDmFirmwareState | R | False |  | The current state of the firmware (active, downloading, etc.) |
| name | xs:string | R | False |  | The name of the firmware image. |
| version | xs:string | R | False |  | The version of the firmware image. |
| url | xs:string | RW | True |  | The URL from which the firmware image can be downloaded |

Note 1: several [dmFirmware] resources can be created on a device, but the hosting CSE shall ensure that only one can have state ‘active’.

### 5.8.6 dmSoftware

This ModuleClass provides DM capabilities to control and monitor software modules of the device.

An instance of this module represents a software module hosted by the device.

Table 5.8.6-1 Actions of dmSoftware ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | install | none | true | Download if needed and install the software module. |
| none | uninstall | none | true | Uninstall the software module |
| none | activate | none | true | Activate the software module. |
| none | deactivate | none | true | Deactivate the software module. |

Note 1: the software module can be pre-downloaded when the [dmSoftware] resource is created.

Table 5.8.6-2 DataPoints of dmSoftware ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| state | hd:enumDmSoftwareState | R | false |  | The current state of the software (ready, downloading, activating, etc.) |
| name | xs:string | R | false |  | The name of the software module. |
| version | xs:string | R | false |  | The version of the software module. |
| url | xs:string | RW | true |  | The URL from which the software module can be downloaded |

### 5.8.7 dmEventLog

This ModuleClass provides DM capabilities to control and monitor event logs of the device.

Table 5.8.7-1 Actions of dmEventLog ModuleClass

| Return Type | Name | Argument | Optional | Documentation |
| --- | --- | --- | --- | --- |
| none | start | none | true | Start logging. |
| none | stop | none | true | Stop logging. |

Table 5.8.7-2 DataPoints of dmEventLog ModuleClass

| Name | Type | R/W | Optional | Unit | Documentation |
| --- | --- | --- | --- | --- | --- |
| type | m2m:logTypeId | RW | false |  | The type of the log (e.g. security log, system log…). |
| data | xs:string | R | false |  | Raw data of events. No format specified. |
| status | m2m:logStatus | R | false |  | The current status of the logging process (Started, Stopped, Error, etc.) |

### ********************* End of Change 1 ******************************************

### ********************* Start of Change 2 ******************************************

### 5.6.xa hd:enumDataModelType

Used for the “dataModel” DataPoint of the “dmClient” ModuleClass.

Table 5.6.xa-1 Interpretation of hd:enumDataModelType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | OMA DM 1.3 |  |
| 2 | OMA DM 2.0 |  |
| 3 | OMA LwM2M |  |
| 4 | BBF TR-181 CWMP | Version of TR-181 for TR-069 |
| 5 | BBF TR-181 USP | Version of TR-181 for USP |
| 6 | oneM2M | For native oneM2M devices |
| 7 | unknown | To be used for a proprietary/unknown protocol |

### 5.6.xb hd:enumDmClientState

Used for the “state” DataPoint of the “dmClient” ModuleClass.

Table 5.6.xb-1 Interpretation of hd:enumDmClientState

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | ready |  |
| 2 | error |  |
| 3 | offline |  |

### 5.6.xc hd:enumDmFirmwareState

Used for the “state” DataPoint of the “dmFirmware” ModuleClass.

Table 5.6.xc-1 Interpretation of hd:enumDmFirmwareState

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | active |  |
| 2 | uninstalled |  |
| 3 | error |  |
| 4 | downloading |  |
| 5 | installing |  |

### 5.6.xd hd:enumDmSoftwareState

Used for the “state” DataPoint of the “dmSoftware” ModuleClass.

Table 5.6.xd-1 Interpretation of hd:enumDmSoftwareState

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | active |  |
| 2 | uninstalled |  |
| 3 | error |  |
| 4 | downloading |  |
| 5 | installing |  |
| 6 | installed |  |
| 7 | activating |  |
| 8 | deactivating |  |

### 5.6.xe hd:enumRebootType

Used for the “rebootType” argument of the “reboot” action of the “dmClient” ModuleClass.

Table 5.6.xe-1 Interpretation of hd:enumRebootType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | reboot |  |
| 2 | factoryReset |  |

### **********************  End of change 2   **********************

### **********************  Start of change 3   **********************

### 6.2.2 Resource mapping for Device model

When the AE exposes a controlling interface for a home domain device which is specified as an information model in clause Error: Reference source not found, a specialization of the <flexContainer> resource shall be created as the mapping of the model following conversion rules:

- Rule 1-1: Each Device model defined in clause Error: Reference source not found shall be mapped to a specialization of <flexContainer>. The containerDefinition attribute shall be set according to Error: Reference source not found.

- Rule 1-2: Each entry in the 'Module' table shall be mapped to a child resource(s) which is mapped as a specialised <flexContainer> following the rule in clause Error: Reference source not found.

- Rule 1-3: The specialized <flexContainer> resource of the Device model shall contain a mandatory [customAttribute] named nodeLink (as defined in TS-0001[3] and in TS-0004[4]). The value of nodeLink shall be set to the resource identifier of a <node> resource described in Rule 1-5 below.

- Rule 1-4: XSD file for each Device model shall be named  according to Error: Reference source not found.

- Rule 1-5: A <node> resource shall be created on the same hosting CSE as the <flexContainer> representing this Device model. The <node> resource contains all the management information as specialized <mgmtObj> resources (e.g. [firmware]) about the Device model instance for device management purposes.

- Rule 1-6: The specialized <flexContainer> resource of the Device model may contain an optional [customAttribute] named sdtNodeLink. The value of sdtNodeLink shall be set to the resource identifier of a <flexContainer> resource described in Rule 1-7 below.

- Rule 1-7: A [sdtNode] specialization of a <flexContainer> resource may be created on the same hosting CSE as the <flexContainer> representing this Device model. This [sdtNode] resource contains all the management information as specialized <flexContainer> resources defined in 5.8 (e.g. [dmFirmware]) about the Device model instance for device management purposes.

### **********************  End of change 3   **********************

### **********************  Start of change 4   **********************

### Resource types

In protocol bindings resource type names for device models shall be translated into short names of Table 6.3.2-1.

Table 6.3.2-1: Specialization type short names (Device models)

| Resource Type Name | Short Name |
| --- | --- |
| Device3DPrinter | dTDPr |
| deviceAirConditioner | deACr |
| deviceAirPurifier | deAPr |
| deviceAirQualityMonitor | dAQMr |
| deviceAudioReceiver | deARr |
| deviceBloodPressureMonitor | dBPMr |
| deviceCamera | devCa |
| deviceClothesDryer | deCDr |
| deviceClothesWasher | deCWr |
| deviceClothesWasherDryer | dCWDr |
| deviceCoffeeMachine | deCMe |
| deviceCookerHood | deCHd |
| deviceCooktop | devCp |
| deviceDehumidifier | devDr |
| deviceDishWasher | deDWr |
| deviceDoor | devD0 |
| deviceDoorLock | deDLk |
| deviceElectricVehicleCharger | dEVCr |
| deviceFan | devFn |
| deviceFoodProbe | deFPe |
| deviceFreezer | devFr |
| deviceGlucosemeter | devGr |
| deviceHeartRateMonitor | dHRMr |
| deviceHomeCCTV | dHCCT |
| deviceHumidifier | devHr |
| deviceKettle | devKe |
| deviceLight | devLt |
| deviceMicrogeneration | devMn |
| deviceMultiFunctionPrinter | dMFPr |
| deviceOutdoorLamp | deOLp |
| deviceOven | devOn |
| devicePrinter | devPr |
| devicePulseOximeter | dePOr |
| deviceRefrigerator | devRr |
| deviceRobotCleaner | deRCr |
| deviceScanner | devSr |
| deviceSecurityPanel | deSPl |
| deviceSetTopBox | dSTBx |
| deviceSmartElectricMeter | dSEMr |
| deviceSmartPlug | deSPg |
| deviceSteamCloset | deSCt |
| deviceStorageBattery | deSBy |
| deviceSwitch | devSh |
| deviceTelevision | devTn |
| deviceThermometer | devTr |
| deviceThermostat | devTt |
| deviceWaterHeater | deWHr |
| deviceWaterValve | deWVe |
| deviceWeightScaleAndBodyCompositionAnalyser | dWSAB |
| deviceWindowShade | deWSe |
| sdtNode | sdtNe |

In protocol bindings resource type names for SubDevice model shall be translated into short names of Table 6.3.2-2.

Table 6.3.2-2: Specialization type short names (SubDevice models)

| Resource Type Name | Short Name |
| --- | --- |
| cuff0 | cuff0 |
| cuff1 | cuff1 |
| cuff2 | cuff2 |
| cuff3 | cuff3 |
| cuff4 | cuff4 |
| cuff5 | cuff5 |
| powerOutlet0 | powO0 |
| powerOutlet1 | powO1 |
| powerOutlet2 | powO2 |
| powerOutlet3 | powO3 |
| powerOutlet4 | powO4 |
| powerOutlet5 | powO5 |
| subDeviceCuff | suDCf |
| subDevicePowerOutlet | sDPOt |

In protocol bindings resource type names for module classes shall be translated into short names of In protocol bindings resource type names for device models shall be translated into short names of Table 6.3.2-1..

Table 6.3.2-3: Specialization type short names (ModuleClasses and Module Instances)

| Resource Type Name | Short Name |
| --- | --- |
| 3DPrinter | thDPr |
| acousticSensor | acoSr |
| airCleanOperationMode | aCOM0 |
| airConJobMode | aCJMe |
| airConOperationMode | aCOMe |
| airFlow | airFw |
| airPurifierJobMode | aPJMe |
| airPurifierOperationMode | aPOMe |
| airQualitySensor | aiQSr |
| alarmSpeaker | alaSr |
| audioVolume | audVe |
| autoDocumentFeeder | auDFr |
| battery | bat |
| binaryObject | binOt |
| binarySwitch | binSh |
| bioElectricalImpedanceAnalysis | bEIAs |
| bodyCompositionAnalyser | boCAr |
| boiler | boilr |
| boilingSwitch | boiSh |
| brewing | brewg |
| brewingSwitch | breSh |
| brightness | brigs |
| channel | chanl |
| clock | clock |
| clothesDryerJobMode | cDJMe |
| clothesDryerOperationMode | cDOMe |
| clothesWasherDryerJobMode | cWDJM |
| clothesWasherDryerOperationMode | cWDOM |
| clothesWasherJobMode | cWJMe |
| clothesWasherJobModeOption | cWJMO |
| clothesWasherOperationMode | cWOMe |
| colour | color |
| colourSaturation | colSn |
| controlPanelLock | coPLk |
| cookerHoodJobMode | cHJMe |
| credentials | creds |
| customTemperature | cusTe |
| dehumidifierJobMode | deJMe |
| dehumidifierOperationMode | deOMe |
| dishWasherJobMode | dWJMe |
| dmClient | dmClt |
| dmEventLog | dmELg |
| dmFirmware | dmFie |
| dmSoftware | dmSoe |
| doorLock | dooLk |
| doorlock | doork |
| doorStatus | dooSs |
| electricVehicleConnector | elVCr |
| energyConsumption | eneCn |
| energyGeneration | eneGn |
| faultDetection | fauDn |
| filterInfo | filIo |
| foaming | foamg |
| fridgeTemperature | friTe |
| frozenTemperature | froTe |
| geoLocation | geoLn |
| glucometer | glucr |
| grinder | grinr |
| heatingZone | heaZe |
| heatingZone0 | heaZ0 |
| heatingZone1 | heaZ1 |
| heatingZone2 | heaZ2 |
| heatingZone3 | heaZ3 |
| heatingZone4 | heaZ4 |
| heatingZone5 | heaZ5 |
| height | heigt |
| hotWaterSupply | hoWSy |
| impactSensor | impSr |
| keepWarm | keeWm |
| keypad | keypd |
| liquidLevel | liqLl |
| liquidRemaining | liqRg |
| lock | lock |
| mediaInput | medIt |
| mediaOutput | medOt |
| mediaSelect | medSt |
| milkFoaming | milFg |
| milkQuantity | milQy |
| milkStatus | milSs |
| motionSensor | motSr |
| numberValue | numVe |
| openLevel | opeLl |
| operationMode | opeMe |
| overcurrentSensor | oveSr |
| oximeter | oximr |
| ozoneMeter | ozoMr |
| phoneCall | phoCl |
| playerControl | plaCl |
| powerSave | powS0 |
| printerRunState | prRSe |
| printQueue | priQe |
| pulsemeter | pulsr |
| pushButton | pusBn |
| recorder | recor |
| refrigeration | refrn |
| relativeHumidity | relHy |
| remoteControlEnable | reCEe |
| robotCleanerJobMode | rCJMe |
| robotCleanerOperationMode | rCOMe |
| runState | runSe |
| scannerRunState | scRSe |
| securityMode | secMe |
| sessionDescription | sesDn |
| signalStrength | sigSh |
| sleepTimer | sleTr |
| smokeSensor | smoSr |
| sphygmomanometer | sphyr |
| spinLevel | spiLl |
| steamClosetJobMode | sCJMe |
| steamClosetOperationMode | sCOMe |
| televisionChannel | telCl |
| temperature | tempe |
| temperatureAlarm | temAm |
| textMessage | texMe |
| timer | timer |
| turbo | turbo |
| uvSensor | uveSr |
| waterFilterInfo | waFIo |
| waterFlow | watFw |
| waterSensor | watSr |
| waterStatus | watSs |
| weight | weigt |

In protocol bindings resource type names for actions shall be translated into short names of Table 6.3.2-4.

Table 6.3.2-4: Specialization type short names (Actions)

| Resource Type Name | Short Name |
| --- | --- |
| activate | actie |
| activateClockTimer | acCTr |
| answer | answr |
| call | call |
| close | close |
| createDataModel | crDMl |
| deactivate | deace |
| deactivateClockTimer | deCTr |
| decrementNumberValue | deNVe |
| deleteDataModel | deDMl |
| downChannel | dowCl |
| downVolume | dowVe |
| execute | exece |
| hangup | hangp |
| install | instl |
| incrementNumberValue | inNVe |
| nextTrack | nexTk |
| open | open |
| previousTrack | preTk |
| reboot | rebot |
| readDataModel | reDMl |
| resetNumberValue | reNVe |
| resetTextMessage | reTMe |
| start | start |
| start3Dprint | staDt |
| stop | stop |
| stop3Dprint | stoDt |
| toggle | togge |
| uninstall | uninl |
| upChannel | uphCl |
| updateDataModel | upDMl |
| upgrade | upgre |
| upVolume | upoVe |

### Resource attributes for properties and data points

In protocol bindings resource attributes names for properties of module classes shall be translated into short names of Table 6.3.3-5.

Table 6.3.3-5: Resource attribute short names (ModuleClass properties)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| chargingCapacity | electricVehicleConnector | chaCy |
| dischargingCapacity | electricVehicleConnector | disCy |
| electricEnergy | battery | eleEy |
| material | battery | matel |
| voltage | battery | volte |

In protocol bindings resource attributes names for data points of module classes shall be translated into short names of Table 6.3.3-6.

Table 6.3.3-6: Resource attribute short names (ModuleClass data points)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| absoluteEnergyConsumption | energyConsumption | abECn |
| absoluteStartTime | timer | abSTe |
| absoluteStopTime | timer | abST0 |
| acousticStatus | acousticSensor | acoSs |
| adfStates | autoDocumentFeeder | adfSs |
| alarm | motionSensor, smokeSensor, temperatureAlarm, waterSensor | alarm |
| alarmStatus | alarmSpeaker | alaSs |
| altitude | geoLocation | altie |
| automode | airFlow | autoe |
| availableChannels | televisionChannel | avaCs |
| basalMetabolism | bodyCompositionAnalyser | basMm |
| bath | hotWaterSupply | bath |
| batteryLevel | dmClient | batLl |
| batteryStatus | dmClient | batSs |
| batteryThreshold | battery | batTd |
| blue | colour | blue |
| bmi | bodyCompositionAnalyser | bmi |
| bodyLength | bodyCompositionAnalyser | bodLh |
| bone | bioElectricalImpedanceAnalysis | bone |
| brightness | brightness | brigs |
| callerID | phoneCall | calID |
| callState | phoneCall | calSe |
| capacity | battery | capay |
| ch2o | airQualitySensor | ch2o |
| channelId | televisionChannel | chaId |
| channelName | televisionChannel | chaNe |
| charging | battery | charg |
| chargingCapacity | electricVehicleConnector | chaCy |
| co | airQualitySensor | co |
| co2 | airQualitySensor | co2 |
| coarseness | grinder | coars |
| code | faultDetection, filterInfo | code |
| coldWash | clothesWasherJobModeOption | colWh |
| colourSaturation | colourSaturation | colSn |
| concentration | glucometer | concn |
| contextCarbohydratesAmount | glucometer | coCAt |
| contextCarbohydratesSource | glucometer | coCSe |
| contextExercise | glucometer | conEe |
| contextHealth | glucometer | conHh |
| contextLocation | glucometer | conLn |
| contextMeal | glucometer | conMl |
| contextMedication | glucometer | conMn |
| contextTester | glucometer | conTr |
| cpuUsage | dmClient | cpuUe |
| cupsNumber | brewing | cupNr |
| current | energyConsumption | currt |
| currentAdfState | autoDocumentFeeder | cuASe |
| currentDate | clock | curDe |
| currentJobMode | airConJobMode, airPurifierJobMode, clothesDryerJobMode, clothesWasherDryerJobMode, clothesWasherJobMode, cookerHoodJobMode, dehumidifierJobMode, dishWasherJobMode, robotCleanerJobMode, steamClosetJobMode | cuJMe |
| currentJobModeName | airConJobMode, airPurifierJobMode, clothesDryerJobMode, clothesWasherDryerJobMode, clothesWasherJobMode, cookerHoodJobMode, dehumidifierJobMode, dishWasherJobMode, robotCleanerJobMode, steamClosetJobMode | cJMNe |
| currentJobState | runState | cuJSe |
| currentMachineState | runState | cuMSe |
| currentPlayerMode | playerControl | cuPMe |
| currentPlayerModeName | playerControl | cPMNe |
| currentSecurityMode | securityMode | cuSMe |
| currentTemperature | temperature | curT0 |
| currentTime | clock | curTe |
| currentTimeZone | clock | cuTZe |
| currentValue | smokeSensor | crv |
| data | dmEventLog | data |
| dataModel | dmClient | datMl |
| defaultValue | numberValue, textMessage | defVe |
| defrost | refrigeration | defrt |
| description | faultDetection | dc |
| desiredHumidity | relativeHumidity | desHy |
| detectedTime | overcurrentSensor, smokeSensor | detTe |
| deviceId | dmClient | devId |
| diastolicPressure | sphygmomanometer | diaPe |
| discharging | battery | discg |
| dischargingCapacity | electricVehicleConnector | disCy |
| doorState | doorStatus | dooSe |
| duration | overcurrentSensor, recorder | dur |
| electricEnergy | battery | eleEy |
| energy | pulsemeter | enery |
| estimatedTimeToEnd | timer | eTTEd |
| extraRinse | clothesWasherJobModeOption | extRe |
| fat | bioElectricalImpedanceAnalysis | fat |
| fatFreeMass | bodyCompositionAnalyser | faFMs |
| filterLifetime | filterInfo | filLe |
| foamingStrength | foaming | foaSh |
| frequency | energyConsumption | freqy |
| generationSource | energyGeneration | genSe |
| grainsRemaining | grinder | graRg |
| green | colour | green |
| hash | binaryObject | hash |
| hba1c | glucometer | hba1c |
| heading | geoLocation | headg |
| headingAccuracy | geoLocation | heaAy |
| heatingLevel | heatingZone | heaLl |
| height | height | heigt |
| horizontalAccuracy | geoLocation | horAy |
| horizontalDirection | airFlow | horDn |
| impactDirectionHorizontal | impactSensor | imDHl |
| impactDirectionVertical | impactSensor | imDVl |
| impactLevel | impactSensor | impLl |
| impactStatus | impactSensor | impSs |
| impedance | bodyCompositionAnalyser | impee |
| jobModes | airConJobMode, airPurifierJobMode, clothesWasherDryerJobMode, clothesWasherJobMode, cookerHoodJobMode, dehumidifierJobMode, dishWasherJobMode, robotCleanerJobMode steamClosetJobMode, | jobMs |
| jobStates | runState | jobSs |
| kcal | bioElectricalImpedanceAnalysis | kcal |
| keyNumber | keypad | keyNr |
| latitude | geoLocation | latie |
| level | battery | lvl |
| light | alarmSpeaker | light |
| liquidLevel | liquidLevel | liqLl |
| liquidRemaining | liquidRemaining | liqRg |
| lock | lock | lock |
| loginName | credentials | logNe |
| longitude | geoLocation | longe |
| loudness | acousticSensor | louds |
| lowBattery | battery | lowBy |
| lqi | signalStrength | lqi |
| machineStates | runState | macSs |
| material | battery | matel |
| maxHeatingLevel | heatingZone | maHLl |
| maxLength | textMessage | maxLh |
| maxLevel | openLevel | maxLl |
| maxSpeed | airFlow | maxSd |
| maxValue | audioVolume, numberValue, ozoneMeter, temperature | maxVe |
| meanPressure | sphygmomanometer | meaPe |
| measuringScope | energyConsumption | meaSe |
| mediaID | mediaSelect | medID |
| mediaName | mediaSelect | medNe |
| mediaType | mediaSelect | medTe |
| memAvailable | dmClient | memAe |
| memorySize | 3DPrinter | memSe |
| memTotal | dmClient | memTl |
| messageEncoding | textMessage | mesEg |
| minLength | textMessage | minLh |
| minLevel | openLevel | minLl |
| minSpeed | airFlow | minSd |
| minValue | numberValue, temperature | minVe |
| modality | pulsemeter | moday |
| monitoringEnabled | airQualitySensor | monEd |
| multiplyingFactors | energyConsumption, energyGeneration | mulFs |
| muscle | bioElectricalImpedanceAnalysis | musce |
| muscleMass | bodyCompositionAnalyser | musMs |
| muteEnabled | audioVolume | mutEd |
| name | dmFirmware, dmSoftware | name |
| network | 3DPrinter | netwk |
| numberValue | numberValue | numVe |
| object | binaryObject | objet |
| objectType | binaryObject | objTe |
| openAlarm | doorStatus | opeAm |
| openDuration | doorStatus | opeDn |
| openLevel | openLevel | opeLl |
| overcurrentStatus | overcurrentSensor | oveSs |
| oxygenSaturation | oximeter | oxySn |
| ozoneStatus | ozoneMeter | ozoSs |
| ozoneValueMG | ozoneMeter | ozVMG |
| ozoneValuePPM | ozoneMeter | oVPPM |
| password | credentials | pwd |
| power | energyConsumption | power |
| powerGenerationData | energyGeneration | poGDa |
| powerSaveEnabled | powerSave | poSEd |
| powerState | binarySwitch | powSe |
| previousChannel | televisionChannel | preCl |
| preWash | clothesWasherJobModeOption | preWh |
| printingState | printQueue | priSe |
| printSizeX | 3DPrinter | priSX |
| printSizeY | 3DPrinter | priSY |
| printSizeZ | 3DPrinter | priSZ |
| printType | 3DPrinter | priTe |
| progressPercentage | runState | proPe |
| pulseRate | pulsemeter | pulRe |
| pushed | pushButton | pushd |
| ramAvailable | dmClient | ramAe |
| ramTotal | dmClient | ramTl |
| rapidCool | refrigeration | rapCl |
| rapidFreeze | refrigeration | rapFe |
| recipientID | phoneCall | recID |
| red | colour | red |
| referenceTimer | timer | refTr |
| relativeHumidity | relativeHumidity | relHy |
| remoteControlEnabled | remoteControlEnable | reCEd |
| resistance | bioElectricalImpedanceAnalysis | resie |
| roundingEnergyConsumption | energyConsumption | roECn |
| roundingEnergyGeneration | energyGeneration | roEGn |
| rr | pulsemeter | r0 |
| rssi | signalStrength | rssi |
| runningTime | timer | runTe |
| sdp | sessionDescription | sdp |
| securityModes | securityMode | secMs |
| sensitivity | motionSensor | sensy |
| sensorHumidity | airQualitySensor | senHy |
| sensorOdor | airQualitySensor | senOr |
| sensorPM1 | airQualitySensor | sePM1 |
| sensorPM10 | airQualitySensor | sePM0 |
| sensorPM2 | airQualitySensor | sePM2 |
| significantDigits | energyConsumption, energyGeneration | sigDs |
| silentTime | motionSensor | silTe |
| size | binaryObject | size |
| smokeThreshhold | smokeSensor | smoTd |
| softLeanMass | bodyCompositionAnalyser | soLMs |
| soilLevel | clothesWasherJobModeOption | soiLl |
| speed | airFlow | speed |
| speedFactor | playerControl | speFr |
| speedWash | clothesWasherJobModeOption | speWh |
| spinLevelStrength | spinLevel | spLSh |
| spinSpeed | clothesWasherJobModeOption | spiSd |
| startPause | operationMode | staPe |
| state | dmClient, dmFirmware, dmSoftware | state |
| status | boiler, dmEventLog, electricVehicleConnector, faultDetection, filterInf, mediaSelect | sus |
| steamTreat | clothesWasherJobModeOption | steTt |
| step | numberValue | step |
| stepValue | audioVolume, openLevel, temperature | steVe |
| strength | brewing | streh |
| supportedHorizontalDirection | airFlow | suHDn |
| supportedMediaSources | mediaSelect | suMSs |
| supportedMessageValues | textMessage | suMVs |
| supportedPlayerModes | playerControl | suPMs |
| supportedVerticalDirection | airFlow | suVDn |
| systolicPressure | sphygmomanometer | sysPe |
| targetAltitude | geoLocation | tarAe |
| targetDuration | timer | tarDn |
| targetLatitude | geoLocation | tarLe |
| targetLongitude | geoLocation | tarL0 |
| targetTemperature | temperature | tarTe |
| targetTimeToStart | timer | tTTSt |
| targetTimeToStop | timer | tTTSp |
| temperature | temperatureAlarm | tempe |
| temperatureThreshhold | temperatureAlarm | temTd |
| textMessage | textMessage | texMe |
| type | dmEventLog | type |
| time | keepWarm | time |
| token | credentials | tk |
| tone | alarmSpeaker | tone |
| turboEnabled | turbo | turEd |
| unit | temperature | unit |
| uri | printQueue | ur0 |
| url | sessionDescription, dmFirmware, dmSoftware | ur1 |
| useGrinder | grinder | useGr |
| uvStatus | uvSensor | uvtSs |
| uvValue | uvSensor | uvaVe |
| version | dmFirmware, dmSoftware | versn |
| verticalAccuracy | geoLocation | verAy |
| verticalDirection | airFlow | verDn |
| visceraFat | bioElectricalImpedanceAnalysis | visFt |
| voc | airQualitySensor | voc |
| voltage | battery | volte |
| voltage | energyConsumption | volte |
| volumePercentage | audioVolume | volPe |
| washTemp | clothesWasherJobModeOption | wasTp |
| water | bioElectricalImpedanceAnalysis | water |
| waterFlowStrength | waterFlow | waFSh |
| weight | weight | weigt |

### **********************  End of change 4   **********************

### **********************  End of change 5   **********************

- A.2 Example for Device model ‘deviceAirConditioner'

The present clause explains the creation process for the device typed 'deviceAirConditioner' (see clause Error: Reference source not found for device model definition of ‘deviceAirConditioner').

Using the definition, 'deviceAirConditioner' model is mapped to [deviceAirConditioner] resource which is a specialization of <flexContainer> resource (See Figure 7.1.3-1).

Figure 7.1.3-1: Structure of [deviceAirConditioner] resource

The AE creates the [deviceAirConditioner] specialization of <flexContainer> resource for the Device model [deviceAirConditioner] resource.

The [deviceAirConditioner] resource contains the child resource specified in Table 7.1.3-7.

Table 7.1.3-7: Child resources of [deviceAirConditioner] resource

| Child Resources of [deviceAirConditioner] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <flexContainer> as defined in the specialization [binarySwitch] | 0..1 | This resource is used to map 'binarySwith' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [runState] | 0..1 | This resource is used to map 'runState' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airConJobMode] | 0..1 | This resource is used to map ‘airConJobMode’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airConOperationMode] | 0..1 | This resource is used to map ‘airConOperationMode’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airCleanOperationMode] | 0..1 | This resource is used to map ‘airCleanOperationMode’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [temperature] | 0..1 | This resource is used to map ‘temperature’ ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [timer] | 0..1 | This resource is used to map 'timer' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [sleepTimer] | 0..1 | This resource is used to map 'sleepTimer' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [turbo] | 0..1 | This resource is used to map 'turbo' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airFlow] | 0..1 | This resource is used to map 'airFlow' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [powerSave] | 0..1 | This resource is used to map 'powerSave' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [airQualitySensor] | 0..1 | This resource is used to map 'airQualitySensor' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <flexContainer> as defined in the specialization [filterInfo] | 0..1 | This resource is used to map 'filterInfo' ModuleClass defined in clause Error: Reference source not found. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in oneM2M TS-0001 [Error: Reference source not found] |

Editor’s Note: Above table should be updated compliant to present structure of deviceAirConditioner.

The [deviceAirConditioner] resource contains the attributes specified in Table 7.1.3-8.

Table 7.1.3-8: Attributes of [deviceAirConditioner] resource

| Attributes of [deviceAirConditioner] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| resourceID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| resourceName | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| parentID | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| expirationTime | 1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| creationTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| labels | 0..1 | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found]. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| stateTag | 1 | RO | See clause 9.6.1.3 in oneM2M TS-0001 [Error: Reference source not found] |
| creator | 0..1 | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found] |
| containerDefinition | 1 | WO | The value is "org.onem2m.home.device.airconditioner" |
| ontologyRef | 0..1 | RW | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| contentSize | 1 | RO | See clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found]. |
| nodeLink | 1 | RO | nodeLink attribute links to a <node> resource that is hosted on the same hosting CSE of the <flexContainer>. See clause and Error: Reference source not found for more details. |
| sdtNodeLink | 0..1 | RO | sdtNodeLink attribute links to a [sdtNode] specialization of a <flexContainer> resource that is hosted on the same hosting CSE of the <flexContainer>. See clauses 5.8 and for more details. |

### **********************  End of change 5   **********************