| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 43 |
| Source:* | Dale Seed, Convida Wireless , Dale.Seed@convidawireless.com |
| Date:* | 2019-12-05 |
| Reason for Change/s:* | Modify attribute names of some semantic resource attributes |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0034 V4.1.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

During TS-0004 contributions better names for some attributes were proposed.

### -----------------------Start of change 1-------------------------------------------

## <reasoningJobInstance> Operations

### Introduction

A Reasoning Initiator (RI), such as an AE or CSE, may trigger two types of reasoning operations. One type is a “one-time” reasoning operation. This is applicable to the case where a reasoning operation can be executed over a Fact Set (FS) and a Rule Set (RS) that may not change over time. In comparison, the other type is a “continuous” reasoning operation. The second type is applicable to the cases where the input FS and RS for reasoning may change over time, and accordingly the previously inferred knowledge may not be valid anymore. Therefore, new reasoning is executed over the latest version of FS and RS in order to generate up-to-date inferred knowledge.

A <reasoningJobInstance> resource represents a specific reasoning job instance for enabling the two types of reasoning operations. A RI initiates a desired reasoning operation by creating a <reasoningJobInstance> resource as a child resource of a <semanticRuleRepository> resource.

### Create <reasoningJobInstance>

This procedure is used for creating a <reasoningJobInstance> resource as described in Table 6.15.2-1.

Table 6.15.2-1: <reasoningJobInstance> CREATE

| <reasoningJobInstance> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in [1] table 8.1.2-3 apply with the specific details for: Content: The resource content provides the information as defined in the resource definition of <reasoningJobInstance> resource. |
| Processing at Originator before sending Request | According to clause 10.1.2 in oneM2M TS-0001 in [1]. |
| Processing at Receiver | The Receiver follows the basic procedure according to clause 10.1.4 in oneM2M TS-0001 [1], with the following specific details: - The receiver first retrieves the facts from the resources referred to by the factSet attribute. For example, - If a referred resource is a type of <semanticDescriptor> resource, the RDF triples included in the descriptor attribute will be collected. - If a referred resource is a type of <ontology> resource, the data included in the ontologyContent attribute will be collected. - The receiver retrieves all the related reasoning rules for the resources referred to by the ruleSet attribute. For example, - If a referred resource is a <reasoningRules> resource, the rules included in the ruleRepresentation attribute will be collected. - The receiver includes the retrieved facts and rules from the previous steps, as well as optional facts/rules based on local policies, as inputs for the semantic reasoning operation. The receiver performs semantic reasoning processing using these inputs and produces the reasoning result and stores it in the resultRepresentation attribute of the created <reasoningJobInstance> resource. - If the created <reasoningJobInstance> resource represents a continuous reasoning operation (i.e., the reasoningType attribute is set to “continuous”), subsequent reasoning processing will be automatically triggered and performed according to the values of reasoningMode and reasoningPeriod attributes and the resultRepresentation attribute will be overwritten with the latest reasoning result. |
| Information in Response message | All parameters defined in table 8.1.3-1 in [1] apply with the specific details for: Content: Address of the created <reasoningJobInstance> resource, according to clause 10.1.2 in [1]. |
| Processing at Originator after receiving Response | According to clause 10.1.2 in oneM2M TS-0001 in [1]. |
| Exceptions | According to clause 10.1.2 in oneM2M TS-0001 in [1]. |

### Retrieve <reasoningJobInstance>

This procedure is used for retrieving the attributes of a <reasoningJobInstance> resource as described in Table 6.15.3-1.

Table 6.15.3 -1: <reasoningJobInstance> RETRIEVE

| <reasoningJobInstance> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc'. |
| Information in Request message | All parameters defined in table 8.1.2-3 in [1] apply. |
| Processing at Originator before sending Request | According to clause 10.1.3 in oneM2M TS-0001 in [1]. |
| Processing at Receiver | According to clause 10.1.3 in oneM2M TS-0001 in [1]. |
| Information in Response message | All parameters defined in table 8.1.3-1 in [Error: Reference source not found] apply with the specific details for: Content: Attributes of the <reasoningJobInstance> resource. |
| Processing at Originator after receiving Response | According to clause 10.1.3 in oneM2M TS-0001 in [1]. |
| Exceptions | According to clause 10.1.3 in oneM2M TS-0001 in [1]. |

### Update <reasoningJobInstance>

This procedure is used for updating the attributes of a <reasoningJobInstance> resource as described in Table 6.15.4-1.

Table 6.15.4-1: <reasoningJobInstance> UPDATE

| <reasoningJobInstance> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 in [1] apply with the specific details for: Content: Attributes of the <reasoningJobInstance> to be updated. |
| Processing at Originator before sending Request | According to clause 10.1.4 in oneM2M TS-0001 in [1]. |
| Processing at Receiver | According to clause 10.1.4 in oneM2M TS-0001 in [1]. |
| Information in Response message | According to clause 10.1.4 in oneM2M TS-0001 in [1]. |
| Processing at Originator after receiving Response | According to clause 10.1.4 in oneM2M TS-0001 in [1]. |
| Exceptions | According to clause 10.1.4 in oneM2M TS-0001 in [1]. |

### Delete <reasoningJobInstance>

This procedure is used for deleting a <reasoningJobInstance> resource as described in Table 6.15.5-1.

Table 6.15.5-1: <reasoningJobInstance> DELETE

| <reasoningJobInstance> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 in [1] apply. |
| Processing at Originator before sending Request | According to clause 10.1.5 in oneM2M TS-0001 in [1]. |
| Processing at Receiver | According to clause 10.1.5 in oneM2M TS-0001 in [1]. |
| Information in Response message | According to clause 10.1.5 in oneM2M TS-0001 in [1]. |
| Processing at Originator after receiving Response | According to clause 10.1.5 in oneM2M TS-0001 in [1]. |
| Exceptions | According to clause 10.1.5 in oneM2M TS-0001 in [1]. |

### -----------------------End of change 1--------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?