| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 42.2 |
| Source:* | Max Zhang, BOE, zhangqiancto@boe.com.cn |
| Date:* | 2019-09-24 |
| Reason for Change/s:* | See introduction |
| CR against: Release* | R4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v4.2.0 |
| Clauses * | 10.2.21.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR is against the security issue of <action> resource mentioned in SDS-2019-0541R01.

In the current <action> resource creating/updating procedure, there is no privilege check procedure of the originator operating against the object resource. This means once the originator gets the privilege to create/update <action> resource, the originator can access all the resource, because the request which the action triggered is sent by the Hosting CSE.

Proposal is to add privilege check procedure of the originator against the object resource in the create/update <action> procedure.

R02- Add privilege check procedure for other attributes (subjectResourceID/input/outputResourceID)

R03- In the case input attribute is set, and the value of this attribute is a URI of a resource attribute, change the target of the privilege check from ‘atribute’ to ‘resource’.

- Delete ‘Impacted other TS/TR(s)’ information.

R04- Move to SDS 42.1.

R05- Remove redundant privilege check procedure already exists in ‘10.1.2/10.1.4 Processing at Receiver’.

- Add ‘Exceptions’ of privilege check failure situation.

### -----------------------Start of change 1-------------------------------------------

#### 10.2.21.3	<action> Resource Procedures

##### 10.2.21.3.1	Create <action>

The hosting CSE shall use this procedure to create an <action> resource according to the resource identifier of the parent resource.

Table 10.2.21.3.1-1: <action> CREATE

| <action> CREATE |  |
| --- | --- |
| Associated Reference Point | Mca, Mcc and Mcc' |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: The resource content shall provide the information as defined in clause 9.6.x |
| Processing at Originator before sending Request | According to clause 10.1.2 |
| Processing at Receiver | According to clause 10.1.2 with the following: - If the subjectResourceID attribute is set, check if the Originator has RETRIEVE privileges on the subject resource. - If the input attribute is set, and the value of this attribute is a resourceID, check if the Originator has RETRIEVE privileges on the resource with the given resourceID. - If the input attribute is set, and the value of this attribute is a URI of a resource attribute, check if the Originator has RETRIEVE privileges on the resource with the given URI. - If the objectResourceID attribute is set, check if the Originator has appropriate privileges on the object resource, associated Operation parameter shall be derived from actionPrimitive attribute. - If the outputResourceID attribute is set, check if the Originator has RETRIEVE privileges on the resource with the given outputResourceID. After successfully creating the <action> resource, the Receiver shall perform the procedure specified in 10.2.21.2. |
| Information in Response message | According to clause 10.1.2 |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | According to clause 10.1.2 with the following: - In case the subjectResourceID attribute is set, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the input attribute is set and the value of this attribute is a resourceID, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the input attribute is set and the value of this attribute is a URI of a resource attribute, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the objectResourceID attribute is set, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the outputResourceID attribute is set, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. |

##### 10.2.21.3.2	Retrieve <action>

This procedure shall be used for retrieving the attributes of an <action> resource.

Table 10.2.21.3.2-1: <action> RETRIEVE

| <action> RETRIEVE |  |
| --- | --- |
| Associated Reference Point | Mca, and Mcc |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.3 |
| Processing at Receiver | According to clause 10.1.3 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | According to clause 10.1.3 |

##### 10.2.21.3.3	Update <action>

This procedure shall be used for updating attributes of an <action> resource.

Table 10.2.21.3.3-1: <action> UPDATE

| <action> UPDATE |  |
| --- | --- |
| Associated Reference Point | Mca |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | According to clause 10.1.4 with the following: - If the subjectResourceID attribute is set, check if the Originator has RETRIEVE privileges on the subject resource. - If the input attribute is set, and the value of this attribute is a resourceID, check if the Originator has RETRIEVE privileges on the resource with the given resourceID. - If the input attribute is set, and the value of this attribute is a URI of a resource attribute, check if the Originator has RETRIEVE privileges on the resource with the given URI. - If the objectResourceID attribute is set, check if the Originator has appropriate privileges on the object resource, associated Operation parameter shall be derived from actionPrimitive attribute. - If the outputResourceID attribute is set, check if the Originator has RETRIEVE privileges on the resource with the given outputResourceID. After successfully updating the <action> resource, the Receiver shall perform the procedure specified in 10.2.21.2. |
| Information in Response message | According to clause 10.1.4 |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | According to clause 10.1.4 with the following: - In case the subjectResourceID attribute is set, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the input attribute is set and the value of this attribute is a resourceID, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the input attribute is set and the value of this attribute is a URI of a resource attribute, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the objectResourceID attribute is set, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. - In case the outputResourceID attribute is set, the Originator does not have the privileges to retrieve the resource with the given resourceID. The Receiver responds with an error. |

##### 10.2.21.3.4	Delete <action>

This procedure shall be used for deleting an <action> resource.

Table 10.2.21.3.4-1: <action> DELETE

| <action> DELETE |  |
| --- | --- |
| Associated Reference Point | Mca, and Mcc |
| Information in Request message | All parameters defined in table 8.1.2-2 apply |
| Processing at Originator before sending Request | According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 After successfully deleting the <action> resource, the Receiver shall stop the event monitoring procedure specified in 10.2.21.2. |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | According to clause 10.1.5 |

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?