| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 43 |
| Source:* | Peter Niblett, IBM |
| Date:* | 2019-11-27 |
| Reason for Change/s:* | Clarifications to Group Request Identifier |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V2.24.0 |
| Clauses * | 7.2.2.2 7.3.3.19 (new clause) 7.4.14.2.4 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Details of the Group Request Identifier  parameter can be found in the introduction to SDS-2019-0654.

This CR addresses two things:

- A mismatch between TS-0001 and TS-0004. TS-0001 states that a Group Hosting CSE shall generate a new Group Request Identifier  if at least one of the targets being fanned out to is itself a <fanOutPoint>, whereas TS-0004 says that a Group Hosting CSE shall generate a new one regardless of whether the targets are <fanOutPoint>s or not.   TS-0001 is more correct, since this parameter is only really needed in that case, however there’s no harm (other than a small performance overhead) in including the parameter on every fan-out operation, so I propose that TS-0004 say that this is permitted.

- All Hosting CSEs have to process the Group Request Identifier when validating any request. At the moment this processing is described in clause  7.4.14.2.4, but this clause is part of <fanOutPoint> and is really about what the Group Hosting CSE has to do when fanning out a request. The CR moves this text to 7.3.3 (generic Hosting CSE actions)

R01. Changed. 7.3.3.19 to say that the Hosting CSE only rejects a request using "GROUP_REQUEST_IDENTIFIER_EXISTS" if it encounters another request that contains a GroupRequestIdentifier that had already been used against the same target resource.

This is to allow the Hosting CSE to process two requests with the same GroupRequestIdentifier provided that they are targetting different resources  (this is prohibited by the current words)

R02.  Reworded 7.3.3.19 to make it more intelligible, and also moved the validation step that it describes to be Recv-6.2.1 rather than Recv-6.1.1

- R03. Reordered clauses so as to avoid adding a new 7.3.3.19

### -----------------------Start of change 1-------------------------------------------

##### 7.4.14.2.4	Fan out Request to each member

If the parent group has no members, the group-hosting CSE shall reject the request with the Response Status Code indicating "NO_MEMBERS".

If the request to be fanned out does not contain a Group Request Identifier already and if any of the target addresses, as found in step a) below, involves a further <fanOutPoint> the group-hosting CSE shall generate a unique group request identifier and store this group request identifier locally. The group-hosting CSE may choose to generate and store a unique group request identifier whenever the request to be fanned out does not already contain one, regardless of whether any targets involve further <fanOutPoint>s.

If the request to be fanned out already contains a Group Request Identifier parameter, the group-hosting CSE shall validate it, as described in clause 7.3.3.19, and if the validation is successful it shall include this parameter value in all the requests that it fans out in step b) below.

If the request contains a Group Request Target Members parameter, and if any of the member IDs in this parameter is not present in the memberIDs list of the parent group or any of its sub-group's memberIDs lists then the request shall be rejected with BAD_REQUEST Response Status Code. Otherwise the group-hosting CSE shall fan out the request to members contained in the Group Request Target Members parameter only.

The group-hosting CSE shall then perform the following steps for each member:

a)	The group-hosting CSE shall execute "Compose Request primitives". The primitive parameters From and To shall be mapped to the primitive parameters of the corresponding Request to be sent out to each member of the group. The primitive parameter From shall be used directly. The primitive parameter To (i.e. <URI of group resource>/fopt) shall be replaced by resource identifiers present in the memberIDs attribute of the group resource. Any additional relative address that was appended to .../fopt in the original Request shall be appended to each To URI. If the group-hosting CSE received a Group Request Identifier in the incoming request, or if it generated a new one as described above, it shall include this Group Request Identifier primitive parameter in every Request.

b)	"Send the Request to the receiver CSE".

c)	"Wait for Response primitives".

The procedures between group-hosting CSE and member-hosting CSEs shall comply with the corresponding procedures as described in clause Error: Reference source not found. The detailed procedures are according to the type of Resource provided in the Request primitive.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### Hosting CSE actions

#### Check existence of the addressed resource

If the Request Expiration Timestamp is given in the request and expired, the Hosting CSE shall reject the request with an "REQUEST_TIMEOUT" Response Status Code parameter value. Otherwise, the Hosting CSE should handle the request before the time specified in Request Expiration Timestamp.

The Hosting CSE shall check if the resource addressed by the To parameter exists in the repository. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

The Hosting CSE shall also check if the conditions specified in the Filter Criteria parameter in the Retrieve/Update/Delete operation are met. If the condition check fails, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

If the Hosting CSE does not support the content format (i.e. type of serialization) requested by the originator, the request shall be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

If the Hosting CSE does not support the content format sent by the originator, the request shall be rejected with a Response Status Code indicating “UNSUPPORTED_MEDIA_TYPE” error.

#### 7.3.3.2	Check for duplicate group requests

If the Hosting CSE receives a request containing a Group Request Identifier parameter it shall check that it has not already received a request that contains the same Group Request Identifier and that is addressed to the same target resource.

To do this, the Hosting CSE shall maintain a list of the group request identifiers from the requests that it has received. With each group request identifier it shall record the identifiers of the resource(s) that were targeted using it.  It shall compare the incoming request’s Group Request Identifier against this list and if it finds a match (of both group request identifier and target resource) it shall reject the request with the Response Status Code indicating "GROUP_REQUEST_IDENTIFIER_EXISTS" error in the Response primitive. Otherwise, the Hosting CSE shall add the Group Request Identifier, and the identifier of the targetted resource, to the list that it is storing locally. It shall then proceed with processing the request.

The Hosting CSE shall keep the group request identifier in its local store until the expiration time of the request or for a period of time determined by local policy.

#### Check validity of resource representation for CREATE

If the request is a valid CREATE request, but the Hosting CSE does not implement the requested resource type, then the Hosting CSE shall reject the request and return an error response with Response Status Code indicating "NOT IMPLEMENTED".

If the CREATE request has a Resource Type that is not listed in the child resource tables, defined in clause Error: Reference source not found corresponding to the addressed resource, then the request shall be rejected with a Response Status Code indicating "INVALID_CHILD_RESOURCE_TYPE" error.

If no resource representation is present in the CREATE request, then the request is rejected with a Response Status Code indicating "BAD_REQUEST" error.

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

There are three cases where the Hosting CSE shall configure or override an expirationTime value that differs from the value specified in the resource representation (if present):

- The Originator does not specify an expirationTime.

- The Originator requests an expirationTime that is later than expirationTime of the parent.

- The Hosting CSE determines the expirationTime requested by the Originator does not meet its requirements (e.g. based on a local policy).

In each of these cases, the Hosting CSE shall configure an expirationTime into the resource that is less than or equal to the expirationTime of the parent resource. In addition, the Hosting CSE shall communicate the modified value back to the originator in the response if the Result Content parameter permits this.

If the creator attribute is present in the resource representation and supported by the type of resource to be created its value shall be NULL. If the originator provides a value for the creator attribute but resource type does not support the creator attribute, then the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error. If the originator provides a value for the creator attribute within the request, the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

The resource descriptions in section Error: Reference source not found include tables that specify the attributes of each resource and the optionality of the attribute in a CREATE or UPDATE request, see Error: Reference source not found. A request containing an attribute not listed in the table shall be rejected with a “BAD_REQUEST” error.

The handling below shall apply to each attribute in the resource for CREATE request primitives and the handling depends on the "presence in CREATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

M attribute for create request

If the attribute is present in the resource representation in the CREATE request, the Hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted, the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

If the attribute is not present in the resource representation in the CREATE request the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

O attribute for create request

If the attribute is present in the resource representation in the CREATE request, the Hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted then the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

NP attribute for create request

If the attribute is present in the resource representation in the CREATE request, the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

#### Generic procedure for handling a Request at a receiver

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an error response" (refer to clause Error: Reference source not found for details) and then "Send Response primitive" (refer to clause Error: Reference source not found for details). The corresponding Response code shall be included in the Response primitive.

Recv-1.0: “Check the validity of received request primitive”

Recv-2.0: Communication method?

Recv-6.0: Resource handling procedures

Recv-3.0: “Create <request> resource locally”

Recv-4.0: “Create a success Response”

Recv-5.0: “Send Response primitive”

nonBlockingRequestAsynch

nonBlockingRequestSynch

Recv-6.0: Resource handling procedures

Recv-7.0: “Update <request> resource”

Finish

Start

Recv-3.0: “Create <request> resource locally”

Recv-4.0: “Create a success Response”

Recv-5.0: “Send Response primitive”

Recv-6.0: Resource handling procedures

Recv-7.0: “Update <request> resource”

Recv-8.0: “Send Notification”

Recv-9.0: “Wait for Response primitive”

blockingRequest

Recv-10.0: “Send Response Primitive”

Figure 7.2.2.2-1: Generic procedure of Receiver

Recv-1.0 "Check the validity of received request primitive": See clause  for details.

Recv-2.0 "Communication method?": The Receiver CSE checks whether a received request is blockingRequest, nonBlockingRequestSynch or nonBlockingRequestAsynch by using Response Type parameter (see detail in clause 8.1.2 in oneM2M TS-0001 [Error: Reference source not found]). If the request is blockingRequest or Response Type parameter is not included, it goes to step Recv-6.0 "Resource handling procedure". If the request is nonBlockingRequestSynch, it goes to step Recv-3.0 "Create <request> resource locally" If the request is nonBlockingRequestAsynch, it goes to step Recv-3.0 "Create <request> resource locally". If the request is flexBlocking, the Receiver CSE shall make the decision to respond using blocking or non-blocking based on its own local context (memory, processing capability, etc.) unless specified further in the resource-specific procedure.

Recv-3.0 "Create <request> resource locally": Please refer to clause Error: Reference source not found for details.

Recv-4.0 "Create a success Response": Please refer to clause Error: Reference source not found for details.

Recv-5.0 "Send Response Primitive": Please refer to clause Error: Reference source not found for details.

Recv-6.0 "Resource handling procedure": Please refer to Figure 7.2.2.2-2 for details.

Recv-7.0 "Update <request> resource": Please refer to clause Error: Reference source not found for details. This step is only valid when the request is non-blocking.

Recv-8.0 "Send Notification": Please refer to clause Error: Reference source not found for details.

Recv-9.0 "Wait for a Response primitive": Please refer to clause Error: Reference source not found for details.

Recv-10.0 "Send Response Primitive": Please refer to clause Error: Reference source not found for details.

Recv-6.10: "Queue request primitive and execute CMDH message forwarding procedure"

Recv-6.1: Hosting CSE of the targeted resource?

Start

Recv-6.3: "Check authorization of the Originator"

Recv-6.2: "Check existence of the addressed resource"

Finish

Recv-6.4: "Check validity of resource representation for the given resource type"

Recv-6.5: "Create/Update/Retrieve/Delete/Notify operation is performed"

Yes

No

Recv-6.6: "Announce/De-announce the resource"

Recv-6.9: CMDH processing supported?

Recv-6.11: "Forwarding"

No

Yes

Recv-6.0.1: Requested operation is an AE registration?

Recv-6.0.2: "Check Service Subscription Profile"

Yes

No

Else

blockingRequest

Recv-6.6.1: "Communication Method?"

Recv-6.7: "Create a success response"

Recv-6.2.1: "Check for duplicate group requests"

Recv-6.10: "Queue request primitive and execute CMDH message forwarding procedure"

Recv-6.1: Hosting CSE of the targeted resource?

Recv-8.0: “Send Notification”

Recv-10.0: “Send Response Primitive”

Recv-6.5: "Create/Update/Retrieve/Delete/Notify operation is performed"

Recv-6.6: "Announce/De-announce the resource"

Finish

Yes

No

Recv-6.7: "Create a success response"

Recv-6.9: CMDH processing supported?

Recv-6.11: “Forwarding”

No

Yes

Recv-6.0.1: Requested operation is an AE registration?

Recv-6.0.2: "Check Service Subscription Profile"

Yes

No

Recv-6.6.1: "Communication Method?"

Else

blockingRequest

Figure 7.2.2.2-2: Resource handling procedure

Figure 7.2.2.2-2 describes the generic procedure to resource handling procedures.

Recv-6.0.1 "Requested operation is an AE registration?": If the requested operation is an AE registration, then it goes to Recv-6.0.2 "Check Service Subscription Profile". Otherwise, it goes to Recv-6.1.

Recv-6.0.2 "Check Service Subscription Profile": Please refer to clause Error: Reference source not found for details.

Recv-6.1 "Hosting CSE of the targeted resource?": The step checks if the receiver is a transit CSE or the Hosting CSE of the received Request by examining the To parameter of the Request primitive. If the receiver hosts the resource that the address in the To parameter represents, the receiver is the Hosting CSE (goes to Recv-6.2" Check ", Yes branch). Otherwise, the receiver is the Transit CSE (goes to Recv-6.9 "CMDH processing supported?", No branch). Please refer to clause Error: Reference source not found for details.

Recv-6.2 "Check existence of the addressed resource": Please refer to clause 7.3.3.1 for details.

Recv-6.2.1 "Check for duplicate group requests": Refer to clause 7.3.3.2 for details.

Recv-6.3 "Check authorization of the Originator": Please refer to clause Error: Reference source not found for details.

Recv-6.4 "Check validity of resource representation": Please refer to clause 7.3.3.3 and clause  for details. Notify is not applicable for this step.

Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed": The step represents five common operations which are "Create the resource (clause Error: Reference source not found)", "Retrieve the resource (clause Error: Reference source not found)", "Update the resource (clause Error: Reference source not found)", "Delete the resource (clause Error: Reference source not found)" and "Notify processing (clause Error: Reference source not found)".

Recv-6.6 "Announce/De-announce the resource": The step represents two common operations which are "Announce the resource" and "De-announce the resource". Please refer to clause Error: Reference source not found and clause Error: Reference source not foundError: Reference source not found for details. Notify is not applicable for this step.

Recv-6.6.1 "Communication method?": The Receiver CSE checks whether a received request is blockingRequest or not by using Response Type parameter (see detail in clause 8.1.2 in oneM2M TS-0001 [Error: Reference source not found]). If the request was blockingRequest or Response Type parameter was not included, it goes to step Recv-6.7 "Create a success response". Otherwise, it goes back to the generic procedure of the receiver (Figure 7.2.2.2-1).

Recv-6.7 "Create a success response": Please refer to clause Error: Reference source not found for details.

Recv-6.9 "CMDH processing supported?": This step checks whether the Receiver supports the CMDH processing. If the receiver supports CMDH processing, it goes to Recv-6.10 "Queue request primitive and execute CMDH message forwarding procedure" otherwise, it goes to Recv-6.11 "Forwarding".

Recv-6.10 "Queue request primitive and execute CMDH message forwarding procedure": the Receiver CSE shall queue the received request primitive and execute the "CMDH message forwarding procedure". Please refer to clause Error: Reference source not found for details.

Recv-6.11 "Forwarding": carry out message forwarding as defined in clause Error: Reference source not found.

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

#### <subscription> resource specific procedure on CRUD operations

##### Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures (Figure 7.2.2.2-1 in clause 7.2.2.2). The additional procedures shall be inserted from Recv-6.2 to Recv-6.8 as below.

The resource handling procedure for the Hosting CSE which receives <subscription> CREATE request shall perform the following procedures in order:

Recv-6.2:

Recv-6.3:

- Check if the Originator has privileges for retrieving the subscribed-to resource. If the Originator does not have the privilege, the Hosting CSE shall return the response primitive with Response Status Code indicating "ORIGINATOR_HAS_NO_PRIVILEGE" error.

Recv-6.4:

- Check if the subscribed-to resource, addressed in the To parameter in the Request, is subscribable. Subscribable resource types are defined in oneM2M TS-0001 [Error: Reference source not found], they have <subscription> resource types as their child resources. If it is not subscribable, the Hosting CSE shall return the Notify response primitive with a Response Status Code indicating "TARGET_NOT_SUBSCRIBABLE" error instead of the Response Status Code "INVALID_CHILD_RESOURCE_TYPE".

- If any of the notificationURI entries are not the Originator, the Hosting CSE may send a Notify request primitive to each of them with verificationRequest parameter set as TRUE (see clause Error: Reference source not found):

a)	If the Hosting CSE cannot send one or more Notify request primitive, the Hosting CSE shall return the response primitive with a Response Status Code indicating "SUBSCRIPTION_VERIFICATION_INITIATION_FAILED" error.

b)	If the Hosting CSE sent all the Notify request primitives, the Hosting CSE shall check if each Notify response primitive contains a Response Status Code indicating "OK". If not, the Hosting CSE shall return the Create response primitive containing the Response Status Code from the first unsuccessful Notify response primitive.

- If the Originator provides a value of childResourceType which is not a valid child of the subscribed-to resource, then the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If the Originator provides missingData,check that subscribed-to-resource is of type <timeSeries>. If not, the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If both the notificationEventType and operationMonitor are present in the Request, the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If the notificationContentType is invalid for a given operation (refer to oneM2M TS-0001 [Error: Reference source not found] Table 9.6.8-4: Default and allowed values of notificationContentType) the request shall be rejected with a "BAD_REQUEST" Response Status Code.

Recv-6.5:

- If the Originator does not provide notificationContentType, the Hosting CSE shall set it as "all attributes".

- If the notificationURI is not the Originator, the Hosting CSE shall set the Originator’s ID as the creator attribute.

- If the batchNotify attribute is present in the Request but batchNotify/duration is not provided by the Originator, the Hosting CSE shall set the value of batchNotify/duration to the default duration as given by the M2M Service Provider.

Recv-6.6.

Recv-6.7.

Recv-6.8.

### -----------------------End of change 4-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?