| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#44 |
| Source:* | Kenichi Yamamoto, KDDI, kc-yamamoto@kddi.com |
| Date:* | 2020-02-07 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active MNT maintenance / < WI-0058> Is this a mirror CR? Yes No mirror CR number: (SDS-2020-0020) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 3GPP Interworking v3.3.0 |
| Clauses * | 8.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR is the Release 3 mirror of SDS-2019-0020.

This CR proposes to address error handling of Monitoring Event API for the SCEF response. The error handling procedures and corresponding error codes defined in 3GPP TS29.122-f60 are described as follow (highlighted with yellow):

#### 4.4.2.2	Monitoring Events Configuration

##### 4.4.2.2.1	General

Upon receipt of the HTTP POST or PUT request message, if the SCS/AS is authorized to perform the request, the SCEF shall check whether the parameters (e.g. Maximum Number of Reports, Monitoring Duration, Maximum Latency, Maximum Response Time, Suggested number of downlink packets) in the HTTP request body are within the range defined by operator policies, if one or more of these parameters are not within the range, the SCEF shall:

-	either reject the request message by sending an HTTP response to the SCS/AS with a status code set to 403 Forbidden and may indicate the "PARAMETER_OUT_OF_RANGE" error in the "cause" attribute of the "ProblemDetails" structure and indicate which parameters are out of the range in the "invalidParams" attribute of the "ProblemDetails" structure; or

-	modify the parameters which are not within the range by selecting different values which are in the range.

For individual UE configuration request, the SCEF shall also check whether the Idle Status Indication is included for UE reachability event, if the Idle Status Indication is received in the request but not supported by the network, the SCEF may reject the request message by sending an HTTP response to the SCS/AS with a status code set to 403 Forbidden, and may indicate the "IDLE_STATUS_UNSUPPORTED" error in the "cause" attribute of the "ProblemDetails" structure.

Table 5.3.5.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| PARAMETER_OUT_OF_RANGE | 403 Forbidden | Indicates that the resource is not allowed to be created since one or more of the received parameter are out of range defined by operator policies. |
| IDLE_STATUS_UNSUPPORTED | 403 Forbidden | Indicates that the resource is not allowed to be created since the Idle Status Indication is received in the request but not supported by the network. |
| OPERATION_PROHIBITED | 403 Forbidden | Indicates the HTTP method is not supported. |
| EVENT_FEATURE_MISMATCH | 400 Bad Request | Indicates the resource creation is not allowed since the supported feature corresponding to the monitoring event is not supported by the client. |
| EVENT_UNSUPPORTED | 500 Internal Server Error | Indicates the required monitoring event is not supported by the server. |

### -----------------------Start of change 1-------------------------------------------

## 8.3	Error Handling

### 8.3.1	Overview

An IN-CSE that interworks to a 3GPP SCEF via the T8 reference point shall support handling the following types of error conditions:

        - Unavailable SCEF – If the IN-CSE sends a request to the SCEF, but the SCEF does not respond back, the IN-CSE shall be robust to this scenario.  At a minimum, the IN-CSE shall support timing out the request.  The IN-CSE may support retrying requests but these details are out of scope of the present document.

        - Errors returned by the SCEF to the IN-CSE within T8 responses -  Table 8.3-1 provides a list of the supported T8 mandatory error response codes applicable to all T8 APIs defined by 3GPP 29.122 [4].  -40x response codes indicate a problem due to malformed requests from the IN-CSE that need to be addressed.   -50x response codes typically indicate a problem with the SCEF that needs to be addressed.

Table 8.3.1-1: T8 Error response codes

| Response Codes | Remarks |
| --- | --- |
| 400 Bad Request | Incorrect parameters were passed in the request issued by the IN-CSE. |
| 401 Unauthorized | The IN-CSE is not authorized to issue request to SCEF. |
| 403 Forbidden | This represents the case when the SCEF is able to understand the request but unable to fulfil the request due to errors (e.g. the requested parameters are out of range). |
| 404 Not Found | The resource URI was incorrect, for instance because of a wrong “scsAsId” field. |
| 411 Length Required | The code indicates that the SCEF refuses to accept the request without a Content-Length header field. |
| 413 Payload Too Large | The request contains a payload larger than the SCEF is able to process. |
| 415 Unsupported Media Type | The code indicates that the resource is in a format which is not supported by the SCEF for the method. |
| 429 Too Many Requests | The code indicates that due to excessive traffic which, if continued over time, may lead to (or may increase) an overload situation. The HTTP header field "Retry-After" may be added in the response to indicate how long the IN-CSE has to wait before making a new request. |
| 500 Internal Server Error | The SCEF encountered an unexpected condition that prevented it from fulfilling the request. |
| 503 Service Unavailable | The SCEF is unable to handle the request. |

NOTE: Depending on the specific procedure, the IN-CSE performs different error handling actions.  These procedure specific error-handling actions are defined in the respective sub-clauses of this specification.

### 8.3.2	Error handling for Monitoring events

#### 8.3.2.1	Internal Server Error

If a SCEF receives a Monitoring event request to create a subscription resource that it does not support, the SCEF will reject the request by sending a "500 Internal Server Error" HTTP error response with the application error "EVENT_UNSUPPORTED".

Figure 8.3.2.1-1 shows an example of the error handling procedure for the case where the SCEF doesn’t support the event requested by the SCS (IN-CSE). The SCEF supports the monitoring type of UE Reachability and does not support the monitoring type of Location Reporting. If an IN-CSE sends a Monitoring Event Subscription Request with the monitoring type of Location Reporting, the SCEF will reject the request by sending the above error response with the application error.

If the IN-CSE receives the error response from the SCEF, the IN-CSE may obtain the location information of an ASN/MN-CSE or ADN-AE by using Device-based method or Sharing-based method configured with locationSource attributes of <locationPolicy> resource based on the IN-CSE local policy.

Figure 8.3.2.1-1: Error handling procedures for Location Reporting of Monitoring events

Figure 8.3.2.1-2 shows another example of the error handling procedure. The SCEF supports the monitoring type of UE Reachability and does not support the monitoring type of UE Loss of Connectivity. If an IN-CSE sends a Monitoring Event Subscription Request with the monitoring type of UE Loss of Connectivity, the SCEF will reject the request by sending the above error response with the application error.

If the IN-CSE receives the error response from the SCEF, the IN-CSE may retry the request with the monitoring type of UE Reachability for managing unreachable devices, based on the IN-CSE local policy.

Figure 8.3.2.1-2: Error handling procedures for UE Loss of Connectivity of Monitoring events

#### 8.3.2.2	Forbidden

If a SCEF receives a Monitoring event request to create or update a subscription resource, the SCEF will check whether the parameters (e.g. Maximum Number of Reports) in the request body are within the range defined by MNO policies. If one or more of these parameters are not within the range, the SCEF may reject the request by sending a "403 Forbidden " HTTP error response with the application error "PARAMETER_OUT_OF_RANGE".

Figure 8.3.2.2-1 shows an example of the error handling procedure. The SCEF supports the monitoring type of Communication Failure and maximumNumberOfReports parameter is set to “10” defined by MNO policies. If the IN-CSE sends a Monitoring Event Subscription Request for Communication Failure with maximumNumberOfReports parameter set to “20”, the SCEF will reject the request by sending the above error response with the application error.

If the IN-CSE receives the error response from the SCEF, the IN-CSE may retry the request with the parameters set within the range (e.g. 8), based on the IN-CSE local policy.

Figure 8.3.2.2-1: Error handling procedures for Communication Failure of Monitoring events

If the SCEF receives a Monitoring event request to create or update a subscription resource, the SCEF will check whether the Idle Status Indication is included for UE reachability event. If the Idle Status Indication is received in the request but not supported by the network, the SCEF may reject the request by sending sending a "403 Forbidden" HTTP error response with the application error "IDLE_STATUS_UNSUPPORTED" in the "cause" attribute of the "ProblemDetails" structure.

Figure 8.3.2.2-2 shows an example of the error handling procedure. The SCEF supports the monitoring type of UE Reachability and does not support to send notifications to an IN-CSE when the UE transitions into PSM idle mode, connected mode or receives an eDRX paging occasion. If the IN-CSE sends a Monitoring Event Subscription Request for UE Reachability with idleStatusIndication parameter set to “TRUE”, the SCEF will reject the request by sending the above error response with the application error.

If the IN-CSE receives the error response from the SCEF, the IN-CSE may retry the request with idleStatusIndication parameter set to “FALSE” for one-time monitoring request, based on the IN-CSE local policy.

Figure 8.3.2.2-2: Error handling procedures for UE Reachability of Monitoring events

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?