| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 44 |
| Source:* | Seongyun Kim, KETI Seokjun Lee, KETI Jiho Lee, KETI |
| Date:* | 2020-02-21 |
| Reason for Change/s:* | Clarification of Impersonation Prevention |
| CR against: Release* | Release 3 |
| CR against: WI* | Active – WI-0079 - Small Technical enhancements of oneM2M Release 4 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0003 V3.11.0 |
| Clauses * | 7.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The contribution proposes

- To extend ESPrim to be used for AE or CSE impersonation prevention

- Adding note saying why AE impersonation verification is not applicable for CSE Impersonation

### -----------------------Start of change 1-------------------------------------------

## 7.2	Impersonation Prevention

### 7.2.1	Registrar verification of AE-ID

Since several AEs can behave maliciously and pretend to be another AE with their ID changed, the Hosting CSE needs prevention mechanism for AE impersonation. This mechanism works at Registrar CSE since Registrar CSE is an entry point of M2M system.

When the Registrar CSE receives a request, the Registrar CSE shall perform the following procedure.

Figure 7.2.1-1: AE impersonation checking procedure

0.	Security association establishment may be performed. Clause 6.1.2.2.1 describes the scenarios when security association establishment between an AE and CSE is mandatory, and describes the scenarios when security association establishment between an AE and CSE is recommended. The subsequent procedures shall be performed if a security association has been established.

1.	The AE sends a request to Hosting CSE via its Registrar CSE as specified in oneM2M TS-0001 [Error: Reference source not found] (Hosting CSE is not represented on this figure and can either be the Registrar CSE or another CSE).

2.	The Registrar CSE checks if the value in the From parameter is the same as the ID associated in security association:

3.	If the values are not identical, then the Registrar CSE shall send a response with Response Status Code '4106' ("ORIGINATOR_HAS_NOT_REGISTERED").

4.	If the values are identical, then the Registrar CSE shall perform the procedures specified in clause 8.2 of oneM2M TS-0001 [Error: Reference source not found]. Depending on the number of Transit CSEs, the Registrar CSE shall either process the request or forward it to the Hosting CSE or to another Transit CSE.

Note: This impersonation verification procedure is not applicable for CSE. This is because when a Transit CSE forwards a request to another CSE, the From parameter of the request is the identifier of the Originator which is different from the identifier of the Transit CSE.

### 7.2.2	Verification Using End-to-End Security of Primitives (ESPrim)

End-to-End Security of Primitives (ESPrim), clause 8.4, allows a Target (a Hosting CSE or AE) to authenticate the Originator of a request primitives that are processed by Transit CSEs. ESPrim also provides confidentiality and integrity protection of these request and response primitives. The primitives being protected are called the inner primitives. ESPrim encryption is applied to the inner primitives to form ESPrim Objects. Outer primitives are used to transport the ESPrim objects between the Originator and Target CSE or AE. The Originator's Registrar cannot view the encrypted inner primitive, and cannot verify that the From parameter of the inner primitive is correct. Instead, the Target is expected to verify that the From parameter of the inner primitive agrees with the authenticated identity of the Originator.

When the Target receives an ESPrim-protected request, the Target shall perform the following procedure.

Figure 7.2.2-1: Impersonation checking procedure

0.	The Target and Originator have previously established a symmetric pairwiseESPrimKey. The Target associates an identity with the symmetric pairwiseESPrimKey.

1.	The Originator composes the inner request primitive, encrypts it using ESPrim to form an ESPrim Object, and sends it to the Target as described in clause 8.4.

NOTE:	Regardless of whether ESPrim is applied, each Mcc "hop" is always protected using an SAEF, and each Mca "hop" is optionally protected using an SAEF; see clause 6.1.2.2.1.

2.	The Target applies the procedures in clause 8.4 to decrypt the ESPrim Object and obtain the inner request primitive.

3.	The Target checks if the value in the From parameter is the same as the ID associated with the pairwiseESPrimKey:

4.	If the values are not identical, then the Target shall send a response with Response Status Code '4116' ("ESPRIM_IMPERSONATION_ERROR").

5.	If the values are identical, then the Target shall record that the Originator has been authenticated, and performs procedures specified in clause 8.2 of oneM2M TS-0001 [Error: Reference source not found].

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?