| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 44.3 |
| Source:* | Cyrille Bareau, Orange, cyrille.bareau@orange.com Marianne Mohali, Orange, marianne.mohali@orange.com Bob Flynn, Chordant, Bob.Flynn@chordant.io Andreas Kraft, Deutsche Telekom, Andreas.Kraft@t-systems.com |
| Date:* | 2020-03-24 |
| Reason for Change/s:* | See the introduction below |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0084 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 v.4.0.0 |
| Clauses * | 6.3.5.71, 7.4.37.2.1, 7.4.37.2.3, 7.4.68 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The originator attribute is added to the <flexContainerInstance> resource.

It takes the value of the originator of the request on the parent <flexContainer> resource, CREATE or UPDATE, that caused the creation of this <flexContainerInstance> resource.

It therefore allows tracking what are the entities that create or modify a <flexContainer> resource through its history.

### ********************* Start of Change 1 ******************************************

#### 6.3.5.71	m2m:flexContainerInstanceResource

This data type is used as base type of all <flexContainerInstance> children of <flexContainer> specializations listed in clause 9.6.1.2.2 of oneM2M TS-0001 [Error: Reference source not found]. It includes the attributes commonly used by all <flexContainerInstance> resource type specializations.

It consists of the common attributes shown in Error: Reference source not found and the resource specific attributes shared by all <flexContainerInstance> specializations shown in Error: Reference source not found.

Table 6.3.5.xa-1: Type Definition of m2m:flexContainerInstanceResource

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| @resourceName | m2m:resourceName | 1 |  |
| resourceType | m2m:resourceType | 1 |  |
| resourceID | m2m:ID | 1 |  |
| parentID | m2m:nhURI | 1 |  |
| creationTime | m2m:timestamp | 1 |  |
| expirationTime | m2m:expirationTime | 1 |  |
| labels | m2m:labels | 0..1 (L) |  |
| contentSize | xs:nonNegativeInteger | 1 |  |
| originator | m2m:ID | 1 |  |

### ********************* End of Change 1 ***************************************

### ********************* Start of Change 2 ******************************************

##### 7.4.37.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the received resource containerDefinition attribute. If the schema is not available then the Hosting CSE shall return a response primitive with a Response Status Code indicating "SPECIALIZATION_SCHEMA_NOT_FOUND" error. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- The Hosting CSE shall set the contentSize attribute to the sum of the size in bytes of all of the custom attributes.

- If the maxNrOfInstances attribute is present in the request with a non-zero value and the maxByteSize attribute is present in the request and the contentSize attribute is greater than the maxByteSize attribute, the Hosting CSE shall not create the resource. It shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error and skip step 3 below.

- If the created resource maxNrOfInstances, maxByteSize or maxInstanceAge attributes are present with non-zero value, then the Hosting CSE shall create a <flexContainerInstance> resource child of the targeted <flexContainer> resource. The contentSize attribute of the created <flexContainerInstance> resource will have the same values as in the created <flexContainer> resource. The originator attribute of the created <flexContainerInstance> resource shall be set to the value of the originator of this CREATE request. The custom attributes of the created <flexContainer> resource shall be copied into the created <flexContainerInstance> resource. The currentNrOfInstances and currentByteSize attributes of the <flexContainer> resource shall be respectively set to 1 and the value of the contentSize attribute..

No other changes from the generic procedures in clause Error: Reference source not found.

### ********************* End of Change 2 ***************************************

### ********************* Start of Change 3 ******************************************

##### 7.4.37.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the resource containerDefinition attribute. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- If at least one of the maxNrOfInstances, maxByteSize or maxInstanceAge is set to zero value, all children <flexContainerInstances> resources shall be deleted and the currentNrOfInstances, currentByteSize and policy attributes shall be deleted.

- 2)	If a custom attribute of the targeted <flexContainer> resource is modified in the request, and maxByteSize attribute is present with non-zero value, the Hosting CSE shall check whether the size in bytes of the custom attributes of the targeted <flexContainer> resource is greater than the maxByteSize of attribute.

- If true, the Hosting CSE shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error and skip step 3 below.

- If false, the Hosting CSE shall update the contentSize attribute to the sum of the size in bytes of all of the custom attributes.

3)	If a custom attribute of the targeted <flexContainer> resource is modified in the request, and maxNrOfInstances, maxByteSize or maxInstanceAge attributes are present with non-zero value, then after updating the attributes of the targeted <flexContainer> resource:

a)	The Hosting CSE shall create a <flexContainerInstance> resource child of the targeted <flexContainer> resource. The attribute contentSize of the created <flexContainerInstance> resource will have the same value as in the targeted <flexContainer> resource. The custom attributes of the targeted <flexContainer> resource shall be copied into the created <flexContainerInstance> resource. The originator attribute of the created <flexContainerInstance> resource shall be set to the value of the originator of this UPDATE request.

b)	The Hosting CSE shall update the currentNrOfInstances of the targeted <flexContainer > resource with the number of <flexContainerInstance> resources in the targeted <flexContainer > resource. The Hosting CSE shall update the currentByteSize of the targeted <flexContainer > resource with the sum of the contentSize attributes of the <flexContainerInstance> resources in the targeted <flexContainer > resource.

c)	If maxNrOfInstances of the targeted <flexContainer > resource is specified then if the currentNrOfInstances when modified to reflect the addition of the new <flexContainerInstance> exceeds maxNrOfInstances, the Hosting CSE shall remove the oldest <flexContainerInstance> resource from the targeted <flexContainer> resource.

d)	If maxByteSize of the targeted <flexContainer > resource is specified then if the currentByteSize when modified to reflect the addition of the new <flexContainerInstance> exceeds maxByteSize the Hosting CSE shall remove the oldest <flexContainerInstance> resource(s) from the targeted <flexContainer> resource until maxByteSize conditions are met.

e)	When removing the oldest <flexContainerInstance> resources, the Hosting CSE shall not generate notifications even if there exists a <subscription> to the targeted <flexContainer> resource and this <subscription> is configured to generate a notification on "Delete_of_Direct_Child_Resource".

f)	If the maxInstanceAge attribute is present in the targeted <flexContainer> resource, then the Hosting CSE shall set the expirationTime attribute in created <flexContainerInstance> child resource such that the time difference between expirationTime and the creationTime of the <flexContainerInstance>. The <flexContainerInstance> child resource shall not exceed the maxInstanceAge of the targeted <flexContainer> resource.

No other changes from the generic procedures in clause Error: Reference source not found.

### ********************* End of Change 3 ***************************************

### ********************* Start of Change 4 ******************************************

### 7.4.68	Resource Type <flexContainerInstance>

#### 7.4.68.1	Introduction

This resource represents a copy of the custom attributes of its parent <flexContainer>.

The detailed description can be found in clause 9.6.59 in oneM2M TS-0001 [Error: Reference source not found].

The following resource types are allowed to include <flexContainerInstance> specializations as children: <flexContainer>.

Table 7.4.68.1-1: Resource Specific Attributes of <flexContainerInstance> resource

| Attribute Name | Data Type | Default Value and Constraints |
| --- | --- | --- |
| contentSize | xs:nonNegativeInteger | No default |
| originator | m2m:ID | No default |
| [customAttribute] | Name and data type are defined in the specification document or XSD file identified by the value of containerDefinition attribute. | No default |

### ********************* End of Change 4 ***************************************