| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 44.3 |
| Source:* | Preeti Rani (preetir@cdot.in), Poornima Shandilya (poornima@cdot.in), Anupama Chopra (anupama@cdot.in) |
| Date:* | 2019-03-23 |
| Reason for Change/s:* | Addition of new attribute SubjectAltName into <authenticationProfile> |
| CR against: Release* | Release 4 |
| CR against: WI* | Active – WI-0083 - oneM2M Service Subscribers and Users MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0022-V4_1_1 |
| Clauses * | 7.1.4, 7.2.4 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

According to TS-0022, The [authenticationProfile] specialization of the <mgmtObj> is used to convey the configuration information regarding establishing mutually-authenticated secure communications.

The [authenticationProfile] identifies a security framework, credentials to be used. Currently, it has attributes such as symmKeyID and symmKeyValue required for PSK based authentication, MAF related attributes required for MAF based SAEF but it has missing attribute required for certificate based SAEF.

In the certificate based SAEF, the entity B must have trust anchor information and prior the knowledge of subjectAltName used in the certificate as highlighted below

Refer TS-0003 Section

8.1.2.4

#### 8.1.2.4 Information Needed for Certificate Authentication of another Entity Entity A shall be configured to trust the following information in order to authenticate Entity B using the certificate-Based SAEF: - An indication of the public key certificate flavour of other Entity B's Certificate (that is, raw public key certificate, device certificate, CSE-ID certificate or FQDN certificate). - In the case where Entity B's certificate is a raw public key certificate: - A public key identifier for the raw public key in the certificate (see clause 10.1.2 "Public Key Identifiers"). - In the case where other Entity B's certificate is a device certificate, CSE-ID certificate or FQDN certificate: - A Globally unique identifier: The globally unique identifier for the entity which is also present in the subjectAltName extension of the other entity's certificate: - Device Certificate: A globally unique hardware instance identifier (such as the object identifier M2M Device ID in Annex H "Object Identifier Based M2M Device Identifier" oneM2M TS-0001 [2]) that is present in the device certificate. - CSE-ID Certificate: The public domain name representation of the CSE-ID as defined in oneM2M TS-0001 [2]. - Trust Anchor Information: For the trust anchor certificates of Entity B's certificate chain (see clause 8.1.2.2 "Path Validation and Certificate Status Verification"). Entity B shall be configured to trust the following information in order to authenticate Entity A using the Certificate-Based SAEF: - An indication of the public key certificate flavour of Entity A's Certificate (that is, raw public key certificate, device certificate, CSE-ID certificate or AE-ID Certificate). - In the case where Entity A's certificate is a raw public key certificate: - A public key identifier for the raw public key in the certificate (see clause 10.1.2 "Public Key Identifiers"). - In the case where Entity A's certificate is an device certificate, CSE-ID certificate or AE-ID certificate: - Trust Anchor Information: for the trust anchor certificate for Entity A's certificate chain (see clause 8.1.2.2 "Path Validation and Certificate Status Verification"). In order to authenticate the M2M Enrolment Function using the certificate-based RSPF, an Enrolee shall be configured to trust the trust anchor information of the M2M Enrolment Function's certificate chain. An M2M Enrolment Function shall be configured to trust the following information in order to authenticate an Enrolee using the certificate-based RSPF: - An indication of the public key certificate flavour of Entity B's Certificate (that is, raw public key certificate or device certificate). - In the case where the Enrolee's certificate is a raw public key certificate: - A public key identifier for the raw public key in the certificate (see clause 10.1.2 "Public Key Identifiers"). - In the case where the Enrolee's certificate is an device certificate, CSE-ID certificate or AE-ID certificate: - A Globally unique identifier: The globally unique identifier which is also present in the subjectAltName extension of the Enrolee's certificate: - Device Certificate: A globally unique hardware instance identifier (such as the object identifier M2M Device ID in Annex H "Object Identifier Based M2M Device Identifier" oneM2M TS-0001 [2]) that is present in the device certificate. - CSE-ID Certificate: The public domain name representation of the CSE-ID as defined in oneM2M TS-0001 [2]. - AE-ID Certificate: The Absolute AE-ID assigned to the AE. - Trust Anchor Information: for the trust anchor certification for the Enrolee's certificate chain (see clause 8.1.2.2 "Path Validation and Certificate Status Verification"). - Also, a device must have prior information of subjectAltName that shall be used for the generation of its own certificate on the device. 2 - 2 - 2

Also, a device must have prior information of subjectAltName that shall be used for the generation of its own certificate on the device.

Refer TS-0003 Section 10.1.1 Certificate Profiles

A subsection for Device Certificates is refered here in this document below:

##### 10.1.1.4.1 Profile for Device Certificates Device certificates shall conform to the following description: - Device certificates shall conform to clause 10.1.1.3 "Details Common to the Certificates with Certificate Chains". - The subjectAltName extension of device certificates shall include one or more globally unique hardware instance identifiers. EXAMPLE: Annex H "Object Identifier Based M2M Device Identifier" oneM2M TS-0001 [2] defines an object identifier -based M2M Device ID that can be used for providing a one or more globally unique hardware instance identifier. An object identifier -based M2M Device ID can be representing in an otherName field in the subjectAltName extension, where: - otherName "type-ID" component is set to the M2M Device Indication ID (clause H.2.1 "M2M Device Indication ID" oneM2M TS-0001 [2]) arc of the object identifier M2M Device ID; and - the otherName "value" component is set to the remainder of the object identifier M2M Device ID: Manufacturer ID arc, Model ID arc, Serial Number ID arc and optional Expanded ID arc (see Annex H.2 "OID Based M2M Device Identifier" oneM2M TS-0001 [2]). NOTE: Providing the Model ID as part of the M2M Device ID can have privacy implications in some scenarios.

Thus, the CR proposes to add a new attribute “subjectAltName” in the [authenticationProfile] resource as this information is required by a device for generation of its certificates.

### -----------------------Start of change 1-------------------------------------------

### 7.1.4	Resource [authenticationProfile]

The [authenticationProfile] specialization of the <mgmtObj> is used to convey the configuration information regarding establishing mutually-authenticated secure communications. The security principal using this configuration information can be a CSE or AE or the Managed ADN/ASN/MN acting as security principal on behalf of AEs on the Node.

An [authenticationProfile] instance identifies a security framework, TLS cipher suites, and credentials to be used. The applicable security framework is identified by the SUID attribute. The interpretation of SUID is specified in Table 7.1.4-3.i.e.

NOTE 1:	The present document does not support using [authenticationProfile] for identifying ESData credentials.

The [authenticationProfile] resource does not include any credentials, but either identifies credentials which are stored locally on the Managed Entity or identifies an M2M Authentication Function (MAF) which is to be used to facilitate establishing symmetric keys. The intended security principal on the Managed Entity is the security principal which can use either all the credentials identified by the [authenticationProfile] resource, or (in the case that a MAF is identified) all of the credentials required for mutual authentication with the MAF.

NOTE 2:	The other security principal can be any of the following: CSE; AE; a Node terminating the security protocol on behalf of AE on Node; and an M2M Authentication Function (MAF).

Figure 7.1.4-1: Structure of [authenticationProfile]

The [authenticationProfile] resource shall contain the child resource specified in table 7.1.4-1.

Table 7.1.4-1: Child resources of [authenticationProfile] resource

| Child Resources of [authenticationProfile] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 of oneM2M TS-0001 [2] |

The [authenticationProfile] resource shall contain the attributes specified in table 7.1.4-2.

Table 7.1.4-2: Attributes of [authenticationProfile] resource

| Attributes of [authenticationProfile] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| resourceID | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| resourceName | 1 | WO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| parentID | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| expirationTime | 1 | RW | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| creationTime | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| labels | 0..1(L) | RW | See clause 9.6.1.3 of oneM2M TS-0001 [2]. |
| mgmtDefinition | 1 | WO | See clause 9.6.15 of oneM2M TS-0001 [2]. This attribute shall have the fixed value 1022 ("authenticationProfile"). |
| objectIDs | 0..1 (L) | WO | See clause 9.6.15 of oneM2M TS-0001 [2]. |
| objectPaths | 0..1 (L) | WO | See clause 9.6.15 of oneM2M TS-0001 [2]. |
| description | 0..1 | RW | See clause 9.6.15 of oneM2M TS-0001 [2]. |
| SUID | 1 | WO | Describes how the authentication profile is to be used. Further details about interpretation of each SUID are specified in Table 7.1.4-3 of the present document. |
| TLSCiphersuites | 0..1(L) | RW | If the security framework identified by SUID uses TLS, then this attributes provides a list of allowed TLS cipher suites. |
| symmKeyID | 0..1 | WO | Present when a symmetric key is to be used for mutual authentication. Identifier for a symmetric key already stored locally on the Managed Entity, or to be provisioned to the Managed Entity |
| symmKeyValue | 0..1 | WO | Optionally present when a symmetric key is to be used for mutual authentication. Contains the value of the symmetric key to be used for mutual authentication. |
| MAFKeyRegLabels | 0..1(L) | WO | Optionally present when a MAF is to be used to facilitate establishing a symmetric key for mutual authentication. Provides the content of the labels parameter in the MAF Key Registration request; see Table 8.8.2.7-1, oneM2M TS-0003 [3]. |
| MAFKeyRegDuration | 0..1 | WO | Present when a MAF is to be used to facilitate establishing one or more symmetric keys for mutual authentication. Provides the maximum duration for which an established symmetric key may be used. |
| mycertFingerprint | 0..1 | WO | Present when certificate-based authentication is to be used. Provides a hash value for identifying the certificate to be used by the intended security principal on the Managed Entity to authenticate itself to other security principals. |
| rawPubKeyID | 0..1 | WO | Present when certificate-based authentication is to be used and the other security principal will authenticate itself with a Raw Public Key Certificate. |
| M2M-Sub-ID | 0..1 | WO | Provides ID of the Service Subscription profile to which this authentication profile is linked. |
| subjectAltName | 0..1 | WO | Present when certificate-based authentication is to be used. Provides the entity identifier present in the subjectAltName extension of the managed entity’s certificate. |
| mgmtLink | 0..1(L) | RW | Present when MAF is to be used to facilitate establishing one or more symmetric keys for mutual authentication or certificate-based authentication is to be used. In the former case, the list contains one reference to a [MAFClientRegCfg] resource. In the latter case, the list contains one or more references pointing to [trustAnchorCred] resources. |

Table 7.1.4-3: SUID which are currently supported in the [authenticationProfile] resource, along with reference to the authentication procedure in oneM2M TS-0003 [3] and mapping to symmetric key

| Value | Interpretation (see note) | Authentication Procedure in oneM2M TS-0003 [3] | Derived Symmetric Key | DTLS/TLS Notes |
| --- | --- | --- | --- | --- |
| 10 | A pre-provisioned symmetric key intended to be shared with a MEF | 8.3.2.1 | Kpm | See TLS-PSK Profile in clause 10.2.2 of oneM2M TS-0003 [3] |
| 11 | A pre-provisioned symmetric key intended to be shared with a MAF | 8.8.2.2 | Km |  |
| 12 | A pre-provisioned symmetric key intended for use in a Security Associated Establishment Framework (SAEF) | 8.2.2.1 | Kpsa |  |
| 13 | A pre-provisioned symmetric key intended for use in End-to-End Security of Primitives (ESPrim) | 8.4.2 | pairwiseESPrimKey | DTLS/TLS is not used |
| 21 | A symmetric key, provisioned via a Remote Security Provisioning Framework (RSPF), and intended to be shared with a MAF | RSPF: 8.3.1.2 MAF: 8.8.2.2, 8.8.3.1 | Km | See TLS-PSK Profile in clause 10.2.2 of oneM2M TS-0003 [3] |
| 22 | A symmetric key, provisioned via a RSPF, and intended for use in a SAEF | RSPF: 8.3.1.2 SAEF: 8.2.2.1, 9.1.1.1 | Kpsa |  |
| 23 | A symmetric key, provisioned via a RSPF, and intended for use in ESPrim | RSPF: 8.3.1.2 ESPrim: 8.4.2 | pairwiseESPrimKey | DTLS/TLS is not used |
| 32 | A MAF-distributed symmetric key intended for use in a SAEF | MAF: 8.8.2.7, 8.8.3.3 SAEF: 8.2.2.3, 9.1.1.1 | Kpsa | See TLS-PSK Profile in clause 10.2.2 of oneM2M TS-0003 [3] |
| 33 | A MAF-distributed symmetric key intended for use in ESPrim | MAF: 8.8.2.7, 8.8.3.3 ESPrim: 8.4.2 | pairwiseESPrimKey |  |
| 40 | A certificate intended to be shared with a MEF | 8.3.2.2 | NP | See certificate-based TLS profile in clause 10.2.3 of oneM2M TS-0003 [3] |
| 41 | A certificate intended to be shared with a MAF | 8.8.2.2 | NP |  |
| 42 | A certificate intended for use in a Security Associated Establishment Framework (SAEF) | 8.2.2.2 | NP |  |
| 43 | A certificate intended for use in End-to-End Security Certificate-based Key Establishment (ESCertKE) to establish a pairwiseESPrimKey for End-to-End Security of Primitives (ESPrim) | ESCertKE: 8.7 ESPrim: 8.4.2 | NP | For ESCertKE, see certificate-based TLS profile in clause 10.2.3 of oneM2M TS-0003 [3]. For ESPrim, DTLS/TLS is not used |
| NOTE: The interpretation is copied from definition of m2m:suid in oneM2M TS-0004 [4]. The oneM2M TS-0004 [4] description takes precedence. |  |  |  |  |

The Managed Entity shall allow only TLS cipher suites identified in TLSCiphersuites in the TLS Handshakes for a [authenticationProfile] instance. The final column of table 7.1.4-3 provides references to clauses in oneM2M TS-0003 [3] specifying the TLS Profiles to be used with the SUID values. The TLSCiphersuite attribute shall be present only when the value of SUID identifies a security framework that uses TLS or DTLS.

If the value of SUID is 10, 11, 12, 21, 22 or 23, then the symmKeyID attribute shall be present.  The symmKeyID provides the symmetric key identifier for a symmetric key which shall be retrieved from local storage on the Managed Entity for use in the TLS Handshake. The symmetric key value may be configured in the symmKeyValue. Otherwise, the symmetric key, and associated symmetric key identifier, may be provisioned to the Managed Entity before or after the [authenticationProfile] is configured. Pre-provisioning or Remote Security Provisioning Frameworks (RSPFs), specified in oneM2M TS-0003 [3], should be used whenever possible to establish symmetric keys. Special care is recommended to ensure the confidentiality and integrity of the credentials when using the symmKeyValue to configure symmetric keys.

If the value of SUID is 32 or 33, then the MAFKeyRegDuration attribute shall be present, the MAFKeyRegLabels attribute may be present, and a [MAFClientRegCfg] specialization shall be configured as a child of the [authenticationProfile] resource. These attributes provide the configuration controlling how the Managed Entity shall interact with a MAF to establish the symmetric key subsequently used for mutual authentication. The fqdn attribute of the [MAFClientRegCfg] specialization identifies the MAF.

- If the Managed Entity has not already performed MAF Client Registration procedure with the identified MAF, then the MAF shall perform MAF Client Registration procedure in clause 8.8.2.3 of oneM2M TS-0003 [3] using the information in the [MAFClientRegCfg] specialization of the <mgmtObj> specified in clause 7.1.7.

- The Managed Entity shall perform the MAF Key Registration Procedure in clause 8.8.2.7 of oneM2M TS-0003 [3] with the identified MAF, with the parameters of table 8.8.2.7-1 of oneM2M TS-0003 [3] set as follows:

- The MAF-FQDN parameter shall be set to the value of the  fqdn attribute in the [MAFClientRegCfg] specialization which is the child of the [authenticationProfile] resource.

- The expirationTime Parameter shall be set to the time obtained by adding the MAFKeyRegDuration attribute to the present time.

- If MAFKeyRegLabels attribute is present in the [authenticationProfile] resource, then the labels parameter shall be set to the value of the MAFKeyRegLabels attribute. Otherwise, the labels parameter shall not be present.

- The SUID parameter shall be set to the SUID attribute.

- The targetIDs parameter shall be set to the CSE-ID in the [registration] or [dataCollection] resource.

If the value of SUID is 40, 41, 42, or 43, then the mycertFingerprint attribute shall be present, and either the rawPubKeyID attribute shall be present or one or more [trustAnchorCred] specializations shall be configured as children of the [authenticationProfile] resource. The Managed Entity shall use the certificate matching mycertFingerprint to authenticate itself. The hash value portion of mycertFingerprint shall be computed over the X.509 ASN.1 DER encoded certificate:

- If the rawPubKeyID attribute is present, then the Managed Entity shall compare this value against the public key identifier (similar to a certificate fingerprint) generated from the raw public key certificate presented by the other entity, as specified in clause 10.1.2 of oneM2M TS-0003 [3]. If the rawPubKeyID attribute is present, the Managed Entity shall ignore [trustAnchorCred] resource(s) configured as children of the [authenticationProfile].

- If the rawPubKeyID attribute is not present, then the Managed Entity shall use the one or more [trustAnchorCred] resource instance(s) configured as children of the [authenticationProfile] resource instance to retrieve Certificate Authority certificates to be used by the Managed Entity as a trust anchor certificate (also known as a "root CA certificate" or "trust root certificate") when validating the certificate chains provided by other entities. The Managed Entity shall allow the TLS handshake only if the other entity provides a certificate chaining to one of these trust anchors, using the process specified in clause 8.1.2.2 in oneM2M TS-0003 [3].

[authenticationProfile] resources are expected to be protected by a secure environment on the Managed Entity, in order to preserve integrity of the attributes. Optimal protection is provided when the integrity protection of the management protocol message is verified in the secure environment.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.2.4	Resource [authenticationProfile]

#### 7.2.4.1	Introduction

Table 7.2.4.1-1: Data Type Definition of [authenticationProfile]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| authenticationProfile | DCFG-authenticationProfile-v4_0_0.xsd |  |

Table 7.2.4.1-2: Resource specific attributes of [authenticationProfile]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 of oneM2M TS-0004 [4]. | "authenticationProfile" |
| objectID | O | NP | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |
| objectPaths | O | NP | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |
| description | O | O | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |
| SUID | M | NP | m2m:suid | Allowed values are listed in table 7.1.4-3. |
| TLSCiphersuites | O | O | dcfg:listOfTLSCiphersuite |  |
| symmKeyID | O | NP | sec:credentialID |  |
| symmKeyValue | O | NP | xs:hexBinary | The minimum key length is 256 bits. |
| MAFKeyRegLabels | O | NP | m2m:labels |  |
| MAFKeyRegDuration | O | NP | xs:duration |  |
| mycertFingerprint | O | NP | dcfg:niURI or dcfg:nihURI |  |
| rawPubKeyID | O | NP | dcfg:niURI or dcfg:nihURI |  |
| M2M-Sub-ID | O | NP | xs:string |  |
| subjectAltName | O | NP | xs:string |  |
| mgmtLink | O | O | See clause 7.4.15 of oneM2M TS-0004 [4]. |  |

Table 7.2.4.1-3: Child resources of [authenticationProfile] resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.8 of oneM2M TS-0004 [4] |

#### 7.2.4.2	Resource specific procedure on CRUD operations

When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 of oneM2M TS-0004 [4], '<mgmtObj> specific procedures' shall be used. There is no change from the generic procedures in clause 7.2.2 of oneM2M TS-0004 [4] for operations on this resource. oneM2M TS-0005 [5] and oneM2M TS-0006 [6] provide the mapping of these resources into the technology specific protocol data model.

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change n-------------------------------------------

## 9.2	Common and Field Device Configuration specific oneM2M Resource attributes

In protocol bindings, resource attribute names shall be translated into short names of table 9.2-1 and in table 8.2.3-1 of oneM2M TS-0004 [4].

Table 9.2-1: Common and Field Device Configuration specific oneM2M Attribute Short Names

| Attribute Name | Occurs in | Short Name | Notes |
| --- | --- | --- | --- |
| resourceType | All | ty | Defined in oneM2M TS-0004 [4]. |
| resourceID | All | ri | Defined in oneM2M TS-0004 [4]. |
| resourceName | All | rn | Defined in oneM2M TS-0004 [4]. |
| parentID | All | pi | Defined in oneM2M TS-0004 [4]. |
| expirationTime | All | et | Defined in oneM2M TS-0004 [4]. |
| creationTime | All | ct | Defined in oneM2M TS-0004 [4]. |
| labels | All | lbl | Defined in oneM2M TS-0004 [4]. |
| lastModifiedTime | All | lt | Defined in oneM2M TS-0004 [4]. |
| description | All | dc | Defined in oneM2M TS-0004 [4]. |
| mgmtDefinition | All | mgd | Defined in oneM2M TS-0004 [4]. |
| objectIDs | All | obis | Defined in oneM2M TS-0004 [4]. |
| objectPaths | All | obps | Defined in oneM2M TS-0004 [4]. |
| mgmtLink | All | cmlk | Defined in oneM2M TS-0004 [4]. |
| CSE-ID | registration | csi | Defined in oneM2M TS-0004 [4]. |
| CSEBase | registration | cb | Defined in oneM2M TS-0004 [4]. |
| originatorID | registration | oid |  |
| pointOfAccess | registration | poa |  |
| appID | registration | apid |  |
| externalID | registration | eid |  |
| triggerRecipientID | registration | tri |  |
| containerPath | dataCollection | cntp |  |
| reportingSchedule | dataCollection | rpsc |  |
| measurementSchedule | dataCollection | mesc |  |
| SUID | authenticationProfile | suid |  |
| TLSCiphersuites | authenticationProfile | tlcs |  |
| symmKeyID | authenticationProfile | aski |  |
| symmKeyValue | authenticationProfile | skv |  |
| MAFKeyRegLabels | authenticationProfile | mkrl |  |
| MAFKeyRegDuration | authenticationProfile | mkrd |  |
| mycertFingerprint | authenticationProfile | mcfp |  |
| rawPubKeyID | authenticationProfile | rpki |  |
| M2M-Sub-ID | authenticationProfile | m2mSubID |  |
| subjectAltName | authenticationProfile | san |  |
| SUIDs | myCertFileCred | suids |  |
| myCertFileFormat | myCertFileCred | mcff |  |
| myCertFileContent | myCertFileCred | mcfc |  |
| certFingerprint | trustAnchorCred | cfp |  |
| URI | trustAnchorCred | uri | Defined in oneM2M TS-0004 [4]. |
| fqdn | MEFClientRegCfg, MAFClientRegCfg | fq | Defined in oneM2M TS-0032 [7]. |
| adminFQDN | MEFClientRegCfg, MAFClientRegCfg | adfq | Defined in oneM2M TS-0032 [7]. |
| httpPort | MEFClientRegCfg, MAFClientRegCfg | hpt | Defined in oneM2M TS-0032 [7]. |
| coapPort | MEFClientRegCfg, MAFClientRegCfg | cpt | Defined in oneM2M TS-0032 [7]. |
| websocketPort | MEFClientRegCfg, MAFClientRegCfg | wpt | Defined in oneM2M TS-0032 [7]. |

### -----------------------End of change n---------------------------------------------

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0011: "Common Terminology".

[2]	oneM2M TS-0001: "Functional Architecture".

[3]	oneM2M TS-0003: "Security Solutions".

[4]	oneM2M TS-0004: "Service Layer Core Protocol".

[5]	oneM2M TS-0005: "Management Enablement (OMA)".

[6]	oneM2M TS-0006: "Management Enablement (BBF)".

[7]	oneM2M TS-0032: "MAF and MEF Interface Specification".

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?