| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#45 |
| Source:* | Kenichi Yamamoto, KDDI, kc-yamamoto@kddi.com |
| Date:* | 2020-02-07 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active MNT maintenance / < WI-0058> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 3GPP Interworking V4.3.0 |
| Clauses * | 7.4.1, 7.4.5, 7.4.6, 7.4.7 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to inline Monitoring Event API in TS-0026 with 3GPP TS29.122-f60.

The 3GPP TS29.122-f60 defines maxUEAvailabilityTime attribute for UE Reachability monitoring of Monitoring Event API (see Table 1) as a timestamp. However, TS-0026 shows the attribute as duration.

Table 1: Definition of type MonitoringEventReport

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 1) |
| --- | --- | --- | --- | --- |
| maxUEAvailabilityTime | DateTime | 0..1 | If "monitoring-Type" is "UE_REACHABILITY", this parameter may be included to identify the timestamp until which a UE using a power saving mechanism is expected to be reachable for SM delivery. Refer to Subclause 5.3.3.22 of 3GPP TS 29.338 [34]. | Ue-reachability_notification |

The 3GPP TS29.122-f60 defines maximumDetectionTime attribute for UE Loss of Connectivity of Monitoring Event API (see Table 2). However, TS-0026 contains the attribute in other monitoring types, such as IMSI-IMEI(SV) Association, Roaming Status and Location Reporting.

Table 2: Definition of type MonitoringEventSubscription

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 3) |
| --- | --- | --- | --- | --- |
| maximumDetectionTime | DurationSec | 0..1 | If "monitoring-Type" is "LOSS_OF_CONNECTIVITY", this parameter may be included to identify the maximum period of time after which the UE is considered to be unreachable. | Loss_of_connectivity_notification |

R01 updates to remove maxUEAvailabilityTime attribute in clause 7.4.1.

The maxUEAvailabilityTime attribute is included to identify the timestamp until which a UE using a power saving mechanism is expected to be reachable for SM delivery (see Table 1). When a SCS sets ReachabilityType attribute as “SMS” defined in 3GPP TS29.122-f60 (see Table 3), the maxUEAvailabilityTime will be included in the notification from a SCEF.

Table 3: Enumeration ReachabilityType

| Enumeration value | Description | Applicability (NOTE) |
| --- | --- | --- |
| SMS | The SCS/AS requests to be notified when the UE becomes reachable for sending SMS to the UE | Ue-reachability_notification |
| DATA | The SCS/AS requests to be notified when the UE becomes reachable for sending downlink data to the UE | Ue-reachability_notification |

However, an IN-CSE sets ReachabilityType as DATA in Step 3 of clause 7.4.1, and maxUEAvailabilityTime isn’t included in the notification from a SCEF in Step 7 of clause 7.4.1. So the maxUEAvailabilityTime attiribute shall not be included in the notification handling at the IN-CSE in Step 9 of clause 7.4.1.

Note that “ARC-2018-0267R01” which was agreed at ARC37 addresses to remove the maxUEAvailabilityTime attribute in the Step 7 of clause 7.4.1 for the reasons stated above. However, the attribute still remains in the Step 9 of clause 7.4.1 of current TS-0026.

### -----------------------Start of change 1-------------------------------------------

### 7.4.1 	UE Reachability monitoring

The 3GPP SCEF functionality described in 3GPP TS 29.122 [4] supports APIs for monitoring specific events such as UE Reachability status. This allows an SCS to request to receive reports when a device becomes reachable for receiving either SMS or downlink data.  In the 3GPP interworking architecture of oneM2M, the UE hosts an ADN with one or more Aes or an ASN/MN-CSE. The UE Monitoring flow in Figure 7.4.1-1 takes place after the UE has attached to the 3GPP Network and the ADN-AE(s) or ASN/MN-CSE register with the IN-CSE.

Figure 7.4.1 -1: Procedure for UE Reachability monitoring

Step 1: UE attaches to 3GPP network and registers to IN-CSE

The UE attaches to the 3GPP network and the ADN-AE(s) or ASN/MN-CSE hosted on the UE perform the oneM2M registration procedure, as detailed in clause 6.3. The IN-CSE hosts the corresponding <AE> or <remoteCSE> resources for the registree and an associated <node> resource.

Step 2 (Optional): Subscribe to <schedule>

As described in clause 6.3, if at the time of ADN-AE(s) or ASN/MN-CSE registration to the IN-CSE a child <schedule> resource of the <node> resource representing the UE does not exist, the IN-CSE shall create it.  If an IN-AE, ADN-AE(s) or ASN/MN-CSE is interested in receiving notifications when this <schedule> resource is updated by the IN-CSE, based on UE reachability notifications from the underlying 3GPP network, it may subscribe to this <schedule> resource.

Step 3: Request for Monitoring Event Subscription to monitor the UE reachability

The IN-CSE sends a Monitoring Event Subscription request to the SCEF to monitor the reachability of a UE. The creation of one or more subscriptions to the node <schedule> in step 2 may be used to trigger the monitoring request if the networkCoordinated attribute of the <schedule> resource is set to TRUE.

The Monitoring Event Subscription request from the IN-CSE to the SCEF shall comply with Error: Reference source not found3GPP TS 29.122 [4]. The Monitoring Event Subscription request is configured as follows:

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription (API for Monitoring Event Subscription) data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - externalId shall be set to the M2M-Ext-ID of the UE.

  - notificationDestination shall be set to a URI that the SCEF can target UE Reachability notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to UE_REACHABILITY

  - reachabilityType shall be set to DATA

  - idleStatusIndication shall be set to TRUE or FALSE. A setting of TRUE will result in the SCEF sending notifications to the IN-CSE when the UE transitions into PSM idle mode. These notifications will be in addition to any notifications sent to the IN-CSE when the UE transitions into connected mode or receives an eDRX paging occasion.  A setting of FALSE (default) will result in the SCEF not sending notifications when the UE transitions into PSM idle mode.  A setting of TRUE is only applicable for Ues supporting PSM.  How the IN-CSE determines whether a UE supports PSM, eDRX or both is out of scope of the present document and may be configured based on Service Provider and MNO policies.

  - supportedFeatures shall be set to a string value of “2” indicating support for UE-Reachability notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the UE reachability monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency, maximumResponseTime and suggestedNumberOfDlPackets are not supported by the present document and shall not be included.

Step 4: Handling in the 3GPP Network Entities

The SCEF processes the Monitoring Event Subscription request together with the 3GPP network entities, as described in 3GPP TS 29.122 [4].

Step 5: response to Monitoring Event Subscription

If the Monitoring Event Subscription Request is successfully processed, the SCEF responds indicating the request was accepted.  The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 6: Detection of UE changing reachability mode and reporting to SCEF

Later, when the UE transitions to connected mode (for a UE using Power Saving Mode or extended idle mode DRX) or the UE becomes reachable for paging (for a UE using extended idle mode DRX) or the UE transitions to idle mode (for a UE using Power Saving Mode), the 3GPP network entities (e.g. HSS) detect the condition and send a Monitoring Event Report for UE reachability with Idle Status Information including Idle Status Timestamp, Periodic RAU/TAU timer, Active Time, eDRX Cycle Length) to the SCEF.

Step 7: SCEF sends Monitoring Notification to IN-CSE

When the SCEF receives information of status change in step 6, the SCEF creates and sends a Monitoring Notification message for UE reachability to the IN-CSE as specified in 3GPP TS 29.122 [4].

Otherwise, the Monitoring Notification message is sent with appropriate information in accordance with the monitoringType.

The Monitoring Notification report for UE_REACHABILITY is configured as follows:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request.

- The request payload will include a MonitoringNotification (API for Monitoring Notification) data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelInd shall be ignored by the IN-CSE if present since it is not supported by the present document.

  - monitoringEventReports configured with one or more UE Reachability monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [4]:

    - externalIDs configured with the one or more externalId that shall be included the same value of M2M-Ext-ID of the UE at step 3.

    - monitoringType configured with UE_REACHABILITY

    - idleStatusInfo configured with information such as activeTime, edrxCycleLength, suggestedNumberOfDlPackets, idleStatusTimestamp, periodicAUTimer as defined in 3GPP TS 29.122 [4]

    - reachabilityType configured with a type of reachability as defined in 3GPP TS 29.122 [4]

Step 8: UE Reachability Monitoring Notification Response

After receiving a UE Reachability Monitoring Notification, the IN-CSE responds with a 204 NO CONTENT acknowledging the notification.

Step 9: UE Reachability Monitoring Notification Handling at the IN-CSE

The IN-CSE uses the information provided in the UE Reachability Monitoring Event Report as follows:

- If idleStatusInfo information is provided in the report, the IN-CSE shall change the scheduleElement of the UE’s node <schedule> resource such that the duration of the scheduleElement is set to the value of the activeTime parameter configured in the idleStatusInfo.

- Step 10a and 10b: (Optional) Notifications are sent to the entities which subscribed to changes in the <schedule> resources. This may include notifications sent to the ADN-AE(s) or ASN/MN-CSE hosted on the UE, if they have subscriptions to their respective <schedule> resources.

Step 11: Notification of schedule update

Upon receiving notification of <schedule> resource updates, the ADN-AE(s) or ASN/MN-CSE hosted on the UE process the notification and should update their local <schedule> resources (if applicable) with the same values.

Step 12 (Optional): request to delete Monitoring Event Subscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by the IN-CSE when the <node> or <schedule> resources associated with a UE are deleted or when the networkCoordinated attribute of the <schedule> resource affiliated with a UE is set to FALSE.

The request is configured as follows.

- An HTTP DELETE method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId} which is identical to step 3.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} corresponds to the one configured by the SCEF and returned to the IN-CSE when the MonitoringEventSubscription was created.

- The request shall not contain a payload

Step 13 and 14 (Optional): Handling in 3GPP Network Entities and response to delete Monitoring Event Subscription

The SCEF processes the request to delete the Monitoring Event Subscription together with the 3GPP Network Entities.

The SCEF responds to the IN-CSE with a response either a code of 200 OK or 204 NO CONTENT indicating that it has successfully deleted the Monitoring Event Subscription.

### -----------------------End of Change 1 ---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.4.5	Detecting Change of IMSI-IMEI(SV) Association

The 3GPP SCEF Event Monitoring functionality described in 3GPP TS 29.122 [4] supports an API that allows the IN-CSE to be informed when the SIM card of one physical device is placed in another physical device.  This condition is detected by the underlying 3GPP network when the association between the International Mobile Subscriber Identity (IMSI) and International Mobile Equipment Identity (IMEI/IMEISV) changes.

An IN-CSE may request to receive notifications from an underlying 3GPP network when the association between the IMSI and IMEI(SV) changes for a given UE that hosts one or more ASN/MN-CSEs or ADN-Aes registered to the IN-CSE.  Based on this notification, the IN-CSE may then ignore incoming requests from these ASN/MN-CSEs or ADN-Aes.

Figure 7.4.5-1: Change of IMSI-IMEI(SV) Association Monitoring Request and Notification

Pre-conditions:

There is a relationship in place between the IN-CSE and MNO allowing the IN-CSE to request notifications for IMSI-IMEI(SV) association changes.

An ASN/MN-CSE or ADN-AE registers with the IN-CSE and configures the M2M-Ext-ID attribute of its <remoteCSE> or <AE> resource. The IN-CSE examines the M2M-Ext-ID and recognizes that it is associated with an MNO that it has a relationship with.  The relationship allows the IN-CSE to request notifications when the device’s IMSI-IMEI(SV) association changes.

Step 1: IN-CSE requests notification for changes in IMSI-IMEI(SV) association

The IN-CSE determines whether it wants a notification if the IMSI-IMEI(SV) association of a particular UE hosting one or more ASN/MN-CSEs or ADN-Aes changes.  This determination may be based on whether the IN-CSE has an established relationship with the underlying 3GPP network operator that supports this capability and provisioned policies.

Step 2: Monitoring Event Subscription Request

The IN-CSE sends a request to be notified when the device’s IMSI-IMEI(SV) association changes.  The Monitoring Event Subscription Request contains information as specified in 3GPP TS 29.122 [4],Error: Reference source not found which includes the following:

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - externalId shall be set to the M2M-Ext-ID of the UE.

  - notificationDestination shall be set to a URI that the SCEF can target Change of IMSI/IMEI Association notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to CHANGE_OF_IMSI_IMEI_ASSOCIATION

  - supportedFeatures shall be set to a string value of “4” indicating support for Change of IMSI/IMEI Association notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - associationType shall be set to IMEI or IMEISV.  The type that is used is based on IN-CSE policies which may be based on the relationship between the Service Provider and MNO

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency and maximumResponseTime are not supported by the present document and shall not be included.

Step 3: Process Monitoring Event Subscription Request

The SCEF processes the Monitoring Event Subscription request together with 3GPP network entities.

Step 4: Monitoring Event Subscription Response

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 5: Detect change in IMSI-IMEI(SV) association

The underlying 3GPP network entities monitor and detect a IMSI-IMEI(SV) association change and reports Monitoring Event Report to SCEF .

Step 6: Monitoring Notification report

When the SCEF receives information of status change on IMSI-IMEI(SV) association, the SCEF sends a Monitoring Notification to the corresponding notificationDestination  of the IN-CSE that was configured in the Monitoring Event Subcription Request and that contains information as specified in 3GPP TS 29.122 [4]Error: Reference source not found.

Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType.

The Montoring Notification report for CHANGE_OF_IMSI_IMEI_ASSOCIATION includes:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request at step 2.

- The request payload will include a MonitoringNotification data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelind shall be ignored by the IN-CSE if present since it is not supported by the present document

  - monitoringEventReports configured with one or more Loss of Connectivity monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [4]:

    - externalIDs configured with one or more externalID that will be configured with the same value of M2M-Ext-ID of the UE as step 2.

    - monitoringType configured with CHANGE_OF_IMSI_IMEI_ASSOCIATION

    - associationType configured with IMSI-IMEI or IMSI-IMEISV

Step 7: Monitoring Notofication Response

After receiving a Change of IMSI-IMEI Association Monitoring Event Notification, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Step 8: Process Notification

The IN-CSE shall stop servicing requests from any ASN/MN-CSEs or ADN-Aes having an M2M-Ext-ID matching the one indicated in the report.   To block these requests, the IN-CSE may tear down any active security associations (e.g. D/TLS sessions) between the IN-CSE and these ADN-Aes or ASN/MN-CSEs.  In addition, the IN-CSE may also deny new security association establishment requests from any ADN-Aes or ASN/MN-CSEs that have an M2M-Ext-ID matching the one indicated in the report until the restriction is removed via administrative means which are outside the scope of the present document.

Step 9 (Optional): IN-CSE sends request to SCEF to delete Monitoring Event Subscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE.

The request is configured as follows.

- An HTTP DELETE method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId} which is identical to step 2.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} corresponds to the one configured by the SCEF and returned to the IN-CSE when the MonitoringEventSubscription was created.

- The request shall not contain a payload

Step 10 and 11 (Optional): Monitoring Event Subscription Delete Request Handling in the Underlying Network.

The SCEF handles the request to delete Monitoring Event Subscription together with the 3GPP network entities. The SCEF responds to the IN-CSE with a response code of 204 NO CONTENT to acknowledge the Monitoring Event Subscription has been deleted.

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 7.4.6	Roaming Status

The 3GPP SCEF Monitoring functionality described in 3GPP TS 29.122 [4] supports an API to allow an IN-CSE to be informed when the roaming status of a UE in the underlying 3GPP network changes.

An IN-CSE may request to receive indications from an underlying 3GPP network when the roaming status of a 3GPP UE hosting an ASN/MN-CSE or ADN-AE changes.  Based on these indications, the IN-CSE may buffer requests for ASN/MN-CSEs or ADN-Aes that are hosted on roaming Ues and send these requests when they are no longer roaming.  The determination of whether to buffer requests may be based on system policies that are outside the scope of the current document.  An IN-CSE may also make the roaming status of ASN/MN-CSEs or ADN-Aes available to IN-Aes such that they can use this status to determine whether to avoid and or delay communications until they are no longer roaming.

Figure 7.4.6-1: Request for Roaming Status Indications

Pre-conditions:

There is a relationship in place between the IN-CSE and MNO allowing the IN-CSE to request Roaming Status Reports from the underlying 3GPP network.  The method for establishing this relationship is outside the scope of the present document.

An ASN/MN-CSE or ADN-AE registers with the IN-CSE and configures the M2M-Ext-ID attribute of its <remoteCSE> or <AE> resource. The IN-CSE examines the M2M-Ext-ID and recognizes that it is associated with an MNO that it has a relationship with.

The ASN/MN-CSE or ADN-AE or IN-CSE creates a <node> resource. This <node> resource has roamingStatus and networkID attributes.

An IN-AE may subscribe to the IN-CSE to receive notifications when the roaming status of a ASN/MN-CSE or ADN-AE changes.  This subscription is made to the roamingStatus and/or networkID attributes of the <node> resource.

The IN-CSE may be configured with policies to control whether to buffer requests targeting ASN/MN-CSEs or ADN-Aes hosted on 3GPP Ues that are roaming.  The method for configuring these policies is outside the scope of the present document.

Step 1: IN-CSE determines to monitor Roaming Status Request

If the IN-CSE can resolve the M2M-Ext-ID to a SCEF and IN-CSE policies allow for it, the IN-CSE issues a Roaming Status Request.

NOTE:	These IN-CSE policies are outside the scope of the present document.

Step 2: Request Monitoring Event Subscription to monitor Roaming Status

The IN-CSE requests to monitor roaming status reports using Monitoring Event Subscription which has specified in 3GPP TS 29.122 [4] for a particular ASN/MN-CSE or ADN-AE hosted on a UE. The fields of the MonitoringEventSubscription API are populated as follows.

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - externalId shall be set to the M2M-Ext-ID of the UE.

  - notificationDestination shall be set to a URI that the SCEF can target Roaming Status notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to ROAMING_STATUS

  - supportedFeatures shall be set to a string value of “5” indicating support for Roaming Status notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - pLMNIndication shall be set to TRUE or FALSE to indicate whether the IN-CSE wants to know the identity of the PLMN that the UE is attached to.

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency and maximumResponseTime are not supported by the present document and shall not be included.

Step 3: Process Monitoring Event Subscription to monitor Roaming Status

The SCEF processes the Monitoring Event Subscription request together with 3GPP network entities as described in 3GPP TS 29.122 [4].

Step 4: Response for Monitorinf Event Subscription to monitor Roaming Status

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 5: Detect Roaming Status Change in 3GPP network entities

When the 3GPP network entities detect the change of UE roaming status, the 3GPP network entities report Monitoring Event Report to the SCEF.

Step 6: Monitoring Notification for Roaming Status from SCEF to IN-CSE

When the SCEF receives information of roaming status, the SCEF sends a Monitoring Notification to the corresponding notificationDestination of the IN-CSE that was configured in the Monitoring Event Subscription Request and that contains information as specified in 3GPP TS 29.122 [4]Error: Reference source not found.

Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType

The Monitoring Notification report for roaming status includes:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request.

- The request payload will include a MonitoringNotification data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelind shall be ignored by the IN-CSE if present since it is not supported by the present document

  - monitoringEventReports configured with one or more change of Roaming Status monitoring reports wherein each report includes the following fields as defined in 3GPP TS 29.122 [4]:

    - externalIDs configured with one or more externalID that will be configured with the same values of the M2M-Ext-ID of the UE as step 2.

    - monitoringType configured with ROAMING_STATUS

    - plmnId if pLMNIIndication in the Monitoring Event Subscription was set to TRUE, this parameter is included and indicate the UE’s serving PLMN

    - roamingStatus is configured with ROAMING or NOT_ROAMING

Step 7: Response for Monitoring Notification from IN-CSE to SCEF

After receiving a Monitoring Notification on roaming status, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Step 8: Process Monitoring Notification at IN-CSE

The IN-CSE shall update the roamingStatus and networkID attributes of the <node> resource that is linked to <AE> or <remoteCSE> resources having a M2M-Ext-ID attribute that matches the M2M-Ext-ID configured in the Roaming Status Request.

In addition, the IN-CSE may determine to delay or reject the processing of requests targeting ASN/MN-CSEs or ADN-Aes hosted on roaming Ues via one or more of the following approaches.  How the IN-CSE makes this determination is outside the scope of the current document and may be based on policies and agreements with the mobile network operator.

- An IN-CSE may reject requests that target ASN/MN-CSEs or ADN-Aes hosted on roaming Ues.  If an IN-CSE rejects a request due to a roaming UE, it shall return a corresponding response code informing the cause of rejection is due to lack of accessibility to a roaming network node.

- An IN-CSE may delay the processing of requests (i.e. buffer) that target ASN/MN-CSEs or ADN-Aes hosted on roaming Ues.

  - If the request is a blocking request, the IN-CSE should not delay the processing of the request and should instead reject the request with a corresponding response code informing the cause of rejection is due to the destination node is roaming.

  - If the request includes an Event Category that is set to immediate the IN-CSE should not delay the processing of the request and should instead reject the request with a corresponding response code informing the cause of rejection is due to the destination node is roaming.  In this case, the IN-AE may decide to resubmit the request with the Event Category set to “bestEffort” or “latest” to indicate the IN-CSE may buffer the request.

Step 9 (Optional): Request to delete Monitoring Event Subscription on Roaming Status

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE.

The request is configured as follows.

- An HTTP DELETE method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} corresponds to the one configured by the SCEF and returned to the IN-CSE when the MonitoringEventSubscription was created.

- The request shall not contain a payload

Steps 10 and 11 (Optional): Process Roaming Status Cancellation Request

The SCEF handles the request to delete Monitoring Event Subscription together with the 3GPP network entities. The SCEF responds to the IN-CSE with a response code of 204 NO CONTENT to acknowledge the Monitoring Event Subscription has been deleted

### -----------------------End of Change 3 ---------------------------------------------

### -----------------------Start of change 4-------------------------------------------

### 7.4.7	Location Reporting

#### 7.4.7.1 Introduction

The 3GPP SCEF Event Monitoring functionality described in 3GPP TS 29.122 [4] supports an API to allow an IN-CSE to be informed when the 3GPP network entities detect the current location of a UE as well as the last known location of UE.  This clause provides details on how the IN-CSE’s oneM2M <locationPolicy>, <container> and <contentInstance> resources are used to interwork with the T8 API and provide location monitoring capability for an ASN/MN-CSE or ADN-AE hosted on a 3GPP UE. To interwork the 3GPP location reporting functionality with oneM2M, the <locationPolicy>, <container> and <contentInstance> resources and corresponding procedures are used.

#### 7.4.7.2 Location updating triggered by retrieval

Figure 7.4.7.2-1: Location updating triggered by retrieving

Step 1: Request for creation of <locationPolicy>

The IN-AE sends a < locationPolicy> create request which shall include  following parameters as specified in clause 9.6.10 of oneM2M TS-0001[1]:

  - locationSource shall be set to Network Based.

  - locationUpdatePeriod shall not be present or set to 0.

  - locationTargetID shall be set to the M2M-Ext-ID of the targeted UE hosting an ASN/MN-CSE or ADN-AE.

  - locationServer shall not be present.

  - locationContainerID shall not be present.

  - locationInformationType shall be set to position fix.

  - retrieveLastKnownLocation shall be configured with either TRUE or FALSE based on the requirements of the IN-AE.

  - locationUpdateEventCriteria shall not be present.

Step 2:  Responds to creation <locationPolicy>

IN-CSE shall create the <locationPolicy> resource and corresponding <container> resource for storing location information based on the procedure specified in clause 10.2.9 of oneM2M TS-0001[1].

Step 3: Retrieve Request for <latest>

The IN-AE sends a RETRIEVE request to the <latest> child resource of the <container> resource linked to the to <locationPolicy> created in Step 2.

Step 4: Request Monitoring Event Subscription to get Current Location

The IN-CSE checks the locationSource attribute of the <locationPolicy> resource that is linked to the <container> targeted by the RETRIEVE request in Step 3.  If the locationSource attribute is set to Network Based and the locationUpdatePeriod attribute is set to zero or NULL, then the IN-CSE shall request a Monitoring Event Subscription to get current location to the SCEF. The Monitoring Event Subscription request shall contain the following information as specified in 3GPP TS 29.122 [4].

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - externalId shall be set to the locationTargetID.

  - notificationDestination shall be set to a URI that the SCEF can target Location Reporting notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to LOCATION_REPORTING

  - supportedFeatures shall be set to a string value of “3” indicating support for Location Reporting notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - locationType shall be set to CURRENT_LOCATION

  - accuracy shall be set set by the IN-CSE (e.g. CGI_ECGI, ENODEB, TA_RA, PLMN or TWAN_ID)

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency and maximumResponseTime are not supported by the present document and shall not be included.

Steps 5: Process Monitoring Event Subscription to get Current Location

The SCEF handles request for  Monitoring Event Subscription togather with 3GPP network entities based on the procedure defined in 3GPP TS 29.122 [4].

Step 6: Response to Monitoring Event Subscription for Current Location

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId} which is identical to step 4. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 7: Detect Current Location of UE

The 3GPP network entities detect the current location of UE and the SCEF receives a Monitoring Event Report for the location of UE.

Step 8: SCEF sends Monitoring Notification Report to IN-CSE

When the SCEF receives a current location information from the 3GPP network entities, the SCEF sends Monitoring Notification Report to the IN-CSE that contains information as specified in 3GPP TS 29.122 [4]Error: Reference source not found. Otherwise, the MonitoringNotification message is sent with appropriate information in accordance with the monitoringType.

The Monitoring Notification report for LOCATION_REPORTING for current location shall include:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request.

- The request payload will include a MonitoringNotification data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelind shall be ignored by the IN-CSE if present since it is not supported by the present document

  - monitoringEventReports configured Location Reporting monitoring report(s) wherein a report includes the following fields as defined in 3GPP TS 29.122 [4]:

    - externalID configured with  the same value of M2M-Ext-ID of the UE as step 4.

    - monitoringType configured with LOCATION_REPORTING

    - locationInfo configured with the UE’s current location information (e.g. ageOfLocationInfo, cellId, enodeBId, routingAreaId, trackingAreaId, plmmId and twanId)

Step 9: Response to Monitoring Notification for Current Location

After receiving a Monitoring Notification for current location, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Steps 10 and 11: Current Location Event Notification Handling at the IN-CSE

The IN-CSE shall create a new <contentInstance> child resource of the <container> targeted by the RETRIEVE request in Step 3.  The IN-CSE shall store the UE’s current location in this <contentInstance> and send the retrieve response for <latest> which is contains the current location to the IN-AE. The response shall include the newly created <contentInstance>.

Step 12: Request for 2nd attempt Monitoring Event Subscription to get Last Known Location

If the Monitor Indication in Step 6 indicates a failure, then the IN-CSE shall send a Monitoring Event Subscription to the SCEF to get the last known location if the retrieveLastKnownLocation attribute of the <locationPolicy> is set to TRUE.  The Monitoring Event Subscription request shall contain the following information as specified in 3GPP TS 29.122 [4].

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/ which is identical to step 4.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - externalId shall be set to the locationTargetID which shall be identical to step 4.

  - notificationDestination shall be set to a URI that the SCEF can target Location Reporting notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to LOCATION_REPORTING

  - supportedFeatures shall be set to a string value of “3” indicating support for Location Reporting notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - locationType shall be set to LAST_KNOWN_LOCATION

  - accuracy shall be set set by the IN-CSE (e.g. CGI_ECGI, ENODEB, TA_RA, PLMN or TWAN_ID)

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency and maximumResponseTime are not supported by the present document and shall not be included.

Step 13: Process Monitoring Event Subscription to get Last Known Location

The SCEF handles Monitoring Event Subsciption Request togather with 3GPP network entities based on the procedure defined in 3GPP TS 29.122 [4].

Step 14: Response to Monitoring Event Subscription for Last Known Location

The SCEF responds a Monitoring Event Subscription to get Last Known Location information to the IN-CSE to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 15: Detect Last Known Location of UE

The 3GPP network entities detect the last known location of UE and SCEF receives  Monitoring Event Report for the last known location of UE

Step 16: SCEF sends Monitoring Notification for Last Known Location to IN-CSE

When the SCEF receives a last known location information from the 3GPP network entities, the SCEF sends Monitoring Notification Report to the IN-CSE that contains information as specified in 3GPP TS 29.122 [4]Error: Reference source not found. Otherwise, the Monitoring Notification message is sent with appropriate information in accordance with the monitoringType.

The Monitoring Notification report for LOCATION_REPORTING for last known location shall include:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request.

- The request payload will include a MonitoringNotification data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelind shall be ignored by the IN-CSE if present since it is not supported by the present document

  - monitoringEventReports configured Location Reporting monitoring report(s) wherein a report includes the following fields as defined in 3GPP TS 29.122 [4]:

    - externalID configured with the same value of M2M-Ext-ID of the UE as step 12.

    - monitoringType configured with LOCATION_REPORTING

    - locationInfo configured with the UE’s last known location information (e.g. ageOfLocationInfo, cellId, enodeBId, routingAreaId, trackingAreaId, plmmId and twanId)

Step 17: Response to Monitoring Notification

After receiving a Monitoring Notification for last known location, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Steps 18 and 19: Current Location Reporting Event Handling at the IN-CSE

The IN-CSE shall create a new <contentInstance> child resource of the <container> initiated by the RETRIEVE request in Step 3.  The IN-CSE shall store the UE’s last known location in this <contentInstance> and send the retrieve response to IN-AE for <latest> which is contains the last known location. The response shall include the newly created <contentInstance>.

#### 7.4.7.3 Location updating triggered by location change

Figure 7.4.7.3-1: Location updating triggered by location change

Step 1: Request for creation of <locationPolicy>

The IN-AE sends <locationPolicy> CREATE request which shall be included  following parameters as specified in clause 9.6.10 of oneM2M TS-0001[1]:

  - locationSource shall be set to Network Based.

  - locationUpdatePeriod shall not be present or set to 0.

  - locationTargetID shall be set to the M2M-Ext-ID of the targeted UE hosting an ASN/MN-CSE or ADN-AE.

  - locationServer shall not be present.

  - locationContainerID shall not be present.

  - locationInformationType shall be set to position fix.

  - retrieveLastKnownLocation shall not be configured.

  - locationUpdateEventCriteria shall be set to LocationChange

Step 2: Responds to creation <locationPolicy>

IN-CSE shall create the <locationPolicy> resource and corresponding <container> resource for storing location information and return a response based on the procedure specified in clause 10.2.9 of oneM2M TS-0001[1].

Step 3: Request Monitoring Event Subscription to get Current Location

The IN-CSE sends Monitoring Event Subscription to SCEF. The request contains information as specified in 3GPP TS 29.122 [4]. Such information includes :

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - externalId shall be set to the locationTargetID.

  - notificationDestination shall be set to a URI that the SCEF can target Location Reporting notifications towards.  The value of this URI shall be based on internal IN-CSE policies.

  - monitoringType shall be set to LOCATION_REPORTING

  - supportedFeatures shall be set to a string value of “5” indicating support for Location Reporting notifications.

  - maximumNumberOfReports is optional and may be set to a maximum number of event reports to be generated by 3GPP network entities e.g. HSS or MME/SSGN.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire.  If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - locationType shall be set to CURRENT_LOCATION

  - accuracy shall be set set by the IN-CSE (e.g. CGI_ECGI, ENODEB, TA_RA, PLMN or TWAN_ID)

  - msisdn, ipv4Addr, ipv6Addr, externalGroupId, requestTestNotification, websockNotifConfig, groupReportGuardTime, maximumLatency and maximumResponseTime are not supported by the present document and shall not be included.

Step 4: Process Monitoring Event Subscription to get Current Location

The SCEF handles request for Monitoring Event Subscription together with 3GPP network entities based on the procedure defined in 3GPP TS 29.122 [4].

Step 5: Response to Monitoring Event Subscription for Current Location

The SCEF sends a Monitoring Event Subscription Response message to the IN-CSE to acknowledge acceptance of the Monitoring Event Subscription Request.

The message includes the following information.

- A response code of 201 CREATED

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.  The {subscriptionId} segment is configured by the SCEF.

- The response payload will include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - self is configured with a URI to the resource created by the SCEF for the request

  - monitoringEventReport may be included if a monitoring event report is available.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 6: Request for Creation of <subscription>

The IN-AE sends the <subscription> CREATE request of <container> which stores location information to the IN-CSE.

Step 7: Response for Creation of <Subscription>

The IN-CSE sends the response to the IN-AE.

Step 8: Detect Current Location change of UE

The 3GPP network entities send a Monitoring Event Report for the current Indication to SCEF when detecting a change in location of the UE hosting an ASN/MN-CSE or ADN-AE.

Step 9: Report Monitoring Notification for Current Location from SCEF  to IN-CSE

The SCEF sends a Monitoring Notification Report to the IN-CSE that contains information as specified in 3GPP TS 29.122 [4]Error: Reference source not found.

Otherwise, the Monitoring Notification message is sent with appropriate information in accordance with the monitoringType.

The Monitoring Notification report for LOCATION_REPORTING for current location shall include:

- An HTTP POST method is used

- URI is set to {notification_uri}.  The {notification_uri} is configured by the IN-CSE in the Monitoring Event Subscription Request.

- The request payload will include a MonitoringNotification data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - subscription configured with a URI to the subscription resource for which this notification corresponds to

  - configResults is used for group-based monitoring and shall be ignored by the IN-CSE if present since group-based monitoring is not supported by the present document.

  - cancelind shall be ignored by the IN-CSE if present since it is not supported by the present document

  - monitoringEventReports configured Location Reporting monitoring report(s) wherein a report includes the following fields as defined in 3GPP TS 29.122 [4]:

    - externalID configured with the same value of M2M-Ext-ID of the UE as step 3.

    - monitoringType configured with LOCATION_REPORTING

    - locationInfo configured with the UE’s current location information (e.g. ageOfLocationInfo, cellId, enodeBId, routingAreaId, trackingAreaId, plmmId and twanId)

Step 10: Response to Monitring Notification for Current Location

After receiving a Location Reporting Monitoring Notification, the IN-CSE returns a response having a response code of 204 NO CONTENT.

Step 11: Handling Monitoring Notification for Current Location at the IN-CSE

The IN-CSE shall create a new <contentInstance> child resource of the <container> resource created in Step 2.  The IN-CSE shall store the UE’s current location in this <contentInstance> resource.

Step 12: Notify <contentInstance>

IN-CSE sends a NOTIFY request to the IN-AE.  The NOTIFY includes the <contentInstance> with the UE’s current location.

Step 13 (Optional): The IN-AE sends a request to update the locationUpdatePeriod  to 0 or <locationPolicy> is deleted.

Step 14 (Optional): The IN-CSE sends a response to the IN-AE.

Step 15 (Optional): IN-CSE requests to SCEF to delete Monitoring Event Subscription

The IN-CSE sends a DELETE request targeting the URI of the subscription resource corresponding to this Monitoring Event Subscription.  This step may be triggered by IN-CSE when a ADN-AE(s) or ASN/MN-CSE hosted on the UE de-registers from the IN-CSE.

The request is configured as follows.

- An HTTP DELETE method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId} which is identical to step 3.  The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} corresponds to the one configured by the SCEF and returned to the IN-CSE when the MonitoringEventSubscription was created.

- The request shall not contain a payload

Step 16 and 17 (Optional): Monitoring Event Subscription Delete Request Handling in the Underlying Network.

The SCEF handles request to delete the Monitoring Event Subscription together with the 3GPP network entities. The SCEF responds to the IN-CSE with a response code of 204 NO CONTENT to acknowledge the Monitoring Event Subscription has been deleted.

### -----------------------Start of change 4-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?