| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 44 |
| Source:* | Dale Seed, Convida Poornima Shandilya, C-DoT, poornima@cdot.in Anupama Chopra, anupama@cdot.in Prateek Varshney, prateekv@cdot.in Suman, ssheoran@cdot.in Preeti, preetir@cdot.in |
| Date:* | 2020-02-10 |
| Reason for Change/s:* | Add support for service subscripton and service user profiles |
| CR against: Release* | Release 4 |
| CR against: WI* | Active – WI-0083 - oneM2M Service Subscribers and Users MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 V4.3.0 |
| Clauses * | 9.6.25, 10.2.11 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 Rel-4 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

TR-0054 defines a support for the capability to define M2M service profile restrictions for both M2M Service Subscribers and M2M Service Users.  For example, restrictions such as the maximum bytes of data stored in the M2M system or the maximum allowed rate of requests.

M2M service profile restrictions enable a M2M Service Provider to apply restrictions on a M2M Service Subscriber based on the M2M Service Subscription agreement between the M2M Service Provider and the M2M Service Subscriber.

Likewise, M2M service profile restrictions enable a M2M Service Provider and/or a M2M Service Subscriber to apply restrictions on specific M2M Service Users based on the M2M Service Subscription agreement and/or preferences of the M2M Service Subscriber.

R01:

- Changed name of contribution from 2019 to 2020

- Added attribtues to profiles to define max number containers, time series and flexContainers.

- Added exception case for Receiver to validate limts of user profiles do not exceed parent service subscription profile

R02:

- Added C-DOT as contributors

- Changed activationDate and deactivationDate to activationTime and deactivationTime

R03:

- Scaled back on the number and types of profile attributes based on comments received at SDS44

- Improved some of the descriptions for new attributes

- Added status attribute to m2mServiceSubscriptionProfile resource

R04:

- Removed text that was added to state that <m2mServiceSubscriptionProfile> and <serviceSubscribedUserProfile> resources are only used by IN-CSE and not descendant CSEs since this is already covered by the text on line 58 which states “… is only stored on IN-CSE”.

- Replaced activationTime and deactivationTime attributes with an activate attribute.

- Changed the names of several attributes to align the names with the functionality of the attribute (e.g. maxNumContentInstances was changed to maxNumInstancesPerContainer).

- Added maxNumInstancesPerFlexContainer attribute

- Corrected several editorial errors

R05:

Summary of changes:

activate attribute is replaced with activationTime and serviceSubscriptionDuration.

Service Subscription Profile is a contract between M2M Subscriber and M2M Service Provider so with start date and end date it was possible to specify the duration of the contract.

To simplify the date management, instead of two dates, we can just keep one date which is activationTime (which will be non-modifiable) and another can be a duration (which will be modifiable) to indicate the length of contract. So a subscriber can change the duration i.e increase,  but it can not modify the start time of contract i.e. the date from which contract is activated.

Let's take an example of existing platforms e.g. Netflix, AWS etc. when we opt for its services, then there are various plans such as 1 month plan, 6 month plan etc. If we opt for one month plan and pays for it then our contract starts from that time and 1 month becomes the duration of the contract. Even if we unsubscribe before the completion of the duration  , then also the service is provided for the duration. So similarly, in our oneM2M scenario, an M2M service provider may offer various duration plan and a subscriber may opt for same.

### -----------------------Start of change 1-------------------------------------------

### 9.6.19	Resource Type m2mServiceSubscriptionProfile

The <m2mServiceSubscriptionProfile> resource represents an M2M Service Subscription. It is used to represent all data pertaining to the M2M Service Subscription, i.e. the technical part of the contract between an M2M Application Service Provider and an M2M Service Provider and is only stored on IN-CSE. The data is also represented in <serviceSubscribedNode> , <serviceSubscribedAppRule> and <serviceSubscribedUserProfile> resources as well as <m2mServiceSubscriptionProfile> resource. The relationship among those resource types are depicted as follows. Note that the diagram does not capture all attributes and child resources. Those resource types shall only be instantiated on IN-CSE.

Figure 9.6.19-1: Relationship among M2M Service Subscription related resources

The <m2mServiceSubscriptionProfile> resource shall contain the child resources specified in table 9.6.19-1.

Table 9.6.19-1: Child resources of <m2mServiceSubscriptionProfile> resource

| Child Resources of <m2mServiceSubscriptionProfile> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <serviceSubscribedNode> | 0..n | See clause 9.6.20 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |
| [variable] | <serviceSubscribedUserProfile> | 0..n | See clause 9.6.68 |

The <m2mServiceSubscriptionProfile> resource shall contain the attributes specified in table 9.6.19-2.

Table 9.6.19-2: Attributes of <m2mServiceSubscriptionProfile> resource

| Attributes of <m2mServiceSubscriptionProfile> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| M2M-Sub-ID | 1 | WO | The identifier assigned by the M2M Service Provder for this M2M Service Subscription. |
| M2M-SS-ID | 0..1 | RW | The identifier assigned by the M2M Service Provder to the M2M Service Subscriber associated with this M2M Service Subscription. |
| status | 1 | RO | Indicates the active/inactive status of this M2M Service Subscription. The Hosting CSE shall set this attribute to “ACTIVE” when the value of activate attribute is set to TRUE and “INACTIVE” when the value of the activate attribute is set to FALSE. |
| activate | 1 | RW | Used to activate or deactivate this M2M Service Subscription. When this attribute is set to a value of TRUE, the Hosting CSE shall activate this M2M Service Subscription. When this attribute is set to a value of FALSE, the Hosting CSE shall deactivate this M2M Service Subscription. The default value of the attribute is FALSE. The attribute shall be set to value of FALSE by the Hosting CSE on end of serviceSubscriptionDuration. To activate the service subscription again, this attribute shall be set to a value of TRUE. |
| activationTime | 0..1 | RO | M2M Service Subscription Activation Time. It is the time from which an M2M Service Provider activates the services of an M2M subscriber. The value of this attribute shall be set to current time when activate is set to TRUE. |
| serviceSubscriptionDuration | 0..1 | RW | It indicates the duration of the contract between an M2M Subscriber and M2M Service Provider. When a service subscription reaches the end of this duration then contracts between an M2M Service Provider and M2M Subscriber shall end. When an ACTIVE service subscription is deactivated by setting activate to FALSE, the Hosting CSE shall recalculate the duration of time for which the service subscription was active and update the value of this attribute. |
| maxNumAEs | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the maximum number of <AE> resources created on the Hosting CSE to this limit, if configured. If a request is received to create an <AE> resource once this limit is reached, the Hosting CSE shall return an error indicating that the maximum number of allowed <AE> resources has been reached. |
| maxNumNodes | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the maximum number of <node> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <node> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of allowed <node> resources has been reached. M2M Service Subscription |
| maxNumBytes | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the aggregate amount of bytes stored on the Hosting CSE to this limit, if configured. This limit shall be applied to Content Sharing Resources only (i.e. <container>, <contentInstance>, <flexContainer>, <flexContainerInstance>, <timeSeries> and <timeSeriesInstance>). If a request is received to create a Content Sharing Resource that will result in exceeding this limit, the Hosting CSE shall return an error that the maximum number of allowed bytes has been reached. A Hosting CSE may take the value of the maxNumBytes attribute as well as other attributes (e.g. maxNumContainers, maxNumContentInstances, etc.) into account when determining a limit to enforce on the maxByteSize attribute of individual Content Sharing Resources, if applicable. How a Hosting CSE allocates the maxNumBytes across individual Content Sharing Resources is based on local policy. |
| maxNumUsers | 0..1 | RW | Maximum number of allowed M2M Service Users associated with this M2M Service Subscription. |
| maxRequestRate | 0..1 | RW | If a Hosting CSE receives requests for this M2M Service Subscription at a rate which exceeds the limit defined by this attribute, the Hosting CSE may throttle the processing of the requests. The Hosting CSE may throttle processing of requests by delaying responses and/or by returning an error indicating that the maximum allowed request rate has been reached.M2M Service Subscription |
| maxNumContainers | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the maximum number of <container> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <container> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of allowed <container> resources has been reached. |
| maxNumInstancesPerContainer | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the value of the maxNrOfInstances attribute of a <container> resource on the Hosting CSE to this limit, if configured. |
| maxNumTimeSeries | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the maximum number of <timeSeries> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <timeSeries> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of <timeSeries> resources has been reached. |
| maxNumInstancesPerTimeSeries | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the value of the maxNrOfInstances attribute of a <timeSeries> resource on the Hosting CSE to this limit, if configured. |
| maxMembersPerGroup | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the value of the maxNrOfMembers attribute of a <group> resource on the Hosting CSE to this limit, if configured. |
| maxNotificationRate | 0..1 |  | The Hosting CSE shall restrict the maximum rate of notifications for this M2M Service Subscription to this limit, if configured. A Hosting CSE shall also restrict the maximum value of the rateLimit attribute of <subscription> resources associated with this M2M Service Subscription to this limit,if configured. |
| maxNumFlexContainers | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the maximum number of <flexContainer> resources created to this limit, if configured. If a request is received to create a <flexContainer> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of allowed <flexContainer> resources has been reached. |
| maxNumInstancesPerFlexContainer | 0..1 | RW | For this M2M Service Subscription, the Hosting CSE shall limit the value of the maxNrOfInstances attribute of a <flexContainer> resource on the Hosting CSE to this limit, if configured. |
| defaultAccessControlPrivileges | 0..1(L) | RW | A list of resource identifiers of default access control policies to link to a resource if/when the accessControlPolicyIDs attribute of a resource created by an entity associated with this M2M Service Subscription is not configured. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 9.6.68	Resource Type serviceSubscribedUserProfile

The <serviceSubscribedUserProfile> resource contains user profile information for a given M2M Service User such as its M2M-User-ID.  A oneM2M <m2mServiceSubscriptionProfile> resource shall have a <serviceSubscribedUserProfile> child resource for each authorized M2M Service User associated with the M2M Service Subscription.  A <serviceSubscribedAppRule> resource may have an allowedUsers attribute that can be configured with a list of <serviceSubscribedUserProfile> resource identifiers, wherein the allowedUsers represents the users that are allowed to originate requests via the allowed AEs specified by the <serviceSubscribedAppRule>.

The <serviceSubscribedUserProfile> resource shall contain the child resources specified in table 9.6.68-1.

Table 9.6.68-1: Child resources of <serviceSubscribedUserProfile> resource

| Child Resources of <serviceSubscribedUserProfile> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |

The <serviceSubscribedUserProfile> resource shall contain the attributes specified in table 9.6.68-2.

Table 9.6.68-2: Attributes of <serviceSubscribedUserProfile> resource

| Attributes of <serviceSubscribedUserProfile> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| M2M-User-ID | 1 | RW | The M2M-User-ID that is assigned to this M2M Service User. |
| maxNumAEs | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the maximum number of <AE> resources created on the Hosting CSE to this limit, if configured. If a request is received to create an <AE> resource once this limit is reached, the Hosting CSE shall return an error indicating that the maximum number of allowed <AE> resources has been reached. |
| maxNumNodes | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the maximum number of <node> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <node> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of allowed <node> resources has been reached. |
| maxNumBytes | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the aggregate amount of bytes stored on the Hosting CSE to this limit, if configured. This limit shall be applied to Content Sharing Resources only (i.e. <container>, <contentInstance>, <flexContainer>, <flexContainerInstance>, <timeSeries> and <timeSeriesInstance>). If a request is received to create a Content Sharing Resource that will result in exceeding this limit, the Hosting CSE shall return an error that the maximum number of allowed bytes has been reached. A Hosting CSE may take the value of the maxNumBytes attribute as well as other attributes (e.g. maxNumContainers, maxNumContentInstances, etc.) into account when determining a limit to enforce on the maxByteSize attribute of individual Content Sharing Resources, if applicable. How a Hosting CSE allocates the maxNumBytes across individual Content Sharing Resources is based on local policy. |
| maxRequestRate | 0..1 | RW | If a Hosting CSE receives requests from this M2M Service User at a rate which exceeds the limit defined by this attribute, the Hosting CSE may throttle the processing of the requests. The Hosting CSE may throttle processing of requests by delaying responses and/or by returning an error indicating that the maximum allowed request rate has been reached. |
| maxNumContainers | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the maximum number of <container> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <container> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of allowed <container> resources has been reached. |
| maxNumInstancesPerContainer | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the value of the maxNrOfInstances attribute of a <container> resource on the Hosting CSE to this limit, if configured. |
| maxNumTimeSeries | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the maximum number of <timeSeries> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <timeSeries> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of <timeSeries> resources has been reached. |
| maxNumInstancesPerTimeSeries | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the value of the maxNrOfInstances attribute of a <timeSeries> resource on the Hosting CSE to this limit, if configured. |
| maxMembersPerGroup | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the value of the maxNrOfMembers attribute of a <group> resource on the Hosting CSE to this limit, if configured. |
| maxNotificationRate | 0..1 | RW | The Hosting CSE shall restrict the maximum rate of notifications sent to this M2M Service User to this limit, if configured. A Hosting CSE shall also restrict the maximum value of the rateLimit attribute of a <subscription> resource for this M2M Service User to this limit, if configured. |
| maxNumFlexContainers | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the maximum number of <flexContainer> resources created on the Hosting CSE to this limit, if configured. If a request is received to create a <flexContainer> resource once this limit is reached, the Hosting CSE shall return an error indicating the maximum number of allowed <flexContainer> resources has been reached. |
| maxNumInstancesPerFlexContainer | 0..1 | RW | For this M2M Service User, the Hosting CSE shall limit the value of the maxNrOfInstances attribute of a <flexContainer> resource on the Hosting CSE to this limit, if configured. |
| defaultAccessControlPrivileges | 0..1(L) | RW | A list of resource identifiers of default access control policies to link to a resource if/when the accessControlPolicyIDs attribute of a resource created by this M2M Service User is not configured. This list of default access control policies is combined with any default access control policies specified by the defaultAccessControlPrivileges attribute of the parent <m2mServiceSubscriptionProfile> resource. |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

#### 10.2.12.14	Create <serviceSubscribedUserProfile>

This procedure shall be used for creating a <serviceSubscribedUserProfile> resource. The information represented in the attributes of a <serviceSubscribedUserProfile> resource shall be used by a Hosting CSE to restrict the usage of services by a M2M Service User.  For example, a Hosting CSE can use this information to enforce limits on the maximum number of <container> resources or the maximum number of <contentInstance> resources per parent <container> resource that a M2M Service User is permitted.  Instances of <serviceSubscribedUserProfile> resources may be associated with specific AEs by linking them via the allowedUsers attribute of <serviceSubscribedAppRule> resources.

Table 10.2.12.14-1: <serviceSubscribedUserProfile> CREATE

| <serviceSubscribedUserProfile> CREATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Hosting CSE shall be an IN-CSE. Content: The resource content shall provide the information as defined in clause 9.6.68. |
| Processing at Originator before sending Request | According to clause 10.1.2. |
| Processing at Receiver | According to clause 10.1.2. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. If the Receiver overrides and configures any attributes with values different than the values specified in the request by the Originator, the Receiver shall return the values back to the Originator in the response depending on the Result Content value specified by the Originator. |
| Processing at Originator after receiving Response | According to clause 10.1.2. |
| Exceptions | According to clause 10.1.2. The Receiver shall check that any values specified for the attributes of this <serviceSubscribedUserProfile> do not violate the limits (if specified) in the parent <m2mServiceSubscriptionProfile> resource (e.g. maxNumContainers). When performing this check, the Receiver shall check that the aggregated values of all attributes across all the <serviceSubscribedUserProfile> child resources do not violate the limits specified in the parent <m2mServiceSubscriptionProfile> resource. If the limts are exceeded, the Receiver shall reject the request or modify the values specified in the request. If limits are not specified in the parent <m2mServiceSubscriptionProfile> resource, the Receiver may use a limit based on local policy. |

#### 10.2.12.15	Retrieve <serviceSubscribedUserProfile>

This procedure shall be used for retrieving the representation of the <serviceSubscribedUserProfile> resource.

Table 10.2.12.15-1: <serviceSubscribedUserProfile> RETRIEVE

| <serviceSubscribedUserProfile> RETRIEVE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Hosting CSE shall be an IN-CSE. Content: void. |
| Processing at Originator before sending Request | According to clause 10.1.3. |
| Processing at Receiver | According to clause 10.1.3. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. |
| Processing at Originator after receiving Response | According to clause 10.1.3. |
| Exceptions | According to clause 10.1.3. |

#### 10.2.12.16	Update <serviceSubscribedUserProfile>

This procedure shall be used for updating the attributes of the <serviceSubscribedUserProfile> resource.

Table 10.2.12.16-1: <serviceSubscribedUserProfile> UPDATE

| <serviceSubscribedUserProfile> UPDATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Hosting CSE shall be an IN-CSE. Content: Attributes of the <serviceSubscribedUserProfile> resource as defined in clause 9.6.68. |
| Processing at Originator before sending Request | According to clause 10.1.4. |
| Processing at Receiver | According to clause 10.1.4. If the request is to update an attribute specifying a limit (e.g. maxNumNodes, maxNumContainers, etc.) to a reduced value, the Receiver shall update the attribute with the reduced value if the limit corresponding to the reduced value has not yet been exceeded. For example, if maxNumNodes has a value of 10 and the request is to update it to a value of 5 and only 4 <node> resources associated with this user have been created thus far, then the Receiver shall update maxNumNodes attribute to a value of 5. However, if the limit corresponding to the reduced value in the request has already been exceeded, the Receiver shall reject the request or override the value specified in the request. For example, if the maxNumNodes attribute has a value of 10 and the request is to update it to a value of 5 but 6 <node> resources associated with this user have already been created, then the Receiver shall either reject the request or update the value of the maxNumNodes attribute to 6. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. If the Receiver overrides and configures any attributes with values different than the values specified in the request by the Originator, the Receiver shall return the values back to the Originator in the response depending on the Result Content value specified by the Originator. |
| Processing at Originator after receiving Response | According to clause 10.1.4. |
| Exceptions | According to clause 10.1.4. The Receiver shall check that any values specified for the attributes of this <serviceSubscribedUserProfile> do not violate the limits (if specified) in the parent <m2mServiceSubscriptionProfile> resource (e.g. maxNumContainers). When performing this check, the Receiver shall check that the aggregated values of all attributes across all the <serviceSubscribedUserProfile> child resources do not violate the limits specified in the parent <m2mServiceSubscriptionProfile> resource. If the limts are exceeded, the Receiver shall reject the request or modify the values specified in the request. If limits are not specified in the parent <m2mServiceSubscriptionProfile> resource, the Receiver may use a limit based on local policy. |

#### 10.2.12.17	Delete <serviceSubscribedUserProfile>

This procedure shall be used for deleting the <serviceSubscribedUserProfile> resource with all related information.

Table 10.2.12.17-1: <serviceSubscribedUserProfile> DELETE

| <serviceSubscribedUserProfile> DELETE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Hosting CSE shall be an IN-CSE. |
| Processing at Originator before sending Request | According to clause 10.1.5. |
| Processing at Receiver | According to clause 10.1.5. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. |
| Processing at Originator after receiving Response | According to clause 10.1.5. |
| Exceptions | According to clause 10.1.5. |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

#### 10.2.12.2	Create <m2mServiceSubscriptionProfile>

This procedure shall be used for creating a <m2mServiceSubscriptionProfile> resource.

Table 10.2.12.2-1: <m2mServiceSubscriptionProfile> CREATE

| <m2mServiceSubscriptionProfile> CREATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Receiver or Hosting CSE shall be an IN-CSE Content: The resource content shall provide the information as defined in clause 9.6.19 |
| Processing at Originator before sending Request | - According to clause 10.1.2 |
| Processing at Receiver | - According to clause 10.1.2 - The Receiver shall configure the value of the status attribute of this <m2mServiceSubscriptionProfile> resource based on the values of the activate attribute of this <m2mServiceSubscriptionProfile> resource as described in Table 9.6.19. |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: - Content: - Address of the created <m2mServiceSubscriptionProfile> resource, according to clause 10.1.2 - If the Receiver overrides and configures any attributes with values different than the values specified in the request by the Originator, the Receiver shall return the values back to the Originator in the response depending on the Result Content value specified by the Originator. |
| Processing at Originator after receiving Response | - According to clause 10.1.2 |
| Exceptions | - According to clause 10.1.2 |

#### 10.2.12.3	Retrieve <m2mServiceSubscriptionProfile>

This procedure shall be used for retrieving the attributes of a <m2mServiceSubscriptionProfile> resource.

Table 10.2.12.3-1: <m2mServiceSubscriptionProfile> RETRIEVE

| <m2mServiceSubscriptionProfile> RETRIEVE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Receiver or Hosting CSE shall be an IN-CSE Content: Void |
| Processing at Originator before sending Request | According to clause 10.1.3 |
| Processing at Receiver | According to clause 10.1.3 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: Content: Attributes of the <m2mServiceSubscriptionProfile> resource as defined in clause 9.6.19 |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | According to clause 10.1.3 |

#### 10.2.12.4	Update <m2mServiceSubscriptionProfile>

This procedure shall be used for updating the attributes of a <m2mServiceSubscriptionProfile> resource.

Table 10.2.12.4-1: <m2mServiceSubscriptionProfile> UPDATE

| <m2mServiceSubscriptionProfile> UPDATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 are applicable as indicate in the table with the specific details for: To: The Receiver or Hosting CSE shall be an IN-CSE Content: Attributes of the <m2mServiceSubscriptionProfile> resource as defined in clause 9.6.19 which need be updated, with the exception of the following that cannot be modified: - "lastModifiedTime" |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | According to clause 10.1.4 If the request is to update an attribute specifying a limit (e.g. maxNumNodes, maxNumContainers, etc.) to a reduced value, the Receiver shall update the attribute with the reduced value if the limit corresponding to the reduced value has not yet been exceeded. For example, if maxNumNodes has a value of 10 and the request is to update it to a value of 5 and only 4 <node> resources associated with this service subscription have been created thus far, then the Receiver shall update maxNumNodes attribute to a value of 5. However, if the limit corresponding to the reduced value in the request has already been exceeded, the Receiver shall reject the request or override the value specified in the request. For example, if the maxNumNodes attribute has a value of 10 and the request is to update it to a value of 5 but 6 <node> resources associated with this service subscription have already been created, then the Receiver shall either reject the request or update the value of the maxNumNodes attribute to 6. |
| Information in Response message | According to clause 10.1.4 If the Receiver overrides and configures any attributes with values different than the values specified in the request by the Originator, the Receiver shall return the values back to the Originator in the response depending on the Result Content value specified by the Originator. |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | According to clause 10.1.4 |

#### 10.2.12.5	Delete <m2mServiceSubscriptionProfile>

This procedure shall be used for deleting a <m2mServiceSubscriptionProfile> resource residing under a <m2mServiceSubscriptionProfile> resource.

Table 10.2.12.5-1: <m2mServiceSubscriptionProfile> DELETE

| <m2mServiceSubscriptionProfile> DELETE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with specific details for: To: The Receiver or Hosting CSE shall be an IN-CSE |
| Processing at Originator before sending Request | According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | According to clause 10.1.5 |

### -----------------------End of change 4-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?