| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #45 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com |
| Date:* | 2020-04-15 |
| Reason for Change/s:* | Add time synchronization functionality |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0046 – Vehicular Domain Enablement MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.0.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds the protocol level support for the time synchronization functionality introduced in TS-0001 for Release 4.

This includes the following:

- a new <timeSyncBeacon> resource

- a new currentTime attribute added to the <CSEBase> resource

- a new enableTimeCompensation attribute of an <AE> and <remoteCSE> resource

- a new notification format for time synchronization beacons

### -----------------------Start of change 1-------------------------------------------

### 7.4.3	Resource Type <CSEBase>

#### 7.4.3.1	Introduction

A <CSEBase> resource shall represent a CSE. This <CSEBase> resource shall be the root for all the resources that are residing on the CSE. The detailed description can be found in clause 9.6.3 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.3.1-1: Data type definition of <CSEBase> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| CSEBase | CDT-CSEBase-v4_0_0.xsd |  |

Table 7.4.3.1-2: Universal/Common Attributes of <CSEBase> resource

Attribute Name

@resourceName

resourceType

resourceID

parentID

creationTime

lastModifiedTime

labels

location

The value of the parentID attribute for the <CSEBase> resource shall be an empty string since the <CSEBase> resource does not have a parent. The common attributes accessControlPolicyIDs and dynamicAuthorizationConsultationIDs are treated as resource-specific attributes.

Table 7.4.3.1-3: Resource Specific Attributes of <CSEBase> resource

| Attribute Name | Data Type | Default Value and Constraints |
| --- | --- | --- |
| accessControlPolicyIDs | m2m:acpType | No default |
| cseType | m2m:cseTypeID | No default |
| CSE-ID | m2m:ID | No default |
| supportedResourceType | list of m2m:resourceType | No default |
| pointOfAccess | m2m:poaList | No default |
| nodeLink | xs:anyURI | No default |
| dynamicAuthorizationConsultationIDs | list of xs:anyURI | No default |
| contentSerialization | m2m:serializations | No default |
| e2eSecInfo | m2m:e2eSecInfo | No default |
| supportedReleaseVersions | m2m:supportedReleaseVersions | No default |
| currentTime | m2m:timestamp | No default |

Table 7.4.3.1-4: Child resources of <CSEBase> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <remoteCSE> | [variable] | 0..n | Clause |
| <remoteCSEAnnc> | [variable] | 0..n | Clause |
| <node> | [variable] | 0..n | Clause Error: Reference source not found |
| <AE> | [variable] | 0..n | Clause |
| <container> | [variable] | 0..n | Clause Error: Reference source not found |
| <group> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <mgmtCmd> | [variable] | 0..n | Clause Error: Reference source not found |
| <locationPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <statsConfig> | [variable] | 0..n | Clause Error: Reference source not found |
| <statsCollect> | [variable] | 0..n | Clause Error: Reference source not found |
| <request> | [variable] | 0..n | Clause Error: Reference source not found |
| <delivery> | [variable] | 0..n | Clause Error: Reference source not found |
| <schedule> | [variable] | 0..1 | Clause Error: Reference source not found |
| <m2mServiceSubscriptionProfile> | [variable] | 0..n | Clause Error: Reference source not found |
| <serviceSubscribedAppRule> | [variable] | 0..n | Clause Error: Reference source not found |
| <notificationTargetPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <dynamicAuthorizationConsultation> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeries> | [variable] | 0..n | Clause Error: Reference source not found |
| <role> | [variable] | 0..n | Clause Error: Reference source not found |
| <token> | [variable] | 0..n | Clause Error: Reference source not found |
| <authorizationDecision> | [variable] | 0..n | Clause 7.4.43 |
| <authorizationPolicy> | [variable] | 0..n | Clause 7.4.44 |
| <authorizationInformation> | [variable] | 0..n | Clause 7.4.45 |
| <ontologyRepository> | [variable] | 0..1 | Clause 7.4.46 |
| <semanticMashupJobProfile> | [variable] | 0..n | Clause 7.4.49 |
| <semanticMashupInstance> | [variable] | 0..n | Clause 7.4.50 |
| <AEContactList> | [variable] | 0..1 | Clause 7.4.53 |
| <localMulticastGroup> | [variable] | 0..n | Clause 7.4.55 |
| <crossResourceSubscription> | [variable] | 0..n | Clause 7.4.58 |
| <backgroundDataTransfer> | [variable] | 0..n | Clause 7.4.59 |
| <transactionMgmt> | [variable] | 0..n | Clause 7.4.60 |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <timeSyncBeacon> | [variable] | 0..n | Clause 7.4.XX |

#### 7.4.3.2	<CSEBase> resource specific procedures for CRUD+N operations

##### 7.4.3.2.1	Create

Originator:

The <CSEBase> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

##### 7.4.3.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

If the currentTime attribute is supported, the Receiver shall compute its current time and include this time value in the currentTime attribute.

##### 7.4.3.2.3	Update

Originator:

The <CSEBase> resource shall not be updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

##### 7.4.3.2.4	Delete

Originator:

The <CSEBase> resource shall not be deleted via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

a)	"Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

b)	"Send the Response primitive".

##### 7.4.3.2.5	Notify

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.4.5	Resource Type <AE>

#### 7.4.5.1	Introduction

The <AE> resource represents information about an Application Entity known to a given Common Services Entity.

The detailed description can be found in clause 9.6.5 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.5.1-5: Data type definition of <AE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| AE | CDT-AE-v4_0_0.xsd | XSD schema for AE resource |

Table 7.4.5.1-6: Universal/Common Attributes of <AE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

Table 7.4.5.1-7: Resource Specific Attributes of <AE> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| appName | O | O | xs:string | No default |
| App-ID | M | NP | xs:string | No default |
| AE-ID | NP | NP | m2m:ID | No default |
| pointOfAccess | O | O | m2m:poaList | No default |
| ontologyRef | O | O | xs:anyURI | No default |
| nodeLink | O | O | xs:anyURI | No default |
| requestReachability | M | O | xs:boolean | No default |
| contentSerialization | O | O | m2m:serializations | No default |
| e2eSecInfo | O | O | m2m:e2eSecInfo | No default |
| M2M-Ext-ID | O | O | m2m:externalID | No default |
| supportedReleaseVersions | M | O | m2m:supportedReleaseVersions | No default |
| registrationStatus | O | O | m2m:AERegistrationStatus | No default |
| trackRegistrationPoints | O | O | xs:boolean | No default |
| sessionCapabilities | O | O | m2m:sessionCapabilities | No default |
| triggerEnable | O | O | xs:boolean | false |
| activityPatternElements | O | O | m2m:activityPatternElements | No default |
| enableTimeCompensation | O | O | xs:boolean | false |

Table 7.4.5.1-8: Child resources of <AE> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <container> | [variable] | 0..n | Clause Error: Reference source not found |
| <group> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <pollingChannel> | [variable] | 0..n | Clause Error: Reference source not found |
| <semanticDescriptor> | [variable] | 0..n | Clause Error: Reference source not found |
| <dynamicAuthorizationConsultation> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeries> | [variable] | 0..n | Clause Error: Reference source not found |
| <semanticMashupInstance> | [variable] | 0..n | Clause 7.4.50 |
| <multimediaSession> | [variable] | 0..n | Clause 7.4.56 |
| <triggerRequest> | [variable] | 0..n | Clause 7.4.57 |
| <crossResourceSubscription> | [variable] | 0..n | Clause 7.4.58 |
| <transactionMgmt> | [variable] | 0..n | Clause 7.4.60 |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <timeSyncBeacon> | [variable] | 0..n | Clause 7.4.XX |

#### 7.4.5.2	<AE> resource specific procedures for CRUD+N operations

##### 7.4.5.2.0	Introduction

This clause describes AE resource specific behaviour for CRUD+N operations.

The entire AE registration procedure including <AE> resource creation procedure below is defined in clause 10.1.1.2.2 of oneM2M TS-0001 [Error: Reference source not found] ("Application Entity registration procedure").

##### 7.4.5.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found with the with the following exception:

- A CSE shall not originate a Create <AE> resource request.

Receiver:

Same as the generic procedures in clause Error: Reference source not found except replacement of Recv-6.3 procedure as follows:

- The <serviceSubscribedNode> resource associated with the Hosting CSE shall be checked, using the credential provided by the Originator, to determine if the requested registration is allowed by any of <serviceSubscribedAppRule> resources which are linked from the ruleLinks attribute.

- If the check fails, then the Hosting CSE shall return a response primitive with a Response Status Code indicating "SECURITY_ASSOCIATION_REQUIRED" error.

Additional primitive specific operations on Recv-6.0.1 "Requested operation is an AE registration?":

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with "/S" in the From parameter, but no specific AE-ID-Stem was provided with the CREATE request of the Registree AE, then this is an initial registration and the receiver shall execute the following steps in order:

a)	Compose a Create <AEAnnc> Request primitive with the following attribute values:

- The link attribute of the <AEAnnc> resource to be created shall be set to the SP-Relative-Resource-ID format of a - not yet existent - <AE> resource hosted on the Registrar CSE constructed with a Unstructured-CSE-relative-Resource-ID that is equal to the AE-ID-Stem value used for the Registree AE.

- The App-ID attribute of the <AEAnnc> resource to be created shall be set to the App-ID attribute value of the Registree AE.

- The concatenation of the string 'Credential-ID:' and the actual Credential-ID of the Security Association used by the Registree AE - if any - shall be placed into the labels attribute of the <AEAnnc> resource. If no Security Association was used by the Registree AE, a value of 'None' shall be used for Credential-ID.

b)	The From parameter of the CREATE request for the <AEAnnc> resource shall be set to the SP-relative-CSE-ID or Absolute-CSE-ID followed by '/S'.

c)	Send Create request to the IN-CSE that is associated with the Registree AE.

d)	Wait for Response primitive.

- Upon receipt of a successful response from the IN-CSE, the Registrar/Host CSE shall use the Unstructured-CSE-relative-Resource-ID that was used for the <AEAnnc> resource on the IN-CSE also as the assigned Unstructured-CSE-relative-Resource-ID for the <AE> resource to be created on the Registrar/Host CSE.

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with an AE-ID-Stem starting with "S", and this is an initial registration or a re-registration to the same Registrar CSE, then the receiver shall execute the following steps in order:

a)	Determine if <AEAnnc> resource already exists in the IN-CSE for the Registree AE. If so, compose an Update <AEAnnc> Request primitive to update the <AEAnnc> resource. If no <AEAnnc> resource already exists in the IN-CSE for the Registree AE, the receiver shall compose a Create <AEAnnc> Request primitive. In both cases, the request primitive shall have the following attribute values:

  - The link attribute of the <AEAnnc> resource shall be set or updated to the SP-Relative-Resource-ID format of a - not yet existent - <AE> resource hosted on the Registrar CSE constructed with a Unstructured-CSE-relative-Resource-ID that is equal to the AE-ID-Stem value used for the Registree AE.

  - The labels attribute of the <AEAnnc> resource shall be set or updated to the concatenation of the string 'Credential-ID:' and the Credential-ID of the Security Association used by the Registree AE, replacing the existing entry starting with 'Credential-ID:' if present. If no Security Association was used by the Registree AE, a value of 'None' shall be used for Credential-ID.

b)	The From parameter of the CREATE or UPDATE request for the <AEAnnc> resource shall be set to the SP-relative-CSE-ID or Absolute-CSE-ID followed by '/' and the AE-ID-Stem value.

c)	The To parameter shall contain the SP-relative-Resource-ID format of the Resource ID for the <AEAnnc> resource which shall be constructed from the CSE-ID of the IN-CSE and the AE-ID-Stem that the Registree AE provided.

d)	Send Create or Update request to the IN-CSE that is associated with the Registree AE.

e)	Wait for Response primitive.

- Upon receipt of a successful response from the IN-CSE, the Registrar/Host CSE shall use the Unstructured-CSE-relative-Resource-ID equal to the AE-ID-Stem provided by the Registree AE for the <AE> resource to be created on the Registrar/Host CSE.

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with an AE-ID-Stem starting with "S", and this is a re-registration to a new Registrar CSE, then the receiver shall execute the following steps in order:

a)	Determine if <AEAnnc> resource already exists in the IN-CSE for the Registree AE. If so, compose an Update <AEAnnc> Request primitive to update the <AEAnnc> resource. If no <AEAnnc> resource already exists in the IN-CSE for the Registree AE, the receiver shall compose a Create <AEAnnc> Request primitive. In both cases, the request primitive shall have the following attribute values:

- The link attribute of the <AEAnnc> resource shall be set or updated to the SP-Relative-Resource-ID format of a - not yet existent - <AE> resource hosted on the Registrar CSE constructed with an Unstructured-CSE-relative-Resource-ID that is equal to the AE-ID-Stem value used for the Registree AE.

- The labels attribute of the <AEAnnc> resource shall be set or updated to the concatenation of the string 'Credential-ID:' and the Credential-ID of the Security Association used by the Registree AE, replacing the existing entry starting with 'Credential-ID:' if present. If no Security Association was used by the Registree AE, a value of 'None' shall be used for Credential-ID.

b)	The From parameter of the CREATE or UPDATE request for the <AEAnnc> resource shall be set to the SP-relative-CSE-ID or Absolute-CSE-ID followed by '/' and the AE-ID-Stem value.

c)	The To parameter shall contain the SP-relative-Resource-ID format of the Resource ID for the <AEAnnc> resource which shall be constructed from the CSE-ID of the IN-CSE and the AE-ID-Stem that the Registree AE provided.

d)	Send Create or Update request to the IN-CSE that is associated with the Registree AE.

e)	Wait for Response primitive.

- Upon receipt of a successful response from the IN-CSE, the Registrar/Host CSE shall use the Unstructured-CSE-relative-Resource-ID equal to the AE-ID-Stem provided by the Registree AE for the <AE> resource to be created on the Registrar/Host CSE.

- And addition of the following to step Recv-6.4:

- The Hosting CSE shall check for the presence of any resources having an AE-ID that matches the one specified in the request and that have the same parent as the new resource being created.

- If such a resource exists, then the Hosting CSE shall reject the request with a Response Status Code indicating "ORIGINATOR_HAS_ALREADY_REGISTERED" error.

- And the addition of a new step following step Recv-6.11:

- If the enableTimeCompensation attribute of the <AE> resource is set to TRUE, the Hosting CSE shall perform time compensation functionality upon the AE’s requests and responses as defined in clause 10.2.24.3 in oneM2M TS-0001 [6].

##### 7.4.5.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.5.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception.

- The addition of a new step following step Recv-6.11:

- If the enableTimeCompensation attribute of the <AE> resource is updated from FALSE to TRUE, the Hosting CSE shall start performing time compensation upon the AE’s requests and responses as defined in clause 10.2.24.3 in oneM2M TS-0001 [6].

- If the enableTimeCompensation attribute of the <AE> resource is updated from TRUE to FALSE, the Hosting CSE shall stop performing time compensation functionality upon the AE’s requests and responses.

##### 7.4.5.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the with the following addition:

- Primitive Specific operation on Recv-6.6 "Announce/De-announce the resource":

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with AE-ID-Stem starting with "S" the receiver shall execute the following steps in order:

a)	Compose the Update <AEAnnc> Request primitive with the link attribute set to "INACTIVE".

b)	Send Update request to the announced to IN-CSE.

c)	Wait for Response primitive.

##### 7.4.5.2.5	Notify

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 7.4.4	Resource Type <remoteCSE>

#### 7.4.4.1	Introduction

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> of the Registrar CSE.

In addition each registered CSE shall have a <remoteCSE> resource representing its Registrar CSE. This is located directly under the registered CSE's <CSEBase>.

The detailed description can be found in clause 9.6.4 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.4.1-9: Data type definition of <remoteCSE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| remoteCSE | CDT-remoteCSE-v4_0_0.xsd |  |

Table 7.4.4.1-10: Universal/Common Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

Table 7.4.4.1-11: Resource Specific Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| cseType | O | NP | m2m:cseTypeID | No default |
| pointOfAccess | O | O | m2m:poaList | No default |
| CSEBase | M | NP | xs:anyURI | No default |
| CSE-ID | M | NP | m2m:ID | No default |
| M2M-Ext-ID | O | O | m2m:externalID | No default |
| Trigger-Recipient-ID | O | O | m2m:triggerRecipientID | No default |
| requestReachability | M | O | xs:boolean | No default |
| nodeLink | O | O | xs:anyURI | No default |
| triggerReferenceNumber | O | O | xs:unsignedInt | No default |
| contentSerialization | O | O | m2m:serializations | No default |
| e2eSecInfo | O | O | m2m:e2eSecInfo | No default |
| descendantCSEs | O | O | m2m:listOfM2MID | No default |
| supportedReleaseVersions | M | O | m2m:supportedReleaseVersions | No default |
| multicastCapability | O | O | m2m:multicastCapability | No default |
| externalGroupID | O | O | m2m:externalID | No default |
| triggerEnable | O | O | xs:boolean | false |
| activityPatternElements | O | O | m2m:activityPatternElements | No default |
| enableTimeCompensation | O | O | xs:boolean | false |

Table 7.4.4.1-12: Child resources of <remoteCSE> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <container> | [variable] | 0..n | Clause Error: Reference source not found |
| <containerAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainerAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <group> | [variable] | 0..n | Clause Error: Reference source not found |
| <groupAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicyAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <pollingChannel> | [variable] | 0..1 | Clause Error: Reference source not found |
| <nodeAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <dynamicAuthorizationConsultation> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeries> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeriesAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <remoteCSEAnnc> | [variable] | 0..n | Clause |
| <AEAnnc> | [variable] | 0..n | Clause |
| <locationPolicyAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <ontologyRepositoryAnnc> | [variable] | 0..1 | Clause 7.4.46 |
| <semanticMashupJobProfile> | [variable] | 0..n | Clause 7.4.49 |
| <semanticMashupJobProfileAnnc> | [variable] | 0..n | Clause 7.4.49 |
| <semanticMashupInstance> | [variable] | 0..n | Clause 7.4.50 |
| <semanticMashupInstanceAnnc> | [variable] | 0..n | Clause 7.4.50 |
| <crossResourceSubscription> | [variable] | 0..n | Clause 7.4.58 |
| <transactionMgmt> | [variable] | 0..n | Clause 7.4.60 |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <timeSyncBeacon> | [variable] | 0..n | Clause 7.4.XX |

7.4.4.2	<remoteCSE> resource specific procedures for CRUD operations

##### 7.4.4.2.0	Introduction

The entire CSE registration procedure including <remoteCSE> resource creation procedure below is defined in clause 10.1.1.2.1 in oneM2M TS-0001 [Error: Reference source not found] ("CSE registration procedure").

##### 7.4.4.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

- An AE shall not originate a Create <remoteCSE> resource request.

- The Originator, upon receipt of successful CREATE response message, shall create a <remoteCSE> resource locally. It shall populate this resource with all the mandatory attributes for a <remoteCSE>.

- The Originator may issue a RETRIEVE request to the registrar <CSEBase> to fetch additional information about the registrar CSE and add this information to its local <remoteCSE> resource. The Originator may choose which optional attributes to include in its <remoteCSE> resource.

Receiver:

Same as the generic procedures in clause Error: Reference source not found except replacement of Recv-6.3 step with following:

- The Hosting CSE shall check if the credential provided by the Originator is valid.

- If the check fails, then the Hosting CSE shall return a response primitive with a Response Status Code indicating "SECURITY_ASSOCIATION_REQUIRED" error.

- And addition of the following to step Recv-6.4:

- If the Hosting CSE is an ASN-CSE, then it shall reject the request with Response Status Code indicating an “OPERATION_NOT_ALLOWED" error.

- The Hosting CSE shall check for the presence of any resources having a CSE-ID that matches the one specified in the request and that have the same parent as the new resource being created.

- If such a resource exists, then the Hosting CSE shall reject the request with a Response Status Code indicating "ORIGINATOR_HAS_ALREADY_REGISTERED" error.

- And addition of the following to step Recv-6.5:

- If the Receiver CSE has registered to another CSE, the Receiver CSE shall send an update request to its Registrar CSE to add the CSE-IDs of the Originator CSE and the Originator CSE's descendants into the descendantCSEs attribute of the Receiver CSE's <remoteCSE> hosted by the Registrar CSE.

- The addition of a new step following step Recv-6.11:

- If the enableTimeCompensation attribute of the <remoteCSE> resource is set to TRUE, the Hosting CSE shall start performing time compensation upon the Registree CSE’s requests and responses as defined in clause 10.2.24.3 in oneM2M TS-0001 [6].

7.4.4.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.4.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

- An AE shall not originate a Update <remoteCSE> resource request.

Receiver:

Same as the generic procedures in clause Error: Reference source not found with the following exceptions:

The addition of the following to step Recv-6.5:

- If the descendantCSEs attribute is updated, and the Receiver CSE has registered to another CSE, the Receiver CSE shall send an update request to its Registrar CSE to make the corresponding updates to the descendantCSEs attribute of the Receiver CSE's <remoteCSE> hosted by the Registrar CSE.

- The addition of a new step following step Recv-6.11:

- If the enableTimeCompensation attribute of the <remoteCSE> resource is updated from FALSE to TRUE, the Hosting CSE shall start performing time compensation upon the Registree CSE’s requests and responses as defined in clause 10.2.24.3 in oneM2M TS-0001 [6].

- If the enableTimeCompensation attribute of the <remoteCSE> resource is updated from TRUE to FALSE, the Hosting CSE shall stop performing time compensation functionality upon the Registree CSE’s requests and responses.

##### 7.4.4.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- An AE shall not originate a Delete <remoteCSE> resource request.

- The Originator, upon receipt of successful DELETE response message, shall delete the corresponding <remoteCSE> resource locally.

Receiver:

Same as the generic procedures in clause Error: Reference source not found except addition of the following to step Recv-6.5:

If the Receiver CSE has registered to another CSE, the Receiver CSE shall send an update request to its Registrar CSE to delete the CSE-IDs of the Originator CSE and the Originator CSE's descendants from the descendantCSEs attribute of the Receiver CSE's <remoteCSE> hosted by the Registrar CSE.

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

### 7.4.XX	Resource Type <timeSyncBeacon>

#### 7.4. XX.1	Introduction

A <timeSyncBeacon> resource is created by an entity requesting time synchronization services from the Hosting CSE. As a result, the Hosting CSE provides time synchronization services in the form of issuing time synchronization beacon notifications that include time-synchronization information (e.g. current time of the Hosting CSE).

Table 7.4.XX.1-13: Data type definition of <timeSyncBeacon> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| timeSyncBeacon | CDT-timeSyncBeacon-v4_XX_0.xsd |  |

Table 7.4.XX.1-14: Universal/Common Attributes of <timeSyncBeacon> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.XX.1-15: Resource Specific Attributes of <timeSyncBeacon> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| beaconRequestor | O | NP | m2m:ID |  |
| beaconCriteria | M | O | m2m:beaconCriteria |  |
| beaconInterval | O | O | xs:duration |  |
| beaconThreshold | O | O | xs:duration |  |
| beaconNotificationURIs | M | O | list of xs:anyURI |  |

Table 7.4.XX.1-16: Child Resources of <timeSyncBeacon> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

#### 7.4. XX.2	<timeSyncBeacon> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <timeSyncBeacon> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Receiver shall perform the time synchronization beacon notification operations defined in clause 7.5.1.2.XX.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Receiver shall perform the time synchronization beacon notification operations defined in clause 7.5.1.2.XX.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

The Receiver shall stop performing the time synchronization beacon notification operations defined in clause 7.5.1.2.XX.

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

##### 6.3.4.2.YY	m2m:beaconCriteria

Used for the beaconCriteria attribute of the <timeSyncBeacon> resource.

Table 6.3.4.2.YY-17: Interpretation of beaconCriteria

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | PERIODIC | Default |
| 2 | LOSS_OF_SYNCHRONIZATION |  |
| NOTE: See clause Error: Reference source not found. |  |  |

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

## 7.5	Primitive-specific procedures and definitions

### 7.5.1	Notification data object and procedures

#### 7.5.1.1	Notification data object

Notification procedures represent a special case of the generic procedures defined in clause Error: Reference source not found, where the Operation parameter of the request primitive is set to value "N" (Notify). In this case, the request primitive is referred to as Notify request primitive, and the associated response primitive is denoted as Notify response primitive.

A Notify request primitive shall convey a special notification data object in its Content parameter. This notification data object has no resource type representation in the oneM2M TS-0001 [Error: Reference source not found], since it does not represent a resource accessible by any M2M entities. The data type of the notification data object is defined in the tables below. The first column of Table 7.5.1.1-19 defines the permitted names the root element the notification data object can take with the data type listed in the third column.

Table 7.5.1.1-18: Data Type Definition of notification data object

| Data Type ID | File Name | Note |
| --- | --- | --- |
| notification | CDT-notification-v4_0_0.xsd |  |

Table 7.5.1.1-19: Data Types for notification data objects

| Root Element Name | Request Optionality | Data Type | Default Value and Constraints |
| --- | --- | --- | --- |
|  | N |  |  |
| m2m:notification | O | m2m:notification |  |
| m2m:aggregatedNotification | O | m2m:aggregatedNotification |  |
| m2m:responsePrimitive | O | Anonymous data type defined in the responsePrimitive declaration |  |
| m2m:securityInfo | O | m2m:securityInfo |  |
| m2m:timeSyncBeaconNotification | O | m2m:timeSyncBeaconNotification |  |

When an Originator sends a Notify primitive to an AE Receiver, it shall use one of the serializations specified in that <AE>'s contentSerialization attribute. When an Originator sends a Notify primitive to a CSE, as the receiver or a transit CSE, it shall use one of the serializations specified in that <remoteCSE>'s contentSerialization attribute. If there is no contentSerialization value specified the Originator may use any serialization format.

#### 7.5.1.2	Notification procedures

##### 7.5.1.2.1	Introduction

Notification procedures shall be employed for the following use cases:

- to notify Receiver(s) of modifications of a resource for an associated <subscription> resource;

- to request Receiver(s) to perform resource subscription verification;

- to notify deletion of the <subscription> resource;

- to notify Receiver(s) for Asynchronous Non-blocking Request;

- to notify Receiver(s) of modifications of a resource when the subscription relationship is established through the <group> resource;

- to send the response corresponding to a request delivered via service layer long polling (clause Error: Reference source not found Retrieve <pollingChannelURI>);

- to notify Receiver(s)(i.e. IPE) for on-demand discovery request;

- to notify Receiver(s) of the missing Time Series Data points for an associated <subscription> resource;

- to notify Receiver(s) of a security related request (e.g. dynamic authorization and end-to-end security);

- to notify Receivers that an AE has changed registration point;

- to notify an IN-CSE that the Originator has a new/updated/deleted reference to an Application Entity Resource identifier;

- to notify Receiver(s) of a cross-resource notification generated by a <crossResourceSubscription> Hosting CSE;

- to notify a Receiver that a triggered update on the subscribed-to resource has been blocked and retargeted to the Receiver;

- to notify Receiver(s) of the current time value of a <timeSyncBeacon> resource Hosting CSE.

The following clauses specify the notification procedures for each of the above use cases.

### -----------------------End of change 6-------------------------------------------

### -----------------------Start of change 7-------------------------------------------

##### 7.5.1.2.XX	Notification for Time Synchronization Beacon

When an Originator creates a <timeSyncBeacon> resource, the Hosting CSE shall generate time synchronization beacon notifications based on the information specified within the <timeSyncBeacon> resource according to the following steps:

- If the beaconCriteria attribute of the <timeSyncBeacon> resource has a value of PERIODIC, then the Hosting CSE shall periodically generate time synchronizations beacon notifications at a periodic interval defined by the beaconInterval attribute. If the beaconInterval attribute is not configured, then the Hosting CSE shall use a default periodic interval value based on local policies.

- If the beaconCriteria attribute of the <timeSyncBeacon> resource has a value of LOSS_OF_SYNCHRONIZATION, then the Hosting CSE shall monitor the current time of the entity specified by the beaconRequester attribute and compare this time to its own current time. The current time of the entity shall be monitored by the Hosting CSE using Originating Timestamp parameters present in the request and response messages sent by the entity to the Hosting CSE.  If the Hosting CSE computes a time offset between its own current time and the current time of the entity that exceeds the value specified by the beaconThreshold attribute, then the Hosting CSE shall generate a time synchronization beacon notification. If the beaconThreshold attribute is not configured, then the Hosting CSE shall use a default threshold value based on local policies.  If the Originating Timestamp parameters are not present in request or response messages and the Hosting CSE is unable to monitor the current time of the entity, then the Hosting CSE shall not generate time synchronization beacon notifications for the entity.

- For each time synchronization beacon notification generated, the Hosting CSE shall create a time synchronization notification request primitive and configure the Content parameter as follows:

  - Configure the timeSyncBeaconReference element with the resource identifier of the <timeSyncBeacon> resource;

  - Configure the currentTime element with the current time of the Hosting CSE;

  - If the beaconCriteria attribute of the <timeSyncBeacon> resource has a value of LOSS_OF_SYNCHRONIZATION, set the timeOffset element to the computed offset between the Hosting CSE’s current time and the current time of the entity specified by the beaconRequester attribute.

- For each time synchronization beacon notification generated, the Hosting CSE shall send a time synchronization notification request primitive to each Receiver specified by the beaconNotificationURIs attribute;

- "Wait for Response primitive" procedure.

The Receiver shall perform the following steps in order:

- Extract the timeSyncBeaconReference, currentTime and timeOffset (if present) elements from the Content parameter of time synchronization beacon notification;

- "Create a success response" procedure;

- "Send the Response primitive" procedure.

### -----------------------End of change 7-------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 7.5.2	Elements contained in the Content primitive parameter

Clauses Error: Reference source not found and Error: Reference source not found enumerate the forms that the Content primitive parameter takes in various Request and Response cases. Note that the Content primitive parameter is denoted as primitiveContent in both CDT-requestPrimitive-v4_0_0.xsd and CDT-responsePrimitive-v4_0_0.xsd.

This clause details the Objects (elements) used in some of these cases. in the tables below.

The following elements are defined for use in the Content parameter of a request:

Table 7.5.2-1: Elements used for request content

| Element Name | Applicable Operations | Data Type | Defined in |
| --- | --- | --- | --- |
| m2m:<resourceType> {other namespace identifier}:<resourceType> | C U | See element declaration | CDT-<resourceType>-v4_0_0.xsd |
| m2m:notification | N | m2m:notification | CDT-notification-v4_0_0.xsd |
| m2m:aggregatedNotification | N | m2m:aggregatedNotification | CDT-notification-v4_0_0.xsd |
| m2m:securityInfo | N | m2m:securityInfo | CDT-notification-v4_0_0.xsd |
| m2m:attributeList | R | m2m:attributeList | CDT-requestPrimitive-v4_0_0.xsd |
| m2m:responsePrimitive | N | Anonymous data type defined in the responsePrimitive declaration | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:timeSyncBeaconNotification | N | m2m:timeSyncBeaconNotification | CDT-timeSyncBeacon-v4_0_0.xsd |

The following elements are defined for use in the Content parameter of a response sent in reply to a request message with Operation and Result Content (rcn) parameters as given in the column "Applicable Operations" (the settings of the Result Content parameters are defined in clause Error: Reference source not found; NP means the rcn parameter is not present).

Table 7.5.2-2: Elements used for response content

| Element Name | Applicable Operations/rcn | Data Type | Element is Defined in |
| --- | --- | --- | --- |
| m2m:<resourceType> {other namespace identifier}:<resourceType> See note 6 | C/1,9,NP R/1,4,5,6,7,8,NP U/1,9,NP D/1,4,5,6,8 See note 1 | See element declaration | CDT-<resourceType>-v4_0_0.xsd |
| m2m:resource | C/3 | m2m:resourceWrapper | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:URIList | R/NP See note 2 | list of xs:anyURI | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:resourceRefList | R/6 See note 2 | m2m:listOfChildResourceRef | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:aggregatedResponse | C R U D See note 3 | m2m:aggregatedResponse | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:URI | C/2 See note 4 | xs:anyURI | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:requestPrimitive | See note 7 | Anonymous data type defined in the requestPrimitive declaration | CDT-requestPrimitive-v4_0_0.xsd |
| m2m:debugInfo | See note 5 | xs:string | CDT-responsePrimitive-v4_0_0.xsd |
| m2m:securityInfo | N/NP | m2m:securityInfo | CDT-notification-v4_0_0.xsd |
| m2m:queryResult | R/10 See note 8 | xs:string | CDT-responsePrimitive-v4_0_0.xsd |
| NOTE 1: The case rcn = 7 applies to Retrieve operation only (R/7). It retrieves the original resource in case the To parameter points to an announced resource. The case R/NP applies to Retrieve operation (Non-Discovery) only. NOTE 2: This applies to discovery operation only. For discovery, the format of the address (structured, unstructured) depends on the Desired Identifier Result Type parameter setting (see clause Error: Reference source not found). NOTE 3: This applies to CRUD operations on a <fanOutPoint> child resource of a <group> parent resource. The Content parameter of each response primitive included in aggregatedResponse is set as given in one of the other rows of this table. NOTE 4: This also applies to the response ("acknowledgement") to non-blocking requests in asynchronous and synchronous modes for any CRUD operation. NOTE 5: This is a plain text messages which can optionally be included as debugging information in error responses. The language and content of the message is determined by the Service Provider. NOTE 6: "{other namespace identifier}" refers to a namespace other than m2m. NOTE 7: This applies to a polling response that contains a request for polling mechanism (see clause 7.4.22.2.2). NOTE 8: This applies to semantic query operation only. The Originator may use the Accept option to indicate which media types are acceptable for the semantic query result, e.g. application/sparql-results+xml, or application/sparql-results+json. |  |  |  |

The XML schema definition of the Content primitive parameter (i.e. datatype m2m:primitiveContent) allows to include XML wildcard elements. An XML representation of the Content primitive parameter shall include a root element which is associated with an XSD Global Element. The root element shall be prefixed with a namespace prefix identifier (e.g. m2m:) specified in the associated XSD which defines the respective Global Element. The Content primitive parameter allows to include namespaces other than m2m.

### -----------------------End of change 8-------------------------------------------

### -----------------------Start of change 9-------------------------------------------

##### 6.3.4.2.1	m2m:resourceType

Table 6.3.4.2.1-20: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 45 | localMulticastGroup |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | timeSyncBeacon |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 100XX | timeSyncBeaconAnnc |  |
| NOTE: See clause Error: Reference source not found. |  |  |

### -----------------------End of change 9-------------------------------------------

### -----------------------Start of change 10-------------------------------------------

##### 6.3.4.2.11	m2m:memberType

Used for the memberType attribute of the <group> resource.

Table 6.3.4.2.11-21: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | mixed | A mixture of all the resource types (except mixed itself). |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | timeSyncBeacon |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 100XX | timeSyncBeaconAnnc |  |
| 20001 | oldest |  |
| 20002 | latest |  |
| 20003 | mashup |  |
| NOTE: See clause Error: Reference source not found "Resource Type group". |  |  |

### -----------------------End of change 10-------------------------------------------

### -----------------------Start of change 11-------------------------------------------

### 6.5.4	announceableResource

#### 6.5.4.1	Description

This type definition includes the universal and common attributes used by oneM2M resource types that are capable of being announced. In addition to the attributes of a regularResource, it includes (as optional) the common attributes that are used by the announcement mechanism.

#### 6.5.4.2	Reference

See Error: Reference source not found.

#### 6.5.4.3	Usage

This type is used by the following resource types:

<AE>, <container>, <group>, <locationPolicy>, <node>, <remoteCSE>, <semanticDescriptor>, <timeSeries>, <ontologyRepository>, <ontology>, <ontologyMapping>, <ontologyMappingAlgorithm>, <ontologyMappingAlgorithmRepository>, <semanticMashupJobProfile>, <semanticMashupInstance>, <semanticMashupResult>, <multimediaSession>, <schedule>, <semanticRuleRepository>, <reasoningRules>, <reasoningJobInstance>, <timeSyncBeacon>.

It is also used by the specializations of <mgmtObj>.

### 6.5.5	announcedResource

#### 6.5.5.1	Description

This type definition includes the universal and common attributes used by a resource that is announcing an announceable resource. In addition to the attributes of a regularResource, it includes (as optional) the link common attribute.

#### 6.5.5.2	Reference

See Error: Reference source not found.

#### 6.5.5.3	Usage

This type is used by the following resource types:

<AEAnnc>, <containerAnnc>, <groupAnnc>, <locationPolicyAnnc>, <nodeAnnc>, <remoteCSEAnnc>, <semanticDescriptorAnnc>, <[flexContainer]Annc>, <timeSeriesAnnc>, <ontologyRepositoryAnnc>, <ontologyAnnc>, <ontologyMappingAnnc>, <ontologyMappingAlgorithmAnnc>, <ontologyMappingAlgorithmRepositoryAnnc>, <semanticMashupJobProfileAnnc>, <semanticMashupInstanceAnnc>, <semanticMashupResultAnne>, <multimediaSessionAnnc>,<scheduleAnnc>, <semanticRuleRepositoryAnnc>, <reasoningRulesAnnc>, <reasoningJobInstanceAnnc>, <timeSyncBeaconAnnc>.

It is also used by the xxxAnnc variants of the <mgmtObj> specializations.

### -----------------------End of change 11-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

…

Table 8.2.3-22: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| currentTime | CSEBase | ctm |
| enableTimeCompensation | AE, remoteCSE | etc |
| beaconRequester | timeSyncBeacon | bcnr |
| beaconCriteria | timeSyncBeacon | bcnc |
| beaconInterval | timeSyncBeacon | bcni |
| beaconThreshold | timeSyncBeacon | bcnt |

### -----------------------Start of change 12-------------------------------------------

### 8.2.4	Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-23.

Table 8.2.4-23: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| … | … |
| timeSyncBeacon | tsb |

### -----------------------End of change 12-------------------------------------------

### -----------------------Start of change 13-------------------------------------------

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-24: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| timeSyncBeaconReference | timeSyncBeaconNotification | tbr |
| currentTime | timeSyncBeaconNotification | ctm* |
| timeOffset | timeSyncBeaconNotification | tost |

### -----------------------End of change 13-------------------------------------------

### -----------------------Start of change 14-------------------------------------------

#### 6.3.5.XX	m2m:timeSyncBeaconNotification

Table 6.3.5.XX-25: Type Definition of m2m:timeSyncBeaconNotification

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| timeSyncBeaconReference | xs:anyURI | 1 |  |
| currentTime | m2m:timestamp | 1 |  |
| timeOffset | xs:duration | 0..1 |  |

### -----------------------End of change 14-------------------------------------------

### -----------------------Start of change 15-------------------------------------------

### 7.4.26	Announced resource types

#### 7.4.26.1	Introduction

Instances of certain resource types can be announced by the Hosting CSE to one or more remote CSEs to inform the remote CSEs of the existence of the original resource instance. Table 7.4.26.1-26 lists the resource types which are announceable together with its resource type identifier and the name of the XSD file that specifies its data type.

Resource-specific attributes of a resource type may be declared announceable (mandatory or optional) or not announceable. Clause 9.6 in oneM2M TS-0001 [Error: Reference source not found] defines which attributes of each resource type are announceable.

The universal/common attributes of an announced resource type are listed in Table 7.4.26.1-27, together with their request optionality in Create and Update requests (see clause 7.4.1). The announced resource includes a common attribute link which represents a link to the original resource hosted by the original resource-Hosting CSE.

The child resources applicable to each announced resource type are listed in Table 9.6.26.1-1 of oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.26.1-1: Data type definition of announced Resource types

| Data Type ID | File Name | Note |
| --- | --- | --- |
| accessControlPolicyAnnc | CDT-accessControlPolicy-v4_0_0.xsd |  |
| remoteCSEAnnc | CDT-remoteCSE-v4_0_0.xsd |  |
| AEAnnc | CDT-AE-v4_0_0.xsd |  |
| containerAnnc | CDT-container-v4_0_0.xsd |  |
| [flexContainer]Annc | See Annex J and oneM2M TS-0023 [Error: Reference source not found] | Announced variants of flexContainer specializations |
| contentInstanceAnnc | CDT-contentInstance-v4_0_0.xsd |  |
| scheduleAnnc | CDT-schedule-v4_0_0.xsd |  |
| locationPolicyAnnc | CDT-locationPolicy-v4_0_0.xsd |  |
| groupAnnc | CDT-group-v4_0_0.xsd |  |
| nodeAnnc | CDT-node-v4_0_0.xsd |  |
| semanticDescriptorAnnc | CDT-semanticDescriptor-v4_0_0.xsd |  |
| timeSeriesAnnc | CDT-timeSeries-v4_0_0.xsd |  |
| timeSeriesInstanceAnnc | CDT-timeSeriesInstance-v4_0_0.xsd |  |
| ontologyRepositoryAnnc | CDT-ontologyRepository-v4_0_0.xsd |  |
| ontologyAnnc | CDT-ontology-v4_0_0.xsd |  |
| semanticMashupJobProfileAnnc | CDT-semanticMashupJobProfile-v4_0_0.xsd |  |
| semanticMashupInstanceAnnc | CDT-semanticMashupInstance-v4_0_0.xsd |  |
| semanticMashupResultAnnc | CDT-semanticMashupResult-v4_0_0.xsd |  |
| genericInterworkingServiceAnnc | CDT-genericInterworkingService-v4_0_0.xsd |  |
| genericInterworkingOperationInstanceAnnc | CDT-genericInterworkingOperationInstance-v4_0_0.xsd |  |
| svcObjWrapperAnnc | CDT-svcObjWrapper-v4_0_0.xsd |  |
| svcFwWrapperAnnc | CDT-svcFwWrapper-v4_0_0.xsd |  |
| allJoynAppAnnc | CDT-allJoynApp-v4_0_0.xsd |  |
| allJoynSvcObjectAnnc | CDT-allJoynSvcObject-v4_0_0.xsd |  |
| allJoynInterfaceAnnc | CDT-allJoynInterface-v4_0_0.xsd |  |
| allJoynMethodAnnc | CDT-allJoynMethod-v4_0_0.xsd |  |
| allJoynMethodCallAnnc | CDT-allJoynMethodCall-v4_0_0.xsd |  |
| allJoynPropertyAnnc | CDT-allJoynProperty-v4_0_0.xsd |  |
| multimediaSessionAnnc | CDT-multimediaSession-v4_0_0.xsd |  |
| ontologyMappingAnnc | CDT-ontologyMapping-v4_0_0.xsd |  |
| ontologyMappingAlgorithmAnnc | CDT-ontologyMappingAlgorithm-v4_0_0.xsd |  |
| ontologyMappingAlgorithmRepositoryAnnc | CDT-ontologyMappingAlgorithmRepository-v4_0_0.xsd |  |
| semanticRuleRepositoryAnnc | CDT-semanticRuleRepository-v4_0_0.xsd |  |
| reasoningRulesAnnc | CDT-reasoningRules-v4_0_0.xsd |  |
| reasoningJobInstanceAnnc | CDT-reasoningJobInstance-v4_0_0.xsd |  |
| timeSyncBeaconAnnc | CDT-timeSyncBeacon-v4_0_0.xsd |  |

Table 7.4.26.1-2: Universal/Common Attributes of announcedResource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | NP | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | M | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| link | M | O |
| dynamicAuthorizationConsultationIDs | O | O |

Each announced resource type has the resource specific attributes that is the subset of the original resource.

Table 7.4.26.1-3: Resource Specific Attributes of announcedResource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| Name of attribute specified as MA | M | O | Same data type as defined for the original resource | This attribute shall be set to the same value with the attribute at the original resource |
| Name of attribute specified as OA | O | O | Same data type as defined for the original resource | This attribute shall be set to the same value with the attribute at the original resource |

The procedures defined in the following clause 7.4.26.2 apply to all announced resources regardless of the resource type.

### -----------------------End of change 15-------------------------------------------