| Input Contribution |  |
| --- | --- |
| Meeting ID* | SDS#45 |
| Title:* | Analysis on task offloading mechanisms and recommendation |
| Source:* | JaeSeung Song, KETI, jssong@sejong.ac.kr Minbyeong Lee, Hyundai Motors, minbyeong.lee@hyundai.com |
| Date:* | 2020-04-20 |
| Input related to* | WI-0080 Edge and Fog Computing TR-0052 V 0.11.0 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Impacted other TS/TR(s) |  |
| Decision requested or recommendation:* | Agree for inclusion in TR-0052 |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Introduction

This contribution provides an analysis on two proposed solutions for the edge/fog offloading key issue.

After an analysis, the solution 1 shows low complexity, low system impact compared to the solution 2.

Therefore, at the end of this contribution, it is recommended to move forward with the solution 1. In this case, the following extensions to the announcement resource are expected:

- a new attribute indicating the announcement resource is used for resource offloading

- a new attribute to define synchronization modes

Based on the analysis, a corresponding stage 2 contribution is prepared in SDS-2020-0144, Stage 2 offloading work using announcement concept.

### -----------------------Start of change 1-------------------------------------------

- Proposed Solutions

Editor’s Note: The section provides solutions to the Key Issues identified for employing Edge and Fog technologies in oneM2M.

  - Solution A: Edge/Fog Offloading using Resource Announcement

This solution addresses Key Issue 1. A resource can be announced to one or more remote CSEs to inform the remote CSEs of the existence of the original resource. In this case, an announced resource can have a limited set of attributes and a limited set of child resources from the original resource. Through introducing additional attributes to this announced resource, Edge/Fog offloading can be supported.

This solution proposes an Edge/Fog offloading mechanism allowing a resource can be offloaded to a remote CSE to support Edge/Fog computing. In this case all the selected resources are subjected to be transferred to a given remote CSE. A relationship between the original resource and offloaded resource is specified together with a synchronization mechanism.

    - Solution Applicability

This solution addresses the key issue 1 to support.

    - Solution Description

This solution proposes to enhance the resource announcement mechanism standardized in TS-0001 Section 9.6.26 [Error: Reference source not found] to support Edge/Fog resource offloading.

A set of resources can be offloaded to a remote CSE using resource announcement with additional attributes. This offloading type announcement transfer all the necessary information to the remote CSEs for Edge/Fog service. An announced resource with offloading attribute can have all the attributes and child resources from the original resource to support the same service as the original resource. The announced resource includes a link to the original resource hosted by the original resource-Hosting CSE. The original resource includes a link to the announced resource and a synchronization type information. After the original resources are offloaded to the announced resources at the remote CSE, all the requests to the original resources should be handled by the remote CSE to support full Edge/Fog services.

All the operations to the offloaded resources (i.e. announced resources) are synchronized with the original resources. The time for synchronization can be various as follows:

- Periodic: updates to the announced resources are periodically applied to the original resources

- Update: when there is any update to the announced resources, the changes are applied to the original resources immediately

- Finish: the original resources are updated only at the time the announced resources are deleted from the remote CSE and the Edge/Fog service is terminated. If an Application receiving the Edge/Fog service via the announced resources is moved to another Edge/Fog node, all the announced resources to the remote CSE are transferred to a new remote CSE. Then the link at the original resource is also updated to point the new remote CSE.

In case the announceType attribute is indicated as ‘offload’, the synchronization for the attributes between the original resource and the announced resource (i.e. offloaded resource) shall be the responsibility of the announced resource Hosting CSE.

Table 9.1.2-1 : Attributes for Edge/Fog offloading

| Attribute Name | Description |
| --- | --- |
| announceTo | This attribute may be included in a CREATE or UPDATE Request in which case it contains a list of addresses/CSE-IDs where the resource is to be announced. For the case that CSE-IDs are provided, the announced-to CSE shall decide the location of the announced resources based on the rules described in clause 9.6.26 of TS-0001 [Error: Reference source not found]. For the original resource, this attribute shall only be present if it has been successfully announced to other CSEs. This attribute maintains the list of the resource addresses to the successfully announced resources. Updates on this attribute will trigger new resource announcement or de-announcement. If announceTo attribute includes resource address(s), the present document does not provide any means for validating these address(s) for announcement purposes. It is the responsibility of the Hosting-CSE referenced by the resource address(s) to validate the access privileges of the originator of the Request that triggers the announcement. |
| announcedAttribute | This attribute shall only be present at the original resource if some Optional Announced (OA) type attributes have been announced to other CSEs. This attribute maintains the list of the announced Optional Attributes (OA type attributes) in the original resource. Updates to this attribute will trigger new attribute announcement if a new attribute is added or de-announcement if the existing attribute is removed. |
| announceType | This attribute indicates the type of resource announcement. Possible values are as follows: - Announcement: This is the original announcement type - Offload: This is an announcement type for resource offloading |
| synchMode | This attribute provides information about synchronization mechanisms between the original resource and the announced resource. This attribute shall only be presented when the announceType attributed is indicated as ‘Offload’. - Periodic: Synchronization is performed periodically - Update: Synchronization is performed whenever an update to announcedResource is performed - Finish: Synchronization is performed when an announcement is finished |
| postanncAccessMode | This attribute provides information about access policy to the original resource. This attribute shall only be presented when the announceType attributed is indicated as ‘Offload’. - Block: As all the resources are copied and operated at the announced resource, all the operations to the original resource are blocked until the announcement is finished. All the requests to the original resource are forwarded to the offloaded resource. - Readable: Only the RETRIEVE operation is allowed to the original resource. |

Editor’s Note: Consideration of Access Control Policy is TBD

  - Solution B: Edge/Fog Offloading using a New Resource Type

In TS-0001 [Error: Reference source not found], an announced resource is defined to have a limited set of attributes and a limited set of child resources from the original resource. In addition, there exist a fundamental difference in terms of synchronization between Edge/Fog offloading and the resource announcement, mainly because of the place where all the changes are occurring. Please note that all the operations are targeting to the original resource in the resource announcement. On the other hand, in the Edge/Fog offloading, all the operations are actively happening at the offloaded resources. Because of such reasons and potential side effects through introducing new features to the existing resource type for other purposes, a new resource type supporting Edge/Fog offloading is introduced in this section.

This solution addresses Key Issue 1. This solution proposes an Edge/Fog offloading mechanism allowing a resource to be offloaded to a remote CSE to support Edge/Fog computing. In this case selected resources can be offloaded to a given remote CSE. A relationship between the original resource and offloaded resource is specified together with a synchronization mechanism. A new resource type is introduced to support resource offloading. In TS-0001 [Error: Reference source not found], oneM2M resources are categorised into three resources:

- Normal resources

- Virtual resources

- Announced resources

This solution introduces a fourth type of resources called “Offloaded resources”.

An offloaded resource contains a set of attributes of the original resource. An offloaded resource is updated directly by oneM2M applications and the original resource is updated by the CSE hosting the offloaded resource. The offloaded resource contains a link to the original resource.

    - Solution Applicability

This solution addresses the key issue 1.

    - Solution Description

This solution requires a set of extension on different oneM2M entities.

AE: AE can trigger resource offloading via sending a request message to its hosting CSE with the following information.

- Target CSE: is a CSE hosting offloaded resources.

- Target resources: contain a set of resources to be offloaded at the target CSE

- Synchronization mode: specifies how to synchronize between offloaded and original resources

The CSE hosting original resources manages offloaded resources as follows:

- Indicate offloaded resources to show the resources are currently offloaded or not

- Synchronization time shows when the original resources have synchronized with the offloaded resources

- Status of resource offloading to show how to handle requests to the original resource while offloaded

- Link to the target CSE

The target CSE manages its offloaded resources as follows:

- Link to the CSE hosting original resources

- For each offloaded <resourceType>, the addition of suffix “Offload” to the original <resourceType> shall be used to indicate its associated offloaded resource type. For example, resource <containerOffload> shall indicate the offloaded resource type for <container> resource; <groupOffload> shall indicate offloaded resource type for <group> resource, etc.

- Synchronization with its original resources

  - Periodic: updates to the offloaded resources are periodically applied to the original resources

  - Update: when there is any update to the offloaded resources, the changes are applied to the original resources immediately

  - Finish: the original resources are updated only at the time the offloaded resources are deleted from the target CSE and the Edge/Fog service is terminated. If an Application receiving the Edge/Fog service via the offloaded resources is moved to another Edge/Fog node, all the offloaded resources on the target CSE are transferred to a new target CSE. Then the link at the original resource is also updated to point to the new target CSE. (exceptional cases are very critical in this case)

      - Impacted Resources and Attributes

        - Overview

To implement this solution, several resources need to be modified, and some new resource types need to be defined These are further detailed in the following clauses.

        - Modified Common Attributes

In order to make this solution work properly, TS-0001 [Error: Reference source not found]  needs to define additional attributes for resource offloading. Table 9.2.2.1.2-2 that is captured from the existing TS-0001 [Error: Reference source not found]  shows common attributes that are used in multiple resource types in oneM2M.  This solution proposes to add an additional four attributes, i.e., offloadTo, offloadSync, syncTime and syncResponsibility.

Table 9.2.2.1.2-2: Common Attributes from TS-0001 [Error: Reference source not found]

| Attributes Name | Description |
| --- | --- |
| announceTo | This attribute may be included in a CREATE or UPDATE Request in which case it contains a list of addresses/CSE-IDs where the resource is to be announced. For the case that CSE-IDs are provided, the announced-to CSE shall decide the location of the announced resources based on the rules described in clause 9.6.26 [Error: Reference source not found]. For the original resource, this attribute shall only be present if it has been successfully announced to other CSEs. This attribute maintains the list of the resource addresses to the successfully announced resources. Updates on this attribute will trigger new resource announcement or de-announcement. If announceTo attribute includes resource address(s), the present document does not provide any means for validating these address(s) for announcement purposes. It is the responsibility of the Hosting-CSE referenced by the resource address(s) to validate the access privileges of the originator of the Request that triggers the announcement. |
| announcedAttribute | This attributes shall only be present at the original resource if some Optional Announced (OA) type attributes have been announced to other CSEs. This attribute maintains the list of the announced Optional Attributes (OA type attributes) in the original resource. Updates to this attribute will trigger new attribute announcement if a new attribute is added or de-announcement if the existing attribute is removed. |
| offloadTo | This attribute may be included in a CREATE or UPDATE Request in which case it contains the address/CSE-IDs where the resource is to be offloaded. For the case that CSE-IDs are provided, the offloaded-to CSE shall decide the location of the offloaded resources. For the original resource, this attribute shall only be present if it has been successfully offloaded to the target CSE. This attribute maintains the resource identifier to the successfully offloaded resource. Updates on this attribute will trigger migration of an offloaded resource (or handover). |
| offloadSync | Synchronization modes is used to indicate how to synchronize data between the original and offloaded resources. This attribute is optional. The value of the offloadSynch attribute is an individual value as follows: - Periodic: updates to the offloaded resource are periodically applied to the original resources - Update: when there is any update to the offloaded resource, the changes are applied to the original resource immediately - Finish: the original resources are updated only at the time the offloaded resource is deleted from the remote CSE and the Edge/Fog service is terminated. |
| syncTime | If offloadSync attribute is set to ‘Periodic’, this attribute should be configured to set up timer performing synchronization. |
| syncResponsibility | This attribute setup which entity has responsibility to perform synchronization. The attribute can have either: - Original CSE: Original CSE has a responsibility to perform synchronization - Remote CSE: Remote Target CSE has a responsibility to perform synchronization |

        - New Resource Type for Offload

A resource can be offloaded to a remote CSE to such that is located closer to AEs that are accessing the resource. An offloaded resource can have a copy of all the attributes from the original resource. The offloaded resource includes a link to the original resource hosted by the original resource-Hosting CSE.

In case that the original resource is deleted, the offloaded resource for the original resource shall be deleted. If the offloaded resource is not deleted promptly (e.g. the offloaded resource is not reachable or providing a mission critical service, which may cause human security issue if the service stops immediately, for example, injecting insulin to human body), the offloaded resource can be deleted later either by the original resource Hosting CSE or by the expiration of the offloaded resource itself or by finishing a mission critical service. The original resource shall store the link for the offloaded resources for those purposes.

Synchronization between the the original resource and the offloaded resource shall be the responsibility of the original resource Hosting CSE. The access control policy for the offloaded resource shall synchronize with the one from the original resource. In case that the attribute accessControlPolicyIDs is not present in the original resource it is the responsibility of the original resource Hosting CSE to choose the appropriate value depending on the policy for the original resource (e.g. take the parent accessControlPolicyIDs value).

The original resource shall have at least offloadTo attribute present if the resource itself has been offloaded. An AE or other CSE can request the original resource Hosting CSE for offloading the original resource to the target CSE-ID or the address in the offloadTo attribute in the offloading request. An Update to the offloadTo attribute will trigger new resource offload or unload the offloaded resource. After a successful resource offload procedure the attribute offloadTo contains only the address of the offloaded resources.

On successful offload of the resource, the resource itself shall be created at the offloaded resource. Update to the attribute in the original resource is not allowed. The offloaded attributes shall have the same value as the original resource, and synchronization between the value of the offloaded attributes at the original resource and the offloaded resource is the responsibility of the original resource Hosting CSE.

It is also possible for the Hosting CSE of the target offloaded resource to have a responsibility synchronizing the value of the offloaded attributes at the original resource and the offloaded resource. In this case, the Hosting CSE of the target offloaded resource have to update the corresponding original resource.

Table 9.2.2.1.3-3: Offloade Resource Types

| Offloaded Resource Type | Short Description | Child Resource Types | Clause of TS-0001 [Error: Reference source not found] |
| --- | --- | --- | --- |
| accessControlPolicyOffload | Offloaded variant of accessControlPolicy | subscription | 9.6.2 |
| AEOffload | Offloaded variant of AE | subscription, container, containerOffload, flexContainer, flexContainerOffload, group, groupOffload, accessControlPolicy, accessControlPolicyOffload semanticDescriptor, semanticDescriptorOffload, timeSeries, timeSeriesOffload | 9.6.5 |
| containerOffload | Offloaded variant of container | container, containerOffload, flexContainer, flexContainerOffload, group, groupOffload, accessControlPolicy, accessControlPolicyOffload semanticDescriptor, semanticDescriptorOffload, timeSeries, timeSeriesOffload | 9.6.6 |
| contentInstanceOffload | Offloaded variant of contentInstance | semanticDescriptor, semanticDescriptorOffload | 9.6.7 |
| flexContainerOffload | Offloaded variant of flexContainer | container, containerOffload, flexContainer, flexContainerOffload, subscription, semanticDescriptor, semanticDescriptorOffload, timeSeries, timeSeriesOffload | 9.6.35 |
| groupOffload | Offloaded variant of group | subscription, semanticDescriptor, semanticDescriptorOffload | 9.6.13 |
| locationPolicyOffload | Offloaded variant of locationPolicy | None specified | 9.6.10 |
| mgmtObjOffload | Offloaded variant of mgmtObj | Subscription | 9.6.15 |
| nodeOffload | Offloaded variant of node | mgmtObjOffload, subscription, semanticDescriptor, semanticDescriptorOffload, scheduleOffload | 9.6.18 |
| remoteCSEOffload | Offloaded variant of remoteCSE | container, containerOffload, contentInstanceOffload, flexContainer, flexContainerOffload, group, groupOffload, accessControlPolicy, accessControlPolicyOffload, subscription, scheduleOffload, timeSeries, timeSeriesOffload, timeSeriesInstanceOffload, remoteCSEOffload, nodeOffload, mgmtObjOffload, AEAnnc, locationPolicyOffload | 9.6.4 |
| scheduleOffload | Offloaded variant of schedule | None specified | 9.6.9 |
| semanticDescriptorOffload | Offloaded variant of semanticDescriptor | Subscription | 9.6.30 |
| timeSeriesOffload | Offloaded variant of timeSeries | timeSeriesInstance, timeSeriesInstanceOffload, subscription, semanticDescriptor, semanticDescrptorOffload | 9.6.36 |
| timeSeriesInstanceOffload | Offloaded variant of timeSetriesInstance | None specified | 9.6.37 |

        - Universal Attributes for Offloaded Resources

Table 9.2.2.1.4-4 lists the universal attributes for the offloaded resources.

Table 9.2.2.1.4-4: Universal Attributes for Offloaded Resources

| Attributes Name | Mandatory /Optional | Description |
| --- | --- | --- |
| resourceType | Mandatory | Resource Type. As specified in clause 9.2 [Error: Reference source not found], a suffix of "Offload" to the name of the original resource type shall be used to indicate the name for the associated offloaded resource type. |
| resourceID | Mandatory | Identifies the resource at the remote CSE |
| resourceName | Mandatory | See clause 9.6.1.3 [Error: Reference source not found] for information on this attribute |
| parentID | Mandatory | Identifies the parent resource at the remote CSE. |
| creationTime | Mandatory | See clause 9.6.1.3 [Error: Reference source not found] for information on this attribute. |
| lastModifiedTime | Mandatory | See clause 9.6.1.3 [Error: Reference source not found] for information on this attribute. |
| expirationTime | Mandatory | See clause 9.6.1.3.2 [Error: Reference source not found] for information on this attribute. This attribute cannot exceed the value received from the original resource but it can be overridden by the policies of the remote CSE hosting the offloaded resource. |
| Link | Mandatory | Provides the URI to the original resource. |

        - Common Attributes for Offloaded Resources

Table 9.2.2.1.5-5 lists the common attributes for the offloaded resources.

- Table 9.2.2.1.5-5: Commonly Used Attributes for Offloaded Resources

| Attribute Name | Mandatory /Optional | Description |
| --- | --- | --- |
| accessControlPolicyIDs | Conditionally Mandatory | The list of identifiers (either an ID or a URI) of an <accessControlPolicy> resource offloaded by the original resource See clause 9.6.1.3.2 [Error: Reference source not found] for further information on this attribute. If this attribute was not present in the original resource, the original resource shall include this attribute by providing the accessControlPolicyIDs from the original resource's parent resource or from the local policy according at the original resource. |
| stateTag | Conditionally Mandatory | An incremental counter of modification on the resource. See clause 9.6.1.3.2 [Error: Reference source not found] for information on this attribute. |
| labels | Conditionally Mandatory | Tokens used as keys for discovering resources as offloaded by the original resource. See clause 9.6.1.3 [Error: Reference source not found] for further information on this attribute. The attribute is conditionally mandatory, which means that the attribute shall exist in the offloaded resource if it is present in the original resource. |

      - Impacted Information Flow

        - Introduction

This clause describes the procedures for offloading the original resource and  unloading the offloaded resource. A resource may be offloaded from its Hosting CSE to a target CSEs to inform the offloading target CSE of the existence of the original resource. The offloaded resource also may be unloaded from the offloading target CSE.

        - Procedure for AE and CSE to initiate Creation of an Offloaded Resource

This clause describes the procedure for an AE or a CSE to initiate the creation of an offloaded resource.

Figure 9.2.2.2.2-1: Offloaded resource CREATE procedures

The Originator of a Request for initiating resource offload can be either an AE or a CSE. Two methods are supported for initiating the creation of an offloaded resource:

CREATE: The Originator can initiate the creation of an offloaded resource during the creation of the original resource by providing offloadTo attribute in the CREATE Request.

UPDATE: The Originator can initiate the creation of an offloaded resource by using the UPDATE Request to update the offloadTo attribute at the original resource.

Table 9.2.2.2.2-6: Initiate Resource Offloading: UPDATE or CREATE

| Initiate Resource Offloading: CREATE or UPDATE |  |
| --- | --- |
| Associated Reference Points | Mca and Mcc. |
| Information in Request message | All parameters defined in table 8.1.2-3 [Error: Reference source not found] are applicable as indicated in that table. In addition, for the case of the CREATE procedure for a specific resource is described in clause 10.2 [Error: Reference source not found]. The Originator suggests the address(es) or the CSE-ID(s) to which the resource will be offloaded in the Content parameter. |
| Processing at the Originator before sending Request | Content: contains address where the resource needs to be offloaded (within offloadTo attribute): The Originator provides either the address(es) for the offloaded resource or the CSE-ID of the remote CSE where the original resource needs to be offloaded by including such information within the offloadTo attribute of the UPDATE or CREATE Request. |
| Processing at the Receiver | Once the Originator has been successfully authorized, the Receiver (which shall be the original resource Hosting CSE) shall grant the Request after successful validation of the Request: If the Request provides address(es) for the offloaded resource that are not already stored in the offloadTo attribute or for newly created offloadTo attribute, the Receiver shall offload the resource to the offload target CSE. If the Request provides a CSE-ID of the remote CSE that are not already stored in the offloadTo attribute of for the newly created or updated offloadTo attribute, the Receiver shall decide the location at the remote CSE identified by CSE-ID and offload the resource to the offloading target CSE. The original resource Hosting CSE shall first check if it is a Registree or the Registrar of the offloading target CSE. If that is the case, the offloaded resource shall be created as a direct child of the Hosting CSE’s <remoteCSE> resource hosted by the offloading target CSE. If that is not the case, the Hosting CSE shall next check if its <remoteCSE> resource has been offloaded to the offloading target CSE. The Hosting CSE shall perform this check by checking the offloadTo attribute of its <remoteCSE> resource hosted on its Registrar CSE if the offloading target CSE is not a descendent CSE, or the corresponding Registree CSE if the offloading target CSE is a descendent CSE. If it is not offloaded, the Hosting CSE shall request that its Registrar CSE (If the target CSE is not its descendant CSE) or Registree CSE (if the target CSE is its descendant CSE) to create a <remoteCSEOffload> resource representing the Hosting CSE as a direct child of the <CSEBase> of the offloading target CSE. The offloaded resource shall then be created by the Hosting CSE as a direct child resource of the <remoteCSEOffload> resource. |
| Information in Response message | On successful completion of resource offloading as in clause 10.2.3.5 [Error: Reference source not found], the Receiver shall provide all parameters defined in table 8.1.3-1 [Error: Reference source not found] that are applicable as indicated in that table in the Response message: The Receiver shall provide the address(es) of the offloaded resource to the Originator by updating the content of the offloadTo attribute in the original resource and by providing it in the UPDATE or CREATE Response message depending on the type of the Request. |
| Processing at Originator after receiving Response | According to clause 10.1.2 [Error: Reference source not found] in case of CREATE Request. According to clause 10.1.4 [Error: Reference source not found] in case of UPDATE Request. |
| Exceptions | All exceptions described in the basic procedures (clause 10.1.2 [Error: Reference source not found]) are applicable. |

        - Procedure at AE or CSE to Retrieve information from an Offloaded Resource

This clause describes the procedures that shall be used for an AE or a CSE to retrieve information about an offloaded resource or the corresponding original resource.

Figure 9.2.2.2.3-2: Offloaded resource RETRIEVE procedures

The Originator of a Request for initiating retrieval of information about a resource can be either an AE or a CSE. The Originator initiates this procedure by using RETRIEVE Request.

Table 9.2.2.2.3-7: Offloaded Resource Information Retrieval: RETRIEVE

| Resource Retrieval: RETRIEVE |  |
| --- | --- |
| Associated Reference Points | Mca and Mcc. |
| Information in Request message | Clause 8.1.2 [Error: Reference source not found] specifies the information to be included in the Request message. Table 8.1.2-3 [Error: Reference source not found] also describes the parameters that are applicable in the Request message: Specifically, the To parameter is set to the address of the offloaded resource to be retrieved. If a specific attribute is to be retrieved, the address of such attribute is included in the To parameter. The Originator can specify one of the values for the optional Result Content parameter. The Originator can request retrieval of the original resource by targeting the offloaded resource at the Hosting CSE by setting the Result Content parameter to the "original-resource". |
| Processing at the Originator before sending Request | The Originator can request retrieval of information from an offloaded resource at the Hosting CSE. |
| Processing at the Receiver | Once the Originator has been successfully authorized, the Receiver (Hosting CSE) shall grant the Request after successful validation of the Request: Information from the identified offloaded resource (at Hosting CSE) shall be returned to Originator via RETRIEVE Response, as described in clause 8.1.2 [Error: Reference source not found]. If Result Content request message parameter set to "original-resource" is included in the Request message, the Receiver shall provide the representation of the original resource indicated by the link attribute in the offloaded resource. The Receiver shall retrieve the original resource to return the representation of the original resource to the Originator. |
| Information in Response message | Information from the identified offloaded resource (at Hosting CSE), or the original resource shall be returned to Originator via RETRIEVE Response, as described in clause 8.1.3 [Error: Reference source not found]. |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.3 [Error: Reference source not found]) are applicable. |

        - Synchronization Procedures

This clause describes how synchronization is performed between the Originator and the Receiver (Hosting CSE).

Figure 9.2.2.2.4-3: Offloaded resource RETRIEVE procedures

There exist three different types of resource offloading synchronization, i.e., time-, update- and finalization-based synchronization.

Depending on the value of the offloadSync attribute, different synchronization procedure is taken by the Original Resourse Hosting CSE or Offloading Target CSE,

Table 9.2.2.2.4-8: Offloaded Resource Synchronization: UPDATE or RETRIEVE

| Resource Synchronization: UPDATE or RETRIEVE |  |
| --- | --- |
| Associated Reference Points | Mca and Mcc. |
| Information in Request message | All parameters defined in table 8.1.2-3 [Error: Reference source not found] are applicable as indicated in that table. |
| Processing at the Originator before sending Request | The Originator shall perform one of the following for the synchronization depending on the value of the offloadSync attribute: Periodic: The Originator shall request to update the offloaded resource at the original resource Hosting CSE at every time t defined in the syncTime attribute. Update: The Originator shall request to update the offloaded resource at the original resource Hosting CSE whenever there is an update to the offloaded resource at the Offloading Target CSE. Finish: The Originator shall request to retrieve the offloaded resource at the Offloading Target CSE. |
| Processing at the Receiver | Once the Originator has been successfully authorized, the Receiver shall grant the Request after successful validation of the Request: Finish: Information from the identified offloaded resource shall be returned to Originator via RETRIEVE Response. Periodic and Update: Information from the Originator shall be used to update the original resource of the offloading resource |
| Information in Response message | On successful completion of synchronization at the Receiver, the Receiver know that the updated information has been synchronized: Finish: The updated information to the offloaded resource shall be provided to the Originator to be synchronized Periodic and Update: The Receiver shall provide confirmation of the synchronization to the Originator. |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.4 [Error: Reference source not found]) are applicable for UPDATE operation. All exceptions described in the basic procedure (clause 10.1.5 [Error: Reference source not found]) are applicable for DELETE operation. |

        - Finalization Procedures

This clause describes the procedure that shall be used for an AE or a CSE (not the original resource Hosting CSE) to initiate the deletion of an offloaded resource.

Figure 9.2.2.2.5-1 depicts how deletion of an offloaded resource is initiated and the offloaded resource is deleted on an offloading target CSE.

Figure 9.2.2.2.5-4: Finaliztion procedures of offloaded resource

The Originator of a Request for initiating resource offloading finalization can be either an AE or a CSE. Two methods are supported for initiating resource offloading finalization.

UPDATE: The Originator can request to initiate the deletion of an offloaded resource by using UPDATE Request to the offloadTo attribute at the original resource Hosting CSE.

DELETE: Resource offloading finalization (deletion) shall also be performed when the Originator deletes the original resource at the original resource Hosting CSE by using DELETE Request.

Table 9.2.2.2.5-9: Finalization of offloading service: UPDATE and DELETE

| Finalization of offloading: UPDATE or DELETE |  |
| --- | --- |
| Associated Reference Points | Mca and Mcc. |
| Information in Request message | All parameters defined in table 8.1.2-3 [Error: Reference source not found] are applicable as indicated in that table. |
| Processing at the Originator before sending Request (Synchronization Phyase) | The Originator shall perform one of the following for the deletion of an offloaded resource: The Originator shall request to update the offloadTo attribute at the original resource Hosting CSE by providing new content of the offloadTo attribute. The Originator shall request to delete the offloadTo attribute at the original resource Hosting CSE by sending UPDATE Request that sets the value of the offloadTo attribute to NULL for the deletion of the offloaded resource. For DELETE operation, the Originator shall include the resource address of the original resource Hosting CSE that needs to be deleted, in the DELETE Request. Content: Void. |
| Processing at the Receiver (Synchronization Phyase) | Once the Originator has been successfully authorized, the Receiver (which shall be the original resource Hosting CSE) shall grant the Request after successful validation of the Request. The Receiver shall be the resource Hosting CSE. On receiving the UPDATE or DELETE Request, the Receiver shall check any update from the offloaded resources before deleting it. Therefore, the Receiver shall request to retrieve the offloaded resource. If the offloaded resource has been updated at the Offloading Target CSE, the Receiver updates the original resource at the original resource Hosting CSE. |
| Processing at the Originator before sending Request (Finalization Phase) | After the successful synchronization, the Originator shall perform one of the following for the deletion of an offloaded resource: The Originator shall request to update the offloadTo attribute at the original resource Hosting CSE by providing new content of the offloadTo attribute. The Originator shall request to delete the offloadTo attribute at the original resource Hosting CSE by sending UPDATE Request that sets the value of the offloadTo attribute to NULL for the deletion of the offloaded resource. For DELETE operation, the Originator shall include the resource address of the original resource Hosting CSE that needs to be deleted, in the DELETE Request. Content: Void. |
| Processing at the Receiver (Finalization Phase) | Once the Originator has been successfully authorized, the Receiver (which shall be the original resource Hosting CSE) shall grant the Request after successful validation of the Request. The Receiver shall be the resource Hosting CSE. On receiving the UPDATE or DELETE Request, the Receiver shall perform as follows: For UPDATE Request, the Receiver shall request to delete the offloaded resource whose address is not the same in the offloadTo attribute of the request. For DELETE Request, the Receiver shall request to delete the offloaded resource in the offloadTo attribute |
| Information in Response message | On successful completion of resource offloading finalization procedure, the Receiver knows that the offloaded resource has been deleted: The Receiver shall provide confirmation of resource offloading finalization to the Originator. |
| Exceptions | All exceptions described in the basic procedure (clause 10.1.4 [Error: Reference source not found]) are applicable for UPDATE operation. All exceptions described in the basic procedure (clause 10.1.5 [Error: Reference source not found]) are applicable for DELETE operation. |

    - Comparison on Offloading Alternatives

Two solutions are proposed for Key Issue 1. The first solution (see Sec. 9.1) uses the announcement resource while the second solution (see Sec. 9.2) uses a dedicated resource to manage resource offloading. In this section, two solutions are analysed in terms of three criteria, i.e., system impact, complexity and extensibility.

- System impact: checks that how much impact to existing oneM2M system.

- Complexity: checks that how complicated to realize the concept.

- Extensibility: checks that how easy to add new features

Here is a brief summary of two solutions.

- Solution 1: A resource can be offloaded to a remote CSE using the announcement resource. In order to support the necessary features of offloading concept, additional attributes have to be introduced mainly for synchronization between offloaded resource and offloading resource.

- Solution 2: A resource can be offloaded to a remote CSE using a dedicated resource. Similar to the announcement resource, both offloading and offloaded resources should have several attributes, such as the indication of offloading, links between two associated resources, synchronization.

- The table below shows the analysis of two proposed solutions in terms of three criteria.

| Criteria | Result | Solution 1 (announcement resource) | Solution 2 (dedicated resource) |
| --- | --- | --- | --- |
| Impact | Sol1 | Low impact Only added attributes to the announcement resource should be investigated for the system impact. | High impact There may be a various hidden impact on other existing resources. |
| Compexity | Sol1 | Low complexity As the existing resource is used, the complexity stays low. | High complexity The newly introduced resource can increase the complexity of the system. |
| Extensibility | Sol2 | Low extensibility At this moment, the identified requirements for the offloading concept can be supported through the enhancements to the announcement resource. But it is not clear whether further additional features (if identified) can be easily adapted to the announcement resource. | High extensibility As a new resource dedicated to offloading is introduced, additional new features can be easily added. |

Based on the analysis above, solution 1 shows low system impact and complexity compare to solution 1. This means that solution 1 can be easily adapted to the existing oneM2M system.

In the case of solution 1, the following enhancements to the resource announcement are expected:

- Introduce two types of announced resource, i.e., updatable and non-updatable

- Enhanced synchronization mechanism to support changes on announced resource(s)

-----------------------End of change 1-------------------------------------------