| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#45 |
| Source:* | SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr Peter Niblett, IBM, peter_niblett@uk.ibm.com |
| Date:* | 2020-04-22 |
| Reason for Change/s:* | To support Mcc’ communications over MQTT binding |
| CR against: Release* | 2 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: input.doc STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0010 v2.9.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | n/a |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This is the mirror CR to the agreed one (input.doc for Rel-2.

### ********** Start of change 1 **********

### 6.4.2	Topic Name for Requests

A request is transmitted by sending it as an MQTT PUBLISH Packet to the MQTT Server. The MQTT Publish Packet uses a Topic Name that identifies both the Originator and the Receiver of the request as follows:

- /oneM2M/req/<originator>/<receiver>/<type>

- "oneM2M" is a literal string identifying the topic as being used by oneM2M.

- <originator> is the SP-relative-AE-ID or SP-relative-CSE-ID of the entity that sends the request on Mca or Mcc reference point, omitting any leading "/"s and replacing any other "/" characters with ":" characters.

- <originator> is the Absolute-CSE-ID of an IN-CSE on Mcc’ reference point, replacing the leading "//" with ":" and replacing any other "/" characters with ":" characters.

- <receiver> is the SP-relative-AE-ID or SP-relative-CSE-ID of the Receiver (AE, Transit CSE or Hosting CSE) on Mca or Mcc reference point, omitting any leading "/"s and replacing any other "/" characters with ":" characters.

- <receiver> is the Absolute-CSE-ID of an IN-CSE on Mcc’ reference point, replacing the leading "//" with ":" and replacing any other "/" characters with ":" characters..

- "req" is a literal string identifying this as a request.

- <type> is "xml", "json" or "cbor" indicating the MQTT payload data type as described in clause 6.5.4.

### 6.4.3	Listening for and responding to a Request

A Receiver listens for requests arriving via MQTT by subscribing using a wildcarded Topic Filter of the following form:

- /oneM2M/req/+/<receiver>/<type>

- "oneM2M" is a literal string identifying the topic as being used by oneM2M.

- + is a wildcard which matches any entity.

- <receiver> is the SP-relative-AE-ID or SP-relative-CSE-ID of the Receiver (AE, Transit CSE or Hosting CSE) on Mca or Mcc reference point, omitting any leading "/"s and replacing any other "/" characters with ":" characters.

- <receiver> is the Absolute-CSE-ID of an IN-CSE on Mcc’ reference point, replacing the leading "//" with ":" and replacing any other "/" characters with ":" characters.

- "req" is a literal string identifying this as a request.

- <type> is "xml", "json" or "cbor" indicating the MQTT payload data type as described in clause 6.5.4. A wildcard may be used to listen for any type.

When it receives a request, the Receiver shall perform the Core Transport operations associated with the request, including any access control policy checks. In particular it shall check the request expiration timestamp (if any) contained in the request, since it is possible that that time might have passed while the message was being stored by MQTT.

It transmits a response by sending an MQTT PUBLISH Packet to a response topic. This takes the form:

- /oneM2M/resp/<originator>/<receiver>/<type>

- "oneM2M" is a literal string identifying the topic as being used by oneM2M.

- <receiver> is the SP-relative-AE-ID or SP-relative-CSE-ID of the Receiver (AE, Transit CSE or Hosting CSE) on Mca or Mcc reference point, omitting any leading "/"s and replacing any other "/" characters with ":" characters.

- <receiver> is the Absolute-CSE-ID of an IN-CSE on Mcc’ reference point, replacing the leading "//" with ":" and replacing any other "/" characters with ":" characters.

- <originator> is the SP-relative-AE-ID or SP-relative-CSE-ID of the entity that sent the corresponding request on Mca or Mcc reference point, omitting any leading "/"s and replacing any other "/" characters with ":" characters.

- <originator> is the Absolute-CSE-ID of an IN-CSE on Mcc’ reference point, replacing the leading "//" with ":" and replacing any other "/" characters with ":" characters.

- "resp" is a literal string identifying this as a response.

- <type> is "xml", "json" or "cbor" indicating the MQTT payload data type as described in clause 6.5.4.

The Originator shall subscribe to this Topic (either explicitly or using a wildcarded filter) in order to see the response.

The payload of the MQTT PUBLISH packet is used to carry the response primitive, as described in clause 6.5.

### ********** End of change 1 **********

### ********** Start of change 2 **********

### 6.4.5	Request/Response Message Flow (Mca or Mcc)

Figure 6.4.5-1: Initiating Process in MQTT binding

In the MQTT protocol, each client shall subscribe to the MQTT server to receive messages. As shown in figure 6.4.5-1, the AE or CSE initiates the MQTT binding process by trying to connect to the MQTT server, as described in clause 6.3.

After each MQTT client successfully connects to the server, it shall subscribe to the MQTT server. The Topic Filters with which each MQTT client subscribes are "/oneM2M/req/+/<receiver>" (to receive requests) and "/oneM2M/resp/<originator>/+" (to receive replies) where <receiver> and <originator> are both set equal to the ID (SP-relative-AE-ID or SP-relative-CSE-ID as appropriate).

Accordingly the plus sign ('+' U+002B) wildcard and the SP-relative-AE-ID or SP-relative-CSE-ID are used in the Topic Filter within the MQTT SUBSCRIBE Packet. This enables the MQTT client to receive the PUBLISH Packets whose target it is. Therefore, through this process, the AE or CSE receives messages if the request/response messages are published to "/oneM2M/req/<originator>/<receiver>" or "/oneM2M/resp/<originator>/<receiver>".

Figure 6.4.5-2: Request/Response message delivery over MQTT

As an example, Figure 6.4.5-2 illustrates the Request/Response message delivery over MQTT protocol between Originator and Receiver via the Mca/Mcc reference point in oneM2M. The message flow is as follows.

In this flow, the Originator wants to send a Request message to the Receiver. The Originator's MQTT client library sends an MQTT PUBLISH Packet to the MQTT server with "/oneM2M/req/ SP-relative-AE-ID/SP-relative-CSE-ID" as the Topic Name.

The MQTT PUBLISH packet shall include {"op", "fr", "to", "rqi"} and any optional parameters in accordance with the operation (CREATE, RETRIEVE, UPDATE, DELETE, NOTIFY) as specified in clause 7.2.1.1 of oneM2M TS-0004 [Error: Reference source not found] in its payload.

When the MQTT server receives the MQTT PUBLISH Packet from the MQTT client, the server refers to the Topic Name and delivers the message to the intended MQTT client. Finally, the MQTT client library delivers the message to the Receiver.

After that, the Receiver builds a Topic Name for the Response message of the form "/oneM2M/resp/Originator-ID/Receiver-ID". The payload of the MQTT PUBLISH Packet shall include {"rsc", "rqi"} and any optional message parameters in accordance with operation (CREATE, RETRIEVE, UPDATE, DELETE, NOTIFY) as specified in clause 7.2.1.2 of oneM2M TS-0004 [Error: Reference source not found] in its payload.

### ********** End of change 2 **********

### ********** Start of change 3 **********

# 7	Security

## 7.1	Introduction

The MQTT servers authenticate the clients (both CSEs and AEs) that connect to them and authorize access to Topics used to communicate. The clients do not authenticate each other, instead they use the MQTT server to do this.

Background. The MQTT binding makes use of one or more MQTT Servers to transport messages between AEs and CSEs (or between CSEs) as described in clause 5. The AE/CSEs both act as MQTT Clients of an MQTT Server that mediates delivery of messages between the two. As described in clause 6, the topic in the MQTT PUBLISH packet either indicates the Originator's identity and Receiver's identity, or includes the Originator's Credential and the Receiver's Identity (in the case that the Originator has not yet been assigned an identifier).

Trust and the MQTT Server. When the oneM2M binding to MQTT is used, some security functions are performed by the MQTT Server, as described further in this clause. In particular:

- The MQTT Server authenticates the AEs and CSEs as they connect as MQTT Clients. These MQTT Clients themselves never directly authenticate the CSE or AE that is using another MQTT Client - instead they trust the MQTT Server to authenticate the MQTT Clients, and trust the MQTT Server to route the messages between the Originator and Receiver indicated in the topic.

- The MQTT Server enforces access control policies to ensure that unidentified or unauthorized clients are not able to publish messages to oneM2M topics or subscribe to receive messages from them.

The remainder of this clause concerns Authentication and Authorization when the MQTT protocol binding is being used for the Mca or Mcc reference points. Details of Authentication and Authorization over the Mcc’ reference point need to be agreed between the relevant service providers.

### ********** End of change 3 **********

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?