| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 45 |
| Source:* | Dale Seed, Convida, Seed.Dale@convidawireless.com Lu Liu, Convida, Liu.Lu@convidawireless.com |
| Date:* | 2020-04-24 |
| Reason for Change/s:* | Add support for dynamic software management |
| CR against: Release* | Release 4 |
| CR against: WI* | Active – WI-0080 – Study on Edge and Fog Computing in oneM2M MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 V4.4.0 |
| Clauses * | 9.6, 10.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | TS-0004 Rel-4 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This contribution proposes to add support for software management functionality defined in TR-0052 into TS-0001.

### -----------------------Start of change 1-------------------------------------------

##### 9.6.x Resource Type softwarePolicy

The <softwarePolicy> resource provides policies and rules for the management of software by a Hosting CSE. A <softwarePolicy> resource is used to trigger a Hosting CSE to install, uninstall, activate or deactivate software via the creation and management of [software] specialization(s) by the Hosting CSE. An AE can create <softwarePolicy> resource(s) on a Hosting CSE to instruct the Hosting CSE to manage the software versions of its Registrees (e.g. MN/ASN-CSEs) on the AE’s behalf. For example, a service provider (e.g. IN-AE) can create <softwarePolicy> resource(s) on a Hosting CSE (e.g. IN-CSE) to install and manage software on one or more Registree CSEs or AEs of the Hosting CSE.

The <softwarePolilcy> resource shall contain the child resources specified in table 9.6.x-1.

Table 9.6.x-1: Child resources of <softwarePolicy> resource

| Child Resources of <state> | Child Resource Type | Multiplicity | Description | <stateAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <schedule> | 0..n | See clause 9.6.9 | <schedule> |

The <softwarePolicy> resource shall contain the attributes specified in table 9.6.x-2.

Table 9.6.x-2: Attributes of <softwarePolicy> resource

| Attributes of <state> | Multiplicity | RW/ RO/ WO | Description | <stateAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| version | 1 | RW | The version of the software. The Hosting CSE copies the value of this attribute to the version attribute of the [software] specializations that it creates based on this <softwarePolicy> resource. This attribute is read-only when the value of the aggregatedSoftwareStatus attribute is PENDING or IN PROGRESS. | OA |
| name | 1 | RW | The name of the software. The Hosting CSE copies the value of this attribute to the name attribute of the [software] specializations that it creates based on this <softwarePolicy> resource. This attribute is read-only when the value of the aggregatedSoftwareStatus attribute is PENDING or IN PROGRESS. | OA |
| URL | 1 | RW | The URL from which the software package can be downloaded. The Hosting CSE copies the value of this attribute to the URL attribute of the [software] specializations that it creates based on this <softwarePolicy> resource. This attribute is read-only when the value of the aggregatedSoftwareStatus attribute is PENDING or IN PROGRESS. | OA |
| softwareTargets | 1(L) | RW | A list of target entities that the software management operation defined by this resource is to be performed upon. Each entry in the list is specified as a resource identifier of a Registree’s <node> resource corresponding to a Registree targeted for software management by the Hosting CSE. The Hosting CSE targets the specified <node> resource(s) when creating [software] specialization(s) associated with this resource. This attribute is read-only when the value of the aggregatedSoftwareStatus attribute is PENDING or IN PROGRESS. | OA |
| softwareTriggerCriteria | 0..1 | RW | Defines the conditions that the Hosting CSE monitors to detect if/when to perform the specified software management operation defined by this resource. This attribute consists of a subjectResourceID parameter and an evalCriteria parameter having the same format and definitions as those specified in the <action> resource defined in clause 9.6.61. If this attribute is not configured then the Hosting CSE shall attempt to perform the software management operations unconditionally upon creation of this resource. This attribute is read-only when the value of the aggregatedSoftwareStatus attribute is PENDING or IN PROGRESS. | OA |
| softwareOperation | 1 | RW | Defines the software management operation to be performed by the Hosting CSE. The Hosting CSE uses the value of this attribute to configure the install, uninstall, activate and deactivate attributes of the [software] specializations it creates based on this <softwarePolicy> resource. The supported values include: - INSTALL - UNINSTALL - ACTIVATE - DEACTIVATE This attribute is read-only when the value of the aggregatedSoftwareStatus attribute is PENDING or IN PROGRESS. | OA |
| softwareLinks | 1(L) | RO | A list of resource identifiers of the [software] specializations created by the Hosting CSE based on this <softwarePolicy> resource. | OA |
| aggregatedSoftwareStatus | 1 | RO | Defines the aggregated status of the [software] specialization(s) that are managed through this <softwarePolicy> resource. The supported values include: PENDING – The Hosting CSE has not yet performed the software management operation defined by this <softwarePolicy> resource (e.g. the softwareTriggerCriteria have not yet been met). This is the default value when the <softwarePolicy> resource is created. IN PROGRESS – The Hosting CSE has started but not yet completed the software management operation defined by this <softwarePolicy> resource. INSTALLED – The software associated with this <softwarePolicy> resource has been installed, i.e. the installStatus attribute of each [software] specialization linked through this <softwarePolicy> resource has a value of “Installed”. This status value is only applicable when the softwareOperation has a value of INSTALL. UNINSTALLED – The software associated with this <softwarePolicy> resource has been uninstalled, i.e. the installStatus attribute of each [software] specialization linked through this <softwarePolicy> resource has a value of “Uninstalled”. This status value is only applicable when the softwareOperation has a value of UNINSTALL. ACTIVATED – The software associated with this <softwarePolicy> resource has been activated, i.e. the activeStatus attribute of each [software] specialization linked through this <softwarePolicy> resource has been set to “Activated”. This status value is only applicable when the softwareOperation has a value of ACTIVATE. DEACTIVATED – The software associated with this <softwarePolicy> resource has been deactivated, i.e. the activeStatus attribute of each [software] specialization linked through this <softwarePolicy> resource has been set to “Deactivated”. This status value is only applicable when the softwareOperation has a value of DEACTIVATE. ERROR – The software management operation was triggered and initiated by the Hosting CSE but did not complete successfully on all the softwareTargets. An error was encountered on one or more of the softwareTargets. The individual [software] specializations hosted on the softwareTargets should be checked for individual status information. | OA |
| individualSoftwareStatus | 1(L) | RO | A list of status information for each of the softwareTargets. Each entry in the list is a tuple consisting of a resource identifier of a [software] specialization and an individual status. The supported values for the status are the same as those defined for the aggregatedSoftwareStatus attribute. | OA |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

- 10.2.z	Software Management

- 10.2.z.1	Introduction

This clause describes the procedure for software management by a Hosting CSE via the <softwarePolicy> resource.

An Originator (e.g. IN-AE) can instruct a Hosting CSE (e.g. IN-CSE) to install, uninstall, activate or deactivate software instances on its Registree entities (e.g. CSEs or AEs) on behalf of the Originator by creating <softwarePolicy> resource(s) at the Hosting CSE. The <softwarePolicy> resource specifies the version, name and URL of the software to be installed.  The  Hosting CSE shall copy this information to the corresponding attributes when creating [software] specializations. The target nodes where the software is to be installed, uninstalled, activated or deactivated are specified in the softwareTargets attribute. The Hosting CSE shall monitor the conditions defined in the softwareTriggerCriteria attribute to determine when to perform the software management operation on these nodes, while the softwareOperation attribute defines which operation shall be performed.

Upon creation of the <softwarePolicy> resource, the aggregatedSoftwareStatus attribute of the <softwarePolicy> resource shall be set to the default value of PENDING by the Hosting CSE. If the softwareTriggerCriteria attribute is not configured, then the Hosting CSE shall create the [software] specialization(s) on the desired targets and perform the operation specified by the softwareOperation attribute. Otherwise, the Hosting CSE shall monitor the conditions defined by the softwareTriggerCriteria attribtue. When the conditions are met, the Hosting CSE shall create the [software] specialization(s) to perform the operation specified by the softwareOperation attribute. If a <schedule> child resource of the <softwarePolicy> resource exists, the Hosting CSE shall schedule the creation of the [software] specialization(s) to perform the operation specified by the softwareOperation attribute according to the time windows defined in the <schedule> resource. The Hosting CSE shall record the resource identifier of each [software] specialization in the softwareLinks attribute and update the value of aggregatedSoftwareStatus attribute to a value of IN PROGRESS. The Hosting CSE shall also subscribe to the created [software] specialization(s) to get notifications of status information.

If the softwareOperation attribute has a value of INSTALL or UNINSTALL, the Hosting CSE shall accordingly set the install or uninstall attribute to TRUE for each [software] specialization linked through the softwareLinks attribute. When the Hosting CSE receives a notification that the installStatus attribute of a [software] specialization has been updated to a value of installed or uninstalled, the Hosting CSE shall update the corresponding entry in the individualSoftwareStatus attribute to a value of INSTALLED or UNINSTALLED, accordingly. When the Hosting CSE detects that the installStatus attribute of all the [software] specializations have been updated to installed or uninstalled, the Hosting CSE shall update the aggregatedSoftwareStatus attribute to INSTALLED or UNINSTALLED, accordingly.

If the softwareOperation attribute has a value of ACTIVATE or DEACTIVATE, the Hosting CSE shall accordingly set the activate or deactivate attribute to TRUE for each [software] specialization linked through the softwareLinks attribute. When the Hosting CSE receives a notification that the activeStatus attribute of a [software] specialization has been updated to a value of activated or deactivated, the Hosting CSE shall update the corresponding entry in the individualSoftwareStatus attribute to a value of ACTIVATED or DEACTIVATED, accordingly. When the Hosting CSE detects that the activeStatus attribute of all [software] specializations have been updated to a value of activated or deactivated, the Hosting CSE shall update the aggregatedSoftwareStatus attribute to ACTIVATED or DEACTIVATED, accordingly.

If the Hosting CSE is not able to complete the management operations on all the softwareTargets, a timeout occurs,  and/or an error is encountered on one or more of the softwareTargets, the Hosting CSE shall set the aggregatedSoftwareStatus attribute to a value of ERROR.

- 10.2.z.2	Software Management Procedure

This clause describes the software management procedure performed by a Hosting CSE on a Registree entity via the <softwarePolicy> resource:

Figure 10.2.z.2-1: Software Management by a CSE

Step 0:	The Registree(s) register to the Hosting CSE (i.e. Registrar CSE) by creating <remoteCSE> or <AE> resource(s). The nodeLink attribute of a Registree <remoteCSE> or <AE> resource is configured with the resource identifier of the Registree’s <node> resource.

Step 1:	An Originator discovers/retrieves the <remoteCSE> and/or <AE> resource(s) of the Registree(s) to obtain the resource identifiers of the corresponding <node> resources.

Step 2:	The Originator creates <softwarePolicy> resource(s) on the Hosting CSE which the Hosting CSE uses to manage (on the Originator’s behalf) the software of the Registree(s). The Originator configures the attributes of the <softwarePolicy> resource(s) to specify the software operation to perform, the targeted nodes to perform the software operation on and trigger criteria to control if/when the Hosting CSE is to perform the software operation.  When a <softwarePolicy> resource is created, the Hosting CSE shall set the softwareStatus attribute to PENDING.  The Hosting CSE shall also check that the request to create the <softwarePolicy> does not conflict with any existing <softwarePolicy> resources. For example, two <softwarePolicy> resources are targeting the same node and have the same trigger conditions but have different software versions.

Optionally, the Originator may also create a <schedule> child resource of the <softwarePolicy> resource to define the time windows of when software management operations may be performed by the Hosting CSE once triggered.

Optionally, the Originator may also create a <subscription> child resource of the <softwarePolicy> resource to receive notifications if/when the Hosting CSE performs software management operations.

Step 3:	The Hosting CSE shall monitor and detect when the criteria defined by the softwareTriggerCriteria attribute of the <softwarePolicy> resource have been met. If the criteria are met, the Hosting CSE shall check if a <schedule> child resource exists for the <softwarePolicy> resource. If a <schedule> resource exists, the Hosting CSE shall check the defined schedule time windows of the <schedule> resource to determine if the triggered software management operation can be performed immediately. Otherwise, the Hosting CSE shall buffer any triggered software management operations until the defined time windows allow the Hosting CSE to perform the operations.  When the Hosting CSE starts the software management operation, it updates aggregatedSoftwareStatus to a value of IN PROGRESS.

Step 4:	The Hosting CSE shall identify the target nodes to perform software management operations upon based on the softwareTargets attribute.

Step 5:	The Hosting CSE shall create a child [software] specialization under each applicable <node> resource specified by the softwareTargets attribute and shall configure the attributes of the [software] specialization(s) (e.g. version, name, URL, install, uninstall, activate, deactivate, etc.) based on the corresponding attributes and values configured in the <softwarePolicy> resource. After the [software] specialization(s) are created, the Hosting CSE shall subscribe to the [software] specialization(s) to receive notifications for changes to the installStatus or activeStatus attributes of the [software] specialization(s).

NOTE: Before creating a [software] specialization on a targeted node, the Hosting CSE may check the version, name and URL attributes of the <softwarePolicy> resource to determine if the specified software is compatible with the targeted node.  If not compatible, the Hosting CSE may choose not to perform the software management operation on the targeted node.  The method used by the Hosting CSE to determine if the specified software is compatible with the targeted node is out of scope of the present document.

Step 6:	According to the softwareOperation specified by the <softwarePolicy> resource, the Hosting CSE shall perform the specified operation on each [software] specialization by performing an update to the [software] specialization following the create operation defined in Step 5.  The Hosting shall wait to receive notifications that the software management operations for each [software] specialization has completed, or a timeout occurs.

Step 7:	After the specified software management operations are performed, the Hosting CSE shall update the softwareLinks attribute with the resource identifiers of the [software] specializations it created and the aggregatedSoftwareStatus and individualSoftwareStatus attributes with the corresponding status (i.e. INSTALLED, ACTIVATED or ERROR) based on the status it receives from each of the [software] specialization.

Step 8:	If the <softwarePolicy> resource has a <subscription> child resource, the Hosting CSE shall generate notification(s) if the eventNotificationCriteria have been met based on the update of the <softwarePolicy> resource.

- 10.2.z.3	<softwarePolicy> Resource Procedures

- 10.2.z.3.1	Create <softwarePolicy>

The Hosting CSE shall use this procedure to create a <softwarePolicy> resource according to the resource identifier of the parent resource.

Table 10.2.z.3.1-1: <softwarePolicy> CREATE

| <softwarePolicy> CREATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: Content: The resource content shall provide the information as defined in clause 9.6.x |
| Processing at Originator before sending Request | According to clause 10.1.2 |
| Processing at Receiver | According to clause 10.1.2 The Receiver shall check the requested policy does not conflict with any existing <softwarePolicy> resources. If a conflict exists and the requested <softwarePolicy> cannot be created, the Receiver shall respond with an error. The Receiver shall check that the mandatory attributes version, name, URL, softwareTargets and softwareOperation are included in the request. If not, the Receiver shall respond with an error. The Receiver shall check the existence and accessibility of the resources referenced by softwareTriggerCriteria attribute. If any of the referenced resources do not exist or are not accessible by the Originator, the Receiver shall respond with an error. Once the <softwarePolicy> resource is successfully created, the Receiver shall use the <softwarePolicy> resource to perform the software management procedure as defined in 10.2.z.2. |
| Information in Response message | According to clause 10.1.2 |
| Processing at Originator after receiving Response | According to clause 10.1.2 |
| Exceptions | According to clause 10.1.2 |

- 10.2.z.3.2	Retrieve <softwarePolicy>

This procedure shall be used for retrieving the attributes of a <softwarePolicy> resource.

Table 10.2. z.3.2-1: <softwarePolicy> RETRIEVE

| <softwarePolicy> RETRIEVE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.3 |
| Processing at Receiver | According to clause 10.1.3 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply. |
| Processing at Originator after receiving Response | According to clause 10.1.3 |
| Exceptions | According to clause 10.1.3 |

- 10.2.z.3.3	Update <softwarePolicy>

This procedure shall be used for updating attributes of a <softwarePolicy> resource.

Table 10.2. z.3.3-1: <softwarePolicy> UPDATE

| <softwarePolicy> UPDATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-2 apply. |
| Processing at Originator before sending Request | According to clause 10.1.4 |
| Processing at Receiver | According to clause 10.1.4 If the aggregatedSoftwareStatus attribute is “PENDING” or “IN PROGRESS”, and the request is attempting to update the version, name, URL, softwareTargets, softwareOperation or softwareTriggerCriteria attributes, the Receiver shall respond with an error. Once the <softwarePolicy> resource is successfully updated, the Receiver shall use the <softwarePolicy> resource to perform the software management procedure as defined in 10.2.z.2. |
| Information in Response message | According to clause 10.1.4 |
| Processing at Originator after receiving Response | According to clause 10.1.4 |
| Exceptions | According to clause 10.1.4 |

- 10.2.z.3.4	Delete <softwarePolicy>

This procedure shall be used for deleting a <softwarePolicy> resource.

Table 10.2. z.3.4-1: <softwarePolicy> DELETE

| <softwarePolicy> DELETE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-2 apply |
| Processing at Originator before sending Request | According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 The Receiver may delete any [software] specialization(s) associated with this <softwarePolicy> resource (i.e. the [software] specialization(s) linked through the softwareLinks attribute). |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 |
| Exceptions | According to clause 10.1.5 |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3---------------------------------------------

## 9.6	Resource Types

### 9.6.1	Overview

#### 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- flexContainerInstance;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, pollingChannel, semanticDescriptor, timeSeries, transaction, transactionMgmt, triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, locationPolicy, action, multimediaSession, dynamicAuthorizationConsultation, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwarePolicy | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor, transaction | Container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, flexContainerInstance, latest, oldest, subscription, semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.35 |
| flexContainerInstance | Represents a timestamped Data instance in the <flexContainer> resource | transaction, semanticDescriptor | flexContainer | 9.6.59 |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, semanticRuleRepository, flexContainer, timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action, m2mServiceSubscriptionProfile, dynamicAuthorizationConsultation, localMulticastGroup, serviceSubscribedAppRule, authorizationPolicy, authorizationDecision, authorizationInformation, ontologyRepository, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwarePolicy | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription, transaction | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription, transaction | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription, transaction | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance> , <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource | None specified | container, timeSeries, or flexContainer | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription, transaction | CSEBase, AE | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription, transaction | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, transaction, semanticDescriptor | node, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription, transaction, serviceSubscribedUserProfile | CSEBase | 9.6.19 |
| node | Represents specific Node information | mgmtObj, subscription, semanticDescriptor, schedule, transaction, action | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription, transaction | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules, transaction | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource | None specified | container, timeSeries, or flexContainer | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription, transaction | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, contentInstanceAnnc flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, timeSeries, timeSeriesAnnc, timeSeriesInstanceAnnc, mgmtObjAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc, semanticMashupInstance, semanticMashupInstanceAnnc, action, dynamicAuthorizationConsultation, dynamicAuthorizationConsultationAnnc, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwarePolicy | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription, transaction | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription, transaction | subscription, CSEBase, - node, softwarePolicy | 9.6.9 |
| serviceSubscribedNode | Node information | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription, transaction | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription, transaction | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribed-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc, nwMonitoringReq, semanticRuleRepository, reasoningRules, reasoningJobInstance, timeSyncBeacon, primitiveProfile, state, processManagement | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription, transaction | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources. | subscription, transaction | AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource |  | group | 9.6.14a |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities. | timeSeriesInstance, subscription, semanticDescriptor, latest, oldest, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, container, containerAnnc, flexContainer, flexContainerAnnc | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | transaction | timeSeries, timeSeriesAnnc | 9.6.37 |
| role | Represents a role that is assigned to an AE or CSE. | subscription, transaction | authorizationInformation | 9.6.38 |
| token | Used for storing a token that is issued to an AE or CSE. | subscription, transaction | authorizationInformation | 9.6.39 |
| authorizationDecision | Represents an access control decision point | subscription, transaction | CSEBase | 9.6.41 |
| authorizationPolicy | Represents an access control policy retrieval point | subscription, transaction | CSEBase | 9.6.42 |
| authorizationInformation | Represents an access control information point | role token subscription, transaction | CSEBase | 9.6.43 |
| localMulticastGroup | Stores local multicast group information of member hosting CSE. | transaction | CSEBase | 9.6.44 |
| AEContactList | Contains information about a CSE that has resources that referencing an AE-ID | AEContactListPerCSE, subscription, transaction | CSEBase | 9.6.45 |
| AEContactListPerCSE | Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes | None specified | AEContactList | 9.6.46 |
| transactionMgmt |  | subscription | CSEBase, AE, remoteCSE | 9.6.47 |
| transaction |  | action | All non-virtual resource types with the exception of the following: request, delivery, pollingChannel, transactionMgmt, transaction, timeSyncBeacon | 9.6.48 |
| triggerRequest | Used by an AE to initiate, replace or recall a device trigger request | subscription | AE | 9.6.49 |
| ontologyRepository | Represents the collection of the managed ontologies and the semantic validation service | ontology, semanticValidation, subscription | CSEBase | 9.6.50 |
| ontology | Store the representation of an ontology | subscription | ontologyRepository | 9.6.51 |
| semanticValidation | A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology. | None specified | ontologyRepository | 9.6.52 |
| semanticMashupJobProfile | Represents the profile and description of a semantic mashup service | semanticMashupInstance, semanticDescriptor, subscription | CSEBase, remoteCSE | 9.6.53 |
| semanitcMashupInstance | Represents a semantic mashup instance | semanticMashupResult, semanticDescriptor, mashup, subscription | semanticMashupJobProfile, AE, remoteCSE, CSEBase | 9.6.54 |
| mashup | A virtual resource use to trigger the calculation and generation of new mashup result | Not specified | semanticMashupInstance | 9.6.55 |
| semanticMashupResult | Represent semantic mashup results | semanticDescriptor, subscription | semanticMashupInstance | 9.6.56 |
| multimediaSession | Stores a representation of a multimedia session information requested by a registering AE | subscription | AE | 9.6.57 |
| crossResourceSubscription | represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | CSEBase, remoteCSE, AE | 9.6.58 |
| backgroundDataTransfer | Stores information for a background data transfer request | None specified | AE, remoteCSE, CSEBase | 9.6.60 |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | dependency | CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction, state | 9.6.61 |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | None specified | action | 9.6.62 |
| e2eQosSession | Specifies the end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities. | subscription | CSEBase, remoteCSE, AE | 9.6.63 |
| nwMonitoringReq | Specifiesed the request that an AE retrieves the Underlying Network information. The resource provides the characteristics of the Underlying Network status in a particular geographic area such as congestion status and number of devices. | subscription | CSEBase, remoteCSE, AE | 9.6.64 |
| semanticRuleRepository | Specifies one or multiple <reasoningRules> child resources to represent different sets of reasoning rules in the oneM2M system | reasoningRules, reasoningJobInstance | CSEBase | 9.6.65 |
| reasoningRules | Store a set of related reasoning rules | subscription | semanticRuleRepository | 9.6.66 |
| reasoningJobInstance | Represents a specific reasoning job instance for enabling one-time or continuous reasoning operations. | subscription | semanticRuleRepository | 9.6.67 |
| serviceSubscribedUserProfile | Stores user profile information for a given M2M Service User | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.68 |
| timeSyncBeacon | Specifies criteria that a Hosting CSE uses to generate time synchronization beacon notifications | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.69 |
| primitiveProfile | Stores message parameters and/or resource attributes that can be applied (i.e. added, removed or modified) to a request or a response primitive. | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.73 |
| softwarePolicy | Provides policies and rules for the dynamic management of a software by a Hosting CSE. | subscription, transaction, schedule | CSEBase, remoteCSE, AE | 9.6.x |
| NOTE: See clause 9.6.12 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### -----------------------End of change 3---------------------------------------------

### -----------------------Start of change 4---------------------------------------------

### 9.6.3	Resource Type CSEBase

A <CSEBase> resource shall represent a CSE. The <CSEBase> resource shall be the root for all resources that are residing in the CSE. A CSE shall be represented by only one <CSEBase> resource.

The <CSEBase> resource shall contain the child resources specified in table 9.6.3-1.

Table 9.6.3-1: Child resources of <CSEBase> resource

| Child Resources of <CSEBase> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <remoteCSE> | 0..n | See clause 9.6.4 |
| [variable] | <remoteCSEAnnc> | 0..n | Announced variant of <remoteCSE>. Resource with CSE-specific information for a CSE that announced itself to another CSE with which it does not have a registration relationship. |
| [variable] | <node> | 0..n | See clause 9.6.18 |
| [variable] | <AE> | 0..n | See clause 9.6.5 |
| [variable] | <container> | 0..n | See clause 9.6.6 |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 |
| [variable] | <group> | 0..n | See clause 9.6.13 |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <mgmtCmd> | 0..n | See clause 9.6.16 |
| [variable] | <locationPolicy> | 0..n | See clause 9.6.10 |
| [variable] | <statsConfig> | 0..n | See clause 9.6.23 |
| [variable] | <statsCollect> | 0..n | See clause 9.6.25 |
| [variable] | <request> | 0..n | See clause 9.6.12 |
| [variable] | <delivery> | 0..n | See clause 9.6.11 |
| [variable] | <schedule> | 0..1 | This resource defines the reachability schedule information of the entity. The absence of this resource implies the entity is always reachable. See clause 9.6.9 |
| [variable] | <role> | 0..n | See clause 9.6.38 |
| [variable] | <token> | 0..n | See clause 9.6.39 |
| [variable] | <m2mServiceSubscriptionProfile> | 0..n | See clause 9.6.19 |
| [variable] | <serviceSubscribedAppRule> | 0..n | See clause 9.6.29 |
| [variable] | <notificationTargetPolicy> | 0..n | See clause 9.6.32 |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 |
| [variable] | <authorizationDecision> | 0..n | See clause 9.6.41 |
| [variable] | <authorizationPolicy> | 0..n | See clause 9.6.42 |
| [variable] | <authorizationInformation> | 0..n | See clause 9.6.43 |
| [variable] | <localMulticastGroup> | 0..n | See clause 9.6.44 |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |
| [variable] | <ontologyRepository> | 0..1 | See clause 9.6.50 |
| [variable] | <ontologyMapping> | 0..n | See clause 9.6.70 |
| [variable] | <ontologyMappingAlgorithmRepository> | 0..n | See clause 9.6.71 |
| [variable] | <semanticMashupJobProfile> | 0..n | See clause 9.6.53 |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 |
| [variable] | <AEContactList> | 0..n | See clause 9.6.45 |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.63 |
| [variable] | <nwMonitoringReq> | 0..n | See clause 9.6.64 |
| [variable] | <softwarePolicy> | 0..n | See clause 9.6.XX |

### -----------------------End of change 4---------------------------------------------

### -----------------------Start of change 5---------------------------------------------

### 9.6.4	Resource Type remoteCSE

A <remoteCSE> resource shall represent a Registree CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> resource of Registrar CSE.

Similarly <remoteCSE> resource shall also represent a Registrar CSE. <remoteCSE> resource shall be located directly under the <CSEBase> resource of Registree CSE.

For example, when CSE1 (Registree CSE) registers with CSE2 (Registrar CSE), there will be two <remoteCSE> resources created: one in CSE1: <CSEBase1>/<remoteCSE2> and one in CSE2: <CSEBase2>/<remoteCSE1>.

Note that the creation of the two resources does not imply mutual registration. The <CSEBase1>/<remoteCSE2> does not mean CSE2 registered with CSE1 in the example above.

The <remoteCSE> resource shall contain the child resources specified in table 9.6.4-1.

Table 9.6.4-1: Child resources of <remoteCSE> resource

| Child Resources of <remoteCSE> | Child Resource Type | Multiplicity | Description | <remoteCSEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> |
| [variable] | <containerAnnc> | 0..n | Announced variant of <container>. See clause 9.6.6 | <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> |
| [variable] | <flexContaineAnnc> | 0..n | Announced variant of <flexContainer>. See clause 9.6.35 | <flexContainerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> |
| [variable] | <groupAnnc> | 0..n | Announced variant of <group>. See clause 9.6.13 | <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> |
| [variable] | <accessControlPolicyAnnc> | 0..n | Announced variant of <accessControlPolicy>. See clause 9.6.2 | <accessControlPolicyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <pollingChannel> | 0..1 | See clause 9.6.21. If requestReachability is FALSE, the CSE that created this <remoteCSE> resource should create a <pollingChannel> resource and perform long polling. The <pollingChannel> shall be utilized by the parent resource. | None |
| [variable] | <nodeAnnc> | 0..n | Announced variant of <node>. This announced resource is associated with a <node> resource that is hosted on a CSE which is represented by the parent <remoteCSE> or <remoteCSEAnnc> resource. See clause 9.6.18 for <node>. | <nodeAnnc> |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 |  |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries> |
| [variable] | <timeSeriesAnnc> | 0..n | Announced variant of <timeSeries>. See clause 9.6.36 | <timeSeriesAnnc> |
| [variable] | <AEAnnc> | 0..n | Announced variant of <AE>. See clause 9.6.5 | <AEAnnc> |
| [variable] | <locationPolicyAnnc> | 0..n | Announced variant of <locationPolicy>. See clause 9.6.10 | <locationPolicyAnnc> |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 | <transactionMgmt> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <ontologyRepositoryAnnc> | 0..1 | Announced variant of <ontologyRepository>. See clause 9.6.50 | <ontologyRepositoryAnnc> |
| [variable] | <ontologyMappingAnnc> | 0..n | Announced variant of <ontologyMapping>. See clause 9.6.70 | <ontologyMappingAnnc> |
| [variable] | <ontologyMappingAlgorithmRepositoryAnnc> | 0..n | Announced variant of <ontologyMappingAlgorithmRepository>. See clause 9.6.71 | <ontologyMappingAlgorithmRepositoryAnnc> |
| [variable] | <semanticMashupJobProfile> | 0..n | See clause 9.6.53 | <semanticMashupJobProfile> |
| [variable] | <semanticMashupJobProfileAnnc> | 0..n | Announced variant of <semanticMashupJobProfile>. See clause 9.6.53 | <semanticMashupJobProfileAnnc> |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 | <semanticMashupInstance> |
| [variable] | <semanticMashupInstanceAnnc> | 0..n | Announced variant of <semanticMashupInstance >. See clause 9.6.54. | <semanticMashupInstanceAnnc> |
| [variable] | <action> | 0..n | See clause 9.6.61 | <actionAnnc> |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.63 | <e2eQosSession Annc> |
| [variable] | <nwMonitoringReq> | 0..n | See clause 9.6.64 | <nwMonitoringReq> |
| [variable] | <softwarePolicy> | 0..n | See clause 9.6.XX | <softwarePolicyAnnc> |

### -----------------------End of change 5---------------------------------------------

### ----------------------- Start of change 6---------------------------------------------

### 9.6.5	Resource Type AE

An <AE> resource shall represent information about an Application Entity registered to a CSE.

The <AE> resource shall contain the child resources specified in table 9.6.5-1.

Table 9.6.5-1: Child resources of <AE> resource

| Child Resources of <AE> | Child Resource Type | Multiplicity | Description | <AEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> <flexContainerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> <accessControlPolicyAnnc> |
| [variable] | <pollingChannel> | 0..1 | See clause 9.6.21 When the AE is request-unreachable, the AE should create this <pollingChannel> resource and perform long polling. The <pollingChannel> shall be utilized by the parent resource | None |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 | None |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries> <timeSeriesAnnc> |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 | <transactionMgmt> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <triggerRequest> | 0..n | See clause 9.6.49 | None |
| [variable] | <multimediaSession> | 0..n | See Clause 9.6.57. This resources holds information describing the established multimedia session | None |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 | <semanticMashupInstance> <semanticMashupInstanceAnnc> |
| [variable] | <locationPolicy> | 0..n | See clause 9.6.10 | <locationPolicyAnnc> |
| [variable] | <action> | 0..n | See clause 9.6.61 | None |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.63 | <e2eQosSession Annc> |
| [variable] | <nwMonitoringReq> | 0..n | See clause 9.6.64 | <nwMonitoringReq > |
| [variable] | <softwarePolicy> | 0..n | See clause 9.6.XX | <softwarePolicyAnnc> |

### -----------------------End of change 3---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?