| Input contribution Use case |  |
| --- | --- |
| Use Case Title:* | Conflicting Actions Use Case |
| Group Name:* | SDS |
| Source:* | BOE |
| Contact: | Albert Zhao, BOE, zhaojunjie111@boe.com.cn |
| Date:* | 2020-4-20 |
| Abstract:* | Introduce use case for conflicting actions |
| Agenda Item:* |  |
| Work item(s): | WI-0093 |
| Document(s) Impacted* | TR-0060 Study of action triggering enhancements |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* |  |
| 'Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

# Title

Conflicting Actions Use Case

## Description

An IoT system can be used to monitor various sensors and control the various actuators. Take the smart home for example, a smart home IoT system includes sensors utilized to measure parameters (e.g., temperature, humidity, CO, PM2.5) and actuators utilized to change the state of things (e.g., window, switch, light, door).

In order to improve the user experience, many IoT systems have automated the sensing and actuating process using the action mechanism (including the trigger condition, trigger object, trigger operation).A trigger operation against to a trigger object can be triggered when the trigger condition is met. For example, when the CO values is over 100 ppm which will be harmful to human, so the window will be opened automatically.

In some cases, multiple actions target to the same trigger object while having different trigger operation, or even conflict condition. For example, an action states that the window will be opened when the CO value is over 100 ppm, while another action states that the window will be closed when the PM2.5 is over 200 μg/m³. Both of these two actions may be coexist for a user, but it is very dangerous to execute the second action without considering the first action.

## Source

N/A

## Actors

- IoT Requestor: sends out a request to the action manager to enable the management of actions. The management of action includes create action, update action, etc.

- IoT devices: including sensors that are used to monitor events and conditions in the production process, and actuators that are used to take actions during the process.

- Action Manager: performs action management. The manager’s functionalities include taking inputs from IoT entities, managing the resources associated with the process, carrying out actions, sending commands to devices, monitoring status and conditions.

## Pre-conditions

IoT system is deployed with action management including action create, delete, update etc. Action will be performed automatically based on observation of certain events or conditions.

## Triggers

A request for create action management process, or an update of an existing action management process.

## Normal Flow

Figure 1 illustrates the high-level flow of create action, which consists of the following steps:

•	Step 1: The IoT Requestor sends a request to the Action Manager to create an action by specifying required parameters (such as action subject, action object, action trigger condition, operation etc.).

•	Step 2: The Action Manager processes the request and performs necessary validations.

•	Step 3: Based on the request, the Action Manger verifies that if the exiting action has the same action object and has conflicting operation.

•	Step 4: If there is a exiting action with the same action object and has conflicting operation, the Action Manager will invert the action trigger condition of exiting actions and add it to the dependency of newly requested action.

•	Step 5: the Action Manager create the requested action.

•	Step 6: A response is generated and sent to the IoT Requestor.

Figure 1. Normal Flow – Action management

## Alternative flow

N/A

## Post-conditions

N/A

## High Level Illustration

N/A

## Potential requirements

- The oneM2M System shall support the capability for detecting conflicting actions which have the same action object and conflicting operation.

- The oneM2M System shall support the capability for dealing with the conflicting actions by establishing the dependency between the conflicting actions.