| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#45 |
| Source:* | Kenichi Yamamoto, KDDI, kc-yamamoto@kddi.com |
| Date:* | 2020-04-27 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI-0058 - 3GPP & Cellular IoT Interworking MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0026 3GPP Interworking V4.4.0 |
| Clauses * | 7.4.8 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to inline Monitoring Event API (Number of UE in an Area) in TS-0026 with 3GPP TS29.122-f60.

- If a SCEF sends a successful response for the Number of UE in an Area to IN-CSE, the response contains “200 OK” status code to acknowledge the IN-CSE with one aggregated report in the requested area. However, current TS-0026 shows the “201 CREATED” status code as the response.

- Only one-time reporting is supported for the monotoring event Number of UE in an Area with the value of Maximum Number of Reports indicated by "maximumNumberOfReports" sets to 1. However, current TS-0026 does not support the attribute.

TS29.122-f60 defines procedures and attributes for the Number of UE in an Area as follow:

##### 4.4.2.2.3	Monitoring Events Configuration directly via MME/SGSN

The monitoring event Number of UEs in a geographic area is applicable for the monitoring event configuration via MME/SGSN. Only one-time reporting is supported for this event with the value of Maximum Number of Reports indicated by "maximumNumberOfReports" sets to 1.

Upon receipt of the HTTP POST request from the SCS/AS, the SCEF shall

-	resolve the location area to the involved SGSN(s)/MME(s) by local configuration;

-	interact with the HSS via the S6t interface as specified in 3GPP TS 29.336 [11] if the External Group ID(s) is included; and

-	interact with the SGSN(s)/MME(s) via the T6a/b inteface as specified in 3GPP TS 29.128 [12].

After collecting responses from the SGSN(s)/MME(s), if the SCEF does not receive any successful response from the involved SGSN(s)/MME(s), the SCEF shall respond to the SCS/AS with a status code set to 500 Internal Server Error; otherwise the SCEF should send 200 OK status code to acknowledge the SCS/AS with one aggregated report in the requested area by including the total count of number of UEs in the "ueCount" attribute and the External Identifier(s) (if available) or the MSISDN(s) (if available) associated with External Group ID.

NOTE:	It is possible that the number of UEs does not reflect the actual number of UEs in the designated area (e.g. some SGSN(s)/MME(s) do not respond successfully). The SCEF still provides the result to the SCS/AS if at least one SGSN/MME returns successful response.

Table 5.3.3.2.3.4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | MonitoringEventSubscription | 1 | Parameters to register a subscription to notifications about monitoring event with the SCEF. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | MonitoringEventSubscription | 1 | 201 Created | The subscription resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
|  | MonitoringEventReport | 1 | 200 OK | The operation is successful, and corresponding monitoring event report is included. This is only applicable for the one-time monitoring request if report is available in the response. |
|  | ProblemDetails | 1 | 400 Bad Request | The subscription resource is not allowed to be created since the supported feature corresponding to the monitoring event is not supported by the client. (NOTE 4) |
|  | ProblemDetails | 1 | 403 Forbidden | The subscription resource is not allowed to be created since one or more of the received parameters are out of the range defined by operator policies. (NOTE 2) |
|  | ProblemDetails | 1 | 403 Forbidden | The subscription resource is not allowed to be created since the Idle Status Indication is received in the request but not supported by the network. (NOTE 3) |

Table 5.3.2.1.2-1: Definition of type MonitoringEventSubscription

| Attribute name | Data type | Cardinality | Description | Applicability (NOTE 3) |
| --- | --- | --- | --- | --- |
| maximumNumberOfReports | integer | 0..1 | Identifies the maximum number of event reports to be generated by the HSS, MME/SGSN as specified in subclause 5.6.0 of 3GPP TS 23.682 [2]. (NOTE 2) |  |
| NOTE 2: Inclusion of either "maximumNumberOfReports" (with a value higher than 1) or "monitorExpireTime" makes the Monitoring Request a Continuous Monitoring Request, where the SCEF sends Notifications until either the maximum number of reports or the monitoring duration indicated by the property "monitorExpireTime" is exceeded. The "maximumNumberOfReports" with a value 1 makes the Monitoring Request a One-time Monitoring Request. At least one of "maximumNumberOfReports" or "monitorExpireTime" shall be provided. |  |  |  |  |

### -----------------------Start of change 1-------------------------------------------

### 7.4.8	Number of UEs in an Area

The 3GPP SCEF functionality described in 3GPP TS 29.122 [4] supports an API to allow an IN-CSE to ask for the number of UEs that are in the geographic area described by the IN-CSE. Based on the reports, MNOs may be able to predict the general congestion status of each area based on the number of UEs.

A particular group of ASN/MN-CSEs and/or ADN-AEs hosted on UEs in an area may be identified in the Underlying Network by an External Group Identifier available at the IN-CSE. The IN-CSE sends a Monitoring Event Request with the External Group Identifier and a geographic area to the corresponding SCEF. When the IN-CSE receives a Monitoring Event Response from the SCEF, the IN-CSE receives the number of group member UEs found at the area. Note that and the External Identifier(s) of the registree ASN/MN-CSEs/ADN-AEs may or may not be provided in the response, depending on MNO configuration of MME/SGSN. Based on this information, the IN-CSE can take necessary measures such as adjusting other monitoring procedures for the group. The IN-CSE may also take other actions, such as modifying the <schedule> resource of the group members. Upon detecting an updated scheduleElement, the group members will modify when they send requests and make themselves available to receive requests.

Figure 7.4.8-1: Monitoring event for Number of UEs in an Area

Pre-conditions:

There is a relationship in place between the Service Provider and MNO allowing the IN-CSE to request Monitoring events for number of UEs present in an area. The method for establishing this relationship is outside the scope of the present document.

An ASN/MN-CSE or ADN-AE registers with the IN-CSE and configures the M2M-Ext-ID attribute of its <remoteCSE> or <AE> resource. The IN-CSE examines the M2M-Ext-ID and recognizes that it is associated with an MNO that it has a relationship with.

If the deployment uses External Group Identifier (externalGroupId) as described in 3GPP TS29.122 [4], when ASN/MN-CSEs or ADN-AEs register with the IN-CSE, then they use externalGroupId information to configure the externalGroupID of the corresponding <remoteCSE> or <AE> resources (see clause 6.3 when externalGroupID is configured).

The IN-CSE is configured to be able to determine a location area (i.e. locationArea and/or locationArea5G) of interest in the Underlying Network. The IN-CSE may use its location services or other location information. How the location area of interest is determined is outside the scope of the present document.

The IN-CSE is configured with system defaults for:

•	The specified actions to generate the network congestion levels based on the number of UEs in an area.

•	The specified actions to take based on the severity of each congestion level.

The configuration methods for these system defaults are outside the scope of the present document.

The ADN-AE’s or the ASN/MN-CSE’s <node> resource hosted on the IN-CSE has a child <schedule> resource and the IN-CSE has permissions to update it. The ADN-AE or the ASN/MN-CSE has a <subscription> for its <schedule> resource and when it receives a notification from the IN-CSE, it updates its communication schedule accordingly.

Step 1: IN-CSE determines to send a Monitoring Event Request for number of UEs in a geographic area.

The IN-CSE determines to send to a SCEF a Monitoring Event Request for number of UEs present in an area of interest. The IN-CSE may determine the externalGroupID of a group of interest in the request, in which case the Monitoring Event Request is for the number of group-member UEs present in the area of interest.

The IN-CSE gets the externalGroupID information according to the attribute externalGroupID of the resource <remoteCSE> and <AE> of the UEs which location are in the area of interest. If there are multiple externalGroupIDs, the IN-CSE determines by local policy to send this request by each externalGroupID or send this request without externalGroupID.

Step 2: IN-CSE sends a Monitoring Event Request for the number of UEs in the area.

The IN-CSE sends a Monitoring Event Request for the geographical area of interest to the SCEF. The Monitoring Event Subscription request from the IN-CSE to the SCEF shall comply with 3GPP TS 29.122 [4] as follows:

- An HTTP POST method shall be used

- URI shall be set to {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies.

- The request payload shall include a MonitoringEventSubscription data structure as specified in 3GPP TS 29.122 [4] with the following attributes:

  - monitoringType shall be set to NUMBER_OF_UES_IN_AN_AREA indicating the number of UEs in a given geographic area

  - monitorExpireTime is optional and may be set to an absolute time at which the monitoring event request is considered to expire. If used, the IN-CSE shall configure this time based on Service Provider and MNO policies.

  - locationType shall be set to LAST_KNOWN_LOCATION.

  - supportedFeatures shall be set to a string value of “8” and/or “12” indicating support for Location Reporting notifications. If it is set to the value of “8” (Number_of_UEs_in_an_area_notification), the feature supports the pre-5G (e.g. 4G) requirement. If it is set to the value of “12” (Number_of_UEs_in_an_area_notification_5G), the feature supports the 5G requirement (only be supported in 5G).

  - locationArea and/or locationArea5G shall be included to indicate the area of interest within which the IN-CSE requests the number of UEs. If supportedFeatures is set to the value of “8”, the locationArea attribute is applicable. If supportedFeatures is set to the value of “12”, the locationArea5G attribute is applicable.

  - externalGroupId shall be set to the externalGroupID if in step 1 the IN-CSE monitoring request targets identifying the number of UEs from a specific group in the area and the IN-CSE determined an externalGroupID to be monitored.

  - maximumNumberOfReports shall be set to a string value of “1” indicating one-time reporting.

  - requestTestNotification, websockNotifConfig, addExtGroupIds and groupReportGuardTime are not supported by the present document and shall not be included.

Step 3: SCEF processes the Monitoring Event Request.

The SCEF processes the Monitoring Request together with the 3GPP network entities as described in 3GPP TS 29.122 [4].

Step 4: SCEF sends a Monitoring Event Response.

The SCEF sends a Monitoring Event Response to the IN-CSE to acknowledge the request has been accepted. This response is described in 3GPP TS 29.122 [4] and includes the following information.

- A response code of 200 OK.

- The URI of the Monitoring Event Subscription resource created by the SCEF. The URI is returned in the HTTP Location header with a format of {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}. The {apiRoot} and {scsAsId} segments are configured based on Service Provider and MNO policies. The {subscriptionId} segment is configured by the SCEF.

- The response payload shall include a MonitoringEventReport data structure as specified in 3GPP TS 29.122 [4] that includes the attributes present in the request along with the following additional attributes:

  - ueCount is configured to indicate the number of UEs found at the location. If an externalGroupId has been provided in the request, the count indicates the number of UEs from the given group which are found at the location.

  - externalIds is configured to indicate External Identifier(s) of the UEs included in the number of UEs found denoted by ueCount.

Note that and the External Identifier(s) information may or may not be provided in the response, depending on MNO configuration of MME/SGSN.

  - self is configured with a URI to the resource created by the SCEF for the request.

See clause 8.3 for a list of possible error scenarios and error handling options for the IN-CSE.

Step 5: Monitoring Event handling at the IN-CSE.

The IN-CSE may use the information provided in the Monitoring Event Report to modify the <schedule> resource of the group members such that they modify the times they send or receive requests.

How the IN-CSE determines the use of the information received from the monitoring event  is outside the scope of the current document and may be based on agreements with the MNO.

### -----------------------End of Change 1 ---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?