| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#46 |
| Source:* | Rahul, C-DOT, krahul@cdot.in Anupama, C-DOT, anupama@cdot.in Poornima, C-DOT, poornima@cdot.in |
| Date:* | 2020-07-13 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | V3.17.0 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V3.17.0 |
| Clauses * | 7.4.29 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to change the create request optionality of resource specific attribute ‘applicableCredIDs’ of  <serviceSubscribedAppRule> resource from optional to mandatory since multiplicity of this attribute is one as mentioned in TS-0001 v4_4_0 section 9.6.29.

This CR was discussed in the meeting and it was decided to change the data type of  of resource specific attribute ‘applicableCredIDs’ of  <serviceSubscribedAppRule> from ‘m2m:listOfM2MID’ to ‘m2m:URIList’

### -----------------------Start of change 1-------------------------------------------

### 7.4.29	Resource Type <serviceSubscribedAppRule>

#### 7.4.29.1	Introduction

The <serviceSubscribedAppRule> resource represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE. The detailed description can be found in the clause 9.6.29 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.29.1-1: Data type definition of <serviceSubscribedAppRule> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| serviceSubscribedAppRule | CDT-serviceSubscribedAppRule-v4_1_0.xsd |  |

Table 7.4.29.1-2: Universal/Common Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.29.1-3: Resource Specific Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| applicableCredIDs | O | O | m2m:URIList |  |
| allowedApp-IDs | O | O | m2m:listOfM2MID |  |
| allowedAEs | O | O | m2m:listOfM2MID |  |
| allowedRole-IDs | O | O | List of m2m:roleID |  |

Table 7.4.29.1-4: Child resources of <serviceSubscribedAppRule> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

#### 7.4.29.2	<serviceSubscribedAppRule> resource specific procedures for CRUD operations

##### 7.4.29.2.0	Introduction

This clause describes <serviceSubscribedAppRule> resource specific primitive behaviour for CRUD operations.

##### 7.4.29.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.29.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.29.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.29.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?