| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #46.2 |
| Source:* | aeSeung Song, KETI, jssong@sejong.ac.kr Minbyeong Lee, Hyundai Motors, minbyeong.lee@hyundai.com |
| Date:* | 2020-08-27 |
| Reason for Change/s:* | Add a new attribute for announcement synchronization type |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0046 – Vehicular Domain Enablement MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.0.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds an additional attribute to support bi-directional synchronization of resource announcement introduced in TS-0001 for Release 4.

This includes the following:

- a new attribute “announceSyncType”

### -----------------------Start of change 1-------------------------------------------

##### 6.3.4.2.75	m2m:geoSpatialFunctionType

Table 6.3.4.2.75-1: Interpretation of geoSpatialFunctionType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Within |  |
| 2 | Contains |  |
| 3 | Intersects |  |

##### 6.3.4.2.76	m2m:somecastAlgorithm

Used for the somecastAlgorithm attribute of the <group> resource.

Table 6.3.4.2.76-2: Interpretation of somecastAlgorithm

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | ROUND_ROBIN |  |
| 2 | WEIGHTED_ROUND_ROBIN |  |
| 3 | LEAST_WORK_LOAD |  |
| 4 | RANDOM |  |
| NOTE: See clause 7.4.13 Resource Type <group> and clause Error: Reference source not found. |  |  |

##### 6.3.4.2.xx	m2m:announceSyncType

Table 6.3.4.2.xx-3: Interpretation of announceSyncType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | UNI_DIRECTIONAL |  |
| 2 | BI_DIRECTIONAL |  |

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### 6.3.5.34	m2m:mgmtResource

Used This data type is used as base type of all <mgmtObj> specializations specified in Annex D. It includes the attributes commonly used by all <mgmtObj> resource type specializations.

It consists of the common attributes included in m2m:announceableResource as defined Table 6.3.6-7 and the additional common attributes shared by all <mgmtObj> specializations shown in Table 6.3.5.34-4.

Table 6.3.5.34-4: Type Definition of m2m:mgmtResource

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| @resourceName | m2m:resourceName | 1 | common attributes as defined in m2m:announceableResource, see Table 6.3.6-7 |
| resourceType | m2m:resourceType | 1 |  |
| resourceID | m2m:ID | 1 |  |
| parentID | m2m:nhURI | 1 |  |
| creationTime | m2m:timestamp | 1 |  |
| lastModifiedTime | m2m:timestamp | 1 |  |
| labels | m2m:labels | 0..1 |  |
| accessControlPolicyIDs | m2m:acpType | 0..1 |  |
| expirationTime | m2m:timestamp | 1 |  |
| dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
| announceTo | list of xs:anyURI | 0..1 |  |
| announcedAttribute | list of xs:NCName | 0..1 |  |
| announceSyncType | m2m:announceSyncType | 0..1 |  |
| mgmtDefinition | m2m:mgmtDefinition | 1 |  |
| objectIDs | m2m:listOfURIs | 0..1 |  |
| objectPaths | m2m:listOfURIs | 0..1 |  |
| description | xs:string | 0..1 |  |
| mgmtSchema | xs:anyURI | 0..1 |  |

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

#### 6.3.5.38	m2m:flexContainerResource

This data type is used as base type of all <flexContainer> specializations listed in clause 9.6.1.2.2 of oneM2M TS-0001 [Error: Reference source not found]. It includes the attributes commonly used by all <flexContainer> resource type specializations.

It consists of the common attributes shown in Table 7.4.37.1-29 and the resource specific attributes shared by all <flexContainer> specializations shown in Error: Reference source not found.

Table 6.3.5.38-5: Type Definition of m2m:flexContainerResource

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| @resourceName | m2m:resourceName | 1 |  |
| resourceType | m2m:resourceType | 1 |  |
| resourceID | m2m:ID | 1 |  |
| parentID | m2m:nhURI | 1 |  |
| creationTime | m2m:timestamp | 0..1 |  |
| lastModifiedTime | m2m:timestamp | 0..1 |  |
| labels | m2m:labels | 0..1 |  |
| accessControlPolicyIDs | m2m:acpType | 0..1 |  |
| expirationTime | m2m:timestamp | 0..1 |  |
| dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
| announceTo | list of xs:anyURI | 0..1 |  |
| announcedAttribute | list of xs:NCName | 0..1 |  |
| announceSyncType | m2m:announceSyncType | 0..1 |  |
| stateTag | xs:nonNegativeInteger | 1 |  |
| creator | m2m:ID | 0..1 |  |
| containerDefinition | xs:anyURI | 1 |  |
| ontologyRef | xs:anyURI | 0..1 |  |
| contentSize | xs:nonNegativeInteger | 1 |  |
| nodeLink | xs:anyURI | 0..1 |  |
| maxNrOfInstances | xs:nonNegativeInteger | 0..1 |  |
| maxInstanceAge | xs:nonNegativeInteger | 0..1 |  |
| maxByteSize | xs:nonNegativeInteger | 0..1 |  |
| currentNrOfInstances | xs:nonNegativeInteger | 0..1 |  |
| currentByteSize | xs:nonNegativeInteger | 0..1 |  |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

### 6.3.6	Universal and Common attributes

oneM2M TS-0001 [Error: Reference source not found] defines a number of Universal Attributes (which appear in all resources) and Common Attributes (which appear in more than one resource and have the same meaning whenever they do appear). Their types and values are described in Table 6.3.6-6.

If a Resource is represented as an XML document then the resource attributes (if present) appear in the order listed in this table. They appear before any resource-specific attributes.

Table 6.3.6-6: Universal and Common Attributes

| Attribute Name | Data Type | Value restrictions and Notes |
| --- | --- | --- |
| resourceType | m2m:resourceType | This attribute is only determined at creation time by the Hosting CSE |
| resourceID | m2m:ID | This attribute is determined at creation time by the Hosting CSE and used for the non-hierarchical addressing method |
| parentID | m2m:nhURI | This attribute is determined by the Hosting CSE and specified in all resource types. For <CSEBase> however, the value of this attribute shall be an empty string |
| creationTime | m2m:timestamp | This attribute is determined by the Hosting CSE when the resource is created locally |
| lastModifiedTime | m2m:timestamp | This attribute is determined by the Hosting CSE when the addressed resource is modified by means of the UPDATE operation |
| labels | m2m:labels | Absence of this attribute means there are no labels |
| accessControlPolicyIDs | m2m:acpType | accessControlPolicyIDs |
| expirationTime | m2m:timestamp | expirationTime |
| link | xs:anyURI | Absence of this attribute means that this is not an announced resource |
| announceTo | list of xs:anyURI | Absence of this attribute means that this is not an announced resource |
| announcedAttribute | list of xs:NCName | Absence of this attribute means that this is not an announced resource |
| announceSyncType | m2m:announceSyncType | Absence of this attribute means that the uni-directional synchronization mode is supported |
| stateTag | xs:nonNegativeInteger | This attribute is determined by the Hosting CSE. When a resource is created this counter is set to 0 and it will be incremented on every modification of the resource |
| resourceName | m2m:resourceName |  |
| dynamicAuthorizationConsultationIDs | list of xs:anyURI | dynamicAuthorizationConsultationIDs |
| creator | m2m:ID | The AE-ID or CSE-ID of the entity which created the resource containing this attribute |
| location | m2m:geoCoordinates |  |

Table 6.3.6-7 describes some complex types that group together the universal and common attributes, to be used by Resource Type definitions. Note that location, stateTag and creator only appear in a limited number of resource types, and therefore are not included in these definitions, instead they are declared in the corresponding XSD files of the resources that support them.

Table 6.3.6-7: Complex Data Types declaring groups of resource common attributes

| XSD type name | Child Elements | Child Element Datatype | Multiplicity | Description |
| --- | --- | --- | --- | --- |
| m2m:resource | @resourceName | m2m:resourceName | 1 |  |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
| m2m:regularResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes included in the non-announceable resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
| m2m:announceableResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes included in the majority of announceable resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | xs:anyURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
|  | announceTo | list of xs:anyURI | 0..1 |  |
|  | announcedAttribute | list of xs:NCName | 0..1 |  |
|  | announceSyncType | m2m:announceSyncType | 0..1 |  |
| m2m:announcedResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes in the announced variant of the preceding resources. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | link | xs:anyURI | 1 |  |
|  | dynamicAuthorizationConsultationIDs | list of xs:anyURI | 0..1 |  |
| m2m:subordinateResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes included in the non-announceable resource types without accessControlPolicyIDs. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
| m2m:announceableSubordinateResource | @resourceName | m2m:resourceName | 1 | Declares the universal / common attributes used by resource types that are subordinate children of other resources. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | xs:anyURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | announceTo | list of xs:anyURI | 0..1 |  |
|  | announcedAttribute | list of xs: NCName | 0..1 |  |
|  | announceSyncType | m2m:announceSyncType | 0..1 |  |
| m2m:announcedSubordinateResource | @resourceName | m2m:resourceName | 1 | Declares the common / universal attributes used in the announced variants of the subordinate resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | link | xs:anyURI | 1 |  |

NOTE:	In Table 6.3.6-7, names of XML schema attributes are prefixed with a "@" character to differentiate these from Resource attribute names. The "@" character is not part of the actual attribute name.

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

### 7.4.2	Resource type <accessControlPolicy>

#### 7.4.2.1	Introduction

The <accessControlPolicy> resource is comprised of privileges and selfPrivileges attributes which represent a set of access control rules defining which entities (defined as accessControlOriginators) have the privilege to perform certain operations (defined as accessControlOperations) within specified contexts (defined as accessControlContexts) and are used by the CSEs in making access decision to specific resources.

The detailed description can be found in clause 9.6.2 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.2.1-8: Data type definition of <accessControlPolicy> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| accessControlPolicy | CDT-accessControlPolicy-v4_0_0.xsd |  |

Table 7.4.2.1-9: Universal/Common Attributes of <accessControlPolicy> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| labels | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 7.4.4	Resource Type <remoteCSE>

#### 7.4.4.1	Introduction

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> of the Registrar CSE.

In addition each registered CSE shall have a <remoteCSE> resource representing its Registrar CSE. This is located directly under the registered CSE's <CSEBase>.

The detailed description can be found in clause 9.6.4 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.4.1-10: Data type definition of <remoteCSE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| remoteCSE | CDT-remoteCSE-v4_0_0.xsd |  |

Table 7.4.4.1-11: Universal/Common Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

### -----------------------End of change 6-------------------------------------------

### -----------------------Start of change 7-------------------------------------------

### 7.4.5	Resource Type <AE>

#### 7.4.5.1	Introduction

The <AE> resource represents information about an Application Entity known to a given Common Services Entity.

The detailed description can be found in clause 9.6.5 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.5.1-12: Data type definition of <AE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| AE | CDT-AE-v4_0_0.xsd | XSD schema for AE resource |

Table 7.4.5.1-13: Universal/Common Attributes of <AE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

### -----------------------End of change 7-------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 7.4.6	Resource Type <container>

#### 7.4.6.1	Introduction

This resource represents a container for data instances. It is used to share information among other entities and potentially to track the data. A <container> resource has no associated content, only attributes and child resources.

The detailed description can be found in clause 9.6.6 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.6.1-14: Data type definition of <container> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| container | CDT-container-v4_0_0.xsd |  |

Table 7.4.6.1-15: Universal/Common Attributes of <container> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| stateTag | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |
| location | O | O |

### -----------------------End of change 8-------------------------------------------

### -----------------------Start of change 9-------------------------------------------

### 7.4.7	Resource Type <contentInstance>

#### 7.4.7.1	Introduction

The <contentInstance> resource represents a data instance in the container.

The detailed description can be found in clause 9.6.7 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.7.1-16: Data type definition of <contentInstance> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| contentInstance | CDT-contentInstance-v4_0_0.xsd |  |

Table 7.4.7.1-17: Universal/Common Attributes of <contentInstance> resource

| Attribute Name | Request Optionality |
| --- | --- |
|  | Create |
| @resourceName | O |
| resourceType | NP |
| resourceID | NP |
| parentID | NP |
| expirationTime | O |
| creationTime | NP |
| lastModifiedTime | NP |
| stateTag | NP |
| labels | O |
| announceTo | O |
| announcedAttribute | O |
| announceSyncType | O |
| creator | O |

### -----------------------End of change 9-------------------------------------------

### -----------------------Start of change 10-------------------------------------------

### 7.4.9	Resource Type <schedule>

#### 7.4.9.1	Introduction

The <schedule> resource shall represent scheduling information in the context of its parent resource. If a <schedule> resource is not present as a child resource then there are no time-constraints on the context of its parent resource.

The detailed <schedule> resource description can be found in clause 9.6.9 of the TS-0001 [Error: Reference source not found].

Table 7.4.9.1-18: Data type definition of <schedule> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| schedule | CDT-schedule-v4_0_0.xsd |  |

Table 7.4.9.1-19: Universal/Common Attributes of <schedule> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| accessControlPolicyIDs | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 10-------------------------------------------

### -----------------------Start of change 11-------------------------------------------

### 7.4.10	Resource Type <locationPolicy>

#### 7.4.10.1	Introduction

The <locationPolicy> resource represents the method for obtaining and managing geographical location information of an M2M Node. The detailed description can be found in the clause 9.6.10 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.10.1-20: Data type definition of <locationPolicy> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| locationPolicy | CDT-locationPolicy-v4_0_0.xsd |  |

Table 7.4.10.1-21: Universal/Common Attributes of <locationPolicy> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 11-------------------------------------------

### -----------------------Start of change 12-------------------------------------------

### 7.4.13	Resource Type <group>

#### 7.4.13.1	Introduction

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the memberIDs attribute. The <group> resource contains an attribute that represents the members of the group and a virtual resource (the <fanOutPoint>) that allows operations to be applied to the resources represented by those members. The detailed description can be found in clause 9.6.13 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.13.1-22: Data type definition of <group> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| group | CDT-group-v4_0_0.xsd |  |

Table 7.4.13.1-23: Universal/Common Attributes of <group> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 12-------------------------------------------

### -----------------------Start of change 13-------------------------------------------

### 7.4.15	Resource Type <mgmtObj>

#### 7.4.15.1	Introduction

The <mgmtObj> resource contains management data which represents individual M2M management functions. It represents a general structure to map to technology specific data models. There are multiple specializations of <mgmtObj>; these are defined in the Error: Reference source not found. Each of these specializations has its own schema file. There is no separate schema file just for <mgmtObj>, however the XML schema types for the specializations all conform to the pattern described in this clause.

Table 7.4.15.1-24: Universal/Common Attributes of <mgmtObj> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 13-------------------------------------------

### -----------------------Start of change 14-------------------------------------------

### 7.4.18	Resource Type <node>

#### 7.4.18.1	Introduction

The <node> resource represents specific information that provides properties of an oneM2M Node that can be utilized by other oneM2M operations. The <node> resource has <mgmtObj> as its child resources.

Table 7.4.18.1-25: Data type definition of <node> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| node | CDT-node-v4_0_0.xsd |  |

Table 7.4.18.1-26: Universal/Common Attributes of <node> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 14-------------------------------------------

### -----------------------Start of change 15-------------------------------------------

##### 7.4.26.2.4	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found, except that Recv-6.3 is replaced as follows:

Recv-6.3 "Check if the value of the From parameter in Request message is identical to the CSE-ID included in the link attribute in the announced resource".

In addition, announced resource synchronization specific operation on Recv-6.6 “Announce/De-announce the resource”:

If the Receiver is the original resource Hosting CSE:

- The Receiver CSE checks if the Request message is not coming from any of the CSEs hosting the announced resource the announceSyncType attribute of the announced resource has a synchronization value, “UNI_DIRECTIONAL” or “BI_DIRECTIONAL”.

  - The Receiver CSE shall UPDATE all the announced resource(s) listed in the announceTo attribute

- The Receiver CSE checks if the Request message is coming from one of the CSEs hosting the announced resource and the announceSyncType attribute of the announced resource has a synchronization value, “BI_DIRECTIONAL”.

  - Th

  - e Receiver CSE shall UPDATE all other announced resource(s) listed in the announceTo attribute excluding the Originator of the update request.

- If the Receiver is one of CSEs hosting the announced resource:

- The Receiver CSE checks if the Request message is not coming from the original resource Hosting CSE and the announceSyncType attribute of the announced resource has a synchronization value, “BI_DIRECTIONAL”.

- After updating the announced resource, the Receiver CSE shall UPDATE the original resource to synchronize the values of the announced resource attributes to the original resource.

-----------------------End of change 15-------------------------------------------

### -----------------------Start of change 16-------------------------------------------

### 7.4.34	Resource Type <semanticDescriptor>

#### 7.4.34.1	Introduction

The <semanticDescriptor> resource is used to store a semantic description pertaining to a resource and potentially sub-resources. Such a description shall be according to subject-predicate-object triples as defined in the RDF graph-based data model [Error: Reference source not found].

The detailed description can be found in clause 9.6.34 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.34.1-27: Data type definition of <semanticDescriptor> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| semanticDescriptor | CDT-semanticDescriptor-v4_0_0.xsd |  |

Table 7.4.34.1-28: Universal/Common Attributes of <semanticDescriptor> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 16-------------------------------------------

### -----------------------Start of change 17-------------------------------------------

### 7.4.37	Resource Type <flexContainer>

#### 7.4.37.1	Introduction

This resource represents a customizable container for data instances. It is a template for the definition of flexible specializations of data containers.

The detailed description can be found in clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found].

There are multiple specializations of <flexContainer> specified by oneM2M. Each of these specializations has its own schema file. There is no separate schema file just for <flexContainer>, however the XML schema types for the specializations all conform to the pattern described in this clause. The XSD of <flexContainer> specializations may use a targetNamespace other than the one identified by the m2m: prefix. Specializations of <flexContainer> which employ the namespace prefix m2m: are defined in Error: Reference source not found. Specialization of <flexContainer> which employ the namespace prefix hd: for Home Domain use cases are specified in oneM2M TS-0023 [Error: Reference source not found].

The following resource types are allowed to include <flexContainer> specializations as children: <CSEBase>, <AE>, <remoteCSE> and <container>.

Table 7.4.37.1-29: Universal/Common Attributes of <flexContainer> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| stateTag | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| creator | O | NP |
| dynamicAuthorizationConsultationIDs | O | O |
| location | O | O |

### -----------------------End of change 17-------------------------------------------

### -----------------------Start of change 18-------------------------------------------

### 7.4.38	Resource Type <timeSeries>

#### 7.4.38.1	Introduction

The resource represents a container for Time Series Data instances. It is used to share information with other entities and potentially to track, detect and report the missing data in Time Series. A <timeSeries> resource has no associated content, only attributes and child resources.

The detailed description can be found in clause 9.6.36 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.38.1-30: Data type definition of <timeSeries> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| timeSeries | CDT-timeSeries-v4_0_0.xsd |  |

Table 7.4.38.1-31: Universal/Common Attributes of <timeSeries> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |
| location | O | O |

### -----------------------End of change 18-------------------------------------------

### -----------------------Start of change 19-------------------------------------------

### 7.4.39	Resource Type <timeSeriesInstance>

#### 7.4.39.1	Introduction

The <timeSeriesInstance> resource represents a data instance in the <timeSeries> resource.

The detailed description can be found in clause 9.6.37 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.39.1-32: Data type definition of <timeSeriesInstance> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| timeSeriesInstance | CDT-timeSeriesInstance-v4_0_0.xsd |  |

Table 7.4.39.1-33: Universal/Common Attributes of <timeSeriesInstance> resource

| Attribute Name | Request Optionality |
| --- | --- |
|  | Create |
| @resourceName | O |
| resourceType | NP |
| resourceID | NP |
| parentID | NP |
| creationTime | NP |
| expirationTime | O |
| lastModifiedTime | NP |
| labels | O |
| announceTo | O |
| announcedAttribute | O |
| announceSyncType | O |

### -----------------------End of change 19-------------------------------------------

### -----------------------Start of change 20-------------------------------------------

### 7.4.46	Resource Type <ontologyRepository>

#### 7.4.46.1	Introduction

The <ontologyRepository> resource is a repository of ontologies used for reference and management of the associated <ontology> resources. It also provides semantic validation functionality as detailed in clause 6.7 in oneM2M TS-0034 Semantics Support [Error: Reference source not found].

The detailed description can be found in clause 9.6.50 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.46.1-34: Data type definition of <ontologyRepository> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| ontologyRepository | CDT-ontologyRepository-v4_0_0.xsd |  |

Table 7.4.46.1-35: Universal/Common Attributes of <ontologyRepository> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 20-------------------------------------------

### -----------------------Start of change 21-------------------------------------------

### 7.4.47	Resource Type <ontology>

#### 7.4.47.1	Introduction

The <ontology> resource is used to store the representation of a specific ontology. This representation may contain ontology descriptions in a variety of formats.

The detailed description can be found in clause 9.6.51 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.47.1-36: Data type definition of <ontology> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| ontology | CDT-ontology-v4_0_0.xsd |  |

Table 7.4.47.1-37: Universal/Common Attributes of <ontology> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 21-------------------------------------------

### -----------------------Start of change 22-------------------------------------------

### 7.4.49	Resource Type <semanticMashupJobProfile>

#### 7.4.49.1	Introduction

The <semanticMashupJobProfile> resource is used to store a Semantic Mashup Job Profile (SMJP).

The detailed description can be found in clause 6.3 in oneM2M TS-0034: Semantics Support [Error: Reference source not found] and clause 9.6.53 in oneM2M TS-0001: Functional Architecture [Error: Reference source not found].

Table 7.4.49.1-38: Data type definition of <semanticMashupJobProfile> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| semanticMashupJobProfile | CDT-semanticMashupJobProfile-v4_0_0.xsd |  |

Table 7.4.49.1-39: Universal/Common Attributes of <semanticMashupJobProfile> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| creator | O | NP |

### -----------------------End of change 22-------------------------------------------

### -----------------------Start of change 23-------------------------------------------

### 7.4.50	Resource Type <semanticMashupInstance>

#### 7.4.50.1	Introduction

The <semanticMashupInstance> resource is used to represent a Semantic Mashup Instance (SMI) that is instantiated based on a specific SMJP. The detailed description can be found in clause 6.4 in oneM2M TS-0034: Semantics Support [Error: Reference source not found] and clause 9.6.54 in oneM2M TS-0001: Functional Architecture [Error: Reference source not found].

Table 7.4.50.1-40: Data type definition of <semanticMashupInstance> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| semanticMashupInstance | CDT-semanticMashupInstance-v4_0_0.xsd |  |

Table 7.4.50.1-41: Universal/Common Attributes of <semanticMashupInstance> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| creator | O | NP |

### -----------------------End of change 23-------------------------------------------

### -----------------------Start of change 24-------------------------------------------

### 7.4.52	Resource Type <semanticMashupResult>

#### 7.4.52.1	Introduction

<semanticMashupResult> resource stores the mashup result. It is a child resource of a <semanticMashupInstance> resource. A <semanticMashupResult> child resource shall be generated by a Hosting CSE when it executes a semantic mashup operation on the parent <semanticMashupInstance> resource. The further detailed description can be found in clause 6.6 in oneM2M TS-0034: Semantics Support [Error: Reference source not found] and clause 9.6.56 in oneM2M TS-0001: Functional Architecture [Error: Reference source not found].

Table 7.4.52.1-42: Data type definition of <semanticMashupResult> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| semanticMashupInstance | CDT-semanticMashupResult-v4_0_0.xsd |  |

Table 7.4.52.1-43: Universal/Common Attributes of <semanticMashupResult> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | NP | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | NP | NP |
| accessControlPolicyIDs | NP | NP |
| expirationTime | NP | NP |
| dynamicAuthorizationConsultationIDs | NP | NP |
| announceTo | NP | NP |
| announcedAttribute | NP | NP |
| announceSyncType | NP | NP |
| creator | NP | NP |

### -----------------------End of change 24-------------------------------------------

### -----------------------Start of change 25-------------------------------------------

### 7.4.56	Resource Type <multimediaSession>

#### 7.4.56.1	Introduction

The <multimediaSession> resource represents a multimedia session between two AEs. The resource contains the session information which is used by the AEs to manage (e.g. establish, tear-down) the session using non-oneM2M protocols.

Table 7.4.56.1-44: Data type definition of <multimediaSession> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| multimediaSession | CDT-multimediaSession-v4_0_0.xsd |  |

Table 7.4.56.1-45: Universal/Common Attributes of <multimediaSession> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 25-------------------------------------------

### -----------------------Start of change 26-------------------------------------------

### 7.4.62	Resource Type <ontologyMapping>

#### 7.4.62.1	Introduction

The <ontologyMapping> resource is used for the mapping between two different ontologies. The detailed description can be found in clause 9.6.70 of oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.62.1-46: Data type definition of <ontologyMapping> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| ontologyMapping | CDT-ontologyMapping-v4_0_0.xsd |  |

Table 7.4.62.1-47: Universal/Common Attributes of <ontologyMapping> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 26-------------------------------------------

### -----------------------Start of change 27-------------------------------------------

### 7.4.63	Resource Type <ontologyMappingAlgorithmRepository>

#### 7.4.63.1	Introduction

The <ontologyMappingAlgorithmRepository> resource is a repository of ontology mapping algorithms, each represented by a <ontologyMappingAlgorithm>. The detailed description can be found in clause 9.6.71 of oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.63.1-1: Data type definition of <ontologyMappingAlgorithmRepository> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| ontologyMappingAlgorithmRepository | CDT-ontologyMappingAlgorithmRepository-v4_0_0.xsd |  |

Table 7.4.63.1-2: Universal/Common Attributes of <ontologyMappingAlgorithmRepository> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 27-------------------------------------------

### -----------------------Start of change 28-------------------------------------------

### 7.4.64	Resource Type <ontologyMappingAlgorithm>

#### 7.4.64.1	Introduction

The <ontologyMappingAlgorithm> resource is to store the representation of an ontology mapping algorithm. The detailed description can be found in clause 9.6.70 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.64.1-48: Data type definition of <ontologyMappingAlgorithm> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| ontologyMappingAlgorithm | CDT-ontologyMappingAlgorithm-v4_0_0.xsd |  |

Table 7.4.64.1-49: Universal/Common Attributes of <ontologyMappingAlgorithm> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |

### -----------------------End of change 28-------------------------------------------

### -----------------------Start of change 29-------------------------------------------

### 7.4.65	Resource Type <semanticRuleRepository>

#### 7.4.65.1	Introduction

The <semanticRuleRepository> resource is a repository and it may have one or multiple <reasoningRules> child resources to represent different sets of reasoning rules in the oneM2M system. The detailed description can be found in clause 9.6.65 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.65.1-50: Data type definition of <semanticRuleRepository> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| semanticRuleRepository | CDT-semanticRuleRepository-v4_0_0.xsd |  |

Table 7.4.65.1-51: Universal/Common Attributes of <semanticRuleRepository> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 29-------------------------------------------

### -----------------------Start of change 30-------------------------------------------

### 7.4.66	Resource Type <reasoningRules>

#### 7.4.66.1	Introduction

The <reasoningRules> resource stores a set of related reasoning rules (e.g. for supporting a particular application). A <reasoningRules> resource is a child resource of a <semanticRuleRepository> resource. The detailed description can be found in clause 9.6.66 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.66.1-52: Data type definition of <reasoningRules> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| reasoningRules | CDT-reasoningRules-v4_0_0.xsd |  |

Table 7.4.66.1-53: Universal/Common Attributes of <reasoningRules> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 30-------------------------------------------

### -----------------------Start of change 31-------------------------------------------

### 7.4.67	Resource Type <reasoningJobInstance>

#### 7.4.67.1	Introduction

The <reasoningJobInstance> resource represents a specific reasoning job instance for enabling the two types of reasoning operations (one type is a one-time reasoning operation and the other type is a continuous reasoning operation). The detailed description can be found in clause 9.6.67 in oneM2M TS-0001 Functional Architecture [Error: Reference source not found].

Table 7.4.67.1-54: Data type definition of <reasoningJobInstance> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| reasoningJobInstance | CDT-reasoningJobInstance-v4_0_0.xsd |  |

Table 7.4.67.1-55: Universal/Common Attributes of <reasoningJobInstance> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

### -----------------------End of change 31-------------------------------------------

### -----------------------Start of change 32-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

Table 8.2.3-56: Resource attribute short names (1/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| accessControlPolicyIDs | All except accessControlPolicy, contentInstance | acpi |
| announcedAttribute | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule, semanticDescriptor | aa |
| announceTo | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule, semanticDescriptor | at |
| announceSyncType | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule, semanticDescriptor | ast |
| creationTime | All | ct |
| expirationTime | All except contentInstance, CSEBase | et |
| labels | All (optional) | lbl |
| lastModifiedTime | All | lt |
| link | All | lnk |
| parentID | All | pi |
| resourceID | All | ri |
| resourceType | All | ty* |
| stateTag | container, contentInstance, delivery, request | st |
| resourceName | All | rn |
| privileges | accessControlPolicy | pv |
| selfPrivileges | accessControlPolicy | pvs |
| authorizationDecisionResourceIDs | accessControlPolicy | adri |
| authorizationPolicyResourceIDs | accessControlPolicy | apri |
| authorizationInformationResourceIDs | accessControlPolicy | airi |
| App-ID | AE | api |
| AE-ID | AE | aei |
| AE-IDList | AEContactListPerCSE | ail |
| appName | AE | apn |
| pointOfAccess | AE, CSEBase, remoteCSE | poa |
| ontologyRef | AE, container, contentInstance, semanticDescriptor. flexContainer, timeSeries | or |
| nodeLink | AE, CSEBase, remoteCSE, flexContainer | nl |
| contentSerialization | AE | csz |
| registrationStatus | AE | regs |
| trackRegistrationPoints | AE | trps |
| sessionCapabilities | AE | scp |
| activityPatternElements | AE, remoteCSE | ape |
| triggerEnable | AE, remoteCSE | tren |
| creator | container, contentInstance, eventConfig, group, pollingChannel, statsCollect, statsConfig, subscription, semanticDescriptor, notificationTargetPolicy, flexContainer, timeSeries, crossResourceSubscription, backgroundDataTransfer | cr |
| maxNrOfInstances | container, timeSeries, flexContainer | mni |
| maxByteSize | container, timeSeries, flexContainer | mbs |
| maxInstanceAge | container, timeSeries, flexContainer | mia |
| currentNrOfInstances | container, timeSeries, flexContainer | cni |
| location | CSEBase, remoteCSE, AE, container, flexContainer, timeSeries, deviceInfo | loc |

### -----------------------End of change 32-------------------------------------------