| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #47 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com Lu Liu, Convida, liu.lu@convidawireless.com |
| Date:* | 2020-10-11 |
| Reason for Change/s:* | Add <softwareCampaign> resources |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0080 - Edge and Fog Computing MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.2.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds support for the <softwareCampaign> resource introduced in TS-0001 for Release 4.

### -----------------------Start of change 1-------------------------------------------

### 7.4.XX	Resource Type <softwareCampaign>

#### 7.4. XX.1	Introduction

The <softwareCampaign> resource provides policies and rules for the management of software by a Hosting CSE. A <softwareCampaign> resource is used to trigger a Hosting CSE to install, uninstall, activate or deactivate software via the creation and management of [software] specialization(s) by the Hosting CSE. An AE can create <softwareCampaign> resource(s) on a Hosting CSE to instruct the Hosting CSE to manage the software versions of its Registrees (e.g. MN/ASN-CSEs) on the AE’s behalf. For example, a service provider (e.g. IN-AE) can create <softwareCampaign> resource(s) on a Hosting CSE (e.g. IN-CSE) to install and manage software on one or more Registree CSEs or AEs of the Hosting CSE.

Table 7.4.XX.1-1: Data type definition of <softwareCampaign> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| softwareCampaign | CDT-softwareCampaign-v4_XX_0.xsd |  |

Table 7.4.XX.1-2: Universal/Common Attributes of <softwareCampaign> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| owner | O | O |

Table 7.4.XX.1-3: Resource Specific Attributes of <softwareCampaign> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| softwareVersion | M | O | xs:string |  |
| softwareName | M | O | xs:string |  |
| softwareURL | M | O | xs:anyURI |  |
| softwareTargets | M | O | m2m:listOfURIs |  |
| softwareTriggerCriteria | O | O | list of m2m:softwareTriggerCriteria |  |
| softwareOperation | M | O | m2m:listOfsoftwareOperation |  |
| softwareLinks | NP | NP | m2m:listOfURIs |  |
| aggregatedSoftwareStatus | NP | NP | m2m:softwareStatus |  |
| individualSoftwareStatus | NP | NP | list of m2m:individualSoftwareStatus |  |

Table 7.4.XX.1-4: Child Resources of <softwareCampaign> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <schedule> | [variable] | 0..n | Clause Error: Reference source not found |

#### 7.4. XX.2	<softwareCampaign> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <softwareCampaign> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check if any <node> resources referenced by the softwareTargets attribute within the request are present in the softwareTargets attribute of other <softwareCampaign> resource hosted by the Receiver and that have an aggregatedSoftwareStatus attribute with a value of PENDING or IN PROGRESS. If so, the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check the existence and accessibility of the <node> resources referenced by the softwareTargets, attribute.  If any of the referenced <node> resources do not exist or are not accessible by the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check the existence and accessibility of the resources referenced by softwareTriggerCriteria attribute if present. If any of the referenced resources do not exist or are not accessible by the Originator, the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check the accessibility of the software package referenced by the URL attribute.  If the software package is not accessible, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- Process the <softwareCampaign> resource as described in clause 10.2.28 of oneM2M TS-0001 [Error: Reference source not found] after Recv-6.7.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- If the state of the aggregatedSoftwareStatus attribute of the resource is PENDING or IN PROGRESS, the Receiver shall  and reject the request and return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check if any <node> resources referenced by the softwareTargets attribute within the request are present in the softwareTargets attribute of another <softwareCampaign> resource hosted by the Receiver and that has an aggregatedSoftwareStatus attribute with a value of PENDING or IN PROGRESS. If so, the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check the existence and accessibility of the <node> resources referenced by the softwareTargets, attribute.  If any of the referenced <node> resources do not exist or are not accessible by the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check the existence and accessibility of the resources referenced by softwareTriggerCriteria attribute if present. If any of the referenced resources do not exist or are not accessible by the Originator, the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check the accessibility of the software package referenced by the URL attribute.  If the software package is not accessible, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- Process the <softwareCampaign> resource as described in clause 10.2.28 of oneM2M TS-0001 [Error: Reference source not found] after Recv-6.7.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- If the state of the aggregatedSoftwareStatus attribute of the resource is IN PROGRESS, the Receiver, based on local policy, shall either complete processing or pre-empt and cancel the software management operation that is in progress.

- The Receiver may delete any [software] specialization(s) associated with this <softwareCampaign> resource (i.e. the [software] specialization(s) linked through the softwareLinks attribute).

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 6.3.4	oneM2M enumerated data types

#### 6.3.4.2	Enumeration type definitions

##### 6.3.4.2.x	m2m:softwareOperation

Used for the softwareOperation attribute of the <softwareCampaign> resource.

Table 6.3.4.2.x-5: Interpretation of softwareOperation

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | INSTALL |  |
| 2 | UNINSTALL |  |
| 3 | ACTIVATE |  |
| 4 | DEACTIVATE |  |
| NOTE: |  |  |

##### 6.3.4.2.x	m2m:softwareStatus

Used for the aggregatedsoftwareStatus attribute of the <softwareCampaign> resource.

Table 6.3.4.2.x-6: Interpretation of softwareStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | PENDING |  |
| 2 | IN PROGRESS |  |
| 3 | INSTALLED |  |
| 4 | UNINSTALLED |  |
| 5 | ACTIVATED |  |
| 6 | DEACTIVATED |  |
| 7 | ERROR |  |
| NOTE: |  |  |

### 6.3.5	Complex data types

#### 6.3.5.x	m2m:softwareTriggerCriteria

Used for the softwareTriggerCriteria attribute of the <softwareCampaign>.

Table 6.3.5.x-7: Type Definition of m2m:softwareTriggerCriteria

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| subjectResourceID | xs:anyURI | 1 |  |
| evalCriteria | m2m:evalCriteria | 1 |  |

#### 6.3.5.x	m2m:individualSoftwareStatus

Used for the individualSoftwareStatus attribute of the <softwareCampaign> resource.

Table 6.3.5.x-8: Type Definition of m2m:individualSoftwareStatus

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| softwareResourceID | xs:anyURI | 1 |  |
| softwareStatus | m2m:softwareStatus | 1 |  |

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

##### 6.3.4.2.1	m2m:resourceType

Table 6.3.4.2.1-9: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | Container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | Delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 45 | localMulticastGroup |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | softwareCampaign |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 100XX | softwareCampaignAnnc |  |
| NOTE: See clause Error: Reference source not found. |  |  |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

##### 6.3.4.2.11	m2m:memberType

Used for the memberType attribute of the  <group> resource.

Table 6.3.4.2.11-10: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | mixed | A mixture of all the resource types (except mixed itself). |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | softwareCampaign |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 100XX | softwareCampaignAnnc |  |
| 20001 | oldest |  |
| 20002 | latest |  |
| 20003 | mashup |  |
| NOTE: See clause Error: Reference source not found "Resource Type group". |  |  |

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

### 6.5.4	announceableResource

#### 6.5.4.1	Description

This type definition includes the universal and common attributes used by oneM2M resource types that are capable of being announced. In addition to the attributes of a regularResource, it includes (as optional) the common attributes that are used by the announcement mechanism.

#### 6.5.4.2	Reference

See Error: Reference source not found.

#### 6.5.4.3	Usage

This type is used by the following resource types:

<AE>, <container>, <group>, <locationPolicy>, <node>, <remoteCSE>, <semanticDescriptor>, <timeSeries>, <ontologyRepository>, <ontology>, <ontologyMapping>, <ontologyMappingAlgorithm>, <ontologyMappingAlgorithmRepository>, <semanticMashupJobProfile>, <semanticMashupInstance>, <semanticMashupResult>, <multimediaSession>, <schedule>, <semanticRuleRepository>, <reasoningRules>, <reasoningJobInstance>, <softwareCampaign>.

It is also used by the specializations of <mgmtObj>.

### 6.5.5	announcedResource

#### 6.5.5.1	Description

This type definition includes the universal and common attributes used by a resource that is announcing an announceable resource. In addition to the attributes of a regularResource, it includes (as optional) the link common attribute.

#### 6.5.5.2	Reference

See Error: Reference source not found.

#### 6.5.5.3	Usage

This type is used by the following resource types:

<AEAnnc>, <containerAnnc>, <groupAnnc>, <locationPolicyAnnc>, <nodeAnnc>, <remoteCSEAnnc>, <semanticDescriptorAnnc>, <[flexContainer]Annc>, <timeSeriesAnnc>, <ontologyRepositoryAnnc>, <ontologyAnnc>, <ontologyMappingAnnc>, <ontologyMappingAlgorithmAnnc>, <ontologyMappingAlgorithmRepositoryAnnc>, <semanticMashupJobProfileAnnc>, <semanticMashupInstanceAnnc>, <semanticMashupResultAnne>, <multimediaSessionAnnc>,<scheduleAnnc>, <semanticRuleRepositoryAnnc>, <reasoningRulesAnnc>, <reasoningJobInstanceAnnc>, <softwareCampaignAnnc>.

It is also used by the xxxAnnc variants of the <mgmtObj> specializations.

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

…

Table 8.2.3-11: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| sofwtareVersion | firmware, software, token, softwareCampaign | swvr |
| softwareName | software, softwareCampaign | swnm |
| softwareURL | firmware, software, softwareCampaign | swur |
| softwareTargets | softwareCampaign | swts |
| softwareTriggerCriteria | softwareCampaign | swtc |
| softwareOperation | softwareCampaign | swop |
| softwareLinks | softwareCampaign | swlk |
| aggregatedSoftwareStatus | softwareCampaign | asws |
| individualSoftwareStatus | softwareCampaign | isws |

### -----------------------End of change 6-------------------------------------------

### -----------------------Start of change 7-------------------------------------------

### 8.2.4	Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-12.

Table 8.2.4-12: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| … | … |
| softwareCampaign | swcm |

-----------------------End of change 7-------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 7.4.26	Announced resource types

#### 7.4.26.1	Introduction

Instances of certain resource types can be announced by the Hosting CSE to one or more remote CSEs to inform the remote CSEs of the existence of the original resource instance. Table 7.4.26.1-13 lists the resource types which are announceable together with its resource type identifier and the name of the XSD file that specifies its data type.

Resource-specific attributes of a resource type may be declared announceable (mandatory or optional) or not announceable. Clause 9.6 in oneM2M TS-0001 [Error: Reference source not found] defines which attributes of each resource type are announceable.

The universal/common attributes of an announced resource type are listed in Table 7.4.26.1-14, together with their request optionality in Create and Update requests (see clause 7.4.1). The announced resource includes a common attribute link which represents a link to the original resource hosted by the original resource-Hosting CSE.

The child resources applicable to each announced resource type are listed in Table 9.6.26.1-1 of oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.26.1-1: Data type definition of announced Resource types

| Data Type ID | File Name | Note |
| --- | --- | --- |
| accessControlPolicyAnnc | CDT-accessControlPolicy-v4_0_0.xsd |  |
| remoteCSEAnnc | CDT-remoteCSE-v4_0_0.xsd |  |
| AEAnnc | CDT-AE-v4_0_0.xsd |  |
| containerAnnc | CDT-container-v4_0_0.xsd |  |
| [flexContainer]Annc | See Annex J and oneM2M TS-0023 [Error: Reference source not found] | Announced variants of flexContainer specializations |
| contentInstanceAnnc | CDT-contentInstance-v4_0_0.xsd |  |
| scheduleAnnc | CDT-schedule-v4_0_0.xsd |  |
| locationPolicyAnnc | CDT-locationPolicy-v4_0_0.xsd |  |
| groupAnnc | CDT-group-v4_0_0.xsd |  |
| nodeAnnc | CDT-node-v4_0_0.xsd |  |
| semanticDescriptorAnnc | CDT-semanticDescriptor-v4_0_0.xsd |  |
| timeSeriesAnnc | CDT-timeSeries-v4_0_0.xsd |  |
| timeSeriesInstanceAnnc | CDT-timeSeriesInstance-v4_0_0.xsd |  |
| ontologyRepositoryAnnc | CDT-ontologyRepository-v4_0_0.xsd |  |
| ontologyAnnc | CDT-ontology-v4_0_0.xsd |  |
| semanticMashupJobProfileAnnc | CDT-semanticMashupJobProfile-v4_0_0.xsd |  |
| semanticMashupInstanceAnnc | CDT-semanticMashupInstance-v4_0_0.xsd |  |
| semanticMashupResultAnnc | CDT-semanticMashupResult-v4_0_0.xsd |  |
| genericInterworkingServiceAnnc | CDT-genericInterworkingService-v4_0_0.xsd |  |
| genericInterworkingOperationInstanceAnnc | CDT-genericInterworkingOperationInstance-v4_0_0.xsd |  |
| svcObjWrapperAnnc | CDT-svcObjWrapper-v4_0_0.xsd |  |
| svcFwWrapperAnnc | CDT-svcFwWrapper-v4_0_0.xsd |  |
| allJoynAppAnnc | CDT-allJoynApp-v4_0_0.xsd |  |
| allJoynSvcObjectAnnc | CDT-allJoynSvcObject-v4_0_0.xsd |  |
| allJoynInterfaceAnnc | CDT-allJoynInterface-v4_0_0.xsd |  |
| allJoynMethodAnnc | CDT-allJoynMethod-v4_0_0.xsd |  |
| allJoynMethodCallAnnc | CDT-allJoynMethodCall-v4_0_0.xsd |  |
| allJoynPropertyAnnc | CDT-allJoynProperty-v4_0_0.xsd |  |
| multimediaSessionAnnc | CDT-multimediaSession-v4_0_0.xsd |  |
| ontologyMappingAnnc | CDT-ontologyMapping-v4_0_0.xsd |  |
| ontologyMappingAlgorithmAnnc | CDT-ontologyMappingAlgorithm-v4_0_0.xsd |  |
| ontologyMappingAlgorithmRepositoryAnnc | CDT-ontologyMappingAlgorithmRepository-v4_0_0.xsd |  |
| semanticRuleRepositoryAnnc | CDT-semanticRuleRepository-v4_0_0.xsd |  |
| reasoningRulesAnnc | CDT-reasoningRules-v4_0_0.xsd |  |
| reasoningJobInstanceAnnc | CDT-reasoningJobInstance-v4_0_0.xsd |  |
| softwareCampaignAnnc | CDT-softwareCampaign-v4_0_0.xsd |  |

### -----------------------End of change 8-------------------------------------------

### -----------------------Start of change 9-------------------------------------------

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-15: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| subjectResourceID | softwareTriggerCriteria | srid |
| evalCriteria | softwareTriggerCriteria | evcr |
| softwareResourceID | individualSoftwareStatus | swid |
| softwareStatus | individualSoftwareStatus | swst |

### -----------------------End of change 9-------------------------------------------