| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #45 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com |
| Date:* | 2020-04-15 |
| Reason for Change/s:* | Add new and updated resource definitions for M2M Service Subscriber and User |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0083 - oneM2M Service Subscribers and Users MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.0.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds/enhances resource definitions to add support for M2M Service Subscriber and User.  This is in support of the M2M Service Subscriber and User functionality introduced in TS-0001 for Release 4.

A summary of the updates includes:

- Add new <serviceSubscribedUserProfile> resource

- Update <m2mServiceSubscriptionProfile> resource

  - New M2M-Sub-ID attribute

  - New M2M-SS-ID attribute

  - New <serviceSubscribedUserProfile> child resource

- Update <serviceSubscribedAppRule> resource

  - New allowedUsers attribute

R01 – Add additional attributes to <serviceSubscribedUserProfile> resource and <m2mServiceSubscriptionProfile> resource that were recently added.

R02: - Address Peter’s comments:

- There aren't any procedures for the new <m2mServiceSubscriptionProfile> attributes in change 2.  I would think we might need some for  activate and for the Update operation.

### -----------------------Start of change 1-------------------------------------------

### 7.4.XX	Resource Type <serviceSubscribedUserProfile>

#### 7.4. XX.1	Introduction

The <serviceSubscribedUserProfile> resource contains user profile information for a given M2M Service User.

Table 7.4.XX.1-1: Data type definition of <serviceSubscribedUserProfile> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| serviceSubscribedUserProfile | CDT-serviceSubscribedUserProfile-v4_XX_0.xsd |  |

Table 7.4.XX.1-2: Universal/Common Attributes of <serviceSubscribedUserProfile> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.XX.1-3: Resource Specific Attributes of <serviceSubscribedUserProfile> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| M2M-User-ID | M | O | m2m:ID | No default |
| maxNumAEs | O | O | xs:nonNegativeInteger | No default |
| maxNumNodes | O | O | xs:nonNegativeInteger | No default |
| maxNumBytes | O | O | xs:nonNegativeInteger | No default |
| maxRequestRate | O | O | xs:nonNegativeInteger | No default |
| maxNumContainers | O | O | xs:nonNegativeInteger | No default |
| maxNumInstancesPerContainer | O | O | xs:nonNegativeInteger | No default |
| maxNumTimeSeries | O | O | xs:nonNegativeInteger | No default |
| maxNumInstancesPerTimeSeries | O | O | xs:nonNegativeInteger | No default |
| maxMembersPerGroup | O | O | xs:nonNegativeInteger | No default |
| maxNotificationRate | O | O | xs:nonNegativeInteger | No default |
| maxNumFlexContainers | O | O | xs:nonNegativeInteger | No default |
| maxNumInstancesPerFlexContainer | O | O | xs:nonNegativeInteger | No default |
| defaultAccessControlPrivileges | O | O | m2m:setOfAcrs | No default |

Table 7.4.XX.1-4: Child Resources of <serviceSubscribedUserProfile> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

#### 7.4. XX.2	<serviceSubscribedUserProfile> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <serviceSubscribedUserProfile> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

- The Receiver shall check that any values specified for the attributes of this <serviceSubscribedUserProfile> do not violate the limits (if specified) in the parent <m2mServiceSubscriptionProfile> resource (e.g. maxNumContainers).  When performing this check, the Receiver shall check that the aggregated values of all attributes across all the <serviceSubscribedUserProfile> child resources do not violate the limits specified in the parent <m2mServiceSubscriptionProfile> resource. If the limits are exceeded, the Receiver shall reject the request with the "CONTENTS_UNACCEPTABLE" Response Status Code or modify the values specified in the request.  If limits are not specified in the parent <m2mServiceSubscriptionProfile> resource, the Receiver may use a limit based on local policy.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:.

- If the request is to update an attribute specifying a limit (e.g. maxNumNodes, maxNumContainers, etc.) to a reduced value, the Receiver shall update the attribute with the reduced value if the limit corresponding to the reduced value has not yet been exceeded. For example, if maxNumNodes has a value of 10 and the request is to update it to a value of 5 and only 4 <node> resources associated with this user have been created thus far, then the Receiver shall update maxNumNodes attribute to a value of 5.  However, if the limit corresponding to the reduced value in the request has already been exceeded, the Receiver shall reject the request with the "CONTENTS_UNACCEPTABLE" Response Status Code or override the value specified in the request. For example, if the maxNumNodes attribute has a value of 10 and the request is to update it to a value of 5 but 6 <node> resources associated with this user have already been created, then the Receiver shall either reject the request or update the value of the maxNumNodes attribute to 6.

- The Receiver shall check that any values specified for the attributes of this <serviceSubscribedUserProfile> do not violate the limits (if specified) in the parent <m2mServiceSubscriptionProfile> resource (e.g. maxNumContainers).  When performing this check, the Receiver shall check that the aggregated values of all attributes across all the <serviceSubscribedUserProfile> child resources do not violate the limits specified in the parent <m2mServiceSubscriptionProfile> resource. If the limits are exceeded, the Receiver shall reject the request with the "CONTENTS_UNACCEPTABLE" Response Status Code or modify the values specified in the request.  If limits are not specified in the parent <m2mServiceSubscriptionProfile> resource, the Receiver may use a limit based on local policy.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.4.19	Resource Type <m2mServiceSubscriptionProfile>

#### 7.4.19.1	Introduction

The <m2mServiceSubscriptionProfile> resource represents an M2M Service Subscription Profile. It is used to represent all data pertaining to the M2M Service Subscription Profile, i.e. the technical part of the contract between an M2M Application Service Provider and an M2M Service Provider.

The detailed description can be found in clause 9.6.19 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.19.1-5: Data type definition of <m2mServiceSubscriptionProfile> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| m2mServiceSubscriptionProfile | CDT-m2mServiceSubscriptionProfile-v4_0_0.xsd |  |

Table 7.4.19.1-6: Universal/Common Attributes of <m2mServiceSubscriptionProfile>

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| labels | O | O |
| lastModifiedTime | NP | NP |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.19.1-7: Child resources of <m2mServiceSubscriptionProfile>

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <serviceSubscribedNode> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <serviceSubscribedUserProfile> | [variable] | 0..n | Clause 7.4.XX |

Table 7.4.19.1-4: Resource Specific Attributes of <m2mServiceSubscriptionProfile> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| M2M-Sub-ID | M | NP | m2m:ID | No default |
| M2M-SS-ID | O | O | m2m:ID | No default |
| status | NP | NP | m2m:serviceSubscriptionStatus | No default |
| activate | M | O | xs:boolean | FALSE |
| activationTime | NP | NP | m2m:timestamp | No default |
| deactivationTime | NP | NP | m2m:timestamp | No default |
| serviceSubscriptionDuration | O | O | xs:duration | No default |
| currentNumAEs | NP | NP | xs:nonNegativeInteger | No default |
| maxNumAEs | O | O | xs:nonNegativeInteger | No default |
| currentNumNodes | NP | NP | xs:nonNegativeInteger | No default |
| maxNumNodes | O | O | xs:nonNegativeInteger | No default |
| maxNumBytes | O | O | xs:nonNegativeInteger | No default |
| maxNumUsers | O | O | xs:nonNegativeInteger | No default |
| maxRequestRate | O | O | xs:nonNegativeInteger | No default |
| maxNumContainers | O | O | xs:nonNegativeInteger | No default |
| maxNumInstancesPerContainer | O | O | xs:nonNegativeInteger | No default |
| maxNumTimeSeries | O | O | xs:nonNegativeInteger | No default |
| maxNumInstancesPerTimeSeries | O | O | xs:nonNegativeInteger | No default |
| maxMembersPerGroup | O | O | xs:nonNegativeInteger | No default |
| maxNotificationRate | O | O | xs:nonNegativeInteger | No default |
| maxNumFlexContainers | O | O | xs:nonNegativeInteger | No default |
| maxNumInstancesPerFlexContainer | O | O | xs:nonNegativeInteger | No default |
| defaultAccessControlPrivileges | O | O | m2m:setOfAcrs | No default |

#### 7.4.19.2	<m2mServiceSubscriptionProfile> resource specific procedures for CRUD operations

##### 7.4.19.2.0	Introduction

This clause describes <m2mServiceSubscriptionProfile> resource specific behaviour for CRUD operations.

##### 7.4.19.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

- If the activate attribute of the targeted <m2mServiceSubscriptionProfile> resource is set to a value of true, the Receiver shall activate the service subscription profile such that the service subscription limits defined by the service subscription profile are used by the Receiver when processing subsequent requests received from Originators associated with the service subscription profile as defined in clause 7.3.3.XX.

##### 7.4.19.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.19.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- If the activate attribute of the targeted <m2mServiceSubscriptionProfile> resource is updated from a value of false to true, the Receiver shall activate the service subscription profile such that the service subscription limits defined by the service subscription profile are used by the Receiver when processing subsequent requests received from Originators associated with the service subscription profile as defined in clause 7.3.3.XX.

- If the activate attribute of the targeted <m2mServiceSubscriptionProfile> is updated from a value of true to false, the Receiver shall deactivate the service subscription profile such that the Receiver shall reject subsequent requests received from Originators associated with the service subscription profile.

##### 7.4.19.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### ----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 7.4.29	Resource Type <serviceSubscribedAppRule>

#### 7.4.29.1	Introduction

The <serviceSubscribedAppRule> resource represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE. The detailed description can be found in the clause 9.6.29 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.29.1-8: Data type definition of <serviceSubscribedAppRule> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| serviceSubscribedAppRule | CDT-serviceSubscribedAppRule-v4_0_0.xsd |  |

Table 7.4.29.1-9: Universal/Common Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| dynamicAuthorizationConsultationIDs | O | O |

Table 7.4.29.1-10: Resource Specific Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| applicableCredIDs | O | O | m2m:listOfM2MID |  |
| allowedApp-IDs | O | O | m2m:listOfM2MID |  |
| allowedAEs | O | O | m2m:listOfM2MID |  |
| allowedRole-IDs | O | O | List of m2m:roleID |  |
| allowedUsers | O | O | m2m:listOfM2MID |  |

Table 7.4.29.1-11: Child resources of <serviceSubscribedAppRule> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

##### 6.3.4.2.1	m2m:resourceType

Table 6.3.4.2.1-12: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 45 | localMulticastGroup |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | serviceSubscribedUserProfile |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| NOTE: See clause Error: Reference source not found. |  |  |

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

##### 6.3.4.2.11	m2m:memberType

Used for the memberType attribute of the <group> resource.

Table 6.3.4.2.11-13: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | Mixed | A mixture of all the resource types (except mixed itself). |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | Container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | Delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | Group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | Node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | Request |  |
| 18 | Schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | Subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | Role |  |
| 32 | Token |  |
| 33 | Void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | Ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | Transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | serviceSubscribedUserProfile |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 20001 | oldest |  |
| 20002 | latest |  |
| 20003 | mashup |  |
| NOTE: See clause Error: Reference source not found "Resource Type group". |  |  |

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 6.5.3	regularResource

#### 6.5.3.1	Description

This type definition includes the universal and common attributes used by the non-announceable oneM2M resources.

#### 6.5.3.2	Reference

See Error: Reference source not found.

#### 6.5.3.3	Usage

This type is used by the following resource types:

<delivery>, <eventConfig>, <execInstance>, <m2mServiceSubscriptionProfile>, <mgmtCommand>, <request>, <serviceSubscribedNode>, <statsCollect>, <statsConfig>, <subscription>, <serviceSubscribedAppRule>, <notificationTargetMgmtPolicyRef>, <notificationTargetPolicy>, <policyDeletionRules>, <dynamicAuthorizationConsultation>, <role>, <token>, <authorizationDecision>, <authorizationPolicy> <authorizationInformation>, <AEContactList>, <AEContactListPerCSE>, <localMulticastGroup>, <triggerRequest>, <crossResourceSubscription>, <backgroundDataTransfer>, <transactionMgmt>, <transaction>, <serviceSubscribedUserProfile>.

### -----------------------End of change 6-------------------------------------------

### -----------------------Start of change 7-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

…

Table 8.2.3-14: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| activate | software, m2mServiceSubscriptionProfile | act |
| … | … | … |
| status | authorizationDecision, authorizationPolicy, authorizationInformation, m2mServiceSubscriptionProfile | sus |
| … | … | … |
| M2M-Sub-ID | m2mServiceSubscriptionProfile | msui |
| M2M-SS-ID | m2mServiceSubscriptionProfile | mssi |
| M2M-User-ID | serviceSubscribedUserProfile | mui |
| allowedUsers | serviceSubscribedAppRule | allu |
| activationTime | m2mServiceSubscriptionProfile | actt |
| deactivationTime | m2mServiceSubscriptionProfile | dact |
| serviceSubscriptionDuration | m2mServiceSubscriptionProfile | ssd |
| currentNumAEs | m2mServiceSubscriptionProfile | cna |
| currentNumNodes | m2mServiceSubscriptionProfile | cnn |
| maxNumAEs | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mna |
| maxNumNodes | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnns |
| maxNumBytes | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnb |
| maxRequestRate | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mrr |
| maxNumContainers | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnc |
| maxNumInstancesPerContainer | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnis |
| maxNumTimeSeries | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnt |
| maxNumInstancesPerTimeSeries | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnit |
| maxMembersPerGroup | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mmg |
| maxNotificationRate | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnr |
| maxNumFlexContainers | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnf |
| maxNumInstancesPerFlexContainer | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | mnif |
| defaultAccessControlPrivileges | m2mServiceSubscriptionProfile, serviceSubscribedUserProfile | dacp |

### -----------------------End of change 7-------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 8.2.4	Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-15.

Table 8.2.4-15: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| … | … |
| serviceSubscribedUserProfile | ssup |

### -----------------------End of change 8-------------------------------------------

### -----------------------Start of change 9-------------------------------------------

##### 6.3.4.2.XX	m2m:serviceSubscriptionStatus

Used for status attribute in <m2mServiceSubscriptionProfile> resource.

Table 6.3.4.2.XX-16: Interpretation of serviceSubscriptionStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | ACTIVE |  |
| 2 | INACTIVE |  |
| NOTE: See clause 7.4.19 "Resource Type m2mServiceSubscriptionProfile". |  |  |

### -----------------------End of change 9-------------------------------------------