| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #47 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com Lu Liu, Convida, liu.lu@convidawireless.com |
| Date:* | 2020-10-04 |
| Reason for Change/s:* | Maintenance on <action> and <dependency> resource functionality |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0093 – Action Triggering Enhancements MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 V4.7.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes some updates to the <action> and <dependency> to clean-up functionality as part of the Stage 3 work.

R03:

- Update evalControlMode description to remove “ignore” text

- Added text to clarify that applicable <dependency> resources are child resources of <action> resource

R04:

- Changed actionResult from RW to RO

### -----------------------Start of change 1-------------------------------------------

### 9.6.61	Resource Type action

The <action> resource defines an operation (i.e. a oneM2M primitive) performed by the Hosting CSE on a target resource when criteria defined by the <action> resource and any applicable child <dependency> resources of the <action> resource have been met.

The <action> resource and any applicable child <dependency> resources include the following information:

- A subject resource that is monitored by the Hosting CSE to detect if/when the evaluation criteria specified by the <action> resource are met.

- An object resource that the Hosting CSE targets the primitive, defined by the <action> resource, towards if/when the evaluation criteria specified by the <action> resource and any applicable child <dependency> resources are met.

- An input attribute that may provide input parameters for the action.

- Conditions that are monitored by the Hosting CSE to determine if the primary event occurs.

- Dependencies that are evaluated by the Hosting CSE to determine if the action is to be conditionally performed.

- Parameters providing priorities of the action and dependencies.

The <action> resource shall contain the child resources specified in table 9.6.61-1.

Table 9.6.61-1: Child resources of <action> resource

| Child Resources of <action> | Child Resource Type | Multiplicity | Description | <actionAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <dependency> | 0..n | See clause 9.6.62 | <dependencyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |

The <action> resource shall contain the attributes specified in table 9.6.61-2.

Table 9.6.61-2: Attributes of <action> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <actionAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 1 | RO | See clause 9.6.1.3. | NA |
| owner | 0..1 | RW | See clause 9.6.1.3. | NA |
| actionPriority | 0..1 | RW | Attribute indicating the priority level of this action compared with other actions having the same event expression (subjectResourceID and evalCriteria). The priority level is formatted as a number with highest priority indicated by the lowest value. Local policies are applied for equal or unspecified priorities. | OA |
| subjectResourceID | 0..1 | RW | The resource identifier of the resource that is the subject of monitoring for evaluation of the evalCriteria attribute. The subject can be any resource in the system. If this attribute is not configured, the subject is the parent resource of this <action> resource. | OA |
| evalCriteria | 1 | RW | This attribute provides the conditions determining if the action is to be conditionally triggered for the object resource. The evalCriteria attribute, in conjunction with the subjectResourceID, forms the event expression that is being monitored. See further description below and in table 9.6.61-3. | OA |
| evalMode | 1 | RW | This attribute provides the control mode of the evaluation of the <action> resource. The supported values are: 0 = off 1 = once 2 = periodic 3 = continuous | OA |
| evalControlParam | 0..1 | RW | When evalMode has a value of “periodic”, the evalControlParam represents the periodicity in seconds. If no value of this attribute is configured, then the Hosting CSE shall use local policies to determine the periodicity value. When evalMode has a value of “continuous” the evaluation is restarted as soon as an action has been triggered. In this case, if evalControlParam is specified, it determines the number of times the Hosting CSE shall trigger the event, otherwise it is repeated indefinitely. When evalMode has a value of “off” or “once”, this attribute shall not be be present. | OA |
| dependencies | 0..1 (L) | RW | List of dependencies in priority order. This is an ordered list of the resource identifiers of child <dependency> resources. | OA |
| objectResourceID | 1 | RW | The identifier of a resource or attribute that the Hosting CSE targets the actionPrimitive towards when detecting that the evalCriteria and any dependencies defined in the dependencies attribute have been met. | MA |
| actionPrimitive | 1 | RW | This attribute stores the primitive that is performed on the objectResourceID when the Hosting CSE detects that the evalCriteria and any dependencies defined in the dependencies attribute have been met. If the objectResourceID attribute of the <action> resource is configured, the Hosting CSE shall override the value of the To parameter in the actionPrimitive with the resource identifier defined by objectResourceID and the value of the Resource Type parameter with the resource type of the resource referenced by objectResourceID. If the input attribute of the <action> resource is configured, the Hosting CSE shall override the value of the Content parameter in the actionPrimitive with the value stored in the input attribute. | MA |
| input | 0..1 | RW | Input used by the Hosting CSE to override the Content parameter in the actionPrimitive. This attribute may be formatted as: - a string, - a resource identifier, or - the identifier of a resource attribute. The Hosting CSE overrides the Content parameter in the actionPrimitive with the following (respectively): - the given string, - the resource representation of the referenced resource, - the value of the referenced resource attribute | OA |
| outputResourceID | 0..1 | RW | The resource identifier of a resource where the Hosting CSE stores a response primitive after performing the actionPrimitive on the objectResourceID. | OA |
| actionResult | 1 | RO | After performing the actionPrimitive on the objectResourceID, the Hosting CSE stores the response primitive in this attribute. | OA |

The conditions represented in the evalCriteria attribute determine if the action is to be conditionally triggered.

The evalCriteria attribute is comprised of triples (subject, operator, threshold) with the parameters shown and described in table 9.6.61-3.

Table 9.6.61-3: Parameters in evalCriteria triple

| Name | Description |
| --- | --- |
| subject | This parameter shall indicate an attribute of the subject or dependency resource e.g. “content” |
| operator | This parameter is a keyword used to construct the evaluation logic, e.g. ‘equals’, ‘not equals’, ‘greater than’, ‘less than’, ‘greater or equal’, ‘less or equal’, ‘string match’, etc. |
| threshold | This parameter provides a value used to evaluate the criteria. The value type shall match the type of the attribute used as subject. |

### 9.6.62	Resource Type dependency

The <dependency> resource represents conditions that determine if the action specified in the parent <action> resource is triggered (i.e. a oneM2M primitive performed by the Hosting CSE on a target resource). One or more <dependency> resources may exist to provide complex events for triggering. The <dependency> resources provide conditions that always need to be checked before the action is triggered. The difference between the evalCriteria  specified by an <action> resource and the one specified by a  <dependency> resource is that the former applies to the subject resource and triggers an evaluation, while the latter applies to other resources which provide conditions used in evaluation.

The <dependency> resource shall contain the child resources specified in table 9.6.62-1.

Table 9.6.62-1: Child resources of <dependency> resource

| Child Resources of <action> | Child Resource Type | Multiplicity | Description | <dependencyAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |

The <dependency> resource shall contain the attributes specified in table 9.6.62-2.

Table 9.6.62-2: Attributes of <dependency> resource

| Attributes of <action> | Multiplicity | RW/ RO/ WO | Description | <dependencyAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. If no accessControlPolicyIDs value is configured, the accessControlPolicyIDs of the parent resource shall be applied for privilege checking. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3. | NA |
| owner | 0..1 | RW | See clause 9.6.1.3. | NA |
| logic | 1 | RW | Indicates if the dependency is SUFFICIENT or NECESSARY. The operand of the related parent <action> resource needs to match: At least one of the SUFFICIENT dependencies OR All the NECESSARY dependencies | OA |
| evalCriteria | 1 | RW | Provides conditions determining if the dependency matches the criteria. The evalCriteria attribute in conjunction with the dependentResourceID attribute forms the dependency to be evaluated. See further description in table 9.6.61-3. | OA |
| dependentResourceID | 1 | RW | The resource identifier of the resource for which the evalCriteria is evaluated for the dependency. | MA |

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 10.2.21	Action Triggering

#### 10.2.21.1	Introduction

This clause describes the procedure for managing oneM2M action triggering via the <action> and <dependency> resources.

A oneM2M <action> resource is created whenever an Originator desires a primitive to be sent in response to the detection of a oneM2M event. An event has a subject resource associated with it – a change on this subject resource causes the Hosting CSE to conditionally trigger the procedure as outlined in 10.2.21.2. This is referred to as “action triggering”. The subject resource is identified by the subjectResourceID attribute of the <action> resource and the evalCriteria attribute of the <action> resource determines if an action is conditionally triggered. Collectively, this combination is known as the event expression and the event is the primary event when it is triggered.

When an <action> resource is created, the Hosting CSE checks the evalMode attribute to see if an event is enabled for monitoring. The evalMode attribute has one of four valid values: off, once, periodic, or continuous. If evalMode is off, then no event is being monitored by the Hosting CSE. For all other values, the Hosting CSE will begin monitoring events.

When evalMode is once, the Hosting CSE will immediately enable monitoring the event and will only allow the primary event to trigger once. After the event triggers, the Hosting CSE will disable further monitoring.

When evalMode is either periodic or continuous, the Hosting CSE will also evaluate the evalControlParam attribute to determine how to monitor the event. For periodic evalMode, the evalControlParam attribute determines the periodicity of monitoring the event. The Hosting CSE will start monitoring the event at the indicated periodicity and only allow the event to trigger once within the indicated period. For continuous evalMode, the evalControlParam attribute determines the number of times the Hosting CSE should trigger an event. When in this mode, the Hosting CSE resumes monitoring for the occurrence of the next event upon the triggering of a prior event; hence, there is no lapse in monitoring.

To handle more complex events, one or more  <dependency> resources  may be created as a child resources of the <action> resource. Within the <dependency> resource, the dependentResourceID attribute identifies the resource that the event further depends on. The evalCriteria attribute of the <dependency> resource determines whether the dependency condition is met. The logic attribute determines whether the dependency condition is SUFFICIENT or NECCESARY to the action triggering. Finally, the dependencies attribute of the <action> resource determines the priorities to the dependency conditions to be applied when evaluating if the action or operation should be triggered based on the primary event. The dependency conditions shall be evaluated by the Hosting CSE in their dependencies priority order.

Once a Hosting CSE has determined that an event has triggered, it then uses the following attributes to create the  associated action : actionPrimitive, objectResourceID, and input. The actionPrimitive attribute provides the main content for the primitive that the Hosting CSE sends when the event is triggered. The objectResourceID attribute is used to derive the To and Resource Type parameters which are overwritten in the primitive to be sent. If present, the input attribute is used to derive Content parameter which is overwritten in the primitive to be sent. If the input attribute is not present, the primitive sent maintains the parameters provided in the actionPrimitive.

Both the outputResourceID and actionResult attributes of the <action> resource are used by the Hosting CSE to store the response primitive associated with the actionPrimitive. The Hosting CSE shall store the response primitive in the actionResult attribute. If the outputResourceID attribute is configured the Hosting CSE shall also store the response primitive in the resource referenced by the outputResourceID attribute. .

#### 10.2.21.2	Action Triggering Procedure

This clause describes the procedure a Hosting CSE performs to manage the monitoring of events and, when they occur, the conditional triggering of actions and operations specified, as detailed by the following the steps:

Step 1:	Create <action> resource either as the child resource of a subject resource or elsewhere in the resource tree. The subject resource can be any of the resources defined in oneM2M and is indicated by the subjectResourceID attribute. One or more <dependency> resources may be created as child resources of the <action> resource.

Changes to the subject resource trigger the evaluation of the evalCriteria.

Step 2:		The content of subject resource is updated

Step 3:	The Hosting CSE gets the updated result and evaluates the updated subject resource, and if it matches the criteria defined in the evalCriteria attribute of the <action> resource, the primary event is considered triggered. If the subject resource does not match the criteria defined in the evalCriteria attribute of the <action> resource, the Hosting CSE returns to step 2, waiting for further subject resource updates.

Step 4:	After a primary event is triggered in step 3, if there is at least one <dependency> child resource of the <action> resource, the Hosting CSE performs the following step for each <dependency> child resource(s) in the order provided in the dependencies attribute.

For each of the <dependency> child resources, the Hosting CSE uses the evalCriteria attribute and the logic attribute values provided in the current <dependency> resource. The Hosting CSE retrieves the dependent resource specified by the dependentResourceID attribute, evaluates it against the criteria defined in the evalCriteria attribute to check if the dependency is met.

If the dependency is met (i.e. the resource identified by dependentResourceID meets the criteria specified by evalCriteria):

- if the logic attribute is of value SUFFICIENT, the Hosting CSE proceeds to step 5.

- if the logic attribute is of value NECESSARY, the hosting CSE repeats step 4 for the next <dependency> child resource.

If the dependency is not met (i.e. the resource identified by dependentResourceID does not meet the criteria specified by evalCriteria), the hosting CSE repeats step 4 for the next <dependency> child resource.

Step 5:	The Hosting CSE retrieves the input resource if the input attribute of the <action> resource is a resourceID.

NOTE: The procedure outlined in step 4 means that step 5 is reached when, after the primary event is triggered, either:

- the SUFFICIENT dependency with the highest dependencies priority is met; OR

- all the NECCESARY dependencies are met.

Step 6:	The Hosting CSE composes the request primitive for the object resource for triggering the action by populating it with the content of the actionPrimitive attribute.

If the objectResource attribute is present, the Hosting CSE determines its type (e.g. by performing a retrieve operation). The Hosting CSE overwrites the To and Resource Type parameters of the primitive to be sent with the values determined from objectResourceID.

If the input attribute is present and is a string, the Hosting CSE overwrites the Content parameter of the primitive to be sent with the string. If the input attribute is present and is a resource identifier, the Hosting CSE retrieves the resource and overwrites the Content parameter of the primitive to be sent with the retrieved resource representation. If the input attribute is present and is a URI of a resource attribute , the Hosting CSE retrieves the resource attribute and overwrites the Content parameter of the primitive to be sent with the value of the given resource attribute. If the input attribute is not present, the Content parameter of the primitive to be sent remains populated as indicated by the actionPrimitive attribute.

Step 7:	The Hosting CSE sends the composed primitive request to the object resource.

Step 8:	After receiving the response of the request from step 7, the Hosting CSE stores the response primitive in the actionResult attribute.  If the outputResourceID attribute of the <action> resource is set, the Hosting CSE  also stores the response primitive in the resource identified by outputResourceID.

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

#### 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".

Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- flexContainerInstance;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | subscription, transaction, dependency | CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction, state | 9.6.61 |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | subscription, transaction | action | 9.6.62 |
| … | … | … | … | … |

### -----------------------End of change 3-------------------------------------------