| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #47 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com Lu Liu, Convida, liu.lu@convidawireless.com |
| Date:* | 2020-10-04 |
| Reason for Change/s:* | Add <action> and <dependency> resources |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0093 – Action Triggering Enhancements MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.2.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds support for the <action> and <dependency> resources introduced in TS-0001 for Release 4.

R02:

- Changed dependencies attribute to NP on Create

- Add validation check that <dependency> resources are child resources of <action> resource

R03:

- Restrict evalCriteria to be only applicable to attributes having a simple data type.

- Defined a oneM2M “any simple data type”

- Added a validation check for evalCriteria to check that the referenced attribute and the threshold value are of the same data type.

- Removed string match operator since it overlaps with equal operator

R04:

- Backed out proposal to add oneM2M “any simple data type”

- Reworked evalCriteria element definitions

- Updated text for validation checks in CRUD procedures for <action> and <dependency> resources

- Changed actionResult from RW to RO (i.e. NP on UPDATE and CREATE)

R05:

- Removed the term “legal”

- Made corrections/updates to the validation checks based on comments received from others

### -----------------------Start of change 1-------------------------------------------

### 7.4.XX	Resource Type <action>

#### 7.4. XX.1	Introduction

The <action> resource defines an operation (i.e. a oneM2M primitive) performed by the Hosting CSE on a target resource when criteria defined by the <action> resource have been met.

Table 7.4.XX.1-1: Data type definition of <action> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| action | CDT-action-v4_XX_0.xsd |  |

Table 7.4.XX.1-2: Universal/Common Attributes of <action> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |
| holder | O | O |

Table 7.4.XX.1-3: Resource Specific Attributes of <action> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| actionPriority | O | O | xs:positiveInteger |  |
| subjectResourceID | O | O | xs:anyURI |  |
| evalCriteria | M | O | m2m:evalCriteria |  |
| evalMode | M | O | m2m:evalMode |  |
| evalControlParam | O | O | xs:positiveInteger |  |
| dependencies | NP | O | m2m:listOfURIs |  |
| objectResourceID | M | O | xs:anyURI |  |
| actionPrimitive | M | O | m2m:requestPrimitive |  |
| input | O | O | m2m:actionInput |  |
| outputResourceID | M | O | xs:anyURI |  |
| actionResult | NP | NP | m2m:responsePrimitive |  |

Table 7.4.XX.1-4: Child Resources of <action> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <dependency> | [variable] | 0..1 | Clause 7.4.xx |

#### 7.4. XX.2	<action> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <action> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

The following are changes to the Originator procedures described in clause Error: Reference source not found.

Orig-1.0 When composing a request primitive, the Originator shall include within the <action> resource representation contained within the Content of the request primitive, an actionPrimitve attribute containing a oneM2M request primitive.  This request primitive in the actionPrimitve attribute shall be formatted to comply with the rules defined in the procedures described in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check the existence and accessibility of the resources referenced by the subjectResourceID,  objectResourceID and outputResourceID  attributes.  If any of the referenced resources do not exist or are not accessible by the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check if the From parameter contained in the actionPrimitve attribute of the received <action> resource is equal to the Originator of the received request primitive.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check that if the value of the evalMode attribute received in the request is “off” or “once”, then the evalControlParam attribute is not present in the request. If present, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check that the attribute referenced by the subject element of the evalCriteria attribute is an attribute of the resource type referenced by the subjectResourceID attribute. If it is not, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check that the value provided for the threshold element of the evalCriteria attribute is within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. The Receiver shall also check that the value provided for the operator element of the evalCriteria attribute is a valid value based on Table 6.3.4.2.x-5. If either check fails, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- Process the <action> resource as described in clause 10.2.21 of oneM2M TS-0001 [Error: Reference source not found] after Recv-6.7.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

Originator:

The following are changes to the Originator procedures described in clause Error: Reference source not found.

Orig-1.0 When composing a request primitive, the Originator may include within the <action> resource representation contained within the Content of the request primitive, an actionPrimitve attribute containing a oneM2M request primitive.  If present, this request primitive in the actionPrimitve attribute shall be formatted to comply with the rules defined in the procedures described in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check the existence and accessibility of the resources referenced by the subjectResourceID, dependencies, objectResourceID and outputResourceID  attributes.  If any of the referenced resources do not exist or are not accessible by the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check that any <dependency> resources referenced by the dependencies attribute are child resources of the <action> resource.  If any are not child resources, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check if the From parameter contained in the actionPrimitve attribute of the received <action> resource is equal to the Originator of the received request primitive.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- If the evalMode attribute is present in the request and has a value of “off” or “once”, the Receiver shall check that the evalControlParam attribute is not present in the request, or has a value of NULL. If the evalControlParam attribute is not present in the request, then the Receiver shall also check that the value of the  evalControlParam attribute, within the <action> resource that is the target of the request, has a value of “off” or “once”.  If any of these checks fail, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- If the evalCriteria attribute is present in the request, the Receiver shall check that the attribute referenced by the subject element of the evalCriteria attribute is an attribute of the resource type referenced by the subjectResourceID attribute. If it is not, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- If the evalCriteria attribute is present in the request, the Receiver shall check the value provided for the threshold element of the evalCriteria attribute is within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. The Receiver shall also check that the value provided for the operator element of the evalCriteria attribute is a valid value based on Table 6.3.4.2.x-6. If either check fails, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- Process the <action> resource as described in clause 10.2.21 of oneM2M TS-0001 [Error: Reference source not found] after Recv-6.7.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### 7.4.XX	Resource Type <dependency>

#### 7.4. XX.1	Introduction

The <dependency> resource represents conditions that determine if the action specified in the parent <action> resource is triggered. One or more <dependency> resources may exist to provide complex events for triggering. The <dependency> resources provide conditions that always need to be checked before the action is triggered.

Table 7.4.XX.1-7: Data type definition of <dependency> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| dependency | CDT-dependency-v4_XX_0.xsd |  |

Table 7.4.XX.1-8: Universal/Common Attributes of <dependency> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| creator | O | NP |
| holder | O | O |

Table 7.4.XX.1-9: Resource Specific Attributes of <dependency> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| logic | M | O | m2m:logic |  |
| evalCriteria | M | O | m2m:evalCriteria |  |
| dependentResourceID | M | O | xs:anyURI |  |

Table 7.4.XX.1-10: Child Resources of <dependency> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

#### 7.4. XX.2	<dependency> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <dependency> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check the existence and accessibility of the dependent resource defined in the dependentResourceID attribute.  If the resource does not exist or is not accessible by the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check that the attribute referenced by the subject element of the evalCriteria attribute is an attribute of the resource type referenced by the dependentResourceID attribute. If it is not, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- The Receiver shall check that the value provided for the threshold element of the evalCriteria attribute is within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. The Receiver shall also check that the value provided for the operator element of the evalCriteria attribute is a valid value based on Table 6.3.4.2.x-11. If either check fails, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- Process the <dependency> resource as described in clause 10.2.21 of oneM2M TS-0001 [Error: Reference source not found] after Recv-6.7.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- If the evalCriteria attribute is present in the request, the Receiver shall check that the attribute referenced by the subject element of the evalCriteria attribute is an attribute of the resource type referenced by the dependentResourceID attribute. If it is not, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

- If the evalCriteria attribute is present in the request, the Receiver shall check the value provided for the threshold element of the evalCriteria attribute is within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. The Receiver shall also check that the value provided for the operator element of the evalCriteria attribute is a valid value based on Table 6.3.4.2.x-12. If either check fails, the receiver shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error

- .

- Process the <dependency> resource as described in clause 10.2.21 of oneM2M TS-0001 [Error: Reference source not found] after Recv-6.7.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 6.3.4	oneM2M enumerated data types

#### 6.3.4.2	Enumeration type definitions

##### 6.3.4.2.x	m2m:evalCriteriaOperator

Used for the evalCriteria attribute of the <action> and <dependency> resources.

Table 6.3.4.2.x-13: Interpretation of evalCriteriaOperator

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | equal | May be used for evalCriteria with subjects having any of the simple data types defined in Table 6.3.2-1 and 6.3.3-1 or enumerated types defined in clause 6.3.4. When the subject has a simple data type that is a list, then ‘equal’ shall be interpreted as the subject and threshold of the evalCriteria having the same exact number, order and value of elements in their respective lists. |
| 2 | not equal |  |
| 3 | greater than | Shall only be used for evalCriteria with subjects having atomic numeric simple data types including enumerated types (e.g. xs:decimal, xs:float, xs:long, xs:double, xs:duration, xs:integer, xs:nonNegativeInteger, xs:positiveInteger, xs:unsignedInteger, xs:unsignedLong, xs:unsignedInt, xs:unsignedShort or m2m:timestamp) or unions thereof. |
| 4 | less than |  |
| 5 | greater than or equal |  |
| 6 | less than or equal |  |
| NOTE: |  |  |

##### 6.3.4.2.x	m2m:evalMode

Used for the evalMode attribute of the <action> resource.

Table 6.3.4.2.x-14: Interpretation of evalMode

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | off |  |
| 1 | once |  |
| 2 | periodic |  |
| 3 | continuous |  |
| NOTE: |  |  |

##### 6.3.4.2.x	m2m:logic

Used for the logic attribute of the <dependency> resource.

Table 6.3.4.2.x-15: Interpretation of logic

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | SUFFICIENT |  |
| 2 | NECESSARY |  |
| NOTE: |  |  |

### 6.3.5	Complex data types

#### 6.3.5.x	m2m:evalCriteria

Used for the evalCriteria attribute of the <action> and <dependency> resources.

Table 6.3.5.x-16: Type Definition of m2m:evalCriteria

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| operator | m2m:evalCriteriaOperator | 1 |  |
| subject | xs:NCName | 1 | The value provided for the threshold shall be within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. |
| threshold | xs:anySimpleType | 1 |  |

#### 6.3.5.x	m2m:actionInput

Used for the input attribute of the <action> resource.

Table 6.3.5.x-17: Type Definition of m2m:actionInput

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| contentString | xs:NCName | 0..1 | These elements are mutually exclusive from one another. Exactly one of these elements shall be present. |
| resourceID | xs:anyURI | 0..1 |  |
| resourceAttributeID | xs:anyURI | 0..1 |  |

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

##### 6.3.4.2.1	m2m:resourceType

Table 6.3.4.2.1-18: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | Container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | Delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 45 | localMulticastGroup |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | action |  |
| XX | dependency |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 100XX | actionAnnc |  |
| 100XX | dependencyAnnc |  |
| NOTE: See clause Error: Reference source not found. |  |  |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

##### 6.3.4.2.11	m2m:memberType

Used for the memberType attribute of the  <group> resource.

Table 6.3.4.2.11-19: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | mixed | A mixture of all the resource types (except mixed itself). |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| XX | action |  |
| XX | dependency |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10034 | dynamicAuthorizationConsultationAnnc |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 100XX | actionAnnc |  |
| 100XX | dependencyAnnc |  |
| 20001 | oldest |  |
| 20002 | latest |  |
| 20003 | mashup |  |
| NOTE: See clause Error: Reference source not found "Resource Type group". |  |  |

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

### 6.5.4	announceableResource

#### 6.5.4.1	Description

This type definition includes the universal and common attributes used by oneM2M resource types that are capable of being announced. In addition to the attributes of a regularResource, it includes (as optional) the common attributes that are used by the announcement mechanism.

#### 6.5.4.2	Reference

See Error: Reference source not found.

#### 6.5.4.3	Usage

This type is used by the following resource types:

<AE>, <container>, <group>, <locationPolicy>, <node>, <remoteCSE>, <semanticDescriptor>, <timeSeries>, <ontologyRepository>, <ontology>, <ontologyMapping>, <ontologyMappingAlgorithm>, <ontologyMappingAlgorithmRepository>, <semanticMashupJobProfile>, <semanticMashupInstance>, <semanticMashupResult>, <multimediaSession>, <schedule>, <semanticRuleRepository>, <reasoningRules>, <reasoningJobInstance>, <action>, <dependency>.

It is also used by the specializations of <mgmtObj>.

### 6.5.5	announcedResource

#### 6.5.5.1	Description

This type definition includes the universal and common attributes used by a resource that is announcing an announceable resource. In addition to the attributes of a regularResource, it includes (as optional) the link common attribute.

#### 6.5.5.2	Reference

See Error: Reference source not found.

#### 6.5.5.3	Usage

This type is used by the following resource types:

<AEAnnc>, <containerAnnc>, <groupAnnc>, <locationPolicyAnnc>, <nodeAnnc>, <remoteCSEAnnc>, <semanticDescriptorAnnc>, <[flexContainer]Annc>, <timeSeriesAnnc>, <ontologyRepositoryAnnc>, <ontologyAnnc>, <ontologyMappingAnnc>, <ontologyMappingAlgorithmAnnc>, <ontologyMappingAlgorithmRepositoryAnnc>, <semanticMashupJobProfileAnnc>, <semanticMashupInstanceAnnc>, <semanticMashupResultAnne>, <multimediaSessionAnnc>,<scheduleAnnc>, <semanticRuleRepositoryAnnc>, <reasoningRulesAnnc>, <reasoningJobInstanceAnnc>, <actionAnnc>, <dependencyAnnc>.

It is also used by the xxxAnnc variants of the <mgmtObj> specializations.

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

…

Table 8.2.3-20: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| actionPriority | action | apy |
| subjectResourceID | action | sri |
| evalCriteria | action. dependency | evc |
| evalMode | action | evm |
| evalControlParam | action | ecp |
| evalPriority | action | epy |
| objectResourceID | action | orc |
| actionPrimitive | action | apv |
| input | action | ipu |
| outputResourceID | action | opu |
| actionResult | action | air |
| logic | dependency | lgc |
| dependentResourceID | dependency | dpr |

### -----------------------End of change 6-------------------------------------------

### -----------------------Start of change 7-------------------------------------------

### 8.2.4	Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-21.

Table 8.2.4-21: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| … | … |
| action | actr |
| dependency | depr |

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-22: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| operator | evalCriteria | optr |
| subject | evalCriteria | sbjt |
| threshold | evalCriteria | thld |
| contentString | actionInput | cstr |
| resourceID | actionInput | rsid |
| resourceAttributeID | actionInput | raid |

### -----------------------End of change 7-------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 7.4.26	Announced resource types

#### 7.4.26.1	Introduction

Instances of certain resource types can be announced by the Hosting CSE to one or more remote CSEs to inform the remote CSEs of the existence of the original resource instance. Table 7.4.26.1-23 lists the resource types which are announceable together with its resource type identifier and the name of the XSD file that specifies its data type.

Resource-specific attributes of a resource type may be declared announceable (mandatory or optional) or not announceable. Clause 9.6 in oneM2M TS-0001 [Error: Reference source not found] defines which attributes of each resource type are announceable.

The universal/common attributes of an announced resource type are listed in Table 7.4.26.1-24, together with their request optionality in Create and Update requests (see clause 7.4.1). The announced resource includes a common attribute link which represents a link to the original resource hosted by the original resource-Hosting CSE.

The child resources applicable to each announced resource type are listed in Table 9.6.26.1-1 of oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.26.1-1: Data type definition of announced Resource types

| Data Type ID | File Name | Note |
| --- | --- | --- |
| accessControlPolicyAnnc | CDT-accessControlPolicy-v4_0_0.xsd |  |
| remoteCSEAnnc | CDT-remoteCSE-v4_0_0.xsd |  |
| AEAnnc | CDT-AE-v4_0_0.xsd |  |
| containerAnnc | CDT-container-v4_0_0.xsd |  |
| [flexContainer]Annc | See Annex J and oneM2M TS-0023 [Error: Reference source not found] | Announced variants of flexContainer specializations |
| contentInstanceAnnc | CDT-contentInstance-v4_0_0.xsd |  |
| scheduleAnnc | CDT-schedule-v4_0_0.xsd |  |
| locationPolicyAnnc | CDT-locationPolicy-v4_0_0.xsd |  |
| groupAnnc | CDT-group-v4_0_0.xsd |  |
| nodeAnnc | CDT-node-v4_0_0.xsd |  |
| semanticDescriptorAnnc | CDT-semanticDescriptor-v4_0_0.xsd |  |
| timeSeriesAnnc | CDT-timeSeries-v4_0_0.xsd |  |
| timeSeriesInstanceAnnc | CDT-timeSeriesInstance-v4_0_0.xsd |  |
| ontologyRepositoryAnnc | CDT-ontologyRepository-v4_0_0.xsd |  |
| ontologyAnnc | CDT-ontology-v4_0_0.xsd |  |
| semanticMashupJobProfileAnnc | CDT-semanticMashupJobProfile-v4_0_0.xsd |  |
| semanticMashupInstanceAnnc | CDT-semanticMashupInstance-v4_0_0.xsd |  |
| semanticMashupResultAnnc | CDT-semanticMashupResult-v4_0_0.xsd |  |
| genericInterworkingServiceAnnc | CDT-genericInterworkingService-v4_0_0.xsd |  |
| genericInterworkingOperationInstanceAnnc | CDT-genericInterworkingOperationInstance-v4_0_0.xsd |  |
| svcObjWrapperAnnc | CDT-svcObjWrapper-v4_0_0.xsd |  |
| svcFwWrapperAnnc | CDT-svcFwWrapper-v4_0_0.xsd |  |
| allJoynAppAnnc | CDT-allJoynApp-v4_0_0.xsd |  |
| allJoynSvcObjectAnnc | CDT-allJoynSvcObject-v4_0_0.xsd |  |
| allJoynInterfaceAnnc | CDT-allJoynInterface-v4_0_0.xsd |  |
| allJoynMethodAnnc | CDT-allJoynMethod-v4_0_0.xsd |  |
| allJoynMethodCallAnnc | CDT-allJoynMethodCall-v4_0_0.xsd |  |
| allJoynPropertyAnnc | CDT-allJoynProperty-v4_0_0.xsd |  |
| multimediaSessionAnnc | CDT-multimediaSession-v4_0_0.xsd |  |
| ontologyMappingAnnc | CDT-ontologyMapping-v4_0_0.xsd |  |
| ontologyMappingAlgorithmAnnc | CDT-ontologyMappingAlgorithm-v4_0_0.xsd |  |
| ontologyMappingAlgorithmRepositoryAnnc | CDT-ontologyMappingAlgorithmRepository-v4_0_0.xsd |  |
| semanticRuleRepositoryAnnc | CDT-semanticRuleRepository-v4_0_0.xsd |  |
| reasoningRulesAnnc | CDT-reasoningRules-v4_0_0.xsd |  |
| reasoningJobInstanceAnnc | CDT-reasoningJobInstance-v4_0_0.xsd |  |
| actionAnnc | CDT-action-v4_0_0.xsd |  |
| dependencyAnnc | CDT-dependency-v4_0_0.xsd |  |

### -----------------------End of change 8-------------------------------------------