| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #48 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com |
| Date:* | 2020-12-10 |
| Reason for Change/s:* | Add notification recording attributes to <subscription> and <crossResourceSubscription> resources |
| CR against: Release* | Release 4 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.3.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds support for the notification recording feature introduced into the <subscription> and <crossResourceSubscription> resources for Rel-4.

### -----------------------Start of change 1-------------------------------------------

### 7.4.8	Resource Type <subscription>

#### 7.4.8.1	Introduction

The <subscription> resource contains subscription information for its subscribed-to resource. The subscription resource is a child of the subscribed to resource.

The detailed description can be found in clause 9.6.8 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.8.1-1: Data type definition of <subscription> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| subscription | CDT-subscription.xsd |  |

Table 7.4.8.1-2: Universal/Common Attributes of <subscription> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| holder | O | O |
| creator | O | NP |

Table 7.4.8.1-3: Resource Specific Attributes of <subscription> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| eventNotificationCriteria | O | O | m2m:eventNotificationCriteria | Default behaviour is notification on Update_of_Resource |
| expirationCounter | O | O | xs:positiveInteger | No default |
| notificationURI | M | O | list of xs:anyURI | No default In this value, it may contain notification serialization type (i.e. xml, json, cbor) per target. This shall be applied only for the URL formatted target (c.f. resource ID). When the type is set, only one type indication shall be appended in the target as the key-value format with delimiter "?",. If the value already contains "?" character for application queries, the type information shall be appended with "&". The key shall be "ct" (content serialization type). Note that this serialization type is in lower cases. Examples: http://mydomain/notificationHandler?ct=json http://mydomain/notificationHandler?q=true&ct=json |
| groupID | O | O | xs:anyURI | No default |
| notificationForwardingURI | O | O | xs:anyURI | No default |
| batchNotify | O | O | m2m:batchNotify | No default |
| rateLimit | O | O | m2m:rateLimit | No default |
| preSubscriptionNotify | O | NP | xs:positiveInteger | No default |
| pendingNotification | O | O | m2m:pendingNotification | No default |
| notificationStoragePriority | O | O | xs:positiveInteger | No default |
| latestNotify | O | O | xs:boolean | No default |
| notificationContentType | O | O | m2m:notificationContentType | See TS-0001 [Error: Reference source not found]. Table 9.6.8-4. |
| notificationEventCat | O | O | m2m:eventCat | No default |
| subscriberURI | O | NP | xs:anyURI | No default |
| associatedCrossResourceSub | O | O | m2m:listOfURIs | No default. This attribute shall only be modified by a <crossResourceSubscription> Hosting CSE. described in clause 7.4.58 |
| notifStatType | O | O | m2m:notifStatType | No default |
| notifStatInfo | NP | NP | m2m:setOfNotifStatInfo | No default |

Table 7.4.8.1-4: Reference of child resources

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <schedule> | notificationSchedule | 0..1 | Clause Error: Reference source not found |
| <notificationTargetMgmtPolicyRef> | [variable] | 0..n | Clause Error: Reference source not found |
| <notificationTargetSelfReference> | ntsr | 1 | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

#### 7.4.8.2	<subscription> resource specific procedures for CRUD operations

##### 7.4.8.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures (Error: Reference source not found in clause Error: Reference source not found). The additional procedures shall be inserted from Recv-6.2 to Recv-6.5 as below.

Recv-6.3 The following step is in addition to the procedures defined in clause 7.3.3.15:

Check if the Originator has privileges for retrieving the subscribed-to resource. If the Originator does not have the privilege, the Hosting CSE shall return the response primitive with Response Status Code indicating "ORIGINATOR_HAS_NO_PRIVILEGE" error.

Recv-6.4 The following steps are in addition to the procedures defined in clause 7.3.3.3:

- Check if the subscribed-to resource, addressed in the To parameter in the Request, is subscribable. Subscribable resource types are defined in TS-0001 [Error: Reference source not found]; they have <subscription> resource types as their child resources. If it is not subscribable, the Hosting CSE shall return the Notify response primitive with a Response Status Code indicating "TARGET_NOT_SUBSCRIBABLE" error instead of the Response Status Code "INVALID_CHILD_RESOURCE_TYPE".

- Check if the notificationEventType is set to "Blocking_Update".

- If the subscribed-to resource already has a subscription with this notificationEventType the Hosting CSE shall return the response primitive with Response Status Code indicating "BLOCKING_SUBSCRIPTION_ALREADY_EXISTS" error if more than one notification of this type could be sent.

- If there is more than one notificationURI specified, the Hosting CSE shall return the response primitive with Response Status Code indicating "BAD_REQUEST" error.

- If any resource-specific attributes of the <subscription> resource other than eventNotificationCriteria or notificationURI are specified the Hosting CSE shall return the primitive with Response Status Code indicating "BAD_REQUEST" error.

- If any condition tag of the eventNotificationCriteria attribute other than attribute condition tag is specified, the Hosting CSE shall return the response primitive with Response Status Code indicating "BAD_REQUEST" error.

- If any of the notificationURI entries are not the Originator, the Hosting CSE may send a Subscription Verification request primitive to each of them as described in clause 7.5.1.2.3.

a)	If the Hosting CSE cannot send one or more Subscription Verification request primitives, the Hosting CSE shall return the Create <subscription> response primitive with a Response Status Code indicating "SUBSCRIPTION_VERIFICATION_INITIATION_FAILED" error.

b)	If the Hosting CSE sent all the Subscription Verification request primitives, the Hosting CSE shall check if each Notify response primitive contains a Response Status Code indicating "OK". If not, the Hosting CSE shall return the Create <subscription> response primitive containing the Response Status Code indicating "SUBSCRIPTION_VERIFICATION_INITIATION_FAILED" error.

- If the associatedCrossResourceSub is provided, check that the Hosting CSE ID value in the associatedCrossResourceSub is the same as the From parameter of the request. If not, return the response primitive with a Response Status Code indicating "BAD_REQUEST".

- If the Originator provides a value of childResourceType which is not a valid child of the subscribed-to resource, the request shall be rejected with a “BAD_REQUEST” Response Status Code.

- If the Originator provides missingData, check that subscribed-to resource is of type <timeSeries>. If not, the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If both the notificationEventType and operationMonitor are present in the Request, the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If the notificationContentType is invalid for a given operation (refer to oneM2M TS-0001 [Error: Reference source not found] Table 9.6.8-4: Default and allowed values of notificationContentType) the request shall be rejected with a "BAD_REQUEST" Response Status Code.

Recv-6.5: The following steps are in addition to the procedures defined in clause 7.3.3.5:

- If the Originator does not provide notificationContentType, the Hosting CSE shall set it according to the default shown in oneM2M TS-0001 [Error: Reference source not found] Table 9.6.8-4: Default and allowed values of notificationContentType.

- If the notificationURI is not the Originator, the Hosting CSE shall set the Originator’s ID as the <subscription> resource's creator attribute.

- If the batchNotify attribute is present in the Request but batchNotify/duration is not provided by the Originator, the Hosting CSE shall set the value of batchNotify/duration to the default duration as given by the M2M Service Provider.

- Once the <subscription> resource is created, the Hosting CSE shall start recording notification statistics in the notifStatInfo attribute if the notifStatType attribute is configured.

##### 7.4.8.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.8.2.3	Update

Originator:

The following change from the generic procedures in clause Error: Reference source not found.

Orig-1.0: The originator shall not specify notificationEventType set to "Blocking_Update".

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures in clause Error: Reference source not found.

Recv-6.4: The following steps are in addition to the procedures defined in clause 7.3.3.4:

- Check if the notificationEventType in the request is set to "Blocking_Update". If so, the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If the Originator provides a value of childResourceType which is not a valid child of the subscribed-to resource, the request shall be rejected with a “BAD_REQUEST” Response Status Code.

- If the Originator provides missingData, check that the subscribed-to resource is of type <timeSeries>. If not, the request shall be rejected with a "BAD_REQUEST" Response Status Code.

- If the UPDATE operation would result in both operationMonitor and notificationEventType being present in the resource, the request shall be rejected with a “BAD_REQUEST” Response Status Code.

- Check if a new associatedCrossResourceSub is provided. If so, check that the Hosting CSE ID value in the associatedCrossResourceSub is the same as the From parameter of the request.

- If the notificationContentType is invalid for a given operation (refer to oneM2M TS-0001 [Error: Reference source not found] Table 9.6.8-4: Default and allowed values of notificationContentType) the request shall be rejected with a “BAD_REQUEST” Response Status Code.

Recv-6.5. The following step is in addition to the procedures defined in clause 7.3.3.7:

- If a <crossResourceSubscription> Hosting CSE ID is removed from associatedCrossResourceSub, the Hosting CSE shall send a Notify request for Subscription Deletion, using the procedures in clause 7.5.1.2.4, to the <crossResourceSubscription> Hosting CSE.

- If the notifStatType attribute in the resource is configured and the notifStatType attribute in the request is set to NULL, the Hosting CSE shall delete the notifStatType attribute and stop collecting notification statistics for the <subscription> resource. The Hosting CSE shall maintain the current value of the notifStatInfo attribute until a subsequent update of the notifStatType attribute to a value other than NULL is received.

- If the notifStatType attribute is not present in the resource and the notifStatType attribute in the request is set a value other than NULL, the Hosting CSE shall update the value of the notifStatType attribute in the resource, delete any values stored in the notifStatInfo attribute of the resource and then start recording notification statistics.

- If the notifStatType attribute in the resource is configured and the notifStatType attribute in the request is set to a value that is different, the Hosting CSE shall reject the request and return the response primitive with Response Status Code indicating "BAD_REQUEST" error.

##### 7.4.8.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are changes to the receiver procedures described in clause Error: Reference source not found.

Recv-6.5. The Hosting CSE shall send a Notify request for Subscription Deletion using the procedures in clause 7.5.1.2.4 to all <crossResourceSubscription> hosting CSEs indicated in associatedCrossResourceSub.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 7.4.58	Resource Type <crossResourceSubscription>

#### 7.4.58.1	Introduction

The <crossResourceSubscription> resource is used to support cross-resource subscription and cross-resource notification. A cross-resource subscription is made on multiple target resources. A cross-resource notification shall be generated if and only if expected events on all target resources occur within a designated time window. When notifications from all target resources occur within a specified time window the Hosting CSE shall issue a cross-resource notification.

The detailed description can be found in clause 9.6.58 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.58.1-5: Data type definition of <crossResourceSubscription> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| crossResourceSubscription | CDT-crossResourceSubscription.xsd |  |

Table 7.4.58.1-6: Universal/Common Attributes of <crossResourceSubscription> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| holder | O | O |
| creator | M | NP |

Table 7.4.58.1-7: Resource Specific Attributes of <crossResourceSubscription> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| regularResourcesAsTarget | O | O | m2m:listOfURIs | No default |
| subscriptionResourcesAsTarget | O | O | m2m:listOfURIs | No default |
| timeWindowType | M | O | m2m:timeWindowType | PERIODICWINDOW |
| timeWindowSize | M | O | xs:duration | No default |
| eventNotificationCriteriaSet | O | O | m2m:eventNotificationCriteriaSet | Default behaviour is notification on Update_of_Resource |
| notificationEventCat | O | O | m2m:eventCat | No default |
| expirationCounter | O | O | xs:positiveInteger | No default |
| notificationURI | M | O | list of xs:anyURI | No default |
| subscriberURI | O | NP | m2m:ID | No default |
| notifStatType | O | O | m2m:notifStatType | No default |
| notifStatInfo | NP | NP | m2m:setOfNotifStatInfo | No default |

Table 7.4.58.1-8: Child Resources of <crossResourceSubscription> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <notificationTargetMgmtPolicyRef> | [variable] | 0..n | Clause 7.4.30 |
| <notificationTargetSelfReference> | ntsr | 1 | Clause 7.4.33 |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <schedule> | notificationSchedule | 0..1 | Clause 7.4.9 |

#### 7.4.58.2	<crossResourceSubscription> resource specific procedures for CRUD operations

##### 7.4.58.2.0	Introduction

This clause describes <crossResourceSubscription> resource specific primitive behaviour for CRUD operations.

##### 7.4.58.2.1	Create

Originator:

The following are changes to the Originator procedures described in clause Error: Reference source not found:

- Orig-1.0 When composing a request primitive, the Originator shall include regularResourcesAsTarget and/or subscriptionResourcesAsTarget attributes in the resource representation of the <crossResourceSubscription> in the content of the primitive. If regularResourcesAsTarget attribute is included, eventNotificationCriteriaSet attribute shall be included. If eventNotificationCriteriaSet contains only one eventNotificationCriteria, this eventNotificationCriteria shall be applied to all regular resources included in regularResourcesAsTarget attribute; otherwise, eventNotificationCriteriaSet shall contain the same number of eventNotificationCriteria elements as the number of regular target resources contained in regularResourcesAsTarget and each eventNotificationCriteria element shall be sequentially applied to corresponding target resource as listed in the regularResourcesAsTarget.

Receiver:

The following are changes to the receiver procedures described in clause Error: Reference source not found:

- Recv-6.5: The following steps are in addition to the generic Create procedures defined in clause Error: Reference source not found:

a)	The request shall be rejected with a "BAD_REQUEST" Response Status Code if at least one of regularResourcesAsTarget or subscriptionResourcesAsTarget attributes is not present in the request.

b)	If regularResourcesAsTarget is included, the Hosting CSE shall send a CREATE <subscription> request message to each target resource indicated by regularResourcesAsTarget.

i)	In the new CREATE <subscription> request, the receiver shall use the From of the current CREATE request. For this <subscription> to be created:

1)	eventNotificationCriteria attribute shall use the corresponding entry included in eventNotificationCriteriaSet attribute of the <crossResourceSubscription> resource representation

2)	notificationURI attribute shall be set to the resource identifier of this <crossResourceSubscription> resource being created.

3)	associatedCrossResourceSub attribute shall be set to the resource identifier of this <crossResourceSubscription> resource being created.

4)	notificationEventCat attribute shall be set to the same value in the <crossResourceSubscription> resource representation.

ii)	If any <subscription> for a target resource cannot be successfully created, the receiver shall send an unsuccessful response with a "CROSS_RESOURCE_OPERATION_FAILURE" Response Status Code to the Originator; the receiver shall also delete already created <subscription> resources at other target resources that were created based on the presence of regularResourcesAsTarget.

c)	If subscriptionResourcesAsTarget is included, the Hosting CSE shall add the resource identifier of this <crossResourceSubscription> resource to the associatedCrossResourceSub attribute of each <subscription> resource indicated in subscriptionResourcesAsTarget by issuing an UPDATE request to the <subscription> resource host.

iii)	In the UPDATE request, the receiver shall use the From parameter from the current CREATE request.

iv)	notificationURI attribute shall be updated to include the resource identifier of this <crossResourceSubscription> resource being created.

v)	If any <subscription> for a target resource cannot be successfully updated, the receiver shall send an unsuccessful response with a "CROSS_RESOURCE_OPERATION_FAILURE" Response Status Code  to the Originator; the Hosting CSE shall also remove itself from any already successfully associated <subscription> resources using the procedures in clause 7.4.8.2.4 and also delete any already-created <subscription> resources at other target resources.

d)	Once the <crossResourceSubscription> resource is created, the Hosting CSE shall start the time window if the timeWindowType=PERIODICWINDOW; if timeWindowType=SLIDINGWINDOW, the Hosting CSE shall start the time window after the first notification is received from a Target Resource Hosting CSE.

e) Once the <crossResourceSubscription> resource is created, the Hosting CSE shall start recording notification statistics in the notifStatInfo attribute if the notifStatType attribute is configured.

##### 7.4.58.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.58.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are changes to the receiver procedures described in clause Error: Reference source not found.

- Recv-6.5: The following steps are in addition to the generic Update procedures defined in clause Error: Reference source not found:

a)	If regularResourcesAsTarget is updated, the Hosting CSE shall perform the following tasks:

i)	If a target resource has been removed in the new regularResourcesAsTarget attribute value delete the associated <subscription> child resource using the procedure in clause 7.4.8.2.4.

ii)	If the updated regularResourcesAsTarget attribute value contains new target resources, the Hosting CSE shall send a Create <subscription> request message to each new target resource as described in clause 7.4.58.2.1.

b)	If subscriptionResourcesAsTarget is updated, the Hosting CSE shall perform the following tasks:

iii)	If a <subscription> resource has been removed in the new subscriptionResourcesAsTarget attribute value the Hosting CSE shall update the <subscription> resource using the procedure in clause 7.4.8.2.4 to remove this <crossResourceSubscription> from the <subscription> resource's associatedCrossResourceSub attribute.

iv)	If the updated subscriptionResourcesAsTarget attribute value contains new a <subscription>resource, the Hosting CSE shall add the resource identifier of this <crossResourceSubscription> resource to the associatedCrossResourceSub attribute of each <subscription> resource indicated in subscriptionResourcesAsTarget as described in clause 7.4.58.2.1.

c)	If eventNotificationCriteriaSet is updated, the Hosting CSE shall update the eventNotificationCriteria of each previously created <subscription> child resource of the targets listed in the regularResourcesAsTarget attribute to reflect the received eventNotificationCriteria content using the procedures in clause 7.4.8.2.3.

d)	If timeWindowSize or timeWindowType is updated in the resource representation the receiver shall restart the timer as described in clause 7.4.58.2.1.

e) If any of the above Update procedures are unsuccessful the receiver shall send an unsuccessful response with a "CROSS_RESOURCE_OPERATION_FAILURE" Response Status Code to the Originator; the receiver shall also restore all resources to the states they were in prior to this request.

f) 	If the notifStatType attribute in the resource is configured and the notifStatType attribute in the request is set to NULL, the Hosting CSE shall delete the notifStatType attribute and stop collecting notification statistics for the <crossResourceSubsription> resource. The Hosting CSE shall maintain the current value of the notifStatInfo attribute until a subsequent update of the notifStatType attribute to a value other than NULL is received.

g) 	If the notifStatType attribute is not present in the resource and the notifStatType attribute in the request is set a value other than NULL, the Hosting CSE shall update the value of the notifStatType attribute in the resource, delete any values stored in the notifStatInfo attribute of the resource and then start recording notification statistics.

h)	If the notifStatType attribute in the resource is configured and the notifStatType attribute in the request is set to a value that is different, the Hosting CSE shall reject the request and return the response primitive with Response Status Code indicating "BAD_REQUEST" error.

##### 7.4.58.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

The following are changes to the receiver procedures described in clause Error: Reference source not found:

- Recv-6.5: The following steps are in addition to the generic Delete procedures defined in clause Error: Reference source not found:

a)	The Hosting CSE shall delete the previously created <subscription> child resource of each target resource indicated in the regularResourcesAsTarget attribute. The Receiver shall use the From of the current request for these requests.

b)	The Hosting CSE shall UPDATE the <subscription> resource of each target resource indicated in the subscriptionResourcesAsTarget attribute using the procedure in clause 7.4.8.2.3 to remove the resource identifier of this <crossResourceSubscription> from the <subscription> resource's associatedCrossResourceSub attribute. The Receiver shall use the From of the current request for these requests.

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 6.3.4	oneM2M enumerated data types

#### 6.3.4.2	Enumeration type definitions

##### 6.3.4.2.x	m2m:notifStatType

Used for the notifStatType attribute of the <subscription> and <crossResourceSubscription> resources.

Table 6.3.4.2.x-9: Interpretation of notifStatType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | NUM_NOTIF_REQS_SENT_TO_NOTIF_TGT |  |
| 2 | NUM_NOTIF_RESPS_RCVD_FROM_NOTIF_TGT |  |
| NOTE: Combinations of these values are specified by adding them together. For example, the value 3 is interpreted as "NUM_NOTIF_REQS_SENT_TO_NOTIF_TGT and NUM_NOTIF_RESPS_RCVD_FROM_NOTIF_TGT". |  |  |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

### 6.3.5	Complex data types

#### 6.3.5.x	m2m:notifStats

Used for the notifStatInfo attribute of the <subscription> and <crossResourceSubscription> resources.

Table 6.3.5.x-10: Type Definition of m2m:notifStats

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| numNotifRequestsSent | xs:nonNegativeInteger | 0..1 |  |
| numNotifResponsesReceived | xs:nonNegativeInteger | 0..1 |  |

#### 6.3.5.x	m2m:setOfNotifStatInfo

Used for the notifStatInfo attribute of the <subscription> and <crossResourceSubscription> resources.

Table 6.3.5.x-11: Type Definition of m2m:SetOfNotifStatInfo

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| notifStatInfo | (anonymous) | 1..n |  |
| notifStatInfo/notifTarget | xs:anyURI | 1 |  |
| notifStatInfo/notifStats | m2m:notifStats | 1 |  |

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

…

Table 8.2.3-12: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| notifStatType | subscription, crossResourceSubscription | nst |
| notifStatInfo | subscription, crossResourceSubscription | nsi |

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-13: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| notifTarget | setOfNotifStatInfo | ntgt |
| notifStats | setOfNotifStatInfo | nsts |
| numNotifRequestsSent | notifStats | nnrs |
| numNotifResponsesReceived | notifStats | nnrr |

### -----------------------End of change 6-------------------------------------------