| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 48 |
| Source:* | Bob Flynn, Exacta, bob.flynn@exactagss.com |
| Date:* | 2020-11-30 |
| Reason for Change/s:* |  |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V 2.27.0 |
| Clauses * | 7.4.22.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The <pollingChannelURI> virtual resource does not allow CREATE, UPDATE or DELETE operations over Mca/Mcc.

TS-0004 does not specify the RSC to use if it is attempted.

Other Virtual resources have this structure:

Originator:

The <latest> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order:

- "Create an unsuccessful Response primitive" with the Response Status Code indicating an "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

This structure is implemented in this CR.

### -----------------------Start of change 1-------------------------------------------

#### 7.4.22.2	<pollingChannelURI> resource specific procedures for CRUD operations

##### 7.4.22.2.0	Introduction

This clause describes <pollingChannelURI> resource specific behaviour for the Retrieve operation as a service layer long polling request. CUDN requests to the <pollingChannelURI> resource shall be rejected.

##### 7.4.22.2.1	Create

Originator:

The <pollingChannelURI> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order:

- "Create an unsuccessful Response primitive" with the Response Status Code indicating an "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

7.4.22.2.2	Retrieve

Originator:

Shall execute Originator actions in clause Error: Reference source not found as a service layer long polling request. It is the Originator's responsibility to initiate this procedure after it gets long polling response either successful or unsuccessful. The Originator shall send this Retrieve request as blocking request (clause 8.2.1 in oneM2M TS-0001 [Error: Reference source not found]).

Receiver:

Shall execute the following steps in order and these are modifications to the generic procedure from Recv-6.3 to Recv-6.5 in clause Error: Reference source not found:

- Recv-6.3:

- Check if the request Originator is the creator of the parent <pollingChannel> resource. If it is not the creator, the Hosting CSE shall send response primitive with a Response Status Code indicating "ORIGINATOR_HAS_NO_PRIVILEGE" error.

- Recv-6.4:

- No change from the generic procedure.

- Recv-6.5:

- If there is a pending request(s) to be sent to the Originator;

- Create a Response primitive by setting the Content parameter with one of the pending request(s).

- Else:

- Wait for a request for the Originator until the Request Expiration Timestamp from the long polling request. If a request is available before the Request Expiration Timestamp timeout, create a Response primitive by setting the Content parameter to the received pending request. Otherwise, create a response primitive with a Response Status Code indicating "REQUEST_TIMEOUT" error.

##### 7.4.22.2.3	Update

Originator:

The <pollingChannelURI> resource shall not be updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order:

- "Create an unsuccessful Response primitive" with the Response Status Code indicating an "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

7.4.22.2.4	Delete

Originator:

The <pollingChannelURI> resource shall not be deleted via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order:

- "Create an unsuccessful Response primitive" with the Response Status Code indicating an "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

7.4.22.2.5	Notify

Request/response delivery mechanism via <pollingChannelURI> resource is depicted in the TS-0001 [Error: Reference source not found], figure 10.2.13-1 (Request/response delivery via polling channel). In this procedure, the Originator of this Notify request is the Target AE/CSE and the Receiver is the <pollingChannelURI> Hosting CSE, respectively.

Originator:

This procedure follows the procedure specified in clause Error: Reference source not found, with the following <pollingChannelURI> resource-specific updates.

Additional primitive specific operation on Orig-1.0:

- The Originator shall generate and populate a Notify request as described in clause Error: Reference source not found.

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures procedure from Recv-6.3 to Recv-6.5 in clause Error: Reference source not found:

- Recv-6.3:

- Check if the request Originator is the creator of the parent <pollingChannel> resource. If it is not the creator, the Hosting CSE shall send response primitive with a Response Status Code indicating "ORIGINATOR_HAS_NO_PRIVILEGE" error.

- Recv-6.4:

- No change from the generic procedure.

- Recv-6.5:

Forward  the response (step 006 in Figure 10.2.13-1), which was contained in the Content parameter of the Notify request, to the entity that sent the associated request to the Hosting CSE (Originator in the figure). The associated request is the request that the Hosting CSE received and forwarded to the Registree CSE over the polling channel (step 002 and step 004 in the figure). The association shall be done by matching the Request Identifier parameter of the request delivered in <pollingChannelURI> Retrieve response (step 004 in the figure) and the Request Identifier parameter of the response delivered in the Content parameter in a <pollingChannelURI> Notify request (step 005 in the figure).

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### -----------------------End of change 3---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?