| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #49 |
| Source:* | Rahul Kumar, krahul@cdot.in Poornima, poornima@cdot.in Anupama Chopra(anupama@cdot.in) |
| Date:* | 2020-02-01 |
| Reason for Change/s:* | Changes for CSE-ID attribute in <remoteCSE> resource |
| CR against: Release* | V4.4.0 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004, V4.4.0 |
| Clauses * | 7.4.4 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The CR proposes is corresponding to CR .. of TS-0001

Following changes are proposed:

- Request optionality of CSE-ID attribute in <remoteCSE> resource is changed to NP.

- Handling for setting value of CSE-ID is added in <remoteCSE> CREATE procedure.

### -----------------------Start of change 1-------------------------------------------

### 7.4.4	Resource Type <remoteCSE>

#### 7.4.4.1	Introduction

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> of the Registrar CSE.

In addition each registered CSE shall have a <remoteCSE> resource representing its Registrar CSE. This is located directly under the registered CSE's <CSEBase>.

The detailed description can be found in clause 9.6.4 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.4.1-1: Data type definition of <remoteCSE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| remoteCSE | CDT-remoteCSE.xsd |  |

Table 7.4.4.1-2: Universal/Common Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| holder | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| location | O | O |

Table 7.4.4.1-3: Resource Specific Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| cseType | O | NP | m2m:cseTypeID | No default |
| pointOfAccess | O | O | m2m:poaList | No default |
| CSEBase | M | NP | xs:anyURI | No default |
| CSE-ID | NP | NP | m2m:ID | No default |
| M2M-Ext-ID | O | O | m2m:externalID | No default |
| Trigger-Recipient-ID | O | O | m2m:triggerRecipientID | No default |
| requestReachability | M | O | xs:boolean | No default |
| nodeLink | O | O | xs:anyURI | No default |
| triggerReferenceNumber | O | O | xs:unsignedInt | No default |
| contentSerialization | O | O | m2m:serializations | No default |
| e2eSecInfo | O | O | m2m:e2eSecInfo | No default |
| descendantCSEs | O | O | m2m:listOfM2MID | No default |
| supportedReleaseVersions | M | O | m2m:supportedReleaseVersions | No default |
| multicastCapability | O | O | m2m:multicastCapability | No default |
| externalGroupID | O | O | m2m:externalID | No default |
| triggerEnable | O | O | xs:boolean | false |
| activityPatternElements | O | O | m2m:activityPatternElements | No default |
| enableTimeCompensation | O | O | xs:boolean | false |

Table 7.4.4.1-4: Child resources of <remoteCSE> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <container> | [variable] | 0..n | Clause Error: Reference source not found |
| <containerAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainerAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <group> | [variable] | 0..n | Clause Error: Reference source not found |
| <groupAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicy> | [variable] | 0..n | Clause Error: Reference source not found |
| <accessControlPolicyAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <pollingChannel> | [variable] | 0..1 | Clause Error: Reference source not found |
| <nodeAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <dynamicAuthorizationConsultation> | [variable] | 0..n | Clause Error: Reference source not found |
| <flexContainer> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeries> | [variable] | 0..n | Clause Error: Reference source not found |
| <timeSeriesAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <remoteCSEAnnc> | [variable] | 0..n | Clause |
| <AEAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <locationPolicyAnnc> | [variable] | 0..n | Clause Error: Reference source not found |
| <ontologyRepositoryAnnc> | [variable] | 0..1 | Clause 7.4.46 |
| <semanticMashupJobProfile> | [variable] | 0..n | Clause 7.4.49 |
| <semanticMashupJobProfileAnnc> | [variable] | 0..n | Clause 7.4.49 |
| <semanticMashupInstance> | [variable] | 0..n | Clause 7.4.50 |
| <semanticMashupInstanceAnnc> | [variable] | 0..n | Clause 7.4.50 |
| <crossResourceSubscription> | [variable] | 0..n | Clause 7.4.58 |
| <transactionMgmt> | [variable] | 0..n | Clause 7.4.60 |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <e2eQosSession> | [variable] | 0..n | Clause 7.4.69 |
| <timeSyncBeacon> | [variable] | 0..n | Clause 7.4.70 |
| <timeSyncBeaconAnnc> | [variable] | 0..n | Clause 7.4.70 |
| <nwMonitoringReq> | [variable] | 0..n | Clause 7.4.71 |

#### 7.4.4.2	<remoteCSE> resource specific procedures for CRUD operations

##### 7.4.4.2.0	Introduction

The entire CSE registration procedure including <remoteCSE> resource creation procedure below is defined in clause 10.1.1.2.1 in oneM2M TS-0001 [Error: Reference source not found] ("CSE registration procedure").

##### 7.4.4.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

- An AE shall not originate a Create <remoteCSE> resource request.

- The Originator, upon receipt of successful CREATE response message, shall create a <remoteCSE> resource locally. It shall populate this resource with all the mandatory attributes for a <remoteCSE>.

- The Originator may issue a RETRIEVE request to the registrar <CSEBase> to fetch additional information about the registrar CSE and add this information to its local <remoteCSE> resource. The Originator may choose which optional attributes to include in its <remoteCSE> resource.

Receiver:

Same as the generic procedures in clause Error: Reference source not found except replacement of Recv-6.3 step with following:

- The Hosting CSE shall check if the credential provided by the Originator is valid.

- If the check fails, then the Hosting CSE shall return a response primitive with a Response Status Code indicating "SECURITY_ASSOCIATION_REQUIRED" error.

- And addition of the following to step Recv-6.4:

- If the Hosting CSE is an ASN-CSE, then it shall reject the request with Response Status Code indicating an “OPERATION_NOT_ALLOWED" error.

- The Hosting CSE shall check for the presence of any resources having a CSE-ID that matches the one specified in the From parameter in the request and that have the same parent as the new resource being created.

- If such a resource exists, then the Hosting CSE shall reject the request with a Response Status Code indicating "ORIGINATOR_HAS_ALREADY_REGISTERED" error.

- And addition of the following to step Recv-6.5:

- The Receiver CSE shall assign a value equal to the value carried in the From request parameter to CSE-ID attribute.

- If the Receiver CSE has registered to another CSE, the Receiver CSE shall send an update request to its Registrar CSE to add the CSE-IDs of the Originator CSE and the Originator CSE's descendants into the descendantCSEs attribute of the Receiver CSE's <remoteCSE> hosted by the Registrar CSE.

- And addition of new steps following step Recv-6.11:

- If the enableTimeCompensation attribute within the request is set to true and the Registrar CSE does not support time compensation functionality, the Registrar CSE shall reject the request with the "NOT_IMPLEMENTED" Response Status Code.

- If the Registrar CSE supports time compensation functionality and the enableTimeCompensation attribute of the <remoteCSE> resource is set to true, the Registrar CSE shall start performing time compensation upon the Registree CSE’s requests and responses as defined in clause 10.2.24.3 of oneM2M TS-0001 [6].

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?