| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #50 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com |
| Date:* | 2021-05-08 |
| Reason for Change/s:* | Add <state> and <processManagement> resources |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0093 – Action Triggering Enhancements MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.5.0 |
| Clauses * | Various |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR adds support for the <state> and <processManagement> resources introduced in TS-0001 for Release 4.

R01:

- Changed stateStatus to stateActive and defined it as a Boolean

R02:

- Add error response code “INVALID_PROCESS_CONFIGURATION”

### -----------------------Start of change 1-------------------------------------------

### 7.4.XX	Resource Type <state>

#### 7.4. XX.1	Introduction

A <state> resource is used to define an individual state within a process defined by the parent <processManagement> resource. A <state> resource includes status information maintained for the state and actions performed by the Hosting CSE when a process is in the state. A <state> resource also defines criteria used by a Hosting CSE for transitioning to the next <state> resource in a process.

Table 7.4.XX.1-1: Data type definition of <state> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| state | CDT-state-v4_XX_0.xsd |  |

Table 7.4.XX.1-2: Universal/Common Attributes of <state> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| holder | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| creator | O | NP |

Table 7.4.XX.1-3: Resource Specific Attributes of <state> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| stateActive | NP | NP | xs:boolean | False |
| stateAction | NP | O | xs:anyURI | No default, must reference a child <action> resource |
| stateTransitions | O | O | m2m:listOfStateTransitions | No default |

Table 7.4.XX.1-4: Child Resources of <state> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <action> | [variable] | 0..1 | Clause 7.4.73 |

#### 7.4. XX.2	<state> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <state> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check if the processStatus attribute of the parent <processManagement> resource is equal to “Disabled”.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- The Receiver shall check the existence and accessibility of any <state> resources referenced by the stateTransitions attribute of the request.  If any of the referenced <state> child resources do not exist, do not have retrieve privileges for the Originator or are not child resources of the parent <processManagement> resource, then the Receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" error.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check if the processStatus attribute of the parent <processManagement> resource is equal to “Disabled”.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error

- If the request is attempting to update the stateTransitions attribute, the Receiver shall check if after processing the request, the value of the stateActive attribute of the <state> resource will equal “False”.  If not, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- The Receiver shall check the existence and accessibility of an <action> child resource, if any, referenced by the stateAction attribute of the request.  If the referenced <action> child resource does not exist, does not have retrieve privileges for the Originator, or is not a child resource of this <state> resource, then the Receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" error.

- The Receiver shall check the existence and accessibility of any <state> resources referenced by the stateTransitions attribute of the request.  If any of the referenced <state> child resources do not exist, do not have retrieve privileges for the Originator or are not child resources of the parent <processManagement> resource, then the Receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" error.

- For any evalCriteria defined in the stateTransitions attribute of the request, the Receiver shall check the existence and accessibility of any resource and attribute referenced by the subject element of the evalCriteria. If any of the referenced resources or attributes do not exist or do not have retrieve privileges for the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" error.

- For any evalCriteria defined in the stateTransitions attribute of the request, the Receiver shall check the value provided for the threshold element of the evalCriteria attribute is within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. The Receiver shall also check that the value provided for the operator element of the evalCriteria attribute is a valid value based on Table 6.3.4.2.86-5. If either check fails, the receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" error.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check if the stateActive attribute of the <state> resource is equal to “False”.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- The Receiver shall check if the processStatus attribute of the parent <processManagement> resource is equal to “Disabled”.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

### 7.4.XX	Resource Type <processManagement>

#### 7.4. XX.1	Introduction

A <processManagement> resource defines a process that a Hosting CSE manages on behalf of an AE and that consists of one or more states. Each state in a process is represented using a <state> resource defined in clause 7.4.XX.

Table 7.4.XX.1-6: Data type definition of <processManagement> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| processManagement | CDT-processManagement-v4_XX_0.xsd |  |

Table 7.4.XX.1-7: Universal/Common Attributes of <processManagement> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| holder | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |
| announceSyncType | O | O |
| creator | O | NP |

Table 7.4.XX.1-8: Resource Specific Attributes of <processManagement> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| processStatus | NP | NP | m2m:processStatus | Disabled |
| processControl | NP | O | m2m:processControl | No default |
| currentState | NP | NP | xs:anyURI | No default |
| activateConditions | NP | O | m2m:listOfEvalCriteria | No default |
| endConditions | NP | O | m2m:listOfEvalCriteria | No default |
| initialState | NP | O | xs:anyURI | No default |

Table 7.4.XX.1-9: Child Resources of <processManagement> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |
| <state> | [variable] | 0..n | Clause 7.4.XX |

#### 7.4. XX.2	<processManagement> resource specific procedures for CRUD operations

##### 7.4.XX.2.0	Introduction

This clause describes <processManagement> resource specific primitive behaviour for CRUD operations.

##### 7.4.XX.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

- The Hosting CSE shall set the processStatus attribute to a value of “Disabled” and the currentState attribute to a value of NULL.

##### 7.4.XX.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.XX.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- If the request is attempting to update the initialState, activateConditions or endConditions attributes and the value of the processStatus attribute of the <processManagement> resource is not equal to “Disabled”,  the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- For any evalCriteria defined in the activateConditions or endConditions attributes of the request, the Receiver shall check the existence and accessibility of any resource and attribute referenced by the subject element of the evalCriteria. If any of the referenced resources or attributes do not exist or do not have retrieve privileges for the Originator, then the Receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" error.

- For any evalCriteria defined in the activateConditions or endConditions attributes of the request, the Receiver shall check the value provided for the threshold element of the evalCriteria attribute is within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. The Receiver shall also check that the value provided for the operator element of the evalCriteria attribute is a valid value based on Table 6.3.4.2.86-10. If either check fails, the receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION " error.

- If the request is attempting to update the initialState attribute, the Receiver shall check the existence and accessibility of the referenced <state> resource. If this <state> resource does not exist, does not have retrieve privileges for the Originator or is not a child resource of the parent <processManagement> resource, then the Receiver shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION " error.

- If the value of the processControl attribute in the request is equal to “Enable” and the value of the processStatus attribute of the <processManagement> resource is not equal to “Disabled”, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- If the value of the processControl attribute in the request is equal to “Disable” and the value of the processStatus attribute of the <processManagement> resource is equal to “Disabled”, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- If the value of the processControl attribute in the request is equal to “Pause” and the value of the processStatus attribute of the <processManagement> resource is not equal to “Activated”, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- If the value of the processControl attribute in the request is equal to “Reactivate” and the value of the processStatus attribute of the <processManagement> resource is not equal to “Paused”, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- If the value of the processControl attribute in the request is equal to “Enable”, the Hosting CSE shall check that initialState attribute of the <processManagement> resource references a <state> resource that exists, is a child resource of the <processManagement> resource and has retrieve privileges for the Originator. The Hosting CSE shall also check that the <state> and <action> resources referenced by the <state> and <action> children of this process exist, have the proper create, retrieve, update and delete privileges for the Originator such that the Hosting CSE can perform the process on behalf of the Originator, and these resources are all descendent resources of the <processManagement> resource.  If these checks are successful, the Hosting CSE shall update the value of the processStatus attribute to "Enabled". The Hosting CSE shall then start to monitor and manage the process as defined in clause 10.2.27 of oneM2M TS-0001 [Error: Reference source not found]. If these checks are not successful, the Hosting CSE shall return a response primitive with a Response Status Code indicating "INVALID_PROCESS_CONFIGURATION" erro".

- If the value of the processControl attribute in the request is equal to “Pause” and the value of the processStatus attribute of the <processManagement> resource is equal to “Activated”, the Hosting CSE shall set the processStatus attribute to "Paused" and pause the processing of the process.

- If the value of the processControl attribute in the request is equal to “Reactivate” and the value of the processStatus attribute of the <processManagement> resource is equal to “Paused”, the Hosting CSE shall set the processStatus attribute to "Activated" and continue processing of the process.

- If the value of the processControl attribute in the request is equal to “Disable”, the Hosting CSE shall set the processStatus attribute to "Disabled" and the currentState attribute to NULL.   The Hosting CSE shall also set the stateActive attributes of the current child <state> resource to "False". The Hosting CSE shall also stop monitoring the criteria defined in the endConditions attribute.

##### 7.4.XX.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found with the following exceptions:

- The Receiver shall check if the processStatus attribute of the <processManagement> resource is equal to “Disabled”.  If it is not, the receiver shall return a response primitive with a Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

### 6.3.4	oneM2M enumerated data types

#### 6.3.4.2	Enumeration type definitions

…

##### 6.3.4.2.XX	m2m:processStatus

Used for the processStatus attribute of the <processManagement> resource.

Table 6.3.4.2.XX-11: Interpretation of processStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Disabled |  |
| 2 | Enabled |  |
| 3 | Activated |  |
| 4 | Paused |  |
| 5 | Completed |  |
| 6 | Aborted |  |

##### 6.3.4.2.XX	m2m:processControl

Used for the processControl attribute of the <processManagement> resource.

Table 6.3.4.2.XX-12: Interpretation of processControl

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Enable |  |
| 2 | Disable |  |
| 3 | Pause |  |
| 4 | Reactivate |  |

### 6.3.5	Complex data types

…

#### 6.3.5.77	m2m:evalCriteria

Used for the evalCriteria attribute of the <action> and <dependency> resources, the stateTransitions attribute of the <state> resource and the activateConditions and endConditions attributes of the <processManagement> resource.

Table 6.3.5.77-13: Type Definition of m2m:evalCriteria

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| operator | m2m:evalCriteriaOperator | 1 |  |
| subject | xs:NCName | 1 | The value provided for the threshold shall be within the value space (as defined in [3]) of the data type of the subject element of the evalCriteria attribute. |
| threshold | xs:anySimpleType | 1 |  |

…

#### 6.3.5.XX	m2m:listOfStateTransitions

Used for the stateTransitions attribute of the <state> resource.

Table 6.3.5.XX-14: Type Definition of m2m:listOfStateTransitions

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| stateTransition | m2m:stateTransition | 1..n |  |

#### 6.3.5.XX	m2m:stateTransition

Used for the stateTransitions attribute of the <state> resource.

Table 6.3.5.XX-15: Type Definition of m2m:stateTransition

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| evalCriteria | m2m:evalCriteria | 1 |  |
| nextState | xs:anyURI | 1 |  |

#### 6.3.5.XX	m2m:listOfEvalCriteria

Used for the activateConditions and endConditions attributes of the <processManagement> resource.

Table 6.3.5.XX-16: Type Definition of m2m:listOfEvalCriteria

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| evalCriteria | m2m:evalCriteria | 1..n |  |

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

##### 6.3.4.2.1	m2m:resourceType

Table 6.3.4.2.1-17: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 45 | localMulticastGroup |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| 59 | e2eQosSession |  |
| 60 | timeSyncBeacon |  |
| 61 | nwMonitoringReq |  |
| 62 | primitiveProfile |  |
| 63 | action |  |
| 64 | dependency |  |
| XX | state |  |
| XX | processManagement |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10005 | CSEBaseAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 10060 | timeSyncBeaconAnnc |  |
| 10062 | primitiveProfileAnnc |  |
| 10063 | actionAnnc |  |
| 10064 | dependencyAnnc |  |
| 100XX | stateAnnc |  |
| 100XX | processManagementAnnc |  |
| NOTE: See clause Error: Reference source not found. |  |  |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

##### 6.3.4.2.11	m2m:memberType

Used for the memberType attribute of the  <group> resource.

Table 6.3.4.2.11-18: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | mixed | A mixture of all the resource types (except mixed itself). |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 24 | semanticDescriptor |  |
| 25 | notificationTargetMgmtPolicyRef |  |
| 26 | notificationTargetPolicy |  |
| 27 | policyDeletionRules |  |
| 28 | flexContainer |  |
| 29 | timeSeries |  |
| 30 | timeSeriesInstance |  |
| 31 | role |  |
| 32 | token |  |
| 33 | void |  |
| 34 | dynamicAuthorizationConsultation |  |
| 35 | authorizationDecision |  |
| 36 | authorizationPolicy |  |
| 37 | authorizationInformation |  |
| 38 | ontologyRepository |  |
| 39 | ontology |  |
| 40 | semanticMashupJobProfile |  |
| 41 | semanticMashupInstance |  |
| 42 | semanticMashupResult |  |
| 43 | AEContactList |  |
| 44 | AEContactListPerCSE |  |
| 46 | multimediaSession |  |
| 47 | triggerRequest |  |
| 48 | crossResourceSubscription |  |
| 49 | backgroundDataTransfer |  |
| 50 | transactionMgmt |  |
| 51 | transaction |  |
| 52 | ontologyMapping |  |
| 53 | ontologyMappingAlgorithm |  |
| 54 | ontologyMappingAlgorithmRepository |  |
| 55 | reasoningJobInstance |  |
| 56 | reasoningRules |  |
| 57 | semanticRuleRepository |  |
| 58 | flexContainerInstance |  |
| 59 | e2eQosSession |  |
| 60 | timeSyncBeacon |  |
| 61 | nwMonitoringReq |  |
| 62 | primitiveProfile |  |
| 63 | action |  |
| 64 | dependency |  |
| XX | state |  |
| XX | processManagement |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 10024 | semanticDescriptorAnnc |  |
| 10028 | flexContainerAnnc |  |
| 10029 | timeSeriesAnnc |  |
| 10030 | timeSeriesInstanceAnnc |  |
| 10033 | void |  |
| 10038 | ontologyRepositoryAnnc |  |
| 10039 | ontologyAnnc |  |
| 10040 | semanticMashupJobProfileAnnc |  |
| 10041 | semanticMashupInstanceAnnc |  |
| 10042 | semanticMashupResultAnnc |  |
| 10046 | multimediaSessionAnnc |  |
| 10052 | ontologyMappingAnnc |  |
| 10053 | ontologyMappingAlgorithmAnnc |  |
| 10054 | ontologyMappingAlgorithmRepositoryAnnc |  |
| 10055 | reasoningJobInstanceAnnc |  |
| 10056 | reasoningRulesAnnc |  |
| 10057 | semanticRuleRepositoryAnnc |  |
| 10060 | timeSyncBeaconAnnc |  |
| 10062 | primitiveProfileAnnc |  |
| 10063 | actionAnnc |  |
| 10064 | dependencyAnnc |  |
| 100XX | stateAnnc |  |
| 100XX | processManagementAnnc |  |
| 20001 | oldest |  |
| 20002 | latest |  |
| 20003 | mashup |  |
| NOTE: See clause Error: Reference source not found "Resource Type group". |  |  |

### -----------------------End of change 4-------------------------------------------

### -----------------------Start of change 5-------------------------------------------

### 6.5.4	announceableResource

#### 6.5.4.1	Description

This type definition includes the universal and common attributes used by oneM2M resource types that are capable of being announced. In addition to the attributes of a regularResource, it includes (as optional) the common attributes that are used by the announcement mechanism.

#### 6.5.4.2	Reference

See Error: Reference source not found.

#### 6.5.4.3	Usage

This type is used by the following resource types:

<AE>, <container>, <group>, <locationPolicy>, <node>, <remoteCSE>, <semanticDescriptor>, <timeSeries>, <ontologyRepository>, <ontology>, <ontologyMapping>, <ontologyMappingAlgorithm>, <ontologyMappingAlgorithmRepository>, <semanticMashupJobProfile>, <semanticMashupInstance>, <semanticMashupResult>, <multimediaSession>, <schedule>, <semanticRuleRepository>, <reasoningRules>, <reasoningJobInstance>, <timeSyncBeacon>, <primitiveProfile>, <action>, <dependency>, <state>, <processManagement>.

It is also used by the specializations of <mgmtObj>.

### 6.5.5	announcedResource

#### 6.5.5.1	Description

This type definition includes the universal and common attributes used by a resource that is announcing an announceable resource. In addition to the attributes of a regularResource, it includes (as optional) the link common attribute.

#### 6.5.5.2	Reference

See Error: Reference source not found.

#### 6.5.5.3	Usage

This type is used by the following resource types:

<AEAnnc>, <containerAnnc>,<CSEBaseAnnc>,  <groupAnnc>, <locationPolicyAnnc>, <nodeAnnc>, <remoteCSEAnnc>, <semanticDescriptorAnnc>, <[flexContainer]Annc>, <timeSeriesAnnc>, <ontologyRepositoryAnnc>, <ontologyAnnc>, <ontologyMappingAnnc>, <ontologyMappingAlgorithmAnnc>, <ontologyMappingAlgorithmRepositoryAnnc>, <semanticMashupJobProfileAnnc>, <semanticMashupInstanceAnnc>, <semanticMashupResultAnne>, <multimediaSessionAnnc>,<scheduleAnnc>, <semanticRuleRepositoryAnnc>, <reasoningRulesAnnc>, <reasoningJobInstanceAnnc>, <timeSyncBeaconAnnc>, <primitiveProfileAnnc>, <actionAnnc>, <dependencyAnnc>, <stateAnnc>, <processManagementAnnc>.

It is also used by the xxxAnnc variants of the <mgmtObj> specializations.

### -----------------------End of change 5-------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

…

Table 8.2.3-19: Resource attribute short names (6/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| stateActive | state | sact |
| stateAction | state | stac |
| stateTransitions | state | sttrs |
| processStatus | processManagement | prst |
| processControl | processManagement | prct |
| currentState | processManagement | cust |
| activateConditions | processManagement | atcos |
| endConditions | processManagement | encos |
| initialState | processManagement | inst |

### -----------------------End of change 6-------------------------------------------

### -----------------------Start of change 7-------------------------------------------

### 8.2.4	Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-20.

Table 8.2.4-20: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| … | … |
| state | stte |
| processManagement | prmr |

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-21: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| … | … | … |
| evalCriteria | stateTransition, listOfEvalCriteria | evc* |
| nextState | stateTransition | nxst |
| stateTransition | listOfStateTransitions | sttr |

### -----------------------End of change 7-------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 7.4.26	Announced resource types

#### 7.4.26.1	Introduction

Instances of certain resource types can be announced by the Hosting CSE to one or more remote CSEs to inform the remote CSEs of the existence of the original resource instance. Table 7.4.26.1-22 lists the resource types which are announceable together with its resource type identifier and the name of the XSD file that specifies its data type.

Resource-specific attributes of a resource type may be declared announceable (mandatory or optional) or not announceable. Clause 9.6 in oneM2M TS-0001 [Error: Reference source not found] defines which attributes of each resource type are announceable.

The universal/common attributes of an announced resource type are listed in Table 7.4.26.1-23, together with their request optionality in Create and Update requests (see clause 7.4.1). The announced resource includes a common attribute link which represents a link to the original resource hosted by the original resource-Hosting CSE.

The child resources applicable to each announced resource type are listed in Table 9.6.26.1-1 of oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.26.1-1: Data type definition of announced Resource types

| Data Type ID | File Name | Note |
| --- | --- | --- |
| CSEBaseAnnc | CDT-CSEBase.xsd |  |
| accessControlPolicyAnnc | CDT-accessControlPolicy.xsd |  |
| remoteCSEAnnc | CDT-remoteCSE.xsd |  |
| AEAnnc | CDT-AE.xsd |  |
| containerAnnc | CDT-container.xsd |  |
| [flexContainer]Annc | See Annex J and oneM2M TS-0023 [Error: Reference source not found] | Announced variants of flexContainer specializations |
| contentInstanceAnnc | CDT-contentInstance.xsd |  |
| scheduleAnnc | CDT-schedule.xsd |  |
| locationPolicyAnnc | CDT-locationPolicy.xsd |  |
| groupAnnc | CDT-group.xsd |  |
| nodeAnnc | CDT-node.xsd |  |
| semanticDescriptorAnnc | CDT-semanticDescriptor.xsd |  |
| timeSeriesAnnc | CDT-timeSeries.xsd |  |
| timeSeriesInstanceAnnc | CDT-timeSeriesInstance.xsd |  |
| ontologyRepositoryAnnc | CDT-ontologyRepository.xsd |  |
| ontologyAnnc | CDT-ontology.xsd |  |
| semanticMashupJobProfileAnnc | CDT-semanticMashupJobProfile.xsd |  |
| semanticMashupInstanceAnnc | CDT-semanticMashupInstance.xsd |  |
| semanticMashupResultAnnc | CDT-semanticMashupResult.xsd |  |
| genericInterworkingServiceAnnc | CDT-genericInterworkingService.xsd |  |
| genericInterworkingOperationInstanceAnnc | CDT-genericInterworkingOperationInstance.xsd |  |
| svcObjWrapperAnnc | CDT-svcObjWrapper.xsd |  |
| svcFwWrapperAnnc | CDT-svcFwWrapper.xsd |  |
| allJoynAppAnnc | CDT-allJoynApp.xsd |  |
| allJoynSvcObjectAnnc | CDT-allJoynSvcObject.xsd |  |
| allJoynInterfaceAnnc | CDT-allJoynInterface.xsd |  |
| allJoynMethodAnnc | CDT-allJoynMethod.xsd |  |
| allJoynMethodCallAnnc | CDT-allJoynMethodCall.xsd |  |
| allJoynPropertyAnnc | CDT-allJoynProperty.xsd |  |
| multimediaSessionAnnc | CDT-multimediaSession.xsd |  |
| ontologyMappingAnnc | CDT-ontologyMapping.xsd |  |
| ontologyMappingAlgorithmAnnc | CDT-ontologyMappingAlgorithm.xsd |  |
| ontologyMappingAlgorithmRepositoryAnnc | CDT-ontologyMappingAlgorithmRepository.xsd |  |
| semanticRuleRepositoryAnnc | CDT-semanticRuleRepository.xsd |  |
| reasoningRulesAnnc | CDT-reasoningRules.xsd |  |
| reasoningJobInstanceAnnc | CDT-reasoningJobInstance.xsd |  |
| timeSyncBeaconAnnc | CDT-timeSyncBeacon.xsd |  |
| primitiveProfileAnnc | CDT-primitiveProfile.xsd |  |
| actionAnnc | CDT-action.xsd |  |
| dependencyAnnc | CDT-dependency.xsd |  |
| stateAnnc | CDT-state.xsd |  |
| processManagementAnnc | CDT-processManagement.xsd |  |

### -----------------------End of change 8-------------------------------------------

### -----------------------Start of change 9-------------------------------------------

#### 6.6.3.5	Originator error response class

Table 6.6.3.5-1 specifies the RSCs for Originator error responses.

41xx codes are oneM2M specific.

Table 6.6.3.5-24: RSCs for Originator error response class

| Numeric Code | Description |
| --- | --- |
| 4000 | BAD_REQUEST |
| 4001 | RELEASE_VERSION_NOT_SUPPORTED |
| 4004 | NOT_FOUND |
| 4005 | OPERATION_NOT_ALLOWED |
| 4008 | REQUEST_TIMEOUT |
| 4015 | UNSUPPORTED_MEDIA_TYPE |
| 4101 | SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE |
| 4102 | CONTENTS_UNACCEPTABLE |
| 4103 | ORIGINATOR_HAS_NO_PRIVILEGE |
| 4104 | GROUP_REQUEST_IDENTIFIER_EXISTS |
| 4105 | CONFLICT |
| 4106 | ORIGINATOR_HAS_NOT_REGISTERED |
| 4107 | SECURITY_ASSOCIATION_REQUIRED |
| 4108 | INVALID_CHILD_RESOURCE_TYPE |
| 4109 | NO_MEMBERS |
| 4110 | GROUP_MEMBER_TYPE_INCONSISTENT |
| 4111 | ESPRIM_UNSUPPORTED_OPTION |
| 4112 | ESPRIM_UNKNOWN_KEY_ID |
| 4113 | ESPRIM_UNKNOWN_ORIG_RAND_ID |
| 4114 | ESPRIM_UNKNOWN_RECV_RAND_ID |
| 4115 | ESPRIM_BAD_MAC |
| 4116 | ESPRIM_IMPERSONATION_ERROR |
| 4117 | ORIGINATOR_HAS_ALREADY_REGISTERED |
| 4118 | ONTOLOGY_NOT_AVAILABLE |
| 4119 | LINKED_SEMANTICS_NOT_AVAILABLE |
| 4120 | INVALID_SEMANTICS |
| 4121 | MASHUP_MEMBER_NOT_FOUND |
| 4122 | INVALID_TRIGGER_PURPOSE |
| 4123 | ILLEGAL_TRANSACTION_STATE_TRANSITION_ATTEMPTED |
| 4124 | BLOCKING_SUBSCRIPTION_ALREADY_EXISTS |
| 4125 | SPECIALIZATION_SCHEMA_NOT_FOUND |
| 4126 | APP_RULE_VALIDATION_FAILED |
| 4127 | OPERATION_DENIED_BY_REMOTE_ENTITY |
| 4128 | SERVICE_SUBSCRIPTION_NOT_ESTABLISHED |
| 4130 | ONTOLOGY_MAPPING_ALGORITHM_NOT_AVAILABLE |
| 4131 | ONTOLOGY_MAPPING_POLICY_NOT_MATCHED |
| 4132 | ONTOLOGY_MAPPING_NOT_AVAILABLE |
| 4133 | BAD_FACT_INPUTS_FOR_REASONING |
| 4134 | BAD_RULE_INPUTS_FOR_REASONING |
| 4135 | DISCOVERY_LIMIT_EXCEEDED |
| 4136 | PRIMITIVE_PROFILE_NOT_ACCESSIBLE |
| 4137 | PRIMITIVE_PROFILE_BAD_REQUEST |
| 41XX | INVALID_PROCESS_CONFIGURATION |

### -----------------------End of change 9-------------------------------------------