| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 50.6 |
| Source:* | Rahul, C-DOT, krahul@cdot.in Sachin, C-DOT, sachin@cdot.in Anupama, C-DOT, anupama@cdot.in |
| Date:* | 2021-07-09 |
| Reason for Change/s:* | See the Introduction |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.6.0 |
| Clauses * | 6.3.4.2 Enumeration type definitions, Table D.5.1-1, CDT-areaNwkInfo-v3_22_0.xsd, CDT-enumerationTypes-v3_22_0.xsd |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR aims to provide an enumeration for various possible values of areaNwkType attribute of <areaNwkInfo> resource. Presently, the value of areaNwkType can be any string whereas this CR aims to bind value of this attribute to a specific set of values only.

This CR aims to close follow up action item ‘A-SDS-39-02’ related to SDS-2019-0050 on areaNwkInfo.

Agenda:

| A-SDS-39-02 * | Follow up action related to SDS-2019-0050 on areaNwkInfo, C-DOT to investigate the concepts of areaNwkInfo and the area network device | C-DOT | Need C-DOT to be present to close this action Dale asked C-DOT to look into this. |
| --- | --- | --- | --- |

TS-0001 reference:

| areaNwkType | 1 | RW | The areaNwkType is a value that indicates the type of M2M Area Network. This attribute is a specialization of [objectAttribute] attribute. |
| --- | --- | --- | --- |

Background:

SDS-2019-0563-enum_areaNwkType_R3-TS0004

SDS-2019-0564-enum_areaNwkType_R2 - TS0004

In these CRs, we had proposed new enum to define set of area network types but

wasn't agreed because of data type issue with one of the field in different different external mgmt technologies.

There was apprehension that making areaNwkType field an int value may cause problems in TR-069/OMA mapping. But there is always an MS interface between Management Server of that external Mgmt technology and Management Adapter at CSE. This interface converts oneM2M request to corresponding mgmt technological specific data model request. So, similarly, this interface can also convert int value into implementation specific String according to TR-069/OMA specifications.

Benefit of keeping this value an INT is that it will help in less payload transmitting over network.

### -----------------------Start of change 1-------------------------------------------

### 6.3.4.2.xx m2m:areaNwkType

This is used for areaNwkType attribute of <areaNwkInfo> resource.

Table 6.3.4.2.68-1: Interpretation of areaNwkType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Zigbee |  |
| 2 | DLMS/COSEM |  |
| 3 | ZWave |  |
| 4 | BACnet |  |
| 5 | ANSI C12 |  |
| 6 | mBus |  |
| 7 | 6LowPAN |  |
| 8 | LoRa |  |
| 9 | Sigfox |  |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

Table D.5.1-2: Resource specific attributes of [areaNwkInfo]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause Error: Reference source not found | 1004 (areaNwkInfo) |
| objectIDs | O | NP | See clause Error: Reference source not found |  |
| objectPaths | O | NP | See clause Error: Reference source not found |  |
| description | O | O | See clause Error: Reference source not found |  |
| areaNwkType | M | O | m2m:areaNwkType |  |
| listOfDevices | O | O | m2m:listOfURIs |  |

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

Changes in CDT-areaNwkInfo-v3_22_0.xsd

<?xml version="1.0" encoding="UTF-8"?>

<!--

Copyright Notification

The oneM2M Partners authorize you to copy this document, provided that you retain all copyright and other proprietary notices

contained in the original materials on any copies of the materials and that you comply strictly with these terms.

This copyright permission does not constitute an endorsement of the products or services, nor does it encompass the granting of

any patent rights. The oneM2M Partners assume no responsibility for errors or omissions in this document.

© 2016, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC). All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand

and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations.

No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE,

GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY

PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS.

NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO

ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES.

oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

-->

<xs:schema xmlns="http://www.w3.org/2001/XMLSchema" targetNamespace="http://www.onem2m.org/xml/protocols"

xmlns:m2m="http://www.onem2m.org/xml/protocols" elementFormDefault="unqualified"

attributeFormDefault="unqualified" xmlns:xs="http://www.w3.org/2001/XMLSchema">

<xs:include schemaLocation="CDT-commonTypes-v3_22_0.xsd" />

<xs:include schemaLocation="CDT-subscription-v3_22_0.xsd" />

<xs:element name="areaNwkInfo" substitutionGroup="m2m:sg_mgmtResource">

<xs:complexType>

<xs:complexContent>

<!-- Inherit Common Attributes from mgmtResource -->

<xs:extension base="m2m:mgmtResource">

<!-- Resource Specific Attributes -->

<xs:sequence>

<xs:element name="areaNwkType" type="m2m:areaNwkType" />

<xs:element name="listOfDevices" type="m2m:listOfURIs" />

<!-- Child Resources -->

<xs:choice minOccurs="0" maxOccurs="1">

<xs:element name="childResource" type="m2m:childResourceRef" maxOccurs="unbounded" />

<xs:element ref="m2m:subscription" maxOccurs="unbounded" />

</xs:choice>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

<xs:element name="areaNwkInfoAnnc" substitutionGroup="m2m:sg_announcedMgmtResource">

<xs:complexType>

<xs:complexContent>

<!-- Inherit Common Attributes from announcedMgmtResource -->

<xs:extension base="m2m:announcedMgmtResource">

<!-- Resource Specific Attributes -->

<xs:sequence>

<xs:element name="areaNwkType" type="xs:string" minOccurs="0" />

<xs:element name="listOfDevices" type="m2m:listOfURIs" minOccurs="0" />

<!-- Child Resources -->

<xs:choice minOccurs="0" maxOccurs="1">

<xs:element name="childResource" type="m2m:childResourceRef" maxOccurs="unbounded" />

<xs:element ref="m2m:subscription" maxOccurs="unbounded" />

</xs:choice>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

### -----------------------End of change 3---------------------------------------------

### -----------------------Start of change 4-------------------------------------------

<xs:simpleType name="geofenceEventCriteria">

<xs:annotation>

<xs:documentation>Used in the geofenceEventCriteria attribute of the locationPolicy resource</xs:documentation>

</xs:annotation>

<xs:restriction base="xs:integer">

<!-- Entering -->

<xs:enumeration value="1" />

<!-- Leaving -->

<xs:enumeration value="2" />

<!-- Inside -->

<xs:enumeration value="3" />

<!-- Outside -->

<xs:enumeration value="4" />

</xs:restriction>

</xs:simpleType>

<xs:simpleType name=”areaNwkType”>

<xs:annotation>

<xs:documentation> This is used for areaNwkType attribute of <areaNwkInfo> resource </xs:documentation>

</xs:annotation>

<xs:restriction base=”xs:integer”>

<!-- Zigbee -->

<xs:enumeration value=”1” />

<!-- DLMS/COSEM -->

<xs:enumeration value=”2” />

<!-- ZWave -->

<xs:enumeration value=”3” />

<!-- BACnet -->

<xs:enumeration value=”4” />

<!-- ANSI C12 -->

<xs:enumeration value=”5” />

<!-- mBus -->

<xs:enumeration value=”6” />

<!-- 6LowPAN -->

<xs:enumeration value=”7” />

<!-- LoRa -->

<xs:enumeration value=”8” />

<!-- Sigfox -->

<xs:enumeration value=”9” />

</xs:restriction>

</xs:simpleType>

</xs:schema>

### -----------------------End of change 4---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?