| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #48 |
| Source:* | Dale Seed, Convida, seed.dale @convidawireless.com |
| Date:* | 2020-11-30 |
| Reason for Change/s:* | Update attribute level ACP support |
| CR against: Release* | Release 4 |
| CR against: WI* | Active - WI-0077 - Attribute Based Access Control Policy MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0003 V4.3.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR updates the attribute level ACP description to align it with the text being added to TS-0004

R02 – Address comments from Peter

- Line 120 "A set acrs of access control rules may consist of a mix of pairs,3-tuples, 4-tuples, 5-tuples or 6-tuples. For pairs, any context parameters associated with a request message are admissible.". Instead of "pairs" it should say "pairs or for any tuples not containing accessControlContexts"

- [Dale] Updated

- Line 140 "Wildcard characters are not applicable to SP domain names, resource-IDs of <group> resources and Role IDs."  I assume that "not applicable to" means "not allowed in" - however this contradicts TS-0004 table 7.3.3.15-1 which says that wildcards are allowed in the SP Domain.

- [Dale] Updated to allow wildcard in M2M-SP-ID

- Line 147,"the Discovery operation is indicated by op = Retrieve in combination with the provisioning of fc and Disrestype parameters in the request message."  This looks wrong: 
i) With Discovery-based operations, the Op can be things other than Retrieve 
ii) Not sure what Disrestype is.  Should this say "Filter Criteria filterUsage is  "Discovery", "Discovery-based Operation " or "IPE On-demand Discovery" ?

- [Dale] Updated

- Line 206.  Same comments as I had on lines 194 and 228 of 0144R06 (see above)

- [Dale] Removed “for which the rule applies” text and replaced “superset” with “union”

- Line 238  This contains words similar to line 169 of TS-0004 (see above)  "1)        If a decision is "Permit" for only a single access control rule included in the privileges (or selfPrivileges) attribute of a single ACP, the result is "Permit". 
1)        Otherwise, the result is "Deny".

- [Dale] Added the text “or there are a set of rules that contain accessControlAttributes that individually satisfy all conditions apart from accessControlAttributes and that collectively as a union satisfy the accessControlAttributes condition”

- Line 250.  Another  place where this kind or problem occurs:

The access decision res_acrs defined in clause 7.1.4 is derived by evaluating whether or not the parameters associated with the request message listed in tables 7.1.2-1 and 7.1.2-2 match any of the access control rules contained in the access control rule set defined in clause 7.1.3 as follows: 
        res_acrs = res_acr(1) OR res_acr(2) ... OR res_acr(k) … OR res_acr(K), 
where res_acr(k) represents the logical evaluation result (i.e. TRUE/FALSE or 1/0) of the request parameters against the kth access control rule 
Trying to modify the algorithm to accommodate what we want to do is tricky, so an alternative is to say that we run the algorithm twice.   In the first pass each rule is considered individually and we only accept a rule with accessControlAttributes if it passes completely - i.e. following the OR.    If this fails to provide a conclusive result we then do a second pass.  This second pass is like an election run-off.  Any rules that failed for conditions other than accessControlAttributes are eliminated and those that are left  (the "applicable rules") are then merged into a single rule, containing the union of the accessControlAttributes from those applicable rules.  I would put the paragraph describing this outside of the description of the Algorithm itself (either at the beginning or the end).

- [Dale] I revised the text for the attribute level checks within the algorithm itself. I then added a separate paragraph after the algorithm which performs an additional set of checks as suggested.

- Line 270. "The third partial logical result res_ctxts(k) "   This should say "The fourth partial logical result.."

- [Dale] Updated

- Line 279. "The fourth partial logical result res_objd(k) "   This should say "The fifth partial logical result.."

- [Dale] Updated

- Line 308. "The fifth partial logical result res_attrs(k) "   This should say "The sixth partial logical result.."

- [Dale] Updated

- Line 310.  "rq_attributes refers to the resource attributes that are in scope for the request. ".   This doesn't define what "in scope" means.  As I understand your intention, you have different definitions for "in scope" depending on what the operation is.

- [Dale] Updated the text

- Lines 314..315.  These words define isMember to ALLOW the access if there's at least one match between the in-scope attributes and the attributes in the rule. That's different from TS-0004. For example: 
a) TS-0004 allows non-partial Retrieve even if there is no match at all (TS-0004 says it returns an empty result) 
b) TS-0004 says that DELETE is not permitted unless ALL the attributes in the resource are included in the rule.

- [Dale] Revamped this part of the contribution to break out each type of operation

- Lines 317 - 324.  This is an attempt to modify the algorithm to do what we want, but it doesn't fit well with the existing text.  Some of the sentences just repeat existing behaviour of the algorithm (e.g. Attribute level access control checks shall only be performed for access-control-rule-tuples that include an accessControlAttributes parameter. If an access-control-rule-tuple does not include an accessControlAttributes parameter, then only resource level access control checks shall be performed for that rule. If there is a rule that that doesn't contain accessControlAttributes and all conditions of the rule are satisfied, then other rules containing accessControlAttributes shall be ignored. )  whereas "Otherwise, the set of rules that contain accessControlAttributes and that satisfy all conditions apart from accessControlAttributes shall be considered." is in contradiction to the reset of the algorithm.   As above I think it would be cleaner to say we run the algorithm twice. 

[Dale] Revised this text to pull out some of the text so it is performed as a separate set of steps after the main algorithm completes.

R03 – Address comments received from SDS49.2 meeting

- Think about all the different Result Content types and how attribute-level ACPs should work with them. Then determine whether any additional clarification needs to be added to TS-0003.  Also consider whether attribute-level ACPs should apply to all Result Content types or if use of attribute-level ACPs should be restricted to a subset of RCN values.

- All the Result Content types were evaluated.  The two types that involve child resources and their attributes are rcn=4 (attributes and child resources) and rcn=8 (child resources).  These two rcn types are applicable for only Retrieve and Delete.

- When these rcn values are used, the response content is defined as m2m:<resourceType> per Table 7.5.2-2 in TS-0004.

- Per TS-0001, the Hosting CSE shall list parent resources before their children. This means that the originator of the request will not receive a descendant resource without having received its parents. The hosting CSE shall also ensure that proper nesting representation of all the children is incorporated in its listing for parents and children the hosting CSE

- The new attribute-level ACP functionality is not changing the mechanism of how ACPs are linked to resources (e.g. via the accessControlPolicyID).  Hence when rcn=4 or rcn=8 is used, the set of ACPs associated with each child resource will still be determined in the same manner as in prior releases.

- If an Originator of a Retrieve or Delete request having rcn=4 or rcn=8 is found not have privileges to Retrieve or Delete a child resource, then the child resource and any of its descendents will not be included in the response.

- If an Originator of a Retrieve request having rcn=4 or rcn=8 is found to have Retrieve privileges on a resource but no privileges to retrieve any of its attributes, then only the root element of that resource will be returned and no attributes will be returned.

Example #1 - In one example, an Originator sends a request with rcn=4 to retrieve a targeted AE resource and its child resources.  If the privileges are configured such that the Originator has Retrieve privileges for the AE resource but no retrieve privileges for the attributes of the AE resource, then an empty AE resource is included in the response. If the privileges for each of the child resources of the AE are configured such that the Originator has Retrieve privileges on each of the children as well as Retrieve privileges for attributes of the children, then the child resources and their attributes are included in the responses.

- { 
 "m2m:ae": { 
   "m2m:cnt":[{"rn":"container1", "ty":3,  …},
              {"rn":"container2", "ty":3,  … ,
 	 		   "m2m:sub":[{"rn":"sub1", "ty":23, …}]}

- ]
  } 
}

Example #2, an Originator send a request with rcn=4 to retrieve a targeted AE resource and its child resources.  If the privileges are configured such that the Originator has Retrieve privileges for the AE resource and Retrieve privileges for the attributes of the AE resource, then an AE resource is included in the response with its attributes. Likewise, if the privileges for each of the child resources of the AE are configured such that the Originator has Retrieve privileges on each of the children as well as Retrieve privileges for attributes of the children, then the child resources and their attributes are included in the responses.

{ 
 "m2m:ae": { 
   "rn": "appname", 
   "aei": "CAE01", 
   "ct": "20160404T132648", 
   "et": "20160408T004648", 
   "lt": "20160404T132648", 
   "pi": "ONET-CSE-02", 
   "ri": "REQID1", 
   "ty": 2,
   "m2m:cnt":[{"rn":"container1", "ty":3,  …},
              {"rn":"container2", "ty":3,  … ,
 	 		   "m2m:sub":[{"rn":"sub1", "ty":23, …}]}

]
  } 
}

- In clause 7.1.4 and 7.1.5, add some more text to more clearly describe that there are two phases to the access control decision algorithm.

- In new text that was added to describe the second phase of access control algorithm, remove the 2nd bullet since this text is duplicated in the third bullet.

R04:

- Changed the conditions for performing the second phase of the algorithm from “more than one access-control-rule-tuple including an accessControlAttributes” to “one or more one access-control-rule-tuple including an accessControlAttributes”.  This was done to support the Retrieve a resource scenario and its corresponding functionality to filter out attributes not defined in the accessControlAttributes.  This filtering filtering capability is defined in the second phase of the algorithm and not the first phase.

- Add a note to clarify that although the access decision algorithm is defined as having two phases, it is left to implementation whether these two phases require one or more passes over the access control rules.

R05:

- Replaced rq_attributes term used within the language describing the re_attrs(k) logical result variable of the access control algorithm with.

- Similar to Retrieve, added text to Update and Create to say that any attributes of the targeted resource not included in the union of accessControlAttributes shall be filtered and not included in the response.

- Reworded the Filter Criteria check

### -----------------------Start of change 1-------------------------------------------

## 7.1	Access Control Mechanism

### 7.1.1	General Description

The M2M authorization procedure controls access to resources and services hosted by CSEs and AEs. The authorization procedure requires that the originator of the resource access request message has been identified to the Authentication Function, and originator and receiver are mutually authenticated with each other.

The resource addressed in a request message has an associated accessControlPolicyIDs attribute (either included explicitly as an attribute of the resource addressed in the request message, implied from the parent of the resource, or set fixed by the system, see clause 9.6.1 of oneM2M TS-0001 [Error: Reference source not found]). The accessControlPolicyIDs attribute contains a list of identifiers of <accessControlPolicy> resources applicable to the resource addressed in the request message.

The overall structure of <accessControlPolicy> resources is described in clause 9.6.2 "Resource Type accessControlPolicy" of oneM2M TS-0001 [Error: Reference source not found]).

Each of these <accessControlPolicy> resources include privileges and selfPrivileges attributes, which comprise the information, denoted as access control rules in the present document, that is evaluated against the parameters associated with the request message to obtain the access decision.

Figure 7.1.1-1 illustrates the relation between <accessControlPolicy> resource instances (ACP) and the instances of the protected resources, denoted Resource_1 to Resource_N.

Resource_2

...

ACP_1

ACP_2

ACP_3

ACP_4

Instances of

accessControlPolicy

resources (ACP)

Example:

ACP set = (ACP_1, ACP_2)

assigned to Resource_1

Each ACP includes one privileges and one selfPrivileges attribute.

privileges and selfPrivileges attributes include a set of access control rules (defined in Section 7.3)

List of IDs in accessControlPolicyIDs

attribute of Resource_1

Resource_1

Resource_3

Resource_N

Figure 7.1.1-1: Relation between Resource Instances and Access Control Policies

Access requests to ACP's itself are evaluated against the selfPrivileges attribute of that ACP. Access requests to instances of all other resource types, are evaluated against the privileges attributes of the ACP set associated with the targeted resource.

For requests to <accessControlPolicy> resource type, authorization is granted if the request is evaluated to "Permit" for at least one selfPrivileges attribute. For other resource types, authorization is granted if the request is evaluated to "Permit" for at least one privileges attribute.

The privileges and selfPrivileges defined in the accessControlPolicy resource determine which request originator is allowed to access the resource containing this attribute, for which specific operation (i.e. Create, Retrieve, Update, Delete, etc.) and for which specific context constraints (i.e. constraints regarding access time, originator's IP address and originator's location).

The access control approach specified here conforms to the concept of Attribute Based Access Control (ABAC) as defined in [Error: Reference source not found].

The policies defined in the <accessControlPolicy> resources are enforced by an access control mechanism which employs the authorization logical architecture outlined in clause 6.2.2.

The access control mechanism assembles the information needed to render the access decision which consists of:

- Information included in the resource access request message as defined in clause 7.1.2 (table 7.1.2-1).

- Contextual information as defined in clause 7.1.2 (table 7.1.2-2).

- Tokens (if any) associated with the resource access request.

- The policies governing the access as defined in clause 7.1.3.

### 7.1.2	Parameters of the Request message

This clause specifies the parameters of a request message which are evaluated by the access control mechanism.

The data types applicable to these parameters are defined in clause 6.4 of oneM2M TS-0004 [Error: Reference source not found].

The parameters are listed in table 7.1.2-1.

Table 7.1.2-1: Parameters indicated in the request message

| Parameter | Description | Mandatory/ Optional | Usage in access control mechanism |
| --- | --- | --- | --- |
| To | URI of target resource | M | Selection of accessControlPolicy associated with the target resource |
| From | Identifier representing the originator of the request | M (see note 1) | Evaluated against accessControlOriginators in privileges and selfPrivileges attributes |
| Role IDs | Role IDs of the originator | O | Evaluated against accessControlOriginators in privileges and selfPrivileges attributes |
| Operation | Requested operation | M | Evaluated against accessControlOperations in privileges and selfPrivileges attributes |
| Resource Type | Type of the target resource | O (see note 2) | Evaluated against accessControlObjectDetails in privileges attributes. Applicable to Create operations only. |
| Filter Criteria | filterUsage condition tag in Filter criteria | O | Differentiation between Retrieve and Discovery operations |
| Tokens | ESData-protected Tokens | O | Contains authorization information (e.g. Role-IDs) to be used in the decision for the request |
| Token IDs | tokenIDs or Local-Token-ID | O | Identifies Tokens containing authorization information (e.g. Role-IDs) to be used in the decision for the request |
| M2M Service User | Identity of a M2M Service User | O | Evaluated against the accessControlUserIDs sub-parameter of the accessControlContexts parameter of the privileges and selfPrivileges attributes |
| NOTE 1: The From primitive parameter is Mandatory in all requests except for AE registration procedure where it is optional, as specified in oneM2M TS-0001 [Error: Reference source not found]. NOTE 2: The resource Type primitive parameter is present in Create request primitives only. |  |  |  |

Table 7.1.2-2 lists the context parameters associated with a request message which are evaluated by the access control mechanism. These parameters are not explicitly included in a request message but can be obtained at the receiver and validated against the context policy parameters as given in table 7.1.2-2.

Table 7.1.2-2: Context parameters associated with a request message

| Parameter | Description | Usage in access control mechanism |
| --- | --- | --- |
| rq_time | Time stamp when the request message was received at the hosting CSE. Obtained by the hosting CSE's system time clock. | Validated against accessControlTimeWindow parameter in an access control rule, see clause 7.1.3. |
| rq_loc | Location information about the originator of the request. Obtained over the Mcn reference point. | Validated against accessControlLocationRegion parameter in an access control rule, see clause 7.1.3. |
| rq_ip | IP source address associated with the IP packets that carry the request message. Obtained over the Mcn reference point. | Validated against accessControlIpAddress parameter in an access control rule, see clause 7.1.3. |

Tokens, as defined in clause 7.3.3.1 "Token Structure", may be associated with a request message. A Token may be associated with a request as a result of being included in the Tokens primitive parameter of the request message or identified in the Token IDs primitive parameter of the request message. If the Hosting CSE obtained a token from the Dynamic Authorization System (DAS) Server using Direct Dynamic Authorization, then this Token shall be associated with a request if the holder parameter in the Token matches the Absolute AE-ID or CSE-ID of the Originator of the request. Dynamic Authorization is specified in clause 7.3.

Table 7.1.2-3 lists the security context parameters associated with a request message.

Table 7.1.2-3: Security Context parameters associated with a request message

| Parameter | Description | Mandatory/Optional | Usage in access control mechanism |
| --- | --- | --- | --- |
| rq_authn | Boolean value (TRUE/FALSE) indicating if the Originator is considered to have been authenticated by the Hosting CSE, and the From parameter matched the authenticated identity of the Originator. | M | Validated against accessControlAuthenticationFlag parameter in an access control rule, see clause 7.1.3. |

The following criteria shall be applied to determine if an Originator is considered to have been authenticated by the Hosting CSE.

- If the Originator is an AE registered to the Hosting CSE, then the criteria for deciding whether the Originator is considered authenticated is deployment and/or implementation specific and depends on the trust guaranteed by the field device's physical and logical embodiment bearing the AE(s) and Hosting CSE (e.g. secure boot and tamper resistance). In many cases it is appropriate to expect a secure channel implying authentication (e.g. a TLS or DTLS session) to be used to protect primitives on the Mca interface, in which case the authentication shall be considered valid for the duration of the TLS session, When this is not the case, e.g. because the physical and logical design is trusted, authentication may be considered to be permanently valid unless it is detected that the device is compromised.

- If the Originator is a CSE registered with the Hosting CSE, then the Originator shall be considered authenticated for the duration of a (D)TLS session because the Mcc is always required to be protected by TLS or DTLS according to a Security Association Establishment Framework (SAEF) as described in clause 8.2. The other CSE may be the Registrar or Registree with respect to the Hosting CSE.

- If the Originator is an AE or CSE registered with a CSE other than the Hosting CSE, then the Originator is considered authenticated by the Hosting CSE if and only if the request primitive is protected using End-to-End Security of Primitives (ESPrim) as described in clause 8.4.

### 7.1.3	Format of privileges and selfPrivileges Attributes

The privileges and selfPrivileges attributes exhibit the same data type format which is specified as follows.

Each privileges or selfPrivileges attribute comprises a set of access control rules. In the following, the set of access control rules is denoted as acrs and an individual access control rule in this set as acr. The access control rules in acrs are indexed with the letter k. The number of access control rules in the set is denoted with the letter K:

acrs = { acr(1), acr(2), ..., acr(k), ..., acr(K) }

Each access control rule acr(k) is comprised of mandatory accessControlOriginators and accessControlOperations components and optional accessControlContexts, accessControlObjectDetails, accessControlAuthenticationFlag and accessControlAttributes components.

Hence, an access control rule acr(k) is either represented as a pair:

acr(k) = {acr(k)_accessControlOriginators, acr(k)_accessControlOperations}

or as a 3-tuple, 4-tuple, 5-tuple or 6-tuple.  For example, a 3-tuple such as the following:

acr(k) = {acr(k)_accessControlOriginators, acr(k)_accessControlOperations, acr(k)_accessControlContexts}

The generic term "access-control-rule-tuple" is used when referring to a rule acr(k).

A set acrs of access control rules may consist of a mix of pairs,3-tuples, 4-tuples, 5-tuples or 6-tuples. For pairs or for any tuples not containing accessControlContexts, any context parameters associated with a request message are admissible.

The six component parameters of an access-control-rule-tuple supported in the present document are shown in table 7.1.3-1.

Table 7.1.3-1: Parameters of an access-control-rule-tuple

| Parameter | Usage Description | Mandatory/Optional | Format |
| --- | --- | --- | --- |
| accessControlOriginators | Set of Originators that can be authorized | M | List of CSE-IDs and/or AE-IDs, or keyword "all" to grant access to all originators |
| accessControlOperations | Set of Operations that can be authorized | M | Enumerated list of operations Create Retrieve, Update, Delete, Discover, Notify |
| accessControlContexts | See table 7.1.3-3 | O | See table 7.1.3-3 |
| accessControlObjectDetails | See table 7.1.3-4 | O | See table 7.1.3-4 |
| accessControlAuthenticationFlag | Indicates whether the rule applies only to Originators which are considered to be authenticated by the Hosting CSE | O | Boolean |
| accessControlAttributes | Set of resource attributes for which access can be authorized | O | List of resource attribute name(s). |

The accessControlOriginators parameter comprises a list of SP domain names, CSE-IDs, AE-IDs, resource-IDs of <group> resources and/or Role IDs of any format defined in oneM2M TS-0001 [Error: Reference source not found]. If access for all originators is to be allowed, the reserved keyword "all" may be included into the value space of accessControlOriginators.

Using a SP domain name in accessControlOriginators means all AE-IDs and CSE-IDs matching the given domain name can be authorized.

It is furthermore allowed to use wildcard character "*", in representations of M2M-SP-ID (i.e., SP domain names), CSE-ID and AE-ID. The scope of a "*" is terminated by a following "/" character. Table 7.1.3-2 shows examples of using wildcard characters in CSE-IDs and AE-IDs.

Wildcard characters are not permitted in resource-IDs of <group> resources and Role IDs.

Table 7.1.3-2: Examples of using wildcard characters in CSE-IDs and AE-IDs of accessControlOriginators

|  | Form of ID | Examples | Meaning |
| --- | --- | --- | --- |
| CSE-ID | Absolute | //m2msp.org/myCSEID //*/myCSEID //*/myCSE* | Any CSE whose ID matches the wild cards |
|  | SP-relative | /myCSEID /myCSE* | Any matching CSE from the SP that is hosting the target resource |
| AE-ID | Absolute | //m2msp.org/S988 //*/myCSEID/C9886 //*/myCSE*/C9886 | Any AE whose ID matches the wild cards |
|  | SP-relative | /myCSEID/C9886 /myCSEID/C98* /myCSE*/C98* /SmyAE* | Any matching AE from the SP that is hosting the target resource |

The data type applicable to accessControlOriginators is defined in oneM2M TS-0004 [Error: Reference source not found].

The accessControlOperations parameter comprises a list of admissible operations which can be any subset of the following elements: Create, Retrieve, Update, Delete, Discover, and Notify. While Create, Retrieve, Update, Delete, and Notify operation are explicitly indicated in the op parameter of a request message, the Discovery operation is indicated by the filterUsage condition of the Filter Criteria request parameter having a value of “Discovery”, “Discovery-based Operation” or “IPE On-Demand Discovery”.

The data type applicable to accessControlOperations is defined in oneM2M TS-0004 [Error: Reference source not found].

The accessControlContexts parameters are listed in table 7.1.3-3.

Table 7.1.3-3: Parameters of accessControlContexts

| Parameter | Usage Description | Mandatory/Optional | Formats |
| --- | --- | --- | --- |
| accessControlTimeWindow | Set of Time Windows that can be authorized | O | List of time intervals where access can be granted in extended crontab format |
| accessControlLocationRegion | Set of Location Regions that can be authorized | O | 1) Latitude/longitude coordinates, and a radius defining a circular region around the coordinates 2) Country code |
| accessControlIpAddress | Set of IPv4 and IPv6 addresses that can be authorized | O | IPv4: dotted-decimal notation with CIDR suffix IPv6: colon separated groups of hexadecimal digits with CIDR suffix |
| accessControlUserIDs | Set of M2M Service Users that can be authorized | O | List of M2M-User-IDs |
| accessControlEvalCriteria | Set of conditions that are factored into authorization decisions. | O | A tuple consisting of a mandatory resource identifier of a subject resource and an set of evaluation criteria applicable to the subject resource. |
| accessControlLimit | Number of times access to a resource can be authorized. | O | A number that indicates how many times access can be granted. |

The accessControlTimeWindow parameter represents a list of elements that comply with the extended crontab syntax as defined in clause 7.3.8 of oneM2M TS-0004 [Error: Reference source not found]. It allows definition of periodically recurring time intervals at which access can be granted, when the rq_time parameter associated with the access request message falls into such interval.

For the elements of accessControlLocationRegion there are two representation choices. These can be represented by a 2-character country code or a circle with radius R centred at a point defined in terms of longitude and latitude parameters. Refer to Annex F for detailed information. Each element of accessControlLocationRegion defines an admissible location region, which is compared with the rq_loc parameter associated with the access request message.

The data types applicable to accessControlLocationRegion and rq_loc are defined in oneM2M TS-0004 [Error: Reference source not found].

The accessControlIpAddress parameter represents a list of IPv4 and IPv6 addresses in dotted-decimal notation with CIDR suffix or colon separated groups of hexadecimal digits with CIDR suffix, respectively. If the rq_loc parameter associated with the access request message matches one of these addresses, access may be granted with regard to this criterion.

The data types applicable to accessControlIpAddress and rq_ip are defined in oneM2M TS-0004 [Error: Reference source not found].

The accessControlUserIDs parameter comprises a list of M2M-User-IDs having a format defined in oneM2M TS-0001 [Error: Reference source not found].

This accessControlEvalCriteria parameter represents the conditions determining if the request operation is to be allowed. It allows conditional access to the resource based on conditions not contained in the received request. The accessControlEvalCriteria parameter is a tuple that consists of a mandatory subjectResourceID element as defined in table 9.6.61-2 in oneM2M TS-0001 [Error: Reference source not found] and an evalCriteria element defined in table 9.6.61-3 in oneM2M TS-0001 [1].

The accessControlLimit parameter represents the number of times that the policy defined by an access-control-rule-tuple can allow authorization to the requested resource. This attribute maintains the number of remaining accesses allowed. The parameter is decremented each time an access to the requested resource is granted. If the value is greater than zero then access is granted, otherwise access is denied. If the accessControlLimit parameter is not present in an access-control-rule-tuple, then there are no restrictions on the number of times access is granted.

Using just a SP domain name in accessControlUserIDs means all M2M-User-IDs matching the given domain name can be authorized. For example, “//m2msp.org”.

It is furthermore allowed to use a wildcard character "*" within the SP-Relative-M2M-User-ID portion of a M2M-User-ID. For example, //m2msp.org/homeowner*.  A wildcard character is not permitted within the SP domain name portion of a M2M-User-ID.

The data type applicable to accessControlUserIDs is defined in oneM2M TS-0004 [Error: Reference source not found].

The accessControlAuthenticationFlag parameter is a Boolean value. If the accessControlAuthenticationFlag parameter is not present, then the value is assumed to be FALSE. If the accessControlAuthenticationFlag parameter is TRUE, then this indicates that the access control rule applies only to Originators considered to have been authenticated by the Hosting CSE. Clause 7.1.2 specifies the criteria used to decide whether or not the Originator is considered to have been authenticated by the Hosting CSE.

The accessControlObjectDetails parameters are listed in table 7.1.3-4.

Table 7.1.3-4: Parameters of accessControlObjectDetails

| Parameter | Usage Description | Mandatory/Optional | Formats |
| --- | --- | --- | --- |
| resourceType | Resource type on which access control rule applies | O | Resource type identifier |
| specializationID | Identifier of mgmtDefinition or containerDefinition | O | mgmtDefinition or containerDefinition represented as a string. |
| childResourceType | Set of resource type identifiers that can be created under the parent resource. | O | Resource type list. |

The accessControlObjectDetails attribute specifies a subset of child resource types of the targeted resource to which the access control rule applies. If an access control rule includes accessControlObjectDetails, then childResourceType is specified. An access control rule which does not include any accessControlObjectDetails parameters applies to all child resource types of the target resource. The accessControlObjectDetails parameter is described in table 9.6.2.4-1 of oneM2M TS-0001 [Error: Reference source not found]. Child resource types listed in the childResourceType component are subject of access control for the Create operation only. Once a child resource is created, the Access Control Policies assigned directly to it apply. The resourceType and specializationID elements are optional. If either the resourceType or specializationID element is present in accessControlObjectDetails, the CSE matches the type of resource or specialization of the targeted resource with the value specified in the resourceType or specializationID element. Further checking of childResourceType is done only if the resourceType or specializationID match occurs. However, if the resourceType and specializationID elements are not provided, then only childResourceType match is performed.

The accessControlAttributes attribute specifies a list of one or more resource attribute names -s.  If there is a rule for which all conditions of the rule are satisfied, then other rules shall be ignored. Otherwise, rules that contain accessControlAttributes and that satisfy all conditions apart from accessControlAttributes are considered to be applicable rules. In this case, the resource attributes associated with the request and its response are evaluated against thetunion of resource attributes defined across all the accessControlAttributes of these applicable rules to determine if access is alloweError: Reference source not found].

### 7.1.4	Access Control Decision

The access decision is derived by comparing the parameters associated with a resource access request message as described in clause 7.1.2 with the access control rules included in the privileges or selfPrivileges attributes of all ACP sets assigned to the protected resource by means of the accessControlPolicyIDs, see figure 7.1.1-1.

The result of the access decision algorithm, i.e. the access decision, is the overall result of evaluating the applicable set of access control rules, acrs, against the parameters associated with the access request message. This access decision can be represented by a value of binary data type. The overall result of the access decision algorithm is denoted here with the variable name res_acrs:

res_acrs=TRUE or 1     if the request matches the access control rules  FALSE or 0   else

The reference access decision algorithm is specified in clause 7.1.5. For any given sets of inputs, an implementation of the access decision processing shall return the same result as the reference access decision algorithm would return for those inputs.

If the access decision algorithm yields the result res_acrs = TRUE, then the access decision for the requested resource shall be "Permit".

If the access decision algorithm yields the result res_acrs = FALSE, or the access decision algorithm is not capable of deriving a final result (e.g. due to indeterminate parameters), then the access decision for the requested resource shall be "Deny".

The access decision algorithm consists of two phases.  The first phase evaluates each of the applicable access control rules on an individual basis to determine whether an individual access control rule permits access to the requested resource. If the access control decision for an individual access control rule results in res_acrs = TRUE, the algorithm stops, and the second phase of the algorithm is not performed. However, if the first phase of the reference access decision algorithm results in res_acrs = FALSE, but during the processing of the first phase of the algorithm, one or more access-control-rule-tuple including an accessControlAttributes condition is processed, then a second phase of the algorithm is performed. In this second phase, the set of access control rules that have accessControlAttributes conditions and that have satisfied all conditions of the first phase of the access decision algorithm, apart from their accessControlAttributes conditions, are collectively evaluated. If the union of resource attributes defined across all their accessControlAttributes conditions permit access to the requested resource then res_acrs = TRUE, otherwise res_acrs = FALSE.

Note, although the access decision algorithm is defined as having two phases, it is left to implementation whether these two phases require one or more passes over the access control rules.

### 7.1.5	Description of the Access Decision Algorithm

The reference access decision algorithm specified in this clause combines partial access control results obtained for each of the individual access control rules contained in a privileges or selfPrivileges attribute. Further, if multiple ACP instances are assigned to the protected resource, the reference access decision algorithm combines the partial access control results obtained for the individual ACPs of an ACP set.

The algorithm specified in this clause adopts a "Permit-overrides" combining algorithm with respect to access control rules and ACPs as defined in XACML [Error: Reference source not found]. This algorithm has the following behaviour:

- A first phase of the algorithm determines if the result is “Permit” if a single access control rule included in the privileges (or selfPrivileges) attribute of a single ACP permits access to the targeted resource.

- A second phase of the algorithm determines if the result is “Permit” if the set of applicable access control rules containing accessControlAttributes collectively as a union permit access to the targeted resource.

- Otherwise, the result is "Deny".

The logic for evaluating a request against a privilege can be described mathematically as follows. A privileges or selfPrivileges attribute included in an <accessControlPolicy> resource represents a set of access control rules, acrs, which is built as in figure 7.1.5-1.

Figure 7.1.5-1: Logic to evaluate privileges in the reference access decision algorithm

The parameters associated with a request, which are evaluated against the parameters contained in the access control rules are specified in clause 7.1.3.

The access decision res_acrs defined in clause 7.1.4 is derived by evaluating whether or not the parameters associated with the request message listed in tables 7.1.2-1 and 7.1.2-2 match any of the access control rules contained in the access control rule set defined in clause 7.1.3 as follows:

res_acrs = res_acr(1) OR res_acr(2) ... OR res_acr(k) … OR res_acr(K),

where res_acr(k) represents the logical evaluation result (i.e. TRUE/FALSE or 1/0) of the request parameters against the kth access control rule in the set acrs, which can be expressed as follows:

res_acr(k) = res_authn(k) AND res_origs(k) AND res_ops(k) AND res_ctxts(k) AND res_objd(k) AND res_attrs(k),

where k = 1…K.

The first partial logical result variable res_authn(k) on the right side of above equation shall be evaluated according to Table 7.1.5-1:

Table 7.1.5-1: Evaluating res_authn(k)

| acr(k)_accessControlAuthenticationFlag | rq_authn | res_authn |
| --- | --- | --- |
| TRUE | TRUE | TRUE |
| TRUE | FALSE | FALSE |
| FALSE | TRUE | TRUE |
| FALSE | FALSE | TRUE |

The next 4 partial logical result variables on the right side of above equation can be defined by using the following set function:

ismember(x,setX)=TRUE or 1     if x∈ setX  FALSE or 0   else

With this definition:

res_origs(k) = ismember(Originator, acr(k)_accessControlOriginators)

res_ops(k) = ismember(Operation, acr(k)_ accessControlOperations)

In the above equation, the Originator variable refers to the authenticated identity of the originator of the request primitive which matches the From parameter.

The fourth partial logical result res_ctxts(k) is derived as follows:

res_ctxts(k) = res_context(k, 1) ... OR res_context(k, m) ... OR res_context(k, M_k),

where:

res_context(k, m) = res_time(k, m) AND res_ip(k, m) AND res_loc (k, m) AND res_uids(k, m), k = 1…K, m = 1…M_k

and

res_time(k, m) = ismember(rq_time, acr(k)_accessControlTimeWindow(m))

res_ip(k, m) = ismember(rq_ip, acr(k)_accessControlIpAddress(m))

res_loc (k, m) = ismember(rq_loc, acr(k)_accessControlLocationRegion(m))

res_uids(k, m) = ismember(M2M Service User, acr(k)_accessControlUserIDs(m))

The fifth partial logical result res_objd(k) applies to Create request primitives only and is derived as

res_ objd(k) = res_ objdetails(k, 1) ... OR res_ objdetails(k, m) ... OR res_ objdetails(k, M_k),

where:

res_ objdetails(k, m) = res_resourceType(k, m) AND res_specializationID(k, m) AND res_childResource(k,m),

for m = 1…M_k. The three logical arguments are defined below.

For each given element acr(k)_accessControlObjectDetails(m) in an access control rule determine if the optional resourceType parameter is present

resourceType = acr(k)_accessControlObjectDetails(m)/resourceType

Depending on the presence of resourceType, res_resourceType(k, m) is derived as

res_resourceType(k,m)=TRUE or 1,    if not present in acr(k)_accessControlObjectDetails(m)TRUE or 1,    if present  and resourceType=targetResourceTypeID

where targetResourceTypeID is the resource type identifier associated with the resource addressed in the To parameter of the Create request primitive.

If the value of the resourceType element is 13 (<mgmtObject> specialization) or 28 (<flexContainer> specialization>), the optional specializationID element shall also be included in accessControlObjectDetails:

specializationID = acr(k)_accessControlObjectDetails(m)/specializationID

If specializationID is present, it shall be matched against the mgmtDefinition or containerDefinition attributes given in the Content parameter of the Create request primitive.

res_specializationID(k.m)=TRUE or 1,    if specializationID not present in acr(k)_accessControlObjectDetails(m)TRUE or 1,    specializationID=mgmtDefinition(resourceType= 13)TRUE or 1,    specializationID=  containerDefinition(resourceType= 28)FALSE or 0,  specializationID≠mgmtDefinition(resourceType= 13)

The childResourceType element is mandatory in any given accessControlObjectDetails element of an access control rule. It includes a list of j = 1…J child resource type identifiers to which the rule applies. The jth list element is denoted as follows

childResourceType(k, m. j) = acr(k)_accessControlObjectDetails(m)/childResourceType(j), j = 1…J

The logical variable res_childResource(k, m) is derived as

res_ childResource (k, m) = ismember(Resource Type, childResourceType(k, m, j))

where Resource Type refers to the value of the parameter of the given Create request primitive.

NOTE:	If resourceType and specializationID are not present in acr(k)_accessControlObjectDetails(m), res_ objdetails(k, m) = res_resourceType(k, m) AND res_specializationID(k, m) AND res_childResource(k,m) = res_childResource(k,m).

The sixth partial logical result res_attrs(k) is derived as follows:

Depending on the type of operation, the requested attribute names defined within the parameters of the request (e.g., To, Content, Filter Criteria) or within the targeted resource shall be compared against the names of attributes present in acr(k)_accessControlAttributes to determine the value of res_attrs(k) as follows:

- For an operation that includes a Filter Criteria parameter and that requires access to the attributes of a resource to process the Filter Criteria (i.e., matching conditions defined within a discovery operation, discovery-based operation, IPE On-demand discovery operation or a conditional operation), acr(k)_accessControlAttributes defines the attributes that can be accessed. If the Filter Criteria includes the names of attributes that are not defined in acr(k)_accessControlAttributes, then then res_attrs(k) is False or 0. Otherwise, if the names of all the attributes are defined in acr(k)_accessControlAttributes, then the value of res_attrs(k) shall be determined by the operation specific steps described below.

- For a Retrieve operation, acr(k)_accessControlAttributes defines the attributes that can be retrieved and included in the response.

  - For a Retrieve operation that targets a resource, in which all the names of the attributes present in the targeted resource are included in acr(k)_accessControlAttributes, then res_attrs(k) is True or 1. Otherwise, if one or more of the names of the attributes present in the targeted resource are not included in acr(k)_accessControlAttributes, then res_attrs(k) is False or 0.

  - For a Retrieve operation that targets one or more individual attributes of a resource (i.e., partial retrieve) and these attributes are all defined in acr(k)_accessControlAttributes, then res_attrs(k) is True or 1.  Otherwise, if one or more individual attributes are not defined in acr(k)_accessControlAttributes, then res_attrs(k) is False or 0.

- For a Delete operation, acr(k)_accessControlAttributes defines the attributes that can be deleted. If all the attributes present in the targeted resource of a Delete operation are defined in acr(k)_accessControlAttributes, then res_attrs(k) is True or 1. Otherwise, if one or more of the attributes are not defined in acr(k)_accessControlAttributes, then res_attrs(k) is False or 0.

- For an Update operation, acr(k)_accessControlAttributes defines the attributes that can be included in the Content parameter of a request and its response. For an Update operation that attempts to create, update or delete one or more attributes of a resource that are all defined in acr(k)_accessControlAttributes, then res_attrs(k) is True or 1, however any attributes of the targeted resource not included in acr(k)_accessControlAttributes shall be filtered and not included in the response. Otherwise, if one or more of the attributes of the Update operation are not defined in acr(k)_accessControlAttributes, then res_attrs(k) is False or 0.

- For a Create operation, acr(k)_accessControlAttributes defines the attributes that can be included in the Content parameter of a request and its response. For a Create operation that attempts to create a resource with attributes that are all defined in acr(k)_accessControlAttributes, then res_attrs(k) is True or 1, however any attributes of the targeted resource not included in acr(k)_accessControlAttributes shall be filtered and not included in the response.  Otherwise, if one or more attributes of the Create operation are not defined in acr(k)_accessControlAttributes, then res_attrs(k) is False or 0.

, then

Thanks to the "Permit-overrides" combining approach, if the access control decision for one access control rule results in res_acr = TRUE, the reference access decision algorithm can stop without evaluating any other applicable access control rules of the current ACP or any other ACPs in the ACP set, and the final access decision is "Permit" (i.e. res_acrs = TRUE).

However, if the first phase of the reference access decision algorithm results in res_acrs = FALSE, and during the processing of the algorithm, one or more access-control-rule-tuple including an accessControlAttributes condition is processed, then a second phase of the access decision algorithm shall determine the final access decision. In the second phase of the access decision algorithm, the following steps shall be performed:

- all access control rules having accessControlAttributes conditions, that have satisfied all conditions of the access decision algorithm apart from their accessControlAttributes condition, shall be collectively considered an applicable set of access control rules,

- Depending on the type of operation, the requested attribute names defined within the parameters of the request (e.g., To, Content, Filter Criteria) or within the targeted resource shall be compared against the names of attributes present in the union of resource attributes defined across all the accessControlAttributes of the applicable set of access control rules to determine the value of res_acrs as follows:

  - If a Retrieve, Delete, Update or Create operation includes a Filter Criteria parameter with names of one or more attributes that are not defined in the union of accessControlAttributes, then the final access decision shall be "Deny". Otherwise, if the names of the attributes are all defined in the union of accessControlAttributes, then the final access decision shall be determined by the operation specific steps described below.

  - For a Retrieve operation that targets a resource, the final access decision shall be "Permit", but any attributes not included in the union of accessControlAttributes shall be filtered and not included in the response. If none of the attributes defined in the union of accessControlAttributes match the names of the attributes present in the targeted resource, then no attributes shall be returned in the response.

- For a Retrieve operation that targets one or more individual attributes of a resource (i.e., partial retrieve) and these attributes are all defined in the union of accessControlAttributes, then the final access decision shall be "Permit".  Otherwise, if one or more individual attributes are not defined in the union of accessControlAttributes, then the final access decision shall be "Deny".

  - For a Delete operation,  if all the attributes present in the targeted resource are defined in the union of accessControlAttributes, then the final access decision shall be "Permit". Otherwise, if one or more of the attributes present in the targeted resource are not defined in union of accessControlAttributes, then the final access decision shall be "Deny".

  - For an Update operation that attempts to create, update or delete one or more attributes of a resource that are all defined in the union of accessControlAttributes, then the final access decision shall be "Permit", however any attributes of the targeted resource not included in the union of accessControlAttributes shall be filtered and not included in the response. Otherwise, if one or more of the attributes of the attempted Update operation are not defined in the union of accessControlAttributes, then the final access decision shall be "Deny".

  - For a Create operation that attempts to create a resource with attributes that are all defined in the union of accessControlAttributes, then the final access decision shall be "Permit", however any attributes of the targeted resource not included in the union of accessControlAttributes shall be filtered and not included in the response.  Otherwise, if one or more attributes of the attempted Create operation are not defined in the union of accessControlAttributes, then the final access decision shall be "Deny".

  - -----------------------End of change 1-------------------------------------------