| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 52 |
| Source:* | Rahul Kumar, C-DOT, krahul@cdot.in Poornima Shandilya, C-DOT, poornima@cdot.in Anupama Chopra, C-DOT, anupama@cdot.in |
| Date:* | 2020-11-10 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | V4_12_0 |
| CR against: WI* | Active A-SDS-50.1-01 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 V4_12_0 |
| Clauses * | 10.2.12.6, 10.2.12.9 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR propses to open action item on “Action to define how to determine relevant service subscription”( refer TS-0004  section 7.3.2.1 and the table below).

| A-SDS-50.1-01 | In TS-0004 clause 7.3.2.1 we need to agree how a Registrar or Hosting CSE determines the relevant service subscription resources and accesses these resources and what privileges are required if they are not provisioned and stored locally. | Convida & CDOT |  |
| --- | --- | --- | --- |

This CR tries to associate link between AE-ID and SSP resourceId.

In order to avail M2M services, an M2M application provider has to come in service agreement with M2M Service Provider. In oneM2M, technical part of this agreement is represented through <m2mServiceSubscriptionProfile> resource and details of onem2m nodes is represented through <serviceSubscribedNode> resource.

For configuration of oneM2M nodes in field, oneM2M has defined field device configuration resources in TS-0022. [registration] and [authenticationProfile] management objects provides registrar details and credentials details to field nodes. It also enables CSE to become aware of the details of field nodes.

From TS-0001 we know that SSN i.e. <serviceSubscribedNode> is a child of <m2mServiceSubscription>

### 9.6.19	Resource Type m2mServiceSubscriptionProfile

The <m2mServiceSubscriptionProfile> resource represents an M2M Service Subscription. It is used to represent all data pertaining to the M2M Service Subscription, i.e. the technical part of the contract between an M2M Application Service Provider and an M2M Service Provider and is only stored on IN-CSE. The data is also represented in <serviceSubscribedNode> , <serviceSubscribedAppRule> and <serviceSubscribedUserProfile> resources as well as <m2mServiceSubscriptionProfile> resource. The relationship among those resource types are depicted as follows. Note that the diagram does not capture all attributes and child resources. Those resource types shall only be instantiated on IN-CSE.

Figure 9.6.19-1: Relationship among M2M Service Subscription related resources

The <m2mServiceSubscriptionProfile> resource shall contain the child resources specified in table 9.6.19-1.

Table 9.6.19-1: Child resources of <m2mServiceSubscriptionProfile> resource

| Child Resources of <m2mServiceSubscriptionProfile> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <serviceSubscribedNode> | 0..n | See clause 9.6.20 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |
| [variable] | <serviceSubscribedUserProfile> | 0..n | See clause 9.6.68 |

### 9.6.20	Resource Type serviceSubscribedNode

The <serviceSubscribedNode> resource represents M2M Node information that is needed as part of the M2M Service Subscription resource and is only stored on IN-CSE. It contains M2M-Node-ID and optionally CSE-ID running on that Node.

The <serviceSubscribedNode> resource shall contain the child resource specified in table 9.6.20-1.

Table 9.6.20-1: Child resources of <serviceSubscribedNode> resource

| Child Resources of <serviceSubscribedNode> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |

The <serviceSubscribedNode> resource shall contain the attributes specified in table 9.6.20-2.

Table 9.6.20-2: Attributes of <serviceSubscribedNode> resource

| Attributes of <serviceSubscribedNode> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| holder | 0..1 | RW | See clause 9.6.1.3. |
| nodeID | 1 | WO | M2M-Node-ID of the node that is represented by this instance. |
| CSE-ID | 0..1 | WO | CSE-ID pertaining to this node (for nodes that have a CSE). |
| deviceIdentifier | 0..1 (L) | WO | A list of device identifiers that uniquely identify a device. The format of a device identifier is one of the following: Case 1: Identify a device using the format <OUI> "-" <ProductClass> "-" <SerialNumber> as defined in section 3.4.4 of BBF TR-069 [Error: Reference source not found]. The format of the URN is urn:dev:ops:<OUI> "-" <ProductClass> "-" <SerialNumber>. Case 2: Identify a device using the format <OUI> "-"<SerialNumber> as defined in section 3.4.4 of BBF TR-069 [Error: Reference source not found]. The format of the URN is urn:dev:os:<OUI> "-"<SerialNumber>. Case 3: Identify a device using an International Mobile Equipment Identifiers of 3GPP TS 23 003 [Error: Reference source not found]. This URN specifies a valid, 15 digit IMEI. The format of the URN is urn:imei:###############. Case 4: Identify a device using an Electronic Serial Number. The ESN specifies a valid, 8 digit ESN. The format of the URN is urn:esn:########. Case 5: Identify a device using a Mobile Equipment Identifier. This URN specifies a valid, 14 digit MEID. The format of the URN is urn:meid:##############. Case 6: Identify a device using an Object IDentifier (OID). This URN specifies a valid OID - see annex H for one possible naming convention. The format of the URN is urn:oid:<OID>. Case 7: Identify a device using a Universally Unique IDentifier (UUID). The UUID specifies a valid, hex digit character string as defined in IETF RFC 4122 [i.26]. The format of the URN is urn:uuid: ########-####-####-####-############. |
| ruleLinks | 0..1 ((L) | RW | This attribute contains a list of links towards <serviceSubscribedAppRule> resources pertaining to this <serviceSubscribedNode>. See clause 9.6.29 for an explanation of the <serviceSubscribedAppRule> resource. This attribute shall exist only when the CSE-ID attribute is present. When the list is empty, it means no applications are allowed to register on the CSE which is indicated by the CSE-ID attribute. |
| niddRequired | 0..1 | RW | Controls whether the IN-CSE configures the underlying network to enable Non-IP Data Delivery for this node. Valid values are "TRUE" or "FALSE". If not configured, then IN-CSE default policy shall apply. See oneM2M TS-0026 [Error: Reference source not found]. |

As per above nodeID is present in <serviceSubscribedNode> and it is also present in <node>

The <node> resource shall contain the attributes specified in table 9.6.18-2.

Table 9.6.18-2: Attributes of <node> resource

| Attributes of <node> | Multiplicity | RW/ RO/ WO | Description | <nodeAnnc> attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| holder | 0..1 | RW | See clause 9.6.1.3 | NA |
| nodeID | 1 | RW | The M2M-Node-ID of the node which is represented by this <node> resource. | MA |
| nodeType | 0..1 | RW | Indicates the type of node. It shall have one of the following values: - IN - MN - ASN - ADN - NoDN - UNSPECIFIED | OA |
| hostedCSELink | 0..1 | RW | This attribute allows to find the <CSEBase> or <remoteCSE> resource representing the CSE that is residing on the node that is represented by this <node> resource. The attribute contains the resource ID of a resource where all of the following applies: The resource is a <CSEBase> resource or a <remoteCSE> resource. The resource represents the CSE which resides on the specific node that is represented by the current <node> resource. In case the node that is represented by this <node> resource does not contain a CSE, this attribute shall not be present. | OA |
| hostedAELinks | 0..1(L) | RW | This attribute allows to find the AEs hosted by the node that is represented by this <node> resource. The attribute shall contain a list of resource identifiers of <AE> resources representing the ADN-AEs residing on the node that is represented by the current <node> resource. In case the node that is represented by this <node> resource does not contain an AE, this attribute shall not be present. | OA |
| hostedServiceLinks | 0..1(L) | RW | This attribute allows to find <flexContainer> resources that have been created by an IPE to represent services hosted on a NoDN, the NoDN being represented by this <node> resource. If the NoDN hosts a set of services represented by <flexContainer>s, then the attribute shall contain the list of resource identifiers of these <flexContainer> resources. In case the node that is represented by this <node> resource does not contain a service that is represented by a <flexContainer>, this attribute shall not be present. | OA |
| mgmtClientAddress | 0..1 | RW | Represents the physical address of management client of the node which is represented by this <node> resource. This attribute is absent if management server is able to acquire the physical address of the management client. | OA |
| roamingStatus | 0..1 | RO | Indicates if the M2M Node is currently roaming from the perspective of the underlying network. The allowed values are "Yes" or "No". | OA |
| networkID | 0..1 | RO | Configured with the identity of the underlying network which the M2M Node is currently attached to. | OA |

From TS-0022 DFCG resource <registration> is child of <node> which has attributes M2M-Sub-ID which is resourceId of <m2mServiceSubscriptionProfile> and originatorID  which has aeId of an ADN

### 7.1.2	Resource [registration]

This specialization of <mgmtObj> is used to convey the service layer configuration information needed to register an AE or CSE with a Registrar CSE.

Figure 7.1.2-1: Structure of [registration] resource

The [registration] resource shall contain the child resource specified in table 7.1.2-1.

Table 7.1.2-1: Child resources of [registration] resource

| Child Resources of [registration] | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 of oneM2M TS-0001 [Error: Reference source not found] |

The [registration] resource shall contain the attributes specified in table 7.1.2-2.

Table 7.1.2-2: Attributes of [registration] resource

| Attributes of [reboot] | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| resourceID | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| resourceName | 1 | WO | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| parentID | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| expirationTime | 1 | RW | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| creationTime | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| labels | 0..1(L) | RW | See clause 9.6.1.3 of oneM2M TS-0001 [Error: Reference source not found]. |
| mgmtDefinition | 1 | WO | See clause 9.6.15 of oneM2M TS-0001 [Error: Reference source not found]. This attribute shall have the fixed value 1020 ("registration"). |
| objectIDs | 0..1 (L) | WO | See clause 9.6.15 of oneM2M TS-0001 [Error: Reference source not found]. |
| objectPaths | 0..1 (L) | WO | See clause 9.6.15 of oneM2M TS-0001 [Error: Reference source not found]. |
| description | 0..1 | RW | See clause 9.6.15 of oneM2M TS-0001 [Error: Reference source not found]. |
| originatorID | 0..1 | RW | CSE-ID of the CSE hosted on the ASN/MN or the AE-ID of an AE hosted on an ASN/MN or ADN node. If the setting is for a CSE, then this attribute shall be present. |
| pointOfAccess | 0..1(L) | RW | The point of access URI of the Registrar CSE. It represents the list of physical address used by AE/CSEs to connect to registrar CSE. Example: http://172.25.0.10:8080/xyz coap://m2m.sp.com:5683 mqtt://172.25.0.10:1883 ws://10.222.254.26:80 See note. |
| CSEBase | 0..1 | RW | The resource identifier of the <CSEBase> resource representing Registrar CSE. |
| CSE-ID | 0..1 | RW | The CSE identifier of Registrar CSEin SP-relative CSE-ID format |
| appID | 0..1 | RW | The App-ID of an AE. This attribute shall only be present when this resource is used for the registration of an AE. |
| externalID | 0..1 | RW | The M2M-Ext-ID of the ASN/MN CSE. This attribute can be present when the originatorID is a CSE-ID and the CSE uses the dynamic registration defined in clause 7.1.10 Trigger Recipient Identifier of oneM2M TS-0001 [Error: Reference source not found]. |
| triggerRecipientID | 0..1 | RW | The Trigger-Recipient-ID of the ASN/MN CSE. This attribute can be present when the originatorID is a CSE-ID and the CSE uses the dynamic registration defined in clause 7.1.10 Trigger Recipient Identifier of oneM2M TS-0001 [Error: Reference source not found]. |
| mgmtLink | 0..1 | RW | A link to a <mgmtObj> resource instance containing the information for establishing a security association with the Registrar CSE. |
| M2M-Sub-ID | 0..1 | RW | The identifier assigned by the M2M Service Provider for the M2M Service Subscription of the registree. |
| NOTE: Protocol binding is determined from the protocol schema in this URI. |  |  |  |

TS-0022: Devices in the Field Domain that host oneM2M AEs and CSEs require configuration that permits the AE or CSE to successfully operate in the M2M Service Layer. oneM2M TS-0001 [Error: Reference source not found] and oneM2M TS-0003 [Error: Reference source not found] specifies much of what is needed to configure these devices in the Field Domain (i.e., ADN, ASN/MN). Specifically, oneM2M TS-0001 [Error: Reference source not found] provides: - Guidance on how a CSE is minimally provisioned in Annex E of the specification including how a user AE is established within a Hosting CSE. - Specification of the general communication flows across the Mca and Mcc reference points in clause 8. - Specifications for how ASN/MN and ADN nodes and M2M Applications are enrolled in the M2M System such that the node in the Field domain can establish connectivity with a CSE. TS-0001 heavily relies on Clause 6 and on the Remote Security Provisioning Framework (RSPF) of oneM2M TS-0003 [Error: Reference source not found] to specify how the security credentials of ASN/MN and ADN nodes and M2M Applications are established in the M2M System for the enrolment of the node or M2M Application in the M2M System. - Specifications for how the ADN and ASN/MN nodes in the Field Domain are managed using external management technologies in clause 6.2.4. - Guidance for how the ADN and ASN/MN nodes in the Field Domain can be configured without the support of external management technologies in clause 8.1.3 in the present document. The above clauses in oneM2M TS-0001 [Error: Reference source not found] assume that, for a M2M Application to operate in the M2M System, all required information needed to establish M2M Service operation between a Registrar or Hosting CSE and the AE or CSE in the Field Domain is configured before registration of the AE or CSE to the M2M System..

Preconfiguration in the case of non secure association from an AE:

In order to establish a link between AE-Id and ssp resourceId below changes in  CREATE procedure of <serviceSubscribeNode>  are required:

- Along with SSN creation i.e. <serviceSubscribedNode> for a particular nodeID , <node> shall also be created.

- And within <node>,  Registration resource [registration] shall be created with originatorID set to AE-ID and m2mSubID set to <m2mServiceSubscriptionProfile> resourceId.

Preconfiguration in the case of secure association from an AE:

In order to establish a link between AE-Id and ssp resourceId below changes in  CREATE procedure of <serviceSubscribeNode>  are required:

- Along with SSN creation i.e. <serviceSubscribedNode> for a particular nodeID , <node> shall also be created.

- And within <node>,  Registration resource [registration] shall be created with originatorID set to AE-ID and m2mSubID set to <m2mServiceSubscriptionProfile> resourceId.

- [authenticationProfile] shall also be created with m2mSubID set to <m2mServiceSubscriptionProfile> resourceId.

### -----------------------Start of change 1-------------------------------------------

#### 10.2.12.6	Create <serviceSubscribedNode>

This procedure shall be used for creating a <serviceSubscribedNode> resource which is sub-resource of <m2mServiceSubscriptionProfile> resource.

Table 10.2.12.6-1: <serviceSubscribedNode> CREATE

| <serviceSubscribedNode> CREATE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Receiver or Hosting CSE shall be an IN-CSE Content: The resource content shall provide the information as defined in clause 9.6.20 |
| Processing at Originator before sending Request | - According to clause 10.1.2 |
| Processing at Receiver | - According to clause 10.1.2 |
| Information in Response message | All parameters defined in table 8.1.3-1 apply with the specific details for: - Content: Address of the created <serviceSubscribedNode> resource, according to clause 10.1.2 |
| Processing at Originator after receiving Response | - According to clause 10.1.2 - The Originator shall request to Create a <node> resource by using the CREATE operation. The request shall address a <CSEBase> of a Hosting CSE. The Originator shall be an AE. - The Originator shall request to Create a [registration] resource by using CREATE operation. The request shall address the above <node> resource. - In case the node which is represented by this <node> resource intends to use Security Association then the Originator shall request to Create an [authenticationProfile] resource by using the CREATE operation. The request shall address the above <node> resource. |
| Exceptions | - According to clause 10.1.2 |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### 10.2.12.9	Delete <serviceSubscribedNode>

This procedure shall be used for deleting a <serviceSubscribedNode> resource residing under a <m2mServiceSubscriptionProfile> resource.

Table 10.2.12.9-1: <serviceSubscribedNode> DELETE

| <serviceSubscribedNode> DELETE |  |
| --- | --- |
| Information in Request message | All parameters defined in table 8.1.2-3 apply with the specific details for: To: The Receiver or Hosting CSE shall be an IN-CSE |
| Processing at Originator before sending Request | - According to clause 10.1.5 |
| Processing at Receiver | According to clause 10.1.5 |
| Information in Response message | According to clause 10.1.5 |
| Processing at Originator after receiving Response | According to clause 10.1.5 - The Originator shall request to Delete a <node> resource by using the DELETE operation. The request shall address the <node> resource of a Hosting CSE. The Originator shall be an AE. |
| Exceptions | According to clause 10.1.5 |

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?