| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 55 |
| Source:* | Rahul Kumar, C-DOT, krahul@cdot.in Poornima Shandilya, C-DOT, poornima@cdot.in Anupama Chopra, C-DOT, anupama@cdot.in |
| Date:* | 2022-09-09 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 V4.10.0 |
| Clauses * | 7.4.29.2, 7.4.5.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to add validation in Create and Update procedure of <serviceSubscribedAppRule> resource for attribute allowedApp-IDs. This CR also adds validation in AE Registration request for App-ID attribute.

### -----------------------Start of change 1-------------------------------------------

### 7.4.29	Resource Type <serviceSubscribedAppRule>

#### 7.4.29.1	Introduction

The <serviceSubscribedAppRule> resource represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE. The <serviceSubscribedAppRule> resource also represents a rule that defines the users that are allowed to issue requests via the allowed AEs defined by this resource. The detailed description can be found in the clause 9.6.29 in oneM2M TS-0001 [Error: Reference source not found].

Table 7.4.29.1-1: Data type definition of <serviceSubscribedAppRule> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| serviceSubscribedAppRule | CDT-serviceSubscribedAppRule.xsd |  |

Table 7.4.29.1-2: Universal/Common Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| accessControlPolicyIDs | O | O |
| expirationTime | O | O |
| dynamicAuthorizationConsultationIDs | O | O |
| custodian | O | O |

Table 7.4.29.1-3: Resource Specific Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| applicableCredIDs | O | O | m2m:listOfM2MID |  |
| allowedApp-IDs | O | O | m2m:listOfM2MID |  |
| allowedAEs | O | O | m2m:listOfM2MID |  |
| allowedRole-IDs | O | O | List of m2m:roleID |  |
| allowedUsers | O | O | m2m:listOfURIs |  |

Table 7.4.29.1-4: Child resources of <serviceSubscribedAppRule> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause Error: Reference source not found |
| <transaction> | [variable] | 0..n | Clause 7.4.61 |

#### 7.4.29.2	<serviceSubscribedAppRule> resource specific procedures for CRUD operations

##### 7.4.29.2.0	Introduction

This clause describes <serviceSubscribedAppRule> resource specific primitive behaviour for CRUD operations.

##### 7.4.29.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Addition to the generic procedures in clause Error: Reference source not found.

If allowedApp-IDs contains identifier pattern  starting with other than ‘*’, 'R' or 'N' then the request shall be rejected with "BAD_REQUEST" Response Status Code.

##### 7.4.29.2.2	Retrieve

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

##### 7.4.29.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Addition to the generic procedures in clause Error: Reference source not found.

If allowedApp-IDs contains identifier pattern  starting with other than ‘*’, 'R' or 'N' then the request shall be rejected with "BAD_REQUEST" Response Status Code.

##### 7.4.29.2.4	Delete

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

No change from the generic procedures in clause Error: Reference source not found.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

#### 7.4.5.2	<AE> resource specific procedures for CRUD+N operations

##### 7.4.5.2.0	Introduction

This clause describes AE resource specific behaviour for CRUD+N operations.

The entire AE registration procedure including <AE> resource creation procedure below is defined in clause 10.1.1.2.2 of oneM2M TS-0001 [Error: Reference source not found] ("Application Entity registration procedure").

##### 7.4.5.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found with the following exception:

- A CSE shall not originate a Create <AE> resource request.

Receiver:

Recv-6.0.1 "Requested operation is an AE registration?" will return Yes so Recv-6.0.2 step “ Check Service Subscription Profile” (refer to clause 7.3.2.7) shall be executed.

The generic procedures in clause Error: Reference source not found shall be executed except for Recv-6.3 which shall be skipped.

The following operations shall be added to step Recv-6-4:

- The Hosting CSE shall check for the presence of any resources having an AE-ID that matches the one specified in the request and that have the same parent as the new resource being created.

- If such a resource exists, then the Hosting CSE shall reject the request with a Response Status Code indicating "ORIGINATOR_HAS_ALREADY_REGISTERED" error.

- If supportedReleaseVersions attribute contains a value indicating release version 4 or above and App-ID attribute is set to a value starting with other than ‘R’ or ‘N’, then the Hosting CSE shall reject this request with a Response Status Code indicating "BAD _REQUEST" error.

- If supportedReleaseVersions attribute contains a value indicating release version 3 or below and App-ID attribute is set to a value starting with other than ‘r’, ‘R’ or  ‘N’, then the Hosting CSE shall reject this request with a Response Status Code indicating "BAD _REQUEST" error.

The following additional primitive specific operations shall be performed in Recv-6.5:

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with "/S" in the From parameter, but no specific AE-ID-Stem was provided with the CREATE request of the Registree AE, then this is an initial registration and the receiver shall execute the following steps in order:

a)	Compose a Create <AEAnnc> Request primitive with the following attribute values:

- The link attribute of the <AEAnnc> resource to be created shall be set to the SP-Relative-Resource-ID format of a - not yet existent - <AE> resource hosted on the Registrar CSE constructed with a Unstructured-CSE-relative-Resource-ID that is equal to the AE-ID-Stem value used for the Registree AE.

- The App-ID attribute of the <AEAnnc> resource to be created shall be set to the App-ID attribute value of the Registree AE.

- The concatenation of the string 'Credential-ID:' and the actual Credential-ID of the Security Association used by the Registree AE - if any - shall be placed into the labels attribute of the <AEAnnc> resource. If no Security Association was used by the Registree AE, then no value shall be used for Credential-ID, i.e. the labels attribute shall include the string ‘Credential-ID:’.

b)	The From parameter of the CREATE request for the <AEAnnc> resource shall be set to the SP-relative-CSE-ID or Absolute-CSE-ID followed by '/S'.

c)	Send Create request to the IN-CSE that is associated with the Registree AE.

d)	Wait for Response primitive.

- Upon receipt of a successful response from the IN-CSE, the Registrar/Host CSE shall use the Unstructured-CSE-relative-Resource-ID that was used for the <AEAnnc> resource on the IN-CSE also as the assigned Unstructured-CSE-relative-Resource-ID for the <AE> resource to be created on the Registrar/Host CSE.

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with an AE-ID-Stem starting with "S", and this is an initial registration or a re-registration to the same Registrar CSE, then the receiver shall execute the following steps in order:

a)	Determine if <AEAnnc> resource already exists in the IN-CSE for the Registree AE. If so, compose an Update <AEAnnc> Request primitive to update the <AEAnnc> resource. If no <AEAnnc> resource already exists in the IN-CSE for the Registree AE, the receiver shall compose a Create <AEAnnc> Request primitive. In both cases, the request primitive shall have the following attribute values:

  - The link attribute of the <AEAnnc> resource shall be set or updated to the SP-Relative-Resource-ID format of a - not yet existent - <AE> resource hosted on the Registrar CSE constructed with a Unstructured-CSE-relative-Resource-ID that is equal to the AE-ID-Stem value used for the Registree AE.

  - The labels attribute of the <AEAnnc> resource shall be set or updated to the concatenation of the string 'Credential-ID:' and the Credential-ID of the Security Association used by the Registree AE, replacing the existing entry starting with 'Credential-ID:' if present. If no Security Association was used by the Registree AE, then no value shall be used for Credential-ID, i.e. the labels attribute shall include the string ‘Credential-ID:’.

b)	The From parameter of the CREATE or UPDATE request for the <AEAnnc> resource shall be set to the SP-relative-CSE-ID or Absolute-CSE-ID followed by '/' and the AE-ID-Stem value.

c)	The To parameter shall contain the SP-relative-Resource-ID format of the Resource ID for the <AEAnnc> resource which shall be constructed from the CSE-ID of the IN-CSE and the AE-ID-Stem that the Registree AE provided.

d)	Send Create or Update request to the IN-CSE that is associated with the Registree AE.

e)	Wait for Response primitive.

- Upon receipt of a successful response from the IN-CSE, the Registrar/Host CSE shall use the Unstructured-CSE-relative-Resource-ID equal to the AE-ID-Stem provided by the Registree AE for the <AE> resource to be created on the Registrar/Host CSE.

- If the request is for an <AE> resource and if it is received at a MN-CSE or ASN-CSE with an AE-ID-Stem starting with "S", and this is a re-registration to a new Registrar CSE, then the receiver shall execute the following steps in order:

a)	Determine if <AEAnnc> resource already exists in the IN-CSE for the Registree AE. If so, compose an Update <AEAnnc> Request primitive to update the <AEAnnc> resource. If no <AEAnnc> resource already exists in the IN-CSE for the Registree AE, the receiver shall compose a Create <AEAnnc> Request primitive. In both cases, the request primitive shall have the following attribute values:

- The link attribute of the <AEAnnc> resource shall be set or updated to the SP-Relative-Resource-ID format of a - not yet existent - <AE> resource hosted on the Registrar CSE constructed with an Unstructured-CSE-relative-Resource-ID that is equal to the AE-ID-Stem value used for the Registree AE.

- The labels attribute of the <AEAnnc> resource shall be set or updated to the concatenation of the string 'Credential-ID:' and the Credential-ID of the Security Association used by the Registree AE, replacing the existing entry starting with 'Credential-ID:' if present. If no Security Association was used by the Registree AE, then no value shall be used for Credential-ID, i.e. the labels attribute shall include the string ‘Credential-ID:’.

b)	The From parameter of the CREATE or UPDATE request for the <AEAnnc> resource shall be set to the SP-relative-CSE-ID or Absolute-CSE-ID followed by '/' and the AE-ID-Stem value.

c)	The To parameter shall contain the SP-relative-Resource-ID format of the Resource ID for the <AEAnnc> resource which shall be constructed from the CSE-ID of the IN-CSE and the AE-ID-Stem that the Registree AE provided.

d)	Send Create or Update request to the IN-CSE that is associated with the Registree AE.

e)	Wait for Response primitive.

- Upon receipt of a successful response from the IN-CSE, the Registrar/Host CSE shall use the Unstructured-CSE-relative-Resource-ID equal to the AE-ID-Stem provided by the Registree AE for the <AE> resource to be created on the Registrar/Host CSE.

- And the addition of new steps following step Recv-6.11:

- If the enableTimeCompensation attribute within the request is set to true and the Registrar/Host CSE does not support time compensation functionality, the Registrar/Host CSE shall reject the request with the "NOT_IMPLEMENTED" Response Status Code.

- If the Hosting CSE supports time compensation functionality and the enableTimeCompensation attribute of the <AE> resource is set to true, the Registrar/Host CSE shall perform time compensation functionality upon the AE’s requests and responses as defined in clause 10.2.24.3 of oneM2M TS-0001 [Error: Reference source not found].

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of Changes to References Clause -------------

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf))

## 2.2	Informative references

[i.1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf)

### -----------------------End of Changes to References  -------------

### -Start of changes to Definition of terms symbols and abbreviations-

# 3	Definition of terms, symbols and abbreviations  3.1	Terms

<defined term>: <definition>

<defined term>[N]: <definition>

## 3.2	Symbols

<symbol>	<Explanation>

## 3.3	Abbreviations

<ABBREVIATION/ACRONYM>	<Explanation>

### ---End of changes to Definition of terms, symbols and abbreviations---

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?