| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #56 |
| Source:* | JaeSeung Song, Sejong University, jssong@sejong.ac.kr Seongjin Lee, Sejong University, lsj000424@naver.com Andreas Kraft, Deutsche Telekom, A.Kraft@telekom.de Bob Flynn, Exactagss, bob.flynn@exactagss.com Cyrille Bareau, Orange, cyrille.bareau@orange.com Franck Le Gall, Franck.le-gall@egm.io |
| Date:* | 2022-11-27 |
| Reason for Change/s:* | Clarify ambiguous of the creation and managing behaviour of flexContainerInstance |
| CR against: Release* | Release 4 or 5 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004-V4.10.0 |
| Clauses * | 6.3.5.72, 7.4.37.2.1, 7.4.37.2.3, 7.4.68.1, 8.2.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to clarify the creation and managing behaviour of the <flexContainerInstance> resource.

This is a companion draft with CR SDS-2022-0xxx-TS0001-clarify_flexContainerInstance_behaviour.

### -----------------------Start of change 1 -------------------------------------------

#### 6.3.5.72	m2m:flexContainerInstanceResource

This data type is used as base type of all <flexContainerInstance> children of <flexContainer> specializations listed in clause 9.6.1.2.2 of oneM2M TS-0001 [Error: Reference source not found]. It includes the attributes commonly used by all <flexContainerInstance> resource type specializations.

It consists of the common attributes shown in Error: Reference source not found and the resource specific attributes shared by all <flexContainerInstance> specializations shown in Error: Reference source not found.

Table 6.3.5.72-1: Type Definition of m2m:flexContainerInstanceResource

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| @resourceName | m2m:resourceName | 1 |  |
| resourceType | m2m:resourceType | 1 |  |
| resourceID | m2m:ID | 1 |  |
| parentID | m2m:nhURI | 1 |  |
| creationTime | m2m:timestamp | 1 |  |
| expirationTime | m2m:expirationTime | 1 |  |
| labels | m2m:labels | 0..1 (L) |  |
| contentSize | xs:nonNegativeInteger | 1 |  |
| originator | m2m:ID | 1 |  |
| stateTag | xs:nonNegativeInteger | 0..1 |  |
| fcinEnabled | xs:boolean | 0..1 |  |

### ----------------------- End of change 1 -------------------------------------------

### -----------------------Start of change 2 -------------------------------------------

##### 7.4.37.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the received resource containerDefinition attribute. If the schema is not available then the Hosting CSE shall return a response primitive with a Response Status Code indicating "SPECIALIZATION_SCHEMA_NOT_FOUND" error. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- The Hosting CSE shall set the contentSize attribute to the sum of the size in bytes of all of the custom attributes.

- If the maxNrOfInstances attribute is present in the request with a non-zero value and the maxByteSize attribute is present in the request and the contentSize attribute is greater than the maxByteSize attribute, the Hosting CSE shall not create the resource. It shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error and skip step 3 below.

- 2b) If the fcinEnabled attribute is present in the request with a value TRUE and no attribute maxNrOfInstances, maxByteSize or maxInstanceAge is present in the request with a value of non-zero value, the Hosting CSE shall not create the resource. It shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error and skip step 3 below.

- If the created resource fcinEnabled attribute is present with value TRUE and one of maxNrOfInstances, maxByteSize or maxInstanceAge attributes is present with non-zero value, then the Hosting CSE shall create a <flexContainerInstance> resource child of the targeted <flexContainer> resource. The contentSize attribute of the created <flexContainerInstance> resource will have the same values as in the created <flexContainer> resource. The originator attribute of the created <flexContainerInstance> resource shall be set to the identifier of the originator of this CREATE request.  The attributes of the created <flexContainer> resource that are specified in Table 9.6.35-3 of TS-0001 shall be copied into the created <flexContainerInstance> resource. The Hosting CSE shall adjust accordingly the attributes of parent <flexContainer> resource related to the instance creation (currentNrOfInstances = 1 and currentByteSize).

No other changes from the generic procedures in clause Error: Reference source not found.

### ----------------------- End of change 2 -------------------------------------------

### -----------------------Start of change 3 ------------------------------------------

##### 7.4.37.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the resource containerDefinition attribute. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- If at least one of the maxNrOfInstances, maxByteSize or maxInstanceAge is present in the request with a value of zero, all child <flexContainerInstance> resources shall be deleted, the currentNrOfInstances, currentByteSize attributes shall be set to zero, and the maxNrOfInstances, maxByteSize, maxInstanceAge and fcinEnabled attributes shall be deleted.

In the following steps, any occurrence of fcinEnabled, maxNrOfInstances, maxByteSize or maxInstanceAge refers to its value in the updated <flexContainer> resource:

- If, as a result of the UPDATE request, the fcinEnabled is present with value TRUE and no maxNrOfInstances, maxByteSize or maxInstanceAge attribute is present with a non-zero value, the Hosting CSE shall return the response primitive with a Response Status Code indicating "BAD_REQUEST" error and skip step 3 below.

- If a custom attribute of the targeted <flexContainer> resource is modified in the request, and the fcinEnabled attribute is present with value TRUE, and the maxByteSize attribute is present, the Hosting CSE shall check whether the size in bytes of the custom attributes of the targeted <flexContainer> resource is greater than the maxByteSize of attribute.

  - If true, the Hosting CSE shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error and skip step 3 below.

  - If false, the Hosting CSE shall update the contentSize attribute to the sum of the size in bytes of all of the custom attributes.

- If the request’s Content parameter contains either no attribute at all or at least one custom attribute of the targeted <flexContainer> resource or the labels attribute, and the fcinEnabled attribute is either present in the targeted <flexContainer> with value TRUE or is modified in the request, and at least one of the maxNrOfInstances, maxByteSize or maxInstanceAge attributes is present with a non-zero value in the targeted <flexContainer> then after updating the attributes of the targeted <flexContainer> resource:

- The Hosting CSE shall create a <flexContainerInstance> resource child of the targeted <flexContainer> resource. The attribute contentSize of the created <flexContainerInstance> resource shall have the same value as in the targeted <flexContainer> resource. The attributes of the targeted <flexContainer> resource that are specified in Table 9.6.35-3 of TS-0001 shall be copied into the created <flexContainerInstance> resource. The originator attribute of the created <flexContainerInstance> resource shall be set to the identifier of the originator of this UPDATE request.

- If maxNrOfInstances of the targeted <flexContainer > resource is present then if the currentNrOfInstances when modified to reflect the addition of the new <flexContainerInstance> exceeds maxNrOfInstances, the Hosting CSE shall remove the oldest <flexContainerInstance> resource from the targeted <flexContainer> resource.

- If maxByteSize of the targeted <flexContainer > resource is present then if the currentByteSize when modified to reflect the addition of the new <flexContainerInstance> exceeds maxByteSize the Hosting CSE shall remove the oldest <flexContainerInstance> resource(s) from the targeted <flexContainer> resource until maxByteSize conditions are met.

- If maxInstanceAge of the targeted <flexContainer > resource is present then all <flexContainerInstance> child resource(s) of the targeted <flexContainer> resource such that the time difference in seconds between the current time and their creationDate attribute is greater than the maxInstanceAge value shall be deleted.

- The Hosting CSE shall update the currentNrOfInstances of the targeted <flexContainer> resource with the number of <flexContainerInstance> resources in the targeted <flexContainer> resource. The Hosting CSE shall update the currentByteSize of the targeted <flexContainer> resource with the sum of the contentSize attributes of the <flexContainerInstance> resources in the targeted <flexContainer> resource.

- When removing the oldest <flexContainerInstance> resources, the Hosting CSE shall not generate notifications even if there exists a <subscription> to the targeted <flexContainer> resource and this <subscription> is configured to generate a notification on “Delete_of_Direct_Child_Resource”.

No other changes from the generic procedures in clause Error: Reference source not found.

### ----------------------- End of change 3 -------------------------------------------

### -----------------------Start of change 4 ------------------------------------------

#### 7.4.68.1	Introduction

This resource represents a copy of the custom attributes of its parent <flexContainer>. Its detailed description can be found in clause 9.6.59 in oneM2M TS-0001 [Error: Reference source not found].

The following resource types are allowed to include <flexContainerInstance> specializations as children:

- <flexContainer>.

There are as many specializations of <flexContainerInstance> specified by oneM2M as there are specializations of <flexContainer>. Each <flexContainerInstance> specializations’s XML schema type is defined in the XML schema file of its parent <flexContainer> resource. The resource type name of the <flexContainerInstance> resource is formed by adding the suffix ‘Inst’ to the parent <flexContainer> resource type name.

Table 7.4.68.1-1: Resource Specific Attributes of <flexContainerInstance> resource

| Attribute Name | Data Type | Default Value and Constraints |
| --- | --- | --- |
| contentSize | xs:nonNegativeInteger | No default |
| originator | m2m:ID | No default |
| [customAttribute] | Name and data type are defined in the specification document or XSD file identified by the value of containerDefinition attribute of the parent <flexContainer>. | No default |
| stateTag | xs:nonNegativeInteger | No default |
| fcinEnabled | xs:boolean | No default |

### ----------------------- End of change 4 -------------------------------------------

### -----------------------Start of change 5 -------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

(…)

Table 8.2.3-2: Resource attribute short names (2/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| currentByteSize | container,timeSeries, flexContainer | cbs |
| locationID | container | li |
| disableRetrieval | container | disr |
| contentInfo | contentInstance, timeSeries | cnf |
| contentSize | contentInstance, timeSeriesInstance, flexContainer, flexContainerInstance | cs |
| contentRef | contentInstance | conr |
| containerDefinition | flexContainer | cnd |
| fcinEnabled | flexContainer | fcied |
| primitiveContent | request | pc* |
| content | contentInstance, timeSeriesInstance | con |
| deletionCnt | contentInstance | dcnt |
| cseType | CSEBase, remoteCSE | cst |
| CSE-ID | CSEBase, remoteCSE, service SubscribedNode, AEContactListPerCSE | csi |
| supportedResourceType | CSEBase | srt |
| notificationCongestionPolicy | CSEBase | ncp |
| currentTime | CSEBase | ctm |
| source | delivery | sr |
| target | delivery, request | tg |
| lifespan | delivery | ls |
| eventCat | delivery | ec |
| deliveryMetaData | delivery | dmd |
| aggregatedRequest | delivery | arq |
| eventID | eventConfig, statsCollect | evi |
| eventType | eventConfig | evt |
| evenStart | eventConfig | evs |
| eventEnd | eventConfig | eve |
| operationType | eventConfig | opt |
| dataSize | eventConfig | ds |
| eventResourceTypes | eventConfig | erts |
| eventResourceIDs | eventConfig | eris |
| execStatus | execInstance | exs |
| execResult | execInstance | exr |
| execDisable | execInstance | exd |
| execTarget | execInstance, mgmtCmd | ext |
| execMode | execInstance, mgmtCmd | exm |
| execFrequency | execInstance, mgmtCmd | exf |
| execDelay | execInstance, mgmtCmd | exy |
| execNumber | execInstance, mgmtCmd | exn |
| execReqArgs | execInstance, mgmtCmd | exra |
| execEnable | mgmtCmd | exe |
| memberType | group | mt |
| specializationType | group | spty |
| currentNrOfMembers | group | cnm |
| maxNrOfMembers | group | mnm |
| memberIDs | group, backgroundDataTransfer | mid |
| membersAccessControlPolicyIDs | group | macp |
| memberTypeValidated | group | mtv |
| consistencyStrategy | group | csy |
| semanticSupportIndicator | group | ssi |
| notifyAggregation | group | nar |
| groupName | group, subscription | gn |
| locationSource | locationPolicy | los |
| locationUpdatePeriod | locationPolicy | lou |
| locationTargetID | locationPolicy | lot |
| locationServer | locationPolicy | lor |
| locationContainerID | locationPolicy | loi |
| locationContainerName | locationPolicy | lon |
| locationStatus | locationPolicy | lost |
| authID | locationPolicy | aid |
| retrieveLastKnownLocation | locationPolicy | rlkl |
| locationUpdateEventCriteria | locationPolicy | luec |
| locationInformationType | locationPolicy | lit |
| geographicalTargetArea | locationPolicy | gta |
| geofenceEventCriteria | locationPolicy | gec |
| description | mgmtCmd, mgmtObj, all management resources from firmware, ontology, ontologyMappingAlgorithm | dc |
| cmdType | mgmtCmd | cmt |
| mgmtDefinition | mgmtObj, all management resources from firmware | mgd |
| objectIDs | mgmtObj | obis |

(…)

### ----------------------- End of change 5 -------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?