| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #60 |
| Source:* | JaeSeung Song, Sejong University, jssong@sejong.ac.kr Seongjin Lee, Sejong University, lsj000424@naver.com Andreas Kraft, Deutsche Telekom, A.Kraft@telekom.de Bob Flynn, Exactagss, bob.flynn@exactagss.com Cyrille Bareau, Orange, cyrille.bareau@orange.com |
| Date:* | 2023-06-28 |
| Reason for Change/s:* | Clarify ambiguous of the creation and managing behaviour of flexContainerInstance |
| CR against: Release* | Release 4 or 5 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004-V4.10.0 |
| Clauses * | 6.3.5.72, 7.4.37.2.1, 7.4.37.2.3, 7.4.68.1, 8.2.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to clarify the creation and managing behaviour of the <flexContainerInstance> resource.

This is a companion draft with CR SDS-2022-0147R02.

R01:

- Clarify steps

- Change NOT_ACCEPTABLE to BAD_REQUEST

- Clarify the case when the fcinEnalbed attribute is set to FALSE in UPDATE then all child <flexContainerInstance> resources shall be deleted

- Allow “unlimited” creation of <flexContainerInstance> resources, i.e. allow creation when fcinEnabled=TRUE and no maxInstanceAge, maxNrOfInstances or maxByteSize is defined.

R02:

- Add a note for CREATE and UPDATE to clarify a case where the fcinEnabled attribute is set ot FALSE

- Delete unnecessary steps as we decouple the fcinEnabled attribute and max * attributes.

R03:

- Remove Notes and make the case as an independent step.

- Several clarifications on max* attributes

R04:

- Prepare a version that shows only changes from the original text.

R05:

- Clarify steps for UPDATE

- Add a case to CREATE for a request with the fcinEnabled is FALSE.

### -----------------------Start of change 1 -------------------------------------------

#### 6.3.5.72	m2m:flexContainerInstanceResource

This data type is used as base type of all <flexContainerInstance> children of <flexContainer> specializations listed in clause 9.6.1.2.2 of oneM2M TS-0001 [Error: Reference source not found]. It includes the attributes commonly used by all <flexContainerInstance> resource type specializations.

It consists of the common attributes shown in Error: Reference source not found and the resource specific attributes shared by all <flexContainerInstance> specializations shown in Table 7.4.37.1-2.

Table 6.3.5.72-21: Type Definition of m2m:flexContainerInstanceResource

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| @resourceName | m2m:resourceName | 1 |  |
| resourceType | m2m:resourceType | 1 |  |
| resourceID | m2m:ID | 1 |  |
| parentID | m2m:nhURI | 1 |  |
| creationTime | m2m:timestamp | 1 |  |
| expirationTime | m2m:expirationTime | 1 |  |
| labels | m2m:labels | 0..1 (L) |  |
| contentSize | xs:nonNegativeInteger | 1 |  |
| originator | m2m:ID | 1 |  |
| stateTag | xs:nonNegativeInteger | 0..1 |  |
| fcinEnabled | xs:boolean | 0..1 |  |

### ----------------------- End of change 1 -------------------------------------------

### -----------------------Start of change 2 -------------------------------------------

### 7.4.37	Resource Type <flexContainer>

#### 7.4.37.1	Introduction

This resource represents a customizable container for data instances. It is a template for the definition of flexible specializations of data containers.

The detailed description can be found in clause 9.6.35 in oneM2M TS-0001 [Error: Reference source not found].

##### …

Table 7.4.37.1-2: Resource Specific Attributes of <flexContainer> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| containerDefinition | M | NP | xs:anyURI | No default |
| ontologyRef | O | O | xs:anyURI | No default |
| contentSize | NP | NP | xs:nonNegativeInteger | No default |
| nodeLink | O | O | xs:anyURI | No default |
| maxNrOfInstances | O | O | xs:nonNegativeInteger | No default |
| maxInstanceAge | O | O | xs:nonNegativeInteger | No default |
| maxByteSize | O | O | xs:nonNegativeInteger | No default |
| currentNrOfInstances | NP | NP | xs:nonNegativeInteger | No default (This is generated by the Hosting CSE and limited by the maxNrOfInstances) |
| currentByteSize | NP | NP | xs:nonNegativeInteger | No default (This is generated by the Hosting CSE and limited by the maxByteSize) |
| [customAttribute] | O | O | Name and data type are defined in the specification document or XSD file identified by the value of containerDefinition attribute. | No default |
| fcinEnabled | O | O | Xs:boolean | Default value is false |

Table 7.4.37.1-3: Child Resources of <flexContainer> resource

| Child Resource Type |  | Child Resource Name |  | Multiplicity |  | Ref. to in Resource Type Definition |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <semanticDescriptor> |  | [variable] |  | 0..n |  | Clause Error: Reference source not found |  |
| <subscription> |  | [variable] |  | 0..n |  | Clause Error: Reference source not found |  |
| <container> |  | [variable] |  | 0..n |  | Clause Error: Reference source not found |  |
| <flexContainer> |  | [variable] |  | 0..n |  | Clause |  |
| <timeSeries> |  | [variable] |  | 0..n |  | Clause 7.4.38 |  |
| <transaction> |  | [variable] |  | 0..n |  | Clause 7.4.61 |  |
| <flexContainerInstance> |  | [variable] |  | 0..n |  | Clause 7.4.68 |  |
| <latest> |  | la |  | 0..1 |  | Clause Error: Reference source not found |  |
| <oldest> |  | ol |  | 0..1 |  | Clause Error: Reference source not found |  |
| <action> | [variable] |  | 0..n |  | Clause 7.4.75 |  |  |

##### 7.4.37.2.1	Create

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the received resource containerDefinition attribute. If the schema is not available then the Hosting CSE shall return a response primitive with a Response Status Code indicating "SPECIALIZATION_SCHEMA_NOT_FOUND" error. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- The Hosting CSE shall set the contentSize attribute to the sum of the size in bytes of all of the custom attributes.

- If any of the maxNrOfInstances, maxByteSize or maxInstanceAge attributes are present in the request with a value zero, then the Hosting CSE shall return the response primitive with a Response Status Code indicating "BAD_REQUEST" error and skip the remaining steps.

- If the fcinEnabled attribute is present in the request then the following steps shall be performed:

  - If the fcinEnabled attribute is with the value of TRUE and if the maxByteSize attribute is present in the request and the contentSize attribute is greater than the value of the maxByteSize attribute, then the hosting CSE shall not create the <flexContainerInstance> resource. Instead it shall return the response primitive with a Response Status Code indicating a “BAD_REQUEST” error and skip the remaining steps.

  - If the fcinEnabled attribute is set to TRUE then the following steps are performed:

- The hosting CSE shall create a <flexContainerInstance> resource as a child resource of the targeted <flexContainer> resource.

- The originator attribute of the created <flexContainerInstance> resource shall be set to the originator of this CREATE request.

- The contentSize attribute of the created <flexContainerInstance> resource shall be set to the same value as the contentSize attribute of the created <flexContainer> resource.

- All the attributes specified in Table 9.6.35-3 of TS-0001 that are present in the created <flexContainer> resource shall be copied into the created <flexContainerInstance> resource.

- In the created <flexContainer> resource, the currentNrOfInstances attribute shall be set to 1, and the currentByteSize attribute shall be set to the value of the contentSize attribute of the <flexContainer>.

  - If the fcinEnabled attribute is with the value of FALSE then the following steps are performed:

- The currentByteSize attribute shall be added to the created <flexContainer> resource and set to 0.

- The currentNrOfInstances attribute shall be added to the created <flexContainer> resource and set to 0.

No other changes from the generic procedures in clause Error: Reference source not found.

### ----------------------- End of change 2 -------------------------------------------

### -----------------------Start of change 3 ------------------------------------------

##### 7.4.37.2.3	Update

Originator:

No change from the generic procedures in clause Error: Reference source not found.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the resource containerDefinition attribute. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- If any of the maxNrOfInstances, maxByteSize or maxInstanceAge is present in the request with a value of zero, then all <flexContainerInstance> child resources of the target <flexContainer> resource shall be deleted, and the currentNrOfInstances and currentByteSize shall be set to zero, and the  maxNrOfInstances, maxByteSize and maxInstanceAge attributes shall be deleted from the target <flexContainer> resource.

- If the maxByteSize present in the request with a value that is less than the flexContainer’s contentSize attribute, the hosting CSE shall return the response primitive with a Response Status Code indicating a “BAD_REQUEST” error and skip the remaining steps

- If the fcinEnabled attribute is present in the request with a value of FALSE, the Hosting CSE shall not create a <flexContainerInstance> resource and step 3) is skipped

- In the following steps, any occurrence of fcinEnabled, maxNrOfInstances, maxByteSize or maxInstanceAge refers to its value in the updated <flexContainer> resource:

- If the fcinEnabled attribute is present with a value of TRUE then perform the following steps:

- If any custom attribute of the targeted <flexContainer> resource is modified in the request, then the Hosting CSE shall update the contentSize attribute to the sum of the size in bytes of all of the custom attributes of the targeted <flexContainer> resource.

- If the request contains no attribute at all or at least one custom attribute of the targeted <flexContainer> resource or the labels attribute, then after updating the attributes of the targeted <flexContainer> resource perform the following steps:

  - The Hosting CSE shall create a new <flexContainerInstance> child resource under the targeted <flexContainer> resource and perform the following steps:

    - The attribute contentSize of the created <flexContainerInstance> resource shall have the same value as the contentSize attribute of the targeted <flexContainer> resource.

    - The value of the contentSize attribute of the targeted <flexContainer> resource shall be added to the currentByteSize attribute of the targeted <flexContainer> resource.

    - The attributes of the targeted <flexContainer> resource that are specified in Table 9.6.35-3 of TS-0001 shall be copied into the created <flexContainerInstance> resource.

    - The originator attribute of the created <flexContainerInstance> resource shall be set to the originator of this UPDATE request.

    - The currentNrOfInstances attribute of the targeted <flexContainer> resource shall be incremented by one.

  - If maxNrOfInstances of the targeted <flexContainer> resource is specified then if the currentNrOfInstances when modified to reflect the addition of the new <flexContainerInstance> exceeds maxNrOfInstances, the Hosting CSE shall remove the oldest <flexContainerInstance> resource from the targeted <flexContainer> resource.

  - If maxByteSize of the targeted <flexContainer> resource is specified then if the currentByteSize when modified to reflect the addition of the new <flexContainerInstance> exceeds maxByteSize the Hosting CSE shall remove the oldest <flexContainerInstance> resources from the targeted <container> resource until maxByteSize conditions are met.

  - When removing the oldest <flexContainerInstance> resources, the Hosting CSE shall not generate notifications even if there exists a <subscription> to the targeted <flexContainer> resource and this <subscription> is configured to generate a notification on "Delete_of_Direct_Child_Resource".

  - If the maxInstanceAge attribute is present in the targeted <flexContainer> resource, then the Hosting CSE shall set the expirationTime attribute in the <flexContainerInstance> resource such that the time difference between expirationTime and the creationTime of the <flexContainerInstance> resource shall not exceed the maxInstanceAge of the targeted parent <flexContainer> resource.

- The Hosting CSE shall increment the stateTag attribute of the targeted <flexContainer> resource and copy the value into the stateTag attribute of the <flexContainerInstance> resource.

No other changes from the generic procedures in clause Error: Reference source not found.

### ----------------------- End of change 3 -------------------------------------------

### -----------------------Start of change 4 ------------------------------------------

#### 7.4.68.1	Introduction

This resource represents a copy of the custom attributes of its parent <flexContainer>. Its detailed description can be found in clause 9.6.59 in oneM2M TS-0001 [Error: Reference source not found].

The following resource types are allowed to include <flexContainerInstance> specializations as children:

- <flexContainer>.

There are as many specializations of <flexContainerInstance> specified by oneM2M as there are specializations of <flexContainer>. Each <flexContainerInstance> specializations’s XML schema type is defined in the XML schema file of its parent <flexContainer> resource. The resource type name of the <flexContainerInstance> resource is formed by adding the suffix ‘Inst’ to the parent <flexContainer> resource type name.

Table 7.4.68.1-1: Resource Specific Attributes of <flexContainerInstance> resource

| Attribute Name | Data Type | Default Value and Constraints |
| --- | --- | --- |
| contentSize | xs:nonNegativeInteger | No default |
| originator | m2m:ID | No default |
| [customAttribute] | Name and data type are defined in the specification document or XSD file identified by the value of containerDefinition attribute of the parent <flexContainer>. | No default |
| stateTag | xs:nonNegativeInteger | No default |
| fcinEnabled | xs:boolean | Default is FALSE |

### ----------------------- End of change 4 -------------------------------------------

### -----------------------Start of change 5 -------------------------------------------

### 8.2.3	Resource attributes

In protocol bindings, resource attributes names shall be translated into short names shown in the following tables.

(…)

Table 8.2.3-4: Resource attribute short names (1/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| accessControlPolicyIDs | All except accessControlPolicy, contentInstance | acpi |
| announcedAttribute | All announceable resources | aa |
| announceTo | All announceable resources | at |
| announceSyncType | All announceable resources | ast |
| creationTime | All | ct |
| expirationTime | All except contentInstance, CSEBase | et |
| labels | All (optional) | lbl |
| lastModifiedTime | All | lt |
| link | All | lnk |
| parentID | All | pi |
| resourceID | All | ri |
| resourceType | All | ty* |
| stateTag | container, contentInstance, delivery, request, flexContainer, flexContainerInstance | st |
| resourceName | All | rn |
| privileges | accessControlPolicy | pv |
| selfPrivileges | accessControlPolicy | pvs |
| authorizationDecisionResourceIDs | accessControlPolicy | adri |

Table 8.2.3-5: Resource attribute short names (2/6)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| currentByteSize | container,timeSeries, flexContainer | cbs |
| locationID | container | li |
| disableRetrieval | container | disr |
| contentInfo | contentInstance, timeSeries | cnf |
| contentSize | contentInstance, timeSeriesInstance, flexContainer, flexContainerInstance | cs |
| contentRef | contentInstance | conr |
| containerDefinition | flexContainer | cnd |
| fcinEnabled | flexContainer, flexContainerInstance | fcied |
| primitiveContent | request | pc* |
| content | contentInstance, timeSeriesInstance | con |
| deletionCnt | contentInstance | dcnt |
| cseType | CSEBase, remoteCSE | cst |
| CSE-ID | CSEBase, remoteCSE, service SubscribedNode, AEContactListPerCSE | csi |
| supportedResourceType | CSEBase | srt |
| notificationCongestionPolicy | CSEBase | ncp |
| currentTime | CSEBase | ctm |
| source | delivery | sr |
| target | delivery, request | tg |
| lifespan | delivery | ls |
| eventCat | delivery | ec |
| deliveryMetaData | delivery | dmd |
| aggregatedRequest | delivery | arq |
| eventID | eventConfig, statsCollect | evi |
| eventType | eventConfig | evt |
| evenStart | eventConfig | evs |
| eventEnd | eventConfig | eve |
| operationType | eventConfig | opt |
| dataSize | eventConfig | ds |
| eventResourceTypes | eventConfig | erts |
| eventResourceIDs | eventConfig | eris |
| execStatus | execInstance | exs |
| execResult | execInstance | exr |
| execDisable | execInstance | exd |
| execTarget | execInstance, mgmtCmd | ext |
| execMode | execInstance, mgmtCmd | exm |
| execFrequency | execInstance, mgmtCmd | exf |
| execDelay | execInstance, mgmtCmd | exy |
| execNumber | execInstance, mgmtCmd | exn |
| execReqArgs | execInstance, mgmtCmd | exra |
| execEnable | mgmtCmd | exe |
| memberType | group | mt |
| specializationType | group | spty |
| currentNrOfMembers | group | cnm |
| maxNrOfMembers | group | mnm |
| memberIDs | group, backgroundDataTransfer | mid |
| membersAccessControlPolicyIDs | group | macp |
| memberTypeValidated | group | mtv |
| consistencyStrategy | group | csy |
| semanticSupportIndicator | group | ssi |
| notifyAggregation | group | nar |
| groupName | group, subscription | gn |
| locationSource | locationPolicy | los |
| locationUpdatePeriod | locationPolicy | lou |
| locationTargetID | locationPolicy | lot |
| locationServer | locationPolicy | lor |
| locationContainerID | locationPolicy | loi |
| locationContainerName | locationPolicy | lon |
| locationStatus | locationPolicy | lost |
| authID | locationPolicy | aid |
| retrieveLastKnownLocation | locationPolicy | rlkl |
| locationUpdateEventCriteria | locationPolicy | luec |
| locationInformationType | locationPolicy | lit |
| geographicalTargetArea | locationPolicy | gta |
| geofenceEventCriteria | locationPolicy | gec |
| description | mgmtCmd, mgmtObj, all management resources from firmware, ontology, ontologyMappingAlgorithm | dc |
| cmdType | mgmtCmd | cmt |
| mgmtDefinition | mgmtObj, all management resources from firmware | mgd |
| objectIDs | mgmtObj | obis |

(…)

### ----------------------- End of change 5 -------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?