| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 60 |
| Source:* | SeungMyeong Jeong, KETI, sm.jeong@keti.re.kr NakMyoung Sung, KETI, nmsung@keti.re.kr ChungJae Choe, KETI, cjchoe1@keti.re.kr |
| Date:* | 2023-06-27 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 3 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v3.32 |
| Clauses * | 8.1.2 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | TS-0004 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

This CR proposes to add a new feature to retireve multiple <contentInstance> resources that are oldest or latest.

On this topic, there has been several discussions before and the candidate solution as the way forward was decided with the contribution “SDS-2023-0032R01-discussion_on_getting_N_latest_oldest_instances”. The agreed option was:

- Opt 3) “base/cnt1?latestInstances=5” & “base/cnt1?oldestInstances=5” (as new Filter Criteria condition)

NOTE: not just release-5 as a new feature, but release 4 and 3 are the targeted releases which do not have certification program yet, so it is still allowable to add new features(s) if necessary in contributor’s opinion.

In R01: removed the change 1, but added change 2 for Filter Criteria

### -----------------------Start of change 2-------------------------------------------

### 8.1.2	Request

- Filter Criteria: optional filter criteria: conditions for filtered operations which are described in table 8.1.2-2. This is used for resource discovery (clause 10.2.6) and general Create, Retrieve, Update, Delete requests (clauses 10.1.3, 10.1.4 and 10.1.5).

The Filter Criteria set includes matching conditions and filter handling conditions.

One of the handling conditions is the filterUsage condition (table 8.1.2-2) which governs the use of the other Filter Criteria conditions and the Discovery-based operations as detailed in clause 10.2.6.

Matching conditions are evaluated against resources and, when true, determine the matched resources which compose the matching result. The filter handling conditions provide additional input used to determine the filtering result (e.g. maximum number of resources to be included in the filtering result). The filtering result may be composed of one or more resources and determines the target resources for Discovery-based operations.

Examples of filter criteria usage are provided in clause 10.2.6.

Table 8.1.2-2: Filter Criteria conditions

| Condition tag | Multiplicity | Description |
| --- | --- | --- |
| Matching Conditions |  |  |
| createdBefore | 0..1 | The creationTime attribute of the matched resource is chronologically before the specified value. |
| createdAfter | 0..1 | The creationTime attribute of the matched resource is chronologically after the specified value. |
| modifiedSince | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically after the specified value. |
| unmodifiedSince | 0..1 | The lastModifiedTime attribute of the matched resource is chronologically before the specified value. |
| stateTagSmaller | 0..1 | The stateTag attribute of the matched resource is smaller than the specified value. |
| stateTagBigger | 0..1 | The stateTag attribute of the matched resource is bigger than the specified value. |
| expireBefore | 0..1 | The expirationTime attribute of the matched resource is chronologically before the specified value. |
| expireAfter | 0..1 | The expirationTime attribute of the matched resource is chronologically after the specified value. |
| labels | 0..1 | The labels attribute of the matched resource matches the specified value. |
| labelsQuery | 0..1 | The value is an expression for the filtering of labels attribute of resource when it is of key-value pair format. The expression is about the relationship between label-key and label-value which may include equal to or not equal to, within or not within a specified set etc. For example, label-key equals to label value, or label-key within {label-value1, label-value2}. Details are defined in oneM2M TS-0004 [Error: Reference source not found]. |
| childLabels | 0..1 | A child of the matched resource has labels attributes matching the specified value. The evaluation is the same as for the labels attribute above. Details are defined in oneM2M TS-0004 [Error: Reference source not found]. |
| parentLabels | 0..1 | The parent of the matched resource has labels attributes matching the specified value. The evaluation is the same as for the labels attribute above. Details are defined in oneM2M TS-0004 [Error: Reference source not found]. |
| resourceType | 0..n | The resourceType attribute of the matched resource is the same as the specified value. It also allows differentiating between normal and announced resources. |
| childResourceType | 0..n | A child of the matched resource has the resourceType attribute the same as the specified value. |
| parentResourceType | 0..1 | The parent of the matched resource has the resourceType attribute the same as the specified value. |
| sizeAbove | 0..1 | The contentSize attribute of the <contentInstance> matched resource is equal to or greater than the specified value. |
| sizeBelow | 0..1 | The contentSize attribute of the <contentInstance> matched resource is smaller than the specified value. |
| contentType | 0..n | The contentInfo attribute of the <contentInstance> matched resource matches the specified value. |
| attribute | 0..n | This is an attribute of resource types (clause 9.6). Therefore, a real tag name is variable and depends on its usage and the value of the attribute can have wild card *. E.g. creator of container resource type can be used as a filter criteria tag as "creator=Sam", "creator=Sam*", "creator=*Sam". |
| childAttribute | 0..n | A child of the matched resource meets the condition provided. The evaluation of this condition is similar to the attribute matching condition above. |
| parentAttribute | 0..n | The parent of the matched resource meets the condition provided. The evaluation of this condition is similar to the attribute matching condition above. |
| semanticsFilter | 0..n | Both semantic resource discovery and semantic query use semanticsFilter to specify a query statement that shall be specified in the SPARQL query language [Error: Reference source not found]. When a CSE receives a RETRIEVE request including a semanticsFilter, and the Semantic Query Indicator parameter is also present in the request, the request shall be processed as a semantic query; otherwise, the request shall be processed as a semantic resource discovery. In the case of semantic resource discovery targeting a specific resource, if the semantic description contained in the <semanticDescriptor> of a child resource matches the semanticFilter, the URI of this child resource will be included in the semantic resource discovery result. In the case of semantic query, given a received semantic query request and its query scope, the SPARQL query statement shall be executed over aggregated semantic information collected from the semantic resource(s) in the query scope and the produced output will be the result of this semantic query. Examples for matching semantic filters in SPARQL to semantic descriptions can be found in oneM2M TR-0007 [Error: Reference source not found]. |
| filterOperation | 0..1 | Indicates the logical operation (AND/OR) to be used for different condition tags. The default value is logical AND. |
| contentFilterSyntax | 0..1 | Indicates the Identifier for syntax to be applied for content-based discovery. |
| contentFilterQuery | 0..1 | The query string shall be specified when contentFilterSyntax parameter is present. |
| Filter Handling Conditions |  |  |
| filterUsage | 0..1 | Indicates how the filter criteria is used. If provided, possible values are 'discovery' and 'IPEOnDemandDiscovery'. If this parameter is not provided, the Retrieve operation is a generic retrieve operation and the content of the child resources fitting the filter criteria is returned. If filterUsage is 'discovery', the Retrieve operation is for resource discovery (clause 10.2.6), i.e. only the addresses of the child resources are returned. If filterUsage is 'IPEOnDemandDiscovery', the other filter conditions are sent to the IPE as well as the discovery Originator ID. When the IPE successfully generates new resources matching with the conditions, then the resource address(es) shall be returned. This value shall only be valid for the Retrieve request targeting an <AE> resource that represents the IPE. |
| limit | 0..1 | The maximum number of resources to be included in the filtering result. This may be modified by the Hosting CSE. When it is modified, then the new value shall be smaller than the suggested value by the Originator. |
| level | 0..1 | The maximum level of resource tree that the Hosting CSE shall perform the operation starting from the target resource (i.e. To parameter). This shall only be applied for Retrieve operation. The level of the target resource itself is zero and the level of the direct children of the target is one. |
| offset | 0..1 | The number of direct child and descendant resources that a Hosting CSE shall skip over and not include within a Retrieve response when processing a Retrieve request to a targeted resource. |
| oldestInstances | 0..1 | Representation of the number to retrieve the number of latest <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> resources. The allowed target resource types are container, timeSeries and flexContainer. <This shall not be present with the latestInstances. |
| latestInstances | 0..1 | Representation of the number to retrieve the number of latest <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> resources. The allowed target resource types are container, timeSeries and flexContainer. This shall not be present with the oldestInstances. |
| applyRelativePath | 0..1 | This attribute contains a resource tree relative path (e.g. ../tempContainer/LATEST). This condition applies after all the matching conditions have been used (i.e. a matching result has been obtained). The attribute determines the set of resource(s) in the final filtering result. The filtering result is computed by appending the relative path to the path(s) in the matching result. All resources whose Resource-IDs match that combined path(s) shall be returned in the filtering result. If the relative path does not represent a valid resource, the outcome is the same as if no match was found, i.e. there is no corresponding entry in the filtering result. |

The rules when multiple matching conditions are used together shall be as follows:

- Different condition tags shall use the "AND/OR" logical operation based on the filterOperation specified;

e.g. createdBefore = "time1" AND unmodifiedSince = "time2" if filterOperation = "AND" or "NULL", or createdBefore = "time1" OR unmodifiedSince = "time2" if filterOperation = "OR".

- Same condition tags shall use the "OR" logical operation, i.e. filterOperation does not apply to same conditions.

No mixed AND/OR filter operation will be supported.

Once the Request is delivered, the Receiver shall analyse the Request to determine the target resource.

If the target resource is addressing another M2M Node, the Receiver shall route the request appropriately.

If the target resource is addressing the Receiver, it shall:

- Check the existence of To addressed resource.

- Identify the resource type by Resource Type.

- Check the privileges for From Originator to perform the requested operation.

- Perform the requested operation (using Content content when provided) according to the provided request parameters as described above.

- Depending on the request result content, respond to the Originator with indication of successful or unsuccessful operation results. In some specific cases (e.g. limitation in the binding protocol or based on application indications), the Response could be avoided.

Table 8.1.2-3 summarizes the parameters specified in this clause for the Request message, showing any differences as applied to C, R, U, D or N operations. "M" indicates mandatory, "O" indicates optional, "N/A" indicates "not applicable".

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?