| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 61 |
| Source:* | Andreas Neubacher, Deutsche Telekom, andreas.neubacher@magenta.at Ingo Friese, Deutsche Telekom, Ingo.Friese@telekom.de |
| Date:* | 2023-08-08 |
| Reason for Change/s:* | Additional input on security considerations and mutual authentication |
| CR against: Release* | Rel.5 |
| CR against: WI* | Active WI-0100 MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0065 V0.3.0 |
| Clauses * | Adding clause 6.5.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | non |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Adding a clause 6.5.1. about security considerations and mutual authentication of communication endpoints

### -----------------------Start of change 1-------------------------------------------

6.5.1 Mutual authentication for different protocol interfaces

Most cloud environments allow for configuration of IP address filter. This way only preconfigured instances with pre-known IP addresses are entitled to communicate with each other. A filter table might be created that allows only data traffic between IP addresses of the OGC / STA Server, IPE and CSE.

Direct VPN connections might be established between the endpoints to protect the communication.

If these kind of “additional” security measurements are not applicable there are protocol specific ways for authentication in HTTP, oneM2M and MQTT.

Mutual Authentication for HTTP APIs

One possibility for mutual authentication between OGC / STA Server, IPE and CSE is to share tokens, certificates, passwords or keys beforehand. Appropriate artifacts might be placed in the “Authorization header” of every HTTP-Request for authentication during operation of the IPE. SensorThings API protocol defines no authentication mechanism.

Mutual Authentication in oneM2M

TS 0003 Security Solutions offers a Security Association Establishment Framework (SAEF) [i.11] and describes possibilities for mutual authentication between the CSE and the IPE. There are two methods described in detail to exchange credentials using “Provisioned Symmetric Key”- or “Certificate-Based Security Association”.

Mutual Authentication in MQTT

Basic functionalities like authentication are described in MQTT standard [i.10]. Mutual authentication is described at section “enhanced Authentication”. MQTT enables client authentication. In the architectural setup the IPE is the client that has to be authenticated at the MQTT Broker of the SensorThings API Server.

### -----------------------End of change 1---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?